IEEE P802.19

Coexistence TAG

| Project | IEEE P802.19 Coexistence TAG |  |
| --- | --- | --- |
| Title | Coexistence Metric 1.0 |  |
| Date Submitted | [13 November, 2002] |  |
| Source | [Rob Poor] [Ember Corporation] [313 Congress St / Boston, MA 02210] | Voice: [617 951-0200] Fax: [617 951-0999] E-mail: [robert.poor@ember.com] |
| Re: | [This is an expansion of the Coexistence Metric 1.0 overview given in IEEE P802.19-009.] |  |
| Abstract | [This document describes Coexistence Metric 1.0 (CM1.0), a proposed methodology for quantifying the coexistence between a wireless transmitter / receiver pair and an interfering transmitter. The Metric answers the question “How close can an interferer come to a receiver while maintaining reasonable packet error rates at the receiver?” CM1.0 strives to make as few assumptions as possible, using published specifications wherever possible to describe interfering and receiving radio models. To do this, CM1.0 focuses specifically on interactions at the PHY level, favoring simplicity over real-world accuracy.] |  |
| Purpose | [The goals of this document are to introduce CM1.0 and to solicit initial comments from P802.19 participants and the IEEE community with the intention of refining and clarifying the Metric.] |  |
| Notice | This document has been prepared to assist the IEEE P802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.19. |  |

# Introduction

## Background

Global availability of the license-free 2.4GHz Industrial, Scientific, Medical (ISM) band has contributed to the rapid adoption of wireless communication standards such as 802.11b and Bluetooth.  But the very popularity of the 2.4GHz ISM band has created its own challenges: co-located wireless devices that share the same airwaves but use different protocols can interfere with one another.  As more protocols are developed and more devices are produced that operate in this band, the greater the likelihood for interference.

The IEEE 802 LAN/MAN Standards Committee (LMSC) develops standards and recommended practices for networking, including wireless networking.  Subgroups within the LMSC focus on wireless communication standards, such as 802.11 (Wireless Local Area Networks) and 802.15 (Wireless Personal Area Networks).

The term coexistence has been adopted by the LMSC to refer to the effects of interference among wireless devices that are physically near one another.  Two wireless devices are said to coexist if they can be brought near one another without significant degradation in their performance.  Observing an opportunity to address the issues of interoperability and interference among multiple wireless standards, the LMSC formed the IEEE 802.19 Coexistence Technical Advisory Group, whose charter is to develop coexistence policy.

## Quantifying Coexistence: Motivation and Approach

The goal of the 802.19 Coexistence TAG is to consider a variety of IEEE wireless standards and to ask questions of the form “Will wireless device A significantly reduce the performance of wireless device B, and if so, what can be done to improve the performance?”

Before this sort of question can be answered, it is important establish a coexistence metric in order to quantify the extent to which device A is expected to interfere with device B.  Without such a metric, one runs the risk of trying to improve performance when performance doesn’t need improving.

The need to quantify coexistence has motivated the formulation of Coexistence Metric 1.0 (CM1.0), which is the topic of this paper.  In general, wireless standards are developed long before hardware implementations are built, so the Coexistence Metric needs to be a theoretic model.

And therein lies the challenge.  Theoretical models are notoriously bad at modeling physical reality.  The most accurate models are the most detailed, taking into account as many physical parameters as possible.  Although the time required computing physically accurate models is significant, the real effort lies in collecting accurate parameter sets for highly detailed models.

Fortunately, the goal of a coexistence metric is simply to anticipate whether or not interference will pose a significant problem or not.  To quantify coexistence, a simple relative model is much more useful than a complex absolute model.

## Goals of CM 1.0

Several principals guided the design of CM 1.0.

Favor quantitative assumptions over qualitative assumptions.  Wherever possible, CM1.0 uses parameters that are readily derived from published specifications.

Adopt a simple model.  A wireless standard is characterized in the CM1.0 model with eight well-defined parameters.  A parameter set for a specific wireless standard can be created quickly and easily.

Produce intuitive results.  The CM1.0 model quantifies coexistence as the minimum distance between a receiving radio and an interfering radio that still produces acceptable error rates at the receiver.

Build upon previous work.  The bulk of the CM1.0 model—notably the path loss model and the concept of spectral coupling—is derived directly from <##TG2> [TG2].

Err on the pessimistic side.  The CM1.0 does not attempt to produce an absolute measure of coexistence.  Rather, it is defined to give a repeatable relative measure of coexistence.  Results from CM1.0 are intentionally designed to be pessimistic so that additional application-specific assumptions, such as limited duty cycle in the interferer, will only improve the expected results.

Use a portable, widely supported framework.  The CM1.0 framework is written entirely in Java, so it can be run on all popular operating systems.

Publish early, revise often.  CM1.0 is expressly not intended as the ultimate metric for coexistence; rather, it should be thought of as a first draft.  By adopting a simple approach and publishing the details, the authors hope CM1.0 will be scrutinized and improved upon by the professional community.

## About This Document

This document proceeds as follows.  Section 2 describes the fundamentals of CM1.0, and introduces the concepts of path loss and spectral coupling.  In CM1.0 a radio is totally modeled using nine methods, grouped in a single Java class.  These nine methods are described in section 3. Section 4 shows how to run using CM1.0, using a simple driver file as an example and presenting the resulting output.  Section 5 offers a summary and suggests future directions for CM1.0.  The Appendices include references, sample driver files, annotated parameter sets, and the complete Java source code listings for CM1.0.

# The CM 1.0 Model

Figure 1 below shows the fundamentals of the CM1.0 model.  The individual components are discussed in subsequent sections.

BER calculation

Transmitter

Interferer

Path Loss(d)

Spectral

Coupling

SIR

PT

PI

(PT = Receiver sensitivity+10dB)

Signal power at receiver

Interference power at receiver

BER

FER calculation

FER

Figure 1: The CM1.0 Model

The CM1.0 model assumes a single transmitting radio is the sole source of signal and a single interfering radio is the sole source of interference.  The interfering energy is assumed to appear as additive white Gaussian noise (AWGN) to the receiver.

Signal Power is defined to be 10dB above the minimum sensitivity of the receiving radio.  The minimum receiver sensitivity is defined in the parameter set for the receiving system—see “3. CM1.0 Parameter Sets” below.

Interference Power is the nominal power for the interfering transmitter.  It is defined in the parameter set for the interfering system.

Path Loss is the amount by which the interferer’s power is attenuated due to physical separation, and increases exponentially with distance. The path loss model used by CM1.0 is derived from [Marquess] and [TG2], and is described in greater detail in section 2.1 below.

Spectral Coupling is the attenuation of the interferer’s transmitted spectrum by the receiver’s input filter, expressed as dB of attenuation.  It is computed by multiplying the interferer’s transmitted spectrum with the receiver’s input filter – these spectra are defined in interferer’s and the receiver’s parameter sets respectively.  The area under the resulting spectrum is found by integration from the lowest to the highest frequencies generated by the interferer – this area corresponds to the amount of the interferer’s energy couples into the receiver.  <## Migrate most of this to section 2.2 below>

Signal to Interference Ratio (SIR) is the ratio of the signal power divided by the interference power attenuated by path loss and by spectral coupling.

Bit Error Rate (BER) is a function of the SIR and by a bit error rate function defined in the parameter set file for the receiver. The relationship between SIR and bit error rate is a function of the modulation scheme used by the transmitter and receiver.

Frame Error Rate (FER) is simply a function of BER and bits per frame.  The average bits per frame is defined in the parameter set for the receiver.

## Path Loss Model

The path loss model used by CM1.0 is based on a heuristic model suitable for indoor systems operating at 2.4GHz described in [Marquess].

Figure 2: Path Loss Curve

Path loss follows free-space propagation (inverse square) for distances up to eight meters, and then attenuates more rapidly with a path loss exponent of 3.3.

The path loss L is given by:

L	= 20 log(4d/) = 40.23 + 20 log(d)	d ≤ 8m
			= 58.3 + 33 log(d/8)	d > 8m

where d is the path length in meters and  is the wavelength at 2.45GHz (0.1224 meters).

## Spectral Coupling

A radio transmitter’s radiated power is limited over a band of frequencies, commonly called its spectral mask.  Similarly, a radio receiver has a filter on its input that attenuates energy outside of a particular range of frequencies, called its passband.  The shapes of the transmitter’s spectral mask and the receiver’s input filter, and degree to which they overlap, determine how much of the transmitter’s energy is available to the receiver.

For ordinary radio communications, the transmitter’s spectral mask and the receiver’s input filter are aligned to one another, so that the receiver picks up the majority of the transmitted energy.

In the case of an interfering radio transmission, the interferer’s spectral mask and the receiver’s input filter form an important defense against interference.  When the interferer’s spectral mask lies outside of the receiver’s passband, most of the interfering energy will be attenuated by the receiver’s input filter, reducing the effects of the interference.

In CM1.0, the attenuation of the interferer’s spectral mask by the receiver’s input filter is known as the Spectral Coupling.  It is computed by multiplying the interferer’s spectral mask with the receiver’s input filter to form a filtered spectrum.  The area under the filtered spectrum represents the amount of the interferer’s energy that remains after filtering.  When normalized by dividing by the area under the interferer’s spectral mask, the result is the Spectral Coupling.

The following figure demonstrates the calculation of Spectral Coupling.

| Area AI Interferer’s Spectrum | Receiver’s Filter | Area AF Filtered Spectrum |
| --- | --- | --- |
| Spectral Coupling = Area AF / Area AI |  |  |

Figure 3: Computation of Spectral Coupling

The area AI represents the total transmitted power of the interferer.  The area AF represents the total energy arriving at the input of the receiver, after filtering by the receiver’s input filter.  The ratio of AF/AI is therefore the fraction of the interferer’s total energy that arrives at the receiver after filtering.  This ratio, expressed in decibels, is the Spectral Coupling.

# CM1.0 Parameter Sets

Each wireless protocol characterized by CM1.0 is defined by a set of nine functions, called a Parameter Set.  These functions are written in the Java programming language and are grouped together in a single Java class.

The nine functions that comprise a Parameter Set are described briefly here; an annotated Parameter Set file appears in Appendix C.

public abstract String description();

The description() function returns a short descriptive string for the protocol, such as “802.11b”.

public abstract boolean isFrequencyHopped();

The isFrequencyHopped() function returns true if the protocol is a frequency hopping protocol, false otherwise.

public abstract Channel[] getChannels();

The getChannels() function returns a vector of Channel objects.  Each Channel object is a {channel_id, frequency} pair that describes the relation between a channel id (a small integer) and a frequency in Hertz.

public abstract double receiveSensitivity();

The receiveSensitivity() function returns the minimum receiver sensitivity in dB.  CM1.0 models the effective signal power as 10dB above this value.

public abstract Spectrum receiveSpectrum();

The receiveSpectrum() function returns a Spectrum object that describes the input filter to the receiver.  The Spectrum class provides constructor mechanisms to simplify the creation of a Spectrum object – see Appendix C for additional details.  CM1.0 combines the receive spectrum with the interferer’s transmit spectrum (q.v.) to calculate the Spectral Coupling.

public abstract double bitErrorRate(double sirDB);

The bitErrorRate() function returns a bit error rate for this receiver as a function of Signal to Interference ratio.  The sirDB parameter is the Signal to Interference ratio expressed in decibels.  The function should take into account the modulation scheme processing gain and bits per symbol.

public abstract int bitsPerPacket();

The bitsPerPacket() function returns the average bits per packet of the receiver.  This value is used to convert between Bit Error Rate and Packet Error Rate.

public abstract double transmitPower();

The transmitPower() function returns the power of the transmitter in dBm.  Note that this value is used only when a transmitter is acting as an interferer.

public abstract Spectrum transmitSpectrum();

The transmitSpectrum() function returns a Spectrum object that describes the transmitted spectral mask. The Spectrum class provides constructor mechanisms to simplify the creation of a Spectrum object – see Appendix C for additional details.  CM1.0 will combine the receive spectrum with the transmit spectrum to calculate the Spectral Coupling.

# Driver Files and Results

<## This section shows how CM1.0 is run to produce data sets that can be plotted.  A simple driver file and the resulting output is presented.>

CM1.0 is a framework, written as a collection of Java classes.  A Driver File, also written in Java, links all the pieces together by running the CM1.0 algorithm

In order to test two communication standards using CM1.0,

This example shows CM1.0 being run on an 802.15.3 receiver with an 802.15.4 interferer.

import ieee.p802.coex.*;

/**

* Test coexistence between

* 802.15.3 Receiver (22Mbps) 802.15.4 Interferer

*/

public class T15d3_15d4 {

public static void main(String args[]) {

// Create an array of distances ranging from .1M to 100M

// (with log spacing)

double dist[] = Util.makeDistances();

// Allocate an 802.15.3 object to be used as the receiver

RM802_15_3 recvr = new RM802_15_3(RM802_15_3.MBPS_22);

// Allocate an 802.15.4 object to be used as the interferer

RM802_15_4 intfr = new RM802_15_4();

// Compute bit error rate as a function of distance

double ber[] = Coexistence.coexistence(recvr, intfr, dist);

// Convert bit error rate into packet error rate

double per[] = Coexistence.packetErrorRate(recvr, ber);

// Print a descriptive header

double offset = recvr.getFrequencySeparation(intfr)/1.0e6;

System.out.println("Packet Error Rate vs Separation"+

", Receiver="+recvr.description()+

", Interferer="+intfr.description()+

", offset="+offset+"MHz");

// Print column header

System.out.println("separation(M)\tBER\tPER");

// Print separation in meters, bit error rate, packet error rate

for (int i=0; i<dist.length; i++) {

System.out.println(Util.formatScientific(dist[i])+

"\t"+Util.formatScientific(ber[i])+

"\t"+Util.formatScientific(per[i]));

}

}

}

When executed, the above Java program produces the following output:

Packet Error Rate vs Separation, Receiver=802.15.3(22Mbps), Interferer=802.15.4, offset=7.0MHz

separation(M)	BER	PER

1.000E-01	4.972E-01	1.000E00

1.072E-01	4.970E-01	1.000E00

1.148E-01	4.968E-01	1.000E00

1.230E-01	4.966E-01	1.000E00

…

1.905E01	1.050E-02	1.000E00

2.042E01	4.850E-03	1.000E00

2.188E01	1.900E-03	1.000E00

2.344E01	6.314E-04	9.943E-01

2.512E01	1.461E-04	6.979E-01

2.692E01	2.393E-05	1.780E-01

2.884E01	2.540E-06	2.060E-02

3.090E01	1.563E-07	1.280E-03

3.311E01	4.851E-09	3.974E-05

3.548E01	6.370E-11	5.218E-07

3.802E01	2.840E-13	2.326E-09

4.074E01	3.261E-16	2.728E-12

4.365E01	6.813E-20	1.000E-100

4.677E01	1.674E-24	1.000E-100

5.012E01	2.797E-30	1.000E-100

5.370E01	1.597E-37	1.000E-100

5.754E01	1.312E-46	1.000E-100

6.166E01	5.237E-58	1.000E-100

6.607E01	2.597E-72	1.000E-100

7.079E01	2.885E-90	1.000E-100

7.586E01	1.000E-100	1.000E-100

8.128E01	1.000E-100	1.000E-100

8.710E01	1.000E-100	1.000E-100

9.333E01	1.000E-100	1.000E-100

Since the fields are separated by tabs, this format can be easily read into a plotting or spreadsheet program to produce a graph such as the following.

Figure 4: Sample BER/PER curves

# Conclusion

<## Standard recap.>

## Future Work

Measure predicted results against actual results, adjust model as appropriate.

Consider Duty Cycle.

Extend to handle different radio models, notably UWB.

Add a Graphical User Interface with plotting package such as [Ptolemy] for interactive testing.

## Acknowledgements

The author would like to thank the following members of the IEEE community:

James Gilb – putting the original suggestion to practice for TG3

Nada Golmie – TG2 models, fruitful discussions

Paul Gorday – patience in answering many questions

Jose Gutierrez – putting me on the task

Bob Heile – diplomacy

Jim Lansford – Formation of 802.19

Steve Shellhammer – proving the need

# Appendix A: References

<##all of these need to be made real>

[Kammerman] A. Kamerman, “Coexistence between Bluetooth and 802.11 CCK Solution to Avoid Mutual Interference,” Lucent Technologies, Jan 1999

[Marquess] Marquess, K., “Physical Model Sub-Group Discussion and Questions,” IEEE 802.15/138R0 November 1999

[Ptolemy] The Ptolemy project from Berkeley, which includes an effective 2D graphing package.

[TG2] TG2 draft standard

# Appendix B: Sample Driver Files

# Appendix C: An Annotated Parameter Set

# Appendix D: CM1.0 Source Listings

<## perhaps best handled through online links?  Who should host that?>>