2003-12-20
IEEE C802.20-04/01

	Project
	IEEE 802.20 Working Group on Mobile Broadband Wireless Access 

<http://grouper.ieee.org/groups/802/20/>

	Title
	802.20 Channel Modeling CG Status Report

	Date Submitted
	2003-12-20

	Source(s)
	Qiang Guo
Editor, Channel Modeling CG

	Phone: 847-523-3217 
Email: qiangguo@motorola.com

	Re:
	IEEE 802.20 WG on MBWA Call for Contributions:  Session # 6  - January 12-15, 2004

	Abstract
	The status of the key work items within 802.20 Channel Modeling CG will be reported.

	Purpose
	This contribution presents the up-to-date results of the key work items within 802.20 Channel Modeling CG.

	Notice
	This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

	Release
	The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20.

	Patent Policy
	The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>.








1

