| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | OA&M Requirements for 802.20 |  |
| Date Submitted | 2004-03-04 |  |
| Source(s) | Scott Migaldi 1303 East Algonquin Road Schaumburg, IL. 60196 | Voice: +1.847.576.0574 Fax: +1.847.576.6758 Email: w10265@motorola.com |
|  | Jörg Schmidt 2501 South Price Rd Chandler, AZ. 85248 | Voice: +1.480.732.6493 Email: j.Schmidt@motorola.com |
| Re: | MBWA Call for Contributions |  |
| Abstract | Review and adopt |  |
| Purpose | To refine the requirements pertaining to the OA&M section of the requirements document being developed in 802.20 |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

#### 3.2	Definition of Interfaces

#### Open interfaces: The AI shall support open interfaces between the base station and any upstream network entities. Any interfaces that may be implemented shall use IETF protocols or 3GPP/3GPP2 standardized as appropriate. Some of the possible interfaces are illustrated below.

#### 4.6	OAM&P Support

#### In general and as applicable the system shall make reuse of network management interface specifications defined by 3GPP & 3GPP2, ensuring common network management for service providers/network operators.

#### The system shall provide support for alarm and event monitoring capabilities using the Alarm IRP mechanism as defined by 3GPP (32-series) & 3GPP2 (S.S0028).

#### The system shall provide support for topology and object monitoring as well as provisioning using the CM IRP mechanisms as defined by 3GPP (32-series) & 3GPP2 (S.S0028) – development of a 802.20 object model (802.20 NRM IRP) should be considered.

### 4.6.1 CPE software upgrade “push”   (Existing Text in a new section)

CPE software upgrade “push” – an operator should have the ability to “push” a software upgrade to CPE that are currently connected to the network.  The packets that make up the software image should be given a very high priority and should be coded heavily such that they have a very high chance of arriving error free at the CPE.  The CPE should be capable of holding 2 software loads (the existing one and a new one) such that an operator can ensure that the “new” software load has arrived safely at the CPE before deciding to switch from the “old” software load to the “new” software load.

### 4.6.2 Measurements OA&M Support

The air interface shall support the collection of the following metrics so that a network operator to can effectively monitor the performance of the 802.20 air interfaces.

The following values must be made available in real-time with redisplay intervals of no less than 1000 msecs, with the option to be displayed in both cumulative and delta modes:

- Paging Channel

- Paging Channel Delivery

- Occupancy/capacity used

- Access Channel

- Access Channel Reception

- Occupancy/ Capacity

- State transitions

- Timing/ delay

- Registrations

- Successful and failed

- Forward Traffic Channel Delivery

- Total and Per user

- MAC  retries

- PHY retries

- MAC/PHY delay

- Total blocks/PDU assigned and delivered

- Uncorrectable Errors

- Signal Strength

- Reverse Traffic Channel Reception

- Total and Per user

- MAC retries

- PHY retries

- MAC/PHY delay

- Total blocks/PDU assigned and delivered

- Uncorrectable Errors

- Signal Strength

- Hand-offs

- UL & DL Power Measurements

- Total and per user

These statistics should be made available via  PM data XML mapping and forwarding mechanisms as defined by 3GPP (32-series) & 3GPP2 (S.S0028)