| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | IEEE 802.20 Common MAC |  |
| Date Submitted | 2004-03-17 |  |
| Source(s) | Name : Dan Gal Address: 67 Whippany Rd, Whippany NJ 07981 | Voice: 973-428-7734 Fax: Email: dgal@lucent.com |
| Re: | MBWA Call for Contributions, Session #7 March 2004 |  |
| Abstract | This contribution proposes a MAC requirement that consists of a common-part and a PHY-specific part. |  |
| Purpose | System Requirements document - baseline text proposal |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Purpose

Proposal for System Requirements document.

2. 	Current SRD (v11) Text

### 3.1.1	MBWA System Reference Architecture

Adopting current communications systems specification principles, 802.20 MBWA systems will be specified using a layered architecture. The 802.20 standards, in conjunction with other 802 standards, will specify the services to be delivered by layers 1 and 2 to an IP based layer 3 or a switching layer, e.g. PPP, MPLS.  To facilitate a layered approach, the 802.20 specification shall incorporate a reference partitioning model consisting of Layers 1 and 2. This layered approach shall be generally consistent with other IEEE 802 standards and shall remain generally within the scope of other IEEE 802 standards as shown in figures 1 & 2.   The 802.20 standard shall also address the needs of logical link control and how and when the 802.2 LLC functionality is used. The 802.20 standards include PHY and MAC layer specifications with a well-defined service interface between the PHY and MAC layer.  To provide the best possible performance, the MAC layer design may be optimized for the specific characteristics of the air interface PHY. Figure 2 shows the relationship of various 802 PHY and MAC layer standards to other 802 architectural components. The 802.20 standards shall clarify how 802.20 fits into this architecture.

3. 	Proposed Baseline Text

### 3.1.1	MBWA System Reference Architecture

Adopting current communications systems specification principles, 802.20 MBWA systems shall be specified using a layered architecture. The 802.20 standards, in conjunction with other 802 standards, shall specify the services to be delivered by layers 1 and 2 to an IP based layer 3 or a switching sub-layer, e.g. PPP, MPLS.  To facilitate a layered  architecture, the 802.20 specification shall incorporate a reference partitioning model consisting of Layers 1 and 2. This layered approach shall be generally consistent with other IEEE 802 standards and shall remain generally within the scope of other IEEE 802 standards as shown in figures 1 & 2.   The 802.20 standard shall also address the needs of logical link control and how and when the 802.2 LLC functionality is used. The 802.20 standard shall include PHY and MAC layer specifications with a well-defined service interface between them.

If more than one PHY technology is adopted for the 802.20 standard, the MAC layer shall be designed such that it consists of two parts:  a common part and a PHY-specific part. To provide the best possible performance, the PHY-specific part of the MAC may be optimized for the specific characteristics of a particular PHY.

Figure 2 shows the relationship of various 802 PHY and MAC layer standards to other 802 architectural components. The 802.20 standard shall clarify how it  fits into this architecture.