| Link Budget Template |  |  |  |
| --- | --- | --- | --- |
| id/ii | Item | Forward link | Reverse link |
|  | Test environment | Suburban/urban macro-cell, micro-cell, indoor pico-cell | Suburban/urban macro-cell, micro-cell, indoor pico-cell |
|  | Test service |  |  |
|  | Multipath channel class | Cases I-V | Cases I-V |
| ii/id | (a0) Average transmitter power per traffic channel (NOTE 1) | dBm | dBm |
| id | (a1) Maximum transmitter power per traffic channel | dBm | dBm |
| id | (a2) Maximum total transmitter power | (i) 43 dBm (3GPP2)* (ii) 43 dBm (3GPP) (iii) 48 dBm (PCS-Shively) (iv) 51 dBm (800-900 MHz) Use 1.9 GHz as the operating frequency | (i) 23 dBm (3GPP2) (ii) 30 dBm (Shively) (iii) 27dBm |
| ii | (b) Cable, connector, and combiner losses (enumerate sources) | (i) 2 dB (3GPP2) (ii) 3 dB (Shively) | 0 dB |
|  | Body Losses (Shively) | 0 dB | 3 dB |
| ii | (c) Transmitter antenna gain | For 1800-1900 MHz and higher frequencies 17 dBi (3GPP2) For 800-900 MHz 14 dBi (3GPP) | (i) -1 dBi (3GPP2) (ii) 0 dBi (3GPP, Shively) (TBR) |
| id | (d1) Transmitter e.i.r.p. per traffic channel  (a1 – b  c) | dBm | dBm |
| id | (d2) Total transmitter e.i.r.p.  (a2 – b  c) | 62 dBm | 27 dBm (Shively) |
|  | Penetration Loss (Ref: 3GPP2) [Determine how to use these numbers for different environments, revisit if 20dB is a reasonable value for building penetration)] | 20 dB (Building) 10 dB (Vehicular) | 20 dB (Building) 10 dB (Vehicular) |
| ii | (e) Receiver antenna gain | 0 dBi (TBR) | For 1800-1900 MHz and higher frequencies 17 dBi (3GPP2) For 800-900 MHz 14 dBi (3GPP) |
| ii | (f) Cable and connector losses With no tower mounted amplifier (TMA) With tower mounted amplifier (TMA) | 0 dB 0 dB | (i) 2 dB (3GPP2) (ii) 3 dB (Shively) 0 dB |
|  | Body Losses (Shively) | 3 dB | 0 dB |
| ii | (g) Receiver noise figure | (i) 10 dB (3GPP2, Shively) (ii) 9 dB (3GPP) | 5 dB |
| ii | (h) Thermal noise density (H) (linear units) | –174 dBm/Hz 3.98  10–18 mW/Hz | –174 dBm/Hz 3.98  10–18 mW/Hz |
| id | (i) Receiver interference density (NOTE 2) (I) (linear units) | dBm/Hz mW/Hz | dBm/Hz mW/Hz |
| id | (j) Total effective noise plus interference density  10 log (10((g  h)/10)  I) | dBm/Hz | dBm/Hz |
| ii | (k) Information rate (10 log (Rb)) | dB(Hz) | dB(Hz) |
| id | (l) Required Eb/(N0  I0) | dB | dB |
| id | (m) Receiver sensitivity = (j  k  l) |  |  |
| id | (n) Hand-off gain | dB | dB |
| id | (o) Explicit diversity gain | dB | dB |
| id | (o) Other gain | dB | dB |
| id | (p) Log-normal fade margin | dB | dB |
| id | (q) Maximum path loss  {d1 – m  (e – f)  o  n  o – p} | dB | dB |
| id | (r) Maximum range | m | m |