| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Power Consumption and Thermal Dissipation Requirement for 802.20 Suberscriber Unit |  |
| Date Submitted | 2004-02-11 |  |
| Source(s) | Ping Liu 13811 Wireless Way Richmond, BC Canada V6V 3A4 | Voice: (604)232-1436 Fax: (604)231-1109 Email: pliu@sierrawireless.com |
| Re: |  |  |
| Abstract | This contribution provides a detailed discussion of setting subscriber unit DC power and thermal dissipation limit as one of 802.20 physical layer technology evaluation criterions based on current and next generation PC card standards. |  |
| Purpose | The major subscriber unit form factor of 802.20 will be PC card that is migrating from traditional PCMCIA PC Card including 16-bit PC Card/CardBus to the next generation PC card named ExpressCard and PCI add-in card called MINI PCI Express Card. This contribution tries to set one of criterions for 802.20 physical layer technology evaluation to make sure the selected technologies are friendly to the host device from power supply capability viewpoint. |  |
| Notice | This document is prepared to assist the IEEE 802.20 Working Group. The material in this document is subject to change in form and content after further discussion. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

PCMCIA and PCI-SIG have released next generation PC external and internal add-in card standards recently. The major form factor of 802.20 WWAN subscriber units will be the traditional or next generation PC card. The selected 802.20 physical layer technologies should well match the current and new PC card standards especially on DC power consumption and thermal dissipation specifications in order to make 802.20 business prosperity. Wireless technologies that are power hungry and high heat dissipation should be ruled out simply because they would make 802.20 subscriber unit design impractical or result in higher implementation cost.

- Background Information on traditional PC Card

| Interface | Form factor | Power Capability | Heat Dissipation |
| --- | --- | --- | --- |
| 8/16 bit parallel bus or 32 bit parallel bus | Type I: 85543.3mm Type II: 85545mm, most popular Type III: 855410.5mm | 5V supply: 660mA (peak), 500mA (average) or 3.3V supply: 1000mA(peak), 750mA (average) ##### Peak power: 3.3W ##### Average power: 2.5W | 3W on each PCMCIA slot |

- Background Information on ExpressCard

| Interface | Form factor | Power Capability | Heat Dissipation |
| --- | --- | --- | --- |
| USB2.0 serial bus (480Mbps max) or PCI Express Serial bus (2.5Gbps, max) | ExpressCard/34: 75345mm ExpressCard/54: 75545mm, | 3.3V supply: 1300mA(max), 1000mA(average) and 1.5V second power supply: 650mA(max), 500mA(average) ##### Max power: 5.26W ##### Average power: 4.05W | 2.1W on ExpressCard/54 slot and 1.3W on ExpressCard/34 slot |

- Background Information on MINI PCI Express Card

| Interface | Form factor | Power Capability | Heat Dissipation |
| --- | --- | --- | --- |
| USB2.0 serial bus (480Mbps max) or PCI Express Serial bus (2.5Gbps, max) | 51305mm | 3.3V supply: 1000mA(peak), 750mA(average) and 1.5V second power supply: 500mA(peak), 375mA(average) ###### Peak power: 4.05W ###### Average power: 3.04W | 2.4W |

- Current WWAN PC card power and thermal dissipation requirement

- GPRS/EDGE

  - GPRS GSM Band (2W)

### Card Power Consumption

2.8W/45%(including loss, 45% PA efficiency) + 1W (baseband)=7.2W max

2.8W/45%(including loss, 45% PA efficiency)/2 + 1W (baseband)=4.1 average

# Thermal Dissipation Power

4.1W-2W/2(radiated power)=3.1W (worst case, 50% transmit)

  - GPRS PCS Band (1W)

### Card Power Consumption

1.4W/40%(including loss, 40% PA efficiency)+ 1W (baseband)=4.5W max

1.4W/40%(including loss, 40% PA efficiency)/2+ 1W (baseband)=2.75W average

## Thermal Dissipation Power

2.75-1W/2(radiated power)=2.25W (worst case on class 12, 50% transmit)

  - GPRS DCS Band (1W)

Similar to PCS band above.

- CDMA/1xRTT

  - PCS Band

#### Card Power Consumption

26.5dBm35%(including loss, 35% PA efficiency)+1.2W(baseband)=2.47W

#### Thermal Dissipation Power

2.47W-0.2239W(radiated power)=2.25W

  - Cellular Band

Similar to PCS band above.

- Evaluation criteria of subscriber unit power consumption for 802.20 physical layer technologies

It is good news for wireless technology that the total power capabilities on the newer PC card standards have been increased. However, the thermal dissipation power is much less than traditional PC card because the newer card form factors are getting smaller. That means the subscriber unit can request more power but it has to transmit majority of the power through air. Otherwise the host device won’t be able to dissipate the remaining heat and modem has to stop transmitting or reduce data throughput to cool itself down. The best wireless technology should have the least power requirement and least power dissipation. In short,

min{PrPd},	where 	Pr  is the power required by subscriber unit,

Pd  is the dissipation power(heat) of subscriber unit.

So, the best wireless technology should have minimum value of PrPd under the condition of same BER, same data throughput and same radio coverage.

Fig 1. Peak power and thermal dissipation power constrain for PC cards

Fig 2. Average power and thermal dissipation power constrain for PC cards

Based on Fig 1 and Fig 2, we proposed the following power consumption and thermal dissipation evaluation criterions for IEEE802.20 physical layer technology.

- Criteria 1

  - The average power consumption of subscriber unit should be less than 4W.

- Criteria 2

  - The thermal dissipation power of subscriber unit should be less than 2.5W.

- Criteria 3

  - Within criteria 1 and 2, the best technology should have the smallest PrPd value.

See PCMCIA PC Card release 8.0, 2001, Peak current: The highest averaged current value over any

10-millisecond period.

See PCMCIA ExpressCard release 1.0, 2004, Max current: absolute max excluding initial inrush current

See MINI PCI Express release 1.0, 2003, Peak current: The highest averaged current value over any

10-millisecond period.