| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for Evaluation Criteria Document – RF Performance Issues |  |
| Date Submitted | 1st submission: 2004-06-28 2nd submission: 2004-08-26 3rd revision: 2004-09-14 4th revision: 2004-11-11 |  |
| Source(s) | Dan Gal 67 Whippany Road, Whippany, NJ 07981 | Voice: 973-428-7734 Fax: 973-386-4555 Email: dgal@lucent.com |
| Re: | MBWA Call for Contributions: Session #10 - Sept 13-17, 2004 |  |
| Abstract | This document provides proposed text on RF performance issues for incorporation in the 802.20 Evaluation Criteria Document. |  |
| Purpose | Address essential RF performance parameters in the EC models. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Purpose

This contribution proposes a baseline text for the IEEE 802.20 Evaluation Criteria document. It proposes values and usage scope for key RF performance parameters that are deemed necessary for establishing a uniform RF transmission model that all proposals would adopt for the evaluation criteria simulations.

The following proposed text builds on previous discussions and contributions [1] and [2] and is recommended for incorporation in section 10 of the Evaluation Criteria document v10 [3].

# Proposed Text for the Evaluation Criteria Document

# Draft 802.20 Permanent Document

# <802.20 Evaluation Criteria – Ver 10>

# 10.    RF Environment

# 10.1  Radio Transceiver Characteristics

The RF environment for the EC model is defined by a set of RF parameters that should be considered as fixed constrains (that apply to all technologies) and by technology-specific performance specifications

Table 12 defines the transmitter and receiver parameters for the base station (BS) and the mobile station (MS) radios. Note that some parameters vary with channel bandwidth. Base values were chosen for a 1 MHz reference channel and adjustments were made for 1.25 MHz, 5MHz and 10 MHz channels. The adjustment for other channel bandwidths can be easily calculated by factoring the channel BW ratio to the 1 MHz channel.

10.1.1	Transmitter

For the evaluation purposes, the maximum base station (BS) transmit power is specified here as peak power per 1 MHz and is +43 dBm/MHz. Likewise, the mobile station (MS) maximum transmit power shall be +27 dBm for all channel bandwidths.

For the evaluation purposes, the out-of-band emission limit (item 3 in table 12) shall be that which is specified for the block edge, by the FCC for the PCS band. The spectral shape of the transmitted modulated carrier is material to the evaluation criteria and shall  be submitted.

In the case of a multi-carrier technology, the transmit power is the total power of all carriers transmitted simultaneously and intended to be received by one receiver simultaneously.

It shall be further required that in the evaluation criteria model, any band-edge guard bands that are used in order to meet the out of band emission limits shall be specified..

Note that when applying the FCC emission attenuation formula (see Table 12, item 3, where P is the transmit power in Watts), {editor note: verify correctness} the maximum permissible out-of-band power is -13dBm for all transmit powers and all channel bandwidths. Also note that the resolution-bandwidth for out-of-band power measurements is 1 MHz.

    - Receiver

Noise Figure:

To achieve a fair performance comparison, different technologies should be constrained by similar environmental interference conditions and have identical receiver noise figure specifications. Thus, for the evaluation purposes it is required that all technology proposals use the same receiver noise figure (NF) specs: 10 dB for the MS and 5 dB for the BS.

Receiver Sensitivity:

The theoretical receiver sensitivity is expected to vary from one technology to another, but again, for the sake of fair comparison, it is required that the receiver sensitivity be specified for raw data bit error rate (BER) of 0.1%.

The receiver sensitivity (in dBm) shall be calculated using the following formula:

Sensitivity = (-174.5 dBm) + NF (in dB) + 10 log (channel-BW in Hz) + C/N min for 0.1% BER).

ACLR, Selectivity and Blocking:

In addition to co-channel interference, the RF environment in which a receiver operates includes interference from adjacent and second-adjacent channels (of same or different technologies). The contribution of the latter signals to the overall C/N level can be calculated from the ACLR, Selectivity and Blocking specifications. The total interference power present in the wanted channel band consists of the aggregate interference power caused by co-channel, adjacent channel and second-adjacent channel signals.

For the evaluation model purposes, items 4, 5, 9 and 10 in Table 12 are the recommended values for ACLR and Selectivity. Individual proposals may specify different values and must commit their entire performance analysis and simulations to their chosen specifications. The Blocking signal levels are mandatory values.

The power level of interference from adjacent channels and from blocking signals shall be calculated by applying the ACLR and Selectivity figures from table 10-1 or from the individual proposal’s technical specifications.

Note 1: 
Typically the interference effect of a blocking signal is measured when the level of the wanted signal is 3 dB above receiver sensitivity (at BER 0.1%). When applying the blocking signal to the receiver input (in addition to the wanted signal) no change in the BER should be detected.

Table 12:  Evaluation Criteria RF Parameters

| # | RF Parameter | Base Value 1 MHz Channel | 1.25 MH Ch BW | 5 MHz Ch BW | 10 MHz Ch BW |
| --- | --- | --- | --- | --- | --- |
| 1 | Transmitter Power -- BS | 43 dBm/MHz | +44 dBm | +50 dBm | +53 dBm |
| 2 | Transmitter Power -- MS | 27 dBm | +27 dBm | +27 dBm | +27 dBm |
| 3 | Out of Band emission limits – BS and MS (emission measured in 1 MHz resolution bandwidth) | Attenuation of the transmit power P by: 43 +10 log(P) dB | -13 dBm | -13 dBm | -13 dBm |
| 4* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) -- BS | 45 dB | 39 dB | 45 dB | 48 dB |
| 5* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) -- MS | 33 dB | 27 dB | 33 dB | 36 dB |
| 6 | Receiver noise figure -- BS | 5 dB | 5 dB | 5 dB | 5 dB |
| 7 | Receiver noise figure -- MS | 10 dB | 10 dB | 10 dB | 10 dB |
| 8 | Receiver reference sensitivity (to be proposed by each technology) | Specify at BER of 0.1% | value 1 (proposal specific) | value 2 (proposal specific) | value 3 (proposal specific) |
| 9* | Receiver Selectivity -- BS | 63 dB | 63 dB | 63 dB | 63 dB |
| 10* | Receiver Selectivity -- MS | 33 dB | 33 dB | 33 dB | 33 dB |
| 11[*] | Receiver Blocking – BS (level of same technology blocking signal at frequency offset of 2*Channel BW) | -40 dBm | -40 dBm | -40 dBm | -40 dBm |
| 12[*] | Receiver Blocking – MS (level of same technology blocking signal at frequency offset of 2*Channel BW) | -56 dBm | -56 dBm | -56 dBm | -56 dBm |

* Recommended values. Proposals may choose (and commit to) different values.

# 10.2  Link Budget

Table 13: Link Budget Template

| id/ii | Item | Downlink | Uplink |
| --- | --- | --- | --- |
|  | Test environment | Suburban/urban macro-cell, micro-cell, indoor pico-cell | Suburban/urban macro-cell, micro-cell, indoor pico-cell |
|  | Operating frequency | 1.9GHz | 1.9GHz |
|  | Test service |  |  |
|  | Multipath channel class | Cases I-V | Cases I-V |
| Ii/id | (a0) Average transmitter power per traffic channel (NOTE 1) | dBm | dBm |
| Id | (a1) Maximum transmitter power per traffic channel | dBm | dBm |
| Id | (a2) Maximum total transmitter power | 43 dBm/MHz | 27dBm |
| Ii | (b) Cable, connector, and combiner losses (enumerate sources) | 3 dB | 0 dB |
|  | Body Losses | 0 dB | 3 dB |
| Ii | (c) Transmitter antenna gain | 17 dBi | 0 dBi |
| Id | (d1) Transmitter e.i.r.p. per traffic channel  (a1 – b  c) | dBm | dBm |
| Id | (d2) Total transmitter e.i.r.p.  (a2 – b  c) | 57 dBm | 27 dBm |
|  | Penetration Loss (Ref: 3GPP2) [Determine how to use these numbers for different environments, revisit if 20dB is a reasonable value for building penetration)] | 20 dB (Building) 10 dB (Vehicular) | 20 dB (Building) 10 dB (Vehicular) |
| Ii | (e) Receiver antenna gain | 0 dBi | 17 dBi |
| Ii | (f) Cable and connector losses | 0 dB | 3 dB |
|  | Body Losses | 3 dB | 0 dB |
| ii | (g) Receiver noise figure | 10 dB | 5 dB |
| ii | (h) Thermal noise density (H) (linear units) | –174 dBm/Hz 3.98  10–18 mW/Hz | –174 dBm/Hz 3.98  10–18 mW/Hz |
| id | (i) Receiver interference density (NOTE 2) (I) (linear units) | dBm/Hz mW/Hz | dBm/Hz mW/Hz |
| id | (j) Total effective noise plus interference density  10 log (10((g  h)/10)  I) | dBm/Hz | dBm/Hz |
| ii | (k) Information rate (10 log (Rb)) | dB(Hz) | dB(Hz) |
| id | (l) Required Eb/(N0  I0) | dB | dB |
| id | (m) Receiver sensitivity = (j  k  l) |  |  |
| id | (n) Hand-off gain * | dB | dB |
| id | (o) Explicit diversity gain * | dB | dB |
| id | (o) Other gain * | dB | dB |
| id | (p) Log-normal fade margin * | dB | dB |
| id | (q) Maximum path loss  {d1 – m  (e – f)  o  n  o – p} | dB | dB |
| id | (r) Maximum range | m | m |

* the following parameters: (n), (o), (o’) and (p) should be specified and justified by the individual proposals

# References

- IEEE 802.20, contribution # C802.20-04-51r2 (MS PowerPoint presentation):  RF Performance Evaluation Criteria

- IEEE 802.20, contribution # C802.20-04-58: Evaluation of 802.20 proposals with adjacent channel interference considerations.

- IEEE 802.20 Evaluation Criteria document version 11r2, July 2004