| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Evaluation of 802.20 proposals with adjacent channel interference considerations –Description text |  |
| Date Submitted | 2004-07-12 |  |
| Source(s) | Anna Tee 1301 E Lookout Dr., Richardson, TX 75082 | Voice: (972) 761-7437 Fax: (972) 761-7909 Email: atee@sta.samsung.com |
|  | Joseph Cleveland 1301 E Lookout Dr., Richardson, TX 75082 | Voice: (972) 761-7981 Fax: (972) 761-7909 Email: jclevela@sta.samsung.com |
| Re: | Description text on evaluation methods for effects of adjacent channel interferences |  |
| Abstract | This is a follow-up contribution to C802.20-04-58 and the discussion in conference call on June 1, 2004, to provide text description on the methodology to evaluate adjacent channel interference effects in 802.20 proposals. |  |
| Purpose | For discussion and adoption in 802.20 evaluation criteria document. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Introduction

The problem of evaluating the degradation effects of adjacent channel interference (ACI) on system performance has been discussed in Contribution C802.20-04-58 during the Interim meeting in May, 2004. Further discussion has continued during the conference call in June. This contribution proposes the description text on the evaluation methodology of the degradation effects caused by ACI.

Proposed Method to evaluate the degradation caused by ACI in system simulation

During the discussion in the 802.20 Interim meeting in May 2004, there have been concerns about the assumptions in the computation of ACI assuming the latter is additive white Gaussian Noise (AWGN). The assumption may not be valid in some cases.

Thus, it would be more appropriate to include non-linearity in the link level simulation in order to accurately model the degradation effects of ACI on the system performance. Proposed text for the evaluation criteria document is as follows:

3.3. Link level simulation model to include the effects of adjacent channel interference

When multiple frequency channels are deployed in an assigned channel block, the effect of adjacent channel interference can be modeled and included in the link level simulation as shown in Figure 1.  A non-linearity model that can be used in the simulation is shown in Figure 2 [TBD]. Link performance of the desired user under the effect of ACI can be obtained through the computation of error probabilities at the receiver. The result of link performance should then be incorporated into the system level simulation.

Figure 1	  Link Simulation Model to include the effects of Adjacent Channel Interference

Figure 2	  Power amplifier model (TBD)

Source: 3GPP TR25.892 v1.1.0