| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for the Channel Models to used for the Link-Level Simulation |  |
| Date Submitted | 2004-09-15 |  |
| Source(s) | David Huo 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 428 7787 Fax: +1 973 386 4555 Email: dhuo@lucent.com |
| Re: | Evaluation criteria |  |
| Abstract | This document proposes the mathematical model and its implementation for the simulation of the statistic process for the Rayleigh and Rician channels in the link-level simulation. |  |
| Purpose | Discuss and adopt. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Proposed Text

The power–delay profile is a set of deterministic value pairs  (bk,τk),k=1,2,...,M, where bk refers to the square root of the power received from propagation path indexed by k and τk refers to the time delay of the k-th path with respect to the first path with index k=0.  For given power-delay profile, the  multi-path channel model can be expressed by a sum of  M propagation paths as follows,

. h(t)=k=1Mbkhk(t)δ(t−τk)

where each path consists of   N  sub-paths and can be expressed by

hk(t)=i=1Naiexpj(ϖcosθk,it+φk,i)

for Rayleigh fading and by

hk(t)=Kexpjφk+i=1Naiexpj(ϖcosθk,it+φk,i)K2+1

for Rician fading.

The parameters occur in the above equations have the following meaning:

- M: number of paths of the channel

- N: number of sub-paths

- ϖ: maximum Doppler radian frequency of the sub-path, computed by ϖ=vcω, where ω radian  RF carrier , v the mobile speed and c the light speed

- bk:  given path amplitude satisfying the power normalization condition i=1Nbk2=1

- τk: time delay of the path relative to the first path and τ0=0

- hk(t): the fading model of the path

- δ(t): the delta distribution

- k: index of the path

- t: time

- θk,i: a uniformly distributed random number in [0, 2π)

- φk,i: a uniformly distributed random number in [0,2π)

- ai: a uniformly distributed random number in [0,1) satisfying the normalization condition i=1Nak,i2=1

- j=−1.

- K: the power ratio of the dominant path versus the rest in case of  Rician fading