| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Outline of the Elementary Issues Regarding the Channel Models |  |
| Date Submitted | 2004-09-15 |  |
| Source(s) | David Huo 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 428 7787 Fax: +1 973 386 4555 Email: dhuo@lucent.com |
| Re: | Evaluation criteria |  |
| Abstract | This document outlines the current issues regarding the definition of the channel models within the evaluation criteria and channel model ad-hocs, and proposes approaches to the open problems. |  |
| Purpose | Discuss and adopt. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Outline of the Elementary Issues

The channel model ad-hoc and the evaluation criteria ad-hoc within IEEE802.20 have be running,  independently of each other,  since their creation .  Now, with the scheduled deadline  for their respective activities approaching ,  it is  necessary  to review the documents and to identify the convergent contents, so that the tools for the evaluation and simulation  can   be provided before the proposals submission.  The work in the context of channel model  some crucial issues are not even addressed in the documents  yet. This contribution attempts to address those issues that are elementary for the simulation, to outline the current status and to show  possible directions that may leads to the solution.  The elementary issues are the following:

- Mathematical Formulation of the Elementary Channel Models to be used for the link level simulation

No link level simulation can be done without a  precise formulation of the mathematical model underlying the channel models to be used for the link level simulation.  Since no system level simulation can be performed without link performance results, this issue is the outmost important. Although, by introducing the parameters for the use of a static channel model such as the ITU models, those models  are assumed, but spelling out the exact formulation may help to remove any ambiguity and avoid misunderstanding in the follow-up development of the evaluation criteria.  A proposal  on such an explicit expression is made  in [3].

2. Channel models to be used for the System Level Simulation

It is already recognized that the current space channel model does not specify the fixed set of parameters to be used, in order for a comparison of the different proposals. Therefore, in order for the current space channel model to be useful for the purpose of comparative evaluation of the merits of different proposals, it is necessary to agree on a set of parameters to used  by  the model.

Since the space channel model does not provide significant advantage against the conventional models with fixed power delay profile, e.g. ITU model,  when the antennas involved are omni-directional, it is necessary to allow proponent to use models with fixed power delay profile.  The proposed approach is:

- Specify the precise formulation of channel models with fixed power delay profile, e.g. ITU model, as one of the options within the channel model specification.

- Specify parameters that correspond to the case when channel model of the fixed power-delay profile, e.g. ITU model, can be used.

- Simplification of the Space Channel model for the System Level Simulation

The space channel model involves implementation of multiple and concatenated random processes and, as such, requires significant more simulation time than the conventional channel models. It is, therefore, desirable to introduce some simplification in terms of modified assumptions, e.g. replacing some of the random processes by static values. Each such simplification will reduce the chain of necessary random processes and thus improves the convergence of the simulation results. Of course each such simplification renders the accuracy of the space channel model, which is developed to fit better the real world situation. But within the context of comparative evaluation of different technologies, properly selected simplifications can still well preserve the capability of the space channel model in differentiating the physical performances of  various antenna systems. Proposals on which simplifications should be introduced are invited and the issue is still open.

Reference

[1] IEEE C802.20-04-21: Proposal for Link-System Interface

[2] IEEE C802.20-03-92: Channel Models for IEEE 802.20 MBWA System Simulations

[3] IEEE C802.20-04-21: IEEE 802.30 Evaluation Criteria (ver 11)

[4] IEEE C802.20-04-71: Simplification of the Space Channel Model for Link Level Simulation

[5] IEEE C802.20-04-76: Proposed Text for the Channel Models to be used for the Link Level Simulations