| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | System-level calibration method for IEEE 802.20 technology evaluation. |  |
| Date Submitted | 2004-1-09 |  |
| Source(s) | Anna Tee 1301 E Lookout Dr., Richardson, TX 75082 | Voice: (972) 761-7437 Fax: (972) 761-7909 Email: atee@sta.samsung.com |
|  | Farooq Khan 1301 E Lookout Dr., Richardson, TX 75082 | Voice: (972) 761-7929 Fax: (972) 761-7909 Email: fkhan@sta.samsung.com |
|  | Arak Sutivong 5775 Morehouse Dr., San Diego, CA 92122 | Voice: (858) 651-6957 Fax: Email: araks@qualcomm.com |
| Re: | Contribution to IEEE 802.20 Evaluation Conference call |  |
| Abstract | This contribution proposes a possible approach for calibration of system-level simulation in the evaluation process of proposals for IEEE 802.20. |  |
| Purpose | For discussion & adoption into 802.20 evaluation criteria document. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Introduction

In the latest version of the IEEE 802.20 Evaluation Criteria Document [1], the section on calibration is still open. It is the objective of this contribution to propose a method in which simulators used by various technology proponents can be calibrated at the system level to enable a fair comparison of simulation results in the future. The calibration procedures/metrics specified in this contribution are intended to be as technology-independent as possible. As indicated in the Evaluation criteria document, calibration should be part of phase 1 of the evaluation process. The following text is proposed for adoption into section 6.1.2 “Phase 1 Calibration".

Proposed Text

6.1.2 	Phase 1 Calibration

6.1.2.1 Simulation assumptions:

The link budget in section 11 and the channel models adopted for phase 1 evaluation should be used in the calibration.

Other assumptions include:

- Uniformly distributed users in 19 3-sectored cells (assuming the wrap-around model specified in Appendix A)

- Base Station (BS) to BS distance: 2.5 km

- Path loss model as specified in the channel model document for suburban macro, urban micro cells etc.

- Lognormal shadowing standard deviation = 8 dB

- 10 users/sector

- No admission control

- Each user selects the sector with the best downlink long-term channel gain (i.e., excluding short-term fading) as the serving sector.

- Full-buffer traffic model

- Maximum C/I = 30 dB , where C/I is defined as the ratio of carrier traffic power to the total interference and noise power at the receiver

- No soft handoff  for the calibration

- No power control on the downlink

- Perfect slow power control on the uplink with the same target received power for all users

- Simulation duration : 10 seconds

- Time interval for fast fading channel update & (C/I) data collection: 40 ms for MS speed ~3 km/h, 1 ms for MS speed ~120 km/h.

6.1.2.2	Outputs for calibration

The simulation results from the calibration should include the following:

- Cumulative distribution (CDF) plots of user C/I, for both Downlink and Uplink, at time = 0 (i.e., excluding short-term fading).

- Cumulative distribution plots of user’s distance from the serving sector. Due to log-normal shadowing, a user may end up being served by a sector that is not geographically closest to it.

- Spatial distribution of user C/I, i.e., a plot of C/I distribution as a function of users’ location in the 19 cells

- Cumulative distribution plots of user C/I for both Downlink and Uplink, at the end of the simulation, including all the C/I samples collected over the simulation duration.

- Relative uplink inter-sector over intra-sector received power (f). Definef=IocIor , where Ior is the average received power from intra-sector users and Ioc is the average received power from users outside the sector. The contribution from thermal noise is excluded from the calculation. All users are given the same system resources on the uplink.

6.1.2.3	Further calibration

The traffic models should be calibrated by plotting and comparing the CDF of packet sizes, inter-packet arrival times and related traffic model parameters, before the start of evaluation based on those models.

Depending on the process of technology evaluation and selection, further calibration may be defined during the evaluation process, e.g., by using a common set of link-level performance curves [2].

References

- “IEEE 802.20 Evaluation Criteria Document Version 11r2”, Oct 24, 2004.

- 3GPP2/TSG-C.R1002, “1xEV-DV Evaluation Methodology (V14)”, June 2003.

- Channel Models for IEEE 802.20 MBWA System Simulations-Rev. 06, Oct 26, 2004.

This value should be consistent with that in the final, adopted version of Channel models document. The value as shown above is based on the latest version of the document [3].

Perfect slow power control is equivalent to channel-inversion power control, excluding short-term fading.