| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for the SCM Evaluation Process |  |
| Date Submitted | 2005-1-17 |  |
| Source(s) | Achilles Kogiantis 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 386 4399 Fax: +1 973 386 2961 Email: achilles@lucent.com |
| Re: | Channel Models |  |
| Abstract | This document proposes an approach to the evaluation process that bridges the non-spatial channel modes and the SCM approach. Proposed text is provided. |  |
| Purpose | Discuss and adopt. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

## Introduction

In the San Antonio 802.20 meeting, (November 2004), the discussion on how to bridge the widely-used ITU channel models with the recently proposed Spatial Channel Model (SCM) concluded in:

- Including in the introduction section of the CM document [1], a paragraph that specified that in Phase I of the simulations, fixed values in the SCM model will be used “in order to produce results as close as possible to the ITU Model. This facilitates comparison with documented performance results of existing systems using the ITUmodels.”

- A new section (Section 2.7.3) to be added in a revised version of [1], where the details, as described in the Introduction (Section 1), will be given.

This contribution proposes the text for the new Section 2.7.3 of the CM document [1] and adds a reference document [25] to section 4.

## Proposed Text

---------------------------------------------Proposed Text starts below------------------------------------------------

### Section 2.7.3      SCM Model Assumption for Correlation to ITU Models

In order to facilitate the comparison between 802.20 proposals of various technologies from SISO to MIMO, three categories of channel scenarios are defined below. A justification for each category is given along with the details on the channel model assumptions.

- Category A: ITU Channel Models. 

These are fixed-parameter models and are defined only in the temporal domain (no spatial components contained). This category is specified in order to compare the performance of the MBWA technology proposals with published performance results of existing systems that have been using the ITU models. Thus, Category A provides reference results for the MBWA proposals. The extensive set of the ITU channel models is defined in all detail in [25]. A limited subset of the typically used models is given in Table 2.7.3 below and the identical entries appear in Table 2.1 in Section 2.7.1. Although propagation models are also defined in [25] only the commonly agreed propagation models in MBWA should be used for uniformity of the results. Phase I of the simulations should start with the Category A channel models.

- Category B: ITU Channel Models with Fixed Spatial Parameters. 

This category uses the temporal parameters of the ITU channel models as given by Category A with the addition of fixed spatial parameters. This will allow the comparison of SISO proposals with MIMO proposals in a channel environment that is close to ITU models in the temporal domain, yet defines the necessary spatial parameters so as to enable a MIMO system to exhibit an anticipated performance. Also, simulation results from the Category B channels can be compared with those from Category A channels, thus establishing the performance difference expected.  Table 2.1 in Section 2.7.1 defines all the necessary parameters for the Category B channels. The table defines ITU temporal profiles, a set of Base Station and Mobile Station angle spreads, angle of arrivals, power azimuth spectra and direction of travel. Table 2.1 offers a variety of static (i.e. fixed) spatial channel descriptions that should be enough to cover a variety of conditions for a multiple- antenna system. Category B are to used in the Phase I of the simulations.

- Category C: System Level Spatial Channel Model (SCM). 

This category describes the full-blown SCM to be used for system level performance evaluation. Each mobile is assigned its own uniquely generated temporal and spatial parameters. This category is defined in complete detail in Section 3.

The above three channel categories provide a phased approach to comparing the new MBWA proposals with performance results of existing systems and also the means to benchmark the SCM against the ITU channel based results.

| Power Delay Profile |  | Pedestrian-A |  | Vehicular-A |  | Pedestrian-B |  | Vehicular-B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Paths |  | 4 |  | 6 |  | 6 |  | 6 |  |
| Relative Path power (dB) | Delay (ns) | 0 | 0 | 0 | 0 | 0 | 0 | -2.5 | 0 |
|  |  | -9.7 | 110 | -1.0 | 310 | -0.9 | 200 | 0 | 300 |
|  |  | -19.2 | 190 | -9.0 | 710 | -4.9 | 800 | -12.8 | 8900 |
|  |  | -22.8 | 410 | -10.0 | 1090 | -8.0 | 1200 | -10.0 | 12900 |
|  |  |  |  | -15.0 | 1730 | -7.8 | 2300 | -25.2 | 17100 |
|  |  |  |  | -20.0 | 2510 | -23.9 | 3700 | -16.0 | 20000 |

Table 2.7.3. Power Delay Profiles of typically used outdoor ITU channel models

4. References

{editor’s note: please add reference document [25] as follows:}

[25] ITU-R Recommendation M.1225: “Guidelines  for  Evaluation  of  Radio Transmission Technologies for  IMT-2000, 1997”

----------------------------------------------Proposed Text ends here--------------------------------------------------

## References

[1]     Contribution “802.20 Channel Model for IEEE MBWA System Simulations” draft v7