| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Modeling Mobility in System Level Simulation |  |
| Date Submitted | 2005-01-15 |  |
| Source(s) | David Huo 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 428 7787 Fax: +1 973 386 4555 Email: dhuo@lucent.com |
| Re: | Performance Evaluation Criteria |  |
| Abstract | This document proposes an approach capable of capturing the mobility for the computer simulation.. |  |
| Purpose | Discuss and adopt. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Introduction

In [1] it is proposed to address the mobility issue during the evaluation process via computer simulation. The simulation model in [2] and [3] so far, however, assumes that the mobile remains in the same location once it is dropped.  As the notion of mobility does not exist in the current system level simulation assumption, it is necessary to introduce a procedure that allows for the evaluation of  the impact of the mobility to the system performance by means of computer simulation. In fact, such a model is already available in [4] [5] and has been successfully deployed in the evaluation of GSM system [6] [7]. The following is a sketch of the proposed method based on [4].

- Model of Slow Fading

As a function of time, the received signal under shadow fading is modeled as a Markov process as follows

w(t)=w(t−Δt)⋅r(Δt)+u(t)⋅[1−r(Δt)]

Equation 1: ARMA(1,1) model for the slow fading process

where w(t)is the middle term averaged of the received signal amplitude at time t, u(t) is a random number of lognormal distribution with std  σ  and Δt is the sample time between two consecutive  sample values. The function r(Δt) is the correlation coefficient between the signal value at time t and  t+Δt, i.e.

r(Δt)=Ew(t+Δt)w(t)

where the initial value is w(0)=0. It is assumed that this value is positive and less than 1.  In order to use this model in the simulation to capture the propagation environment typical for the shadow fading,  a relation between  the distance that the mobile traveled and the morphological characteristics of the environment that causes the shadowing is necessary.  Let λ be the mean distance between two obstacles that cause the shadow fading. Then a Poisson distribution can be assumed for the shadowing

Pr(v⋅Δt<x)=exp(−xλ)

The correlation between two consecutively measured signal instance is proportional to the probability that the values measured at two time instances follow the distribution

Pr(v⋅(t2−t1)<x)

where t1 and t2 are the time instance at the two locations. It is proved in [4] that the probability and the correlation coefficient can be equated within the given context.  Thus

r(Δt)=exp(−vλΔt)

Equation 2: Correlation coefficient

can  be used for equation 1, once the parameter λ is given.

- Recommendation

When mobility is to be simulated in the system level simulation, each mobile is associated with 4  parameters (x,y,vx,vy), where x is the X-coordinate, y the Y-coordinate, vx the x-component of the mobile velocity and vy the y-component of the mobile speed. Formula given in Eq.1 and Eq.2 will be used to determine the instantaneous value of the fading signal, where

v=vx2+vy2

The values for the shadow distance λ used in the formula can be taken from the following table:

| Environment | λ |
| --- | --- |
| Urban | 20 m |
| Suburban | 100 m |
| Hilly Terrain | 300 m |

Table 1:  Shadow distance (negotiable)

The received signal, taking into account of both slow and fast fading, has the expression

r(t)=w(t)+(s∗h)(t)+n(t)

where h is the function for fading channel and n is the short term white noise.

I recommend the inclusion of the proposed model for the mobility simulation into the evaluation criteria document.

- Reference

[1] QC contribution in IEEE802.20 San Antonio Meeting

[2] IEEE 802.20 SCM document

[3] IEEE 802.20  Evaluation Criteria Document

[4]  D. Huo, “Simulating Slow Fading by Means of One Dimensional Stochastic Process”, 46th  IEEE VTC’96, pp620ff, 1996.

[5] D. Huo, “Einsatz des ARMA-Filters bei der Simulation des langsamen Schwundes im Mobilfunk”, URSI Kleinheubacher Berichte, Band 39, pp289-295, 1996.

[6] D. Huo, “Computersimulation zeitveraenderlicher Gleichkanalstoerung zur Untersuchung des GSM-Frequenzsprungsverfahrens”, URSI  Kleinheubacher Berichte,  Band 40, pp 648-656, 1997.

[7] M. Keller, “Comprehensive Simulation Approach to Study FH in the GSM System regarding the Physical Layer Level and the System Level”, 47th IEEE VTC’97, pp1852ff, 1997.