| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Compliance Table for the 802.20 Technology Selection Process (TSP) |  |
| Date Submitted | 2005-03-14 |  |
| Source(s) | Joanne C. Wilson ArrayComm Inc. 2480 N. First Street, Suite 200 San Jose, CA 95131 Mike Youssefmir ArrayComm Inc 2480 N. First Street, Suite 200 San Jose, CA 95131-1014 Mark Klerer Flarion Technologies 135 Route 202/206 South Bedminster, NJ 08817 | Voice: +1 202 669-4006 Fax: +1 253 484-0330 Email: joanne@arraycomm.com Voice: N/A Fax: +1 408-428-9083 Email: mike@arraycomm.com Voice : +1 908 997-2069 Fax : +1 908 997-2050 Email : m.klerer@flarion.com |
| Re: | MBWA Call for Contributions: Session # 13- March 14-17, 2005 |  |
| Abstract | This contribution proposes a Compliance Table that can be use with any Technology Selection Process adopted by the 802.20 Working Group. It is based on the format proposed in IEEE C802.20-04/72r3. |  |
| Purpose | This proposal is presented to expedite the adoption of a process for selection of the technology proposal upon which the IEEE 802.20 standard should be drafted. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Table 2a:  Compliance with the 802.20 System Requirements Document

| # | Requirement | SRD Section # | Requirement Type |  | Compliance Level |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | shall | should | Yes | Notes |
| 1 | PAR requirements | 1.3 | ● |  |  |  |
| 2 | VoIP Services | 2.1 | ● |  |  |  |
| 3 | Broadcast – Multicast services | 2.2 | ● |  |  |  |
| 4 | non-line of sight outdoor to indoor scenarios and indoor coverage | 3.1 | ● |  |  |  |
| 5 | layered architecture and separation of functionality between user, data and control | 3.1 | ● |  |  |  |
| 6 | Spectral efficiency – DL @ 3 km/hr: 2.0b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 7 | Spectral efficiency – DL @ 120km/hr: 1.5b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 8 | Spectral efficiency – UL @ 3km/hr: 1.0b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 9 | Spectral efficiency – UL @ 120km/hr: .75b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 10 | Block assignment support | 4.1.2 | ● |  |  | State what sized block assignment supported. |
| 11 | Duplexing Scheme | 4.1.3 | ● |  |  | State if FDD or TDD scheme is supported. |
| 12 | Support for Half Duplex FDD subscriber station. | 4.1.3 |  | ○ |  |  |
| 13 | Support for different mobility rates | 4.1.4 | ● |  |  | State which mobility rates are supported. |
| 14 | Aggregated data rate consistent with item 6 | 4.1.5 | ● |  |  |  |
| 15 | Aggregated data rate consistent with item 7 | 4.1.5 | ● |  |  |  |
| 16 | Aggregated data rate consistent with item 8 | 4.1.5 | ● |  |  |  |
| 17 | Aggregated data rate consistent with item 9 | 4.1.5 | ● |  |  |  |
| 18 | Peak User Data Rate (DL) of 4.5 Mbps in 1.5 MHz | 4.16 | ● |  |  |  |
| 19 | Peak User Data Rate (UL) of 2.25 Mbps in 1.25 MHz | 4.16 | ● |  |  |  |
| 20 | Peak User Data Rate (DL) of 18 Mbps in 5.0 MHz | 4.16 | ● |  |  |  |
| 21 | Peak User Data Rate (UL) of 9 Mbps in 5.0 MHz | 4.16 | ● |  |  |  |
| 22 | MAC layer to control >100 simultaneous active sessions per sector. (See section for conditions.) | 4.1.7 |  | ○ |  |  |
| 23 | QoS support per requirements in section 4.1.8 | 4.1.8 | ● |  |  | State any deviations from requirements in 4.1.8. |
| 24 | Support the configuration of a flexible set variety of traffic classes (see section 4.1.8.1) | 4.1.8.1 | ● |  |  |  |
| 25 | MAC/PHY features to support multi-antenna capabilities at the BS | 4.1.9 | ● |  |  |  |
| 26 | Base station antenna diversity | 4.1.10 |  | ○ |  |  |
| 27 | Support coverage enhancing technologies | 4.1.11 | ● |  |  |  |
| 28 | BS authentication | 4.1.12 | ● |  |  |  |
| 29 | MT authentication | 4.1.12 | ● |  |  |  |
| 30 | Network and mobile terminal perform mutual entity authentication and session key agreement protocol. | 4.1.12.1 | ● |  |  |  |
| 31 | Privacy and message integrity methods | 4.1.12.2 | ● |  |  |  |
| 32 | Support for encryption across the air interface. | 4.1.12.2 | ● |  |  |  |
| 33 | Protection from unauthorized disclosure of the device permanent identity to passive attackers. | 4.1.12.3 | ● |  |  |  |
| 34 | Protection against Denial of Service (DOS) attacks | 4.1.12.4 | ● |  |  |  |
| 35 | AES Support | 4.1.12.5 | ● |  |  | State any deviation from requirements in 4.1.12.5. |
| 36 | automatic selection of optimized user data rates that are consistent with the RF environment constraints and application requirements | 4.2.1 | ● |  |  |  |
| 37 | Graceful reduction or increase of user data rates, on the downlink and uplink | 4.2.1 | ● |  |  |  |
| 38 | Link adaptation | 4.2.1 | ● |  |  |  |
| 39 | BS and MS transmit power control mechanisms and exchange control and monitoring information | 4.2.1 |  | ○ |  |  |
| 40 | Application in dense urban, urban, suburban, rural, outdoor-indoor, pedestrian, and vehicular environments and the relevant channel models. | 4.2.2 | ● |  |  |  |
| 41 | Physical layer Measurements - BS | 4.2.4 | ● |  |  |  |
| 42 | Physical layer Measurements - MS | 4.2.4 | ● |  |  |  |
| 43 | Design extensible to wider channels. | 4.3 | ● |  |  |  |
| 44 | Mechanisms for quality of service (QOS) control and monitoring. | 4.4.1 | ● |  |  |  |
| 45 | Interfaces and procedures that facilitate the configuration, negotiation, and enforcement of QoS policies | 4.4.1 | ● |  |  |  |
| 46 | Support both IPv4 and IPv6. | 4.5 | ● |  |  |  |
| 47 | Handoff methods | 4.5.1 | ● |  |  |  |
| 48 | Allow the use of either MobileIPv4, MobileIPv6 or of SimpleIP | 4.5.1.1 | ● |  |  |  |
| 49 | Mechanism to enable the provisioning and collection of metrics. | 4.5.2 | ● |  |  |  |
| 50 | Not preclude proprietary scheduling algorithms, so long as the standard control messages, data formats, and system constraints are observed. | 4.6 | ● |  |  |  |
| 51 | Power conservation features to improve battery life for idle mobile terminals. | 4.7 | ● |  |  |  |

Table 2b:  Compliance with the 802.20 Evaluation Criteria Document

| Evaluation Criteria Document Compliance Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ECD section # | Item | Requirement Level |  | Compliance Level |  |
|  |  | Shall | Should | Yes | Notes & Deviations |
| 2 | Link level and System Level Analysis |  |  |  |  |
| 3 | Link level modeling |  |  |  |  |
| 3.1 | Modeling assumptions |  |  |  |  |
| 3.2 | Performance metrics |  |  |  |  |
| 3.3 | Link level simulation model |  |  |  |  |
| 4.3 | Traffic models |  |  |  |  |
| 4.4 | Traffic mix |  |  |  |  |
| 5 | System Level Modeling |  |  |  |  |
| 5.1 | Cell layout |  |  |  |  |
| 5.2 | Fading Models |  |  |  |  |
| 5.3 | Higher Layer Protocol Modeling |  |  |  |  |
| 5.4 | Backhaul Network Modeling |  |  |  |  |
| 5.5 | Mobility Modeling |  |  |  |  |
| 5.6 | Control signaling modeling |  |  |  |  |
| 6.1 | Channel models for Phase 1 of the simulations |  |  |  |  |
| 7 | Link-System Interface (LSI) |  |  |  |  |
| 8 | System Simulation Calibration |  |  |  |  |
| 9 | Channel Modeling |  |  |  |  |
| 9.1 | Channel Mix |  |  |  |  |
| 9.2 | Channel Models |  |  |  |  |
| 10 | RF Environment |  |  |  |  |
| 10.1 | Radio Transceiver Characteristics |  |  |  |  |
| 11 | Link Budget |  |  |  |  |
| 12 | Equipment Characteristics |  |  |  |  |
| 12.1 | Antenna Characteristics |  |  |  |  |
| 12.2 | Hardware Characteristics |  |  |  |  |
| 12.3 | Deployment Characteristics |  |  |  |  |
| 13 | Output Metrics |  |  |  |  |
| 13.1 | System Capacity Metrics |  |  |  |  |
| 14 | Payload Based Evaluation |  |  |  |  |
| 14.1 | Capacity performance evaluation criteria |  |  |  |  |
| 14.2 | Payload transmission delay evaluation criteria |  |  |  |  |
| 15 | Fairness Criteria |  |  |  |  |
| 16 | Simulation and evaluation of various block assignments |  |  |  |  |
| APP. A | 19 Cell Wrap-Around Implementation |  |  |  |  |