| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed text for VoIP traffic model for 802.20 proposal evaluation |  |
| Date Submitted | 2005-3-14 |  |
| Source(s) | Anna Tee 1301 E Lookout Dr., Richardson, TX 75082 | Voice: (972) 761-7437 Fax: (972) 761-7909 Email: atee@sta.samsung.com |
|  | Farooq Khan 1301 E Lookout Dr. Richardson, TX 75082 | Voice: +1 972 761 7929 Fax: +1 972 761 7909 Email: fkhan@sta.samsung.com |
|  | Joseph Cleveland 1301 E Lookout Dr. Richardson, TX 75082 | Voice: (972) 761-7981 Fax: (972) 761-7909 Email: jclevela@sta.samsung.com |
| Re: | Call for Contribution for IEEE 802.20 session #13, March 14-17, 2005 |  |
| Abstract | This contribution proposes clarification texts to section 4.3.5 VoIP traffic model of the evaluation criteria document. QoS criteria for VoIP traffic is also proposed for section 15. |  |
| Purpose | For discussion & adoption into IEEE 802.20 evaluation criteria document. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Introduction

This contribution proposes text for the section on Voice (VoIP) traffic model in the IEEE 802.20 evaluation criteria document [1]. The proposed text below is based on the contributions that have been previously discussed in the working group, listed as follows: C802.20-04/12 [1], C802.20-04/37 [3] and C802.20-05/05 [4].

Current Text

“4.3.5	Voice (VoIP)

The voice traffic model will be implemented as voice over IP (VoIP). Voice will in general follow a Markov source model with different rates (full rate, half rate, etc.) with a corresponding set of transition probabilities between different rates.”

Proposed text to follow the current text

Option 1:

Generate voice traffic with voice activity factor of 0.4 and the following percentage mix of voice data rate:

- 29% at 8.6 kbps (full rate)

- 4% at 4.3 kbps (half rate)

- 7% at 2.1 kbps (quarter rate)

- 60% at 1 kbps (one-eighth rate)

The transition probabilities between each of these 4 states can be found in TIA/EIA/IS-871 [5]. Frame duration of 20 ms as supported by this type of vocoder [6] should be used.

Assumptions on overhead due to protocol headers need to be specified by the proponents.

Option 2:

Generate voice traffic with voice activity factor of 0.4 and the following percentage mix of voice data rate:

- 40% at 8 kbps (Active)

- 60% at 0 kbps (Inactive)

Frame duration of 10 ms as supported by this type of vocoder [7] should be used.

The Markov model is shown in Fig. 1. Assume that the average dwell time in the active state and inactive (idle) states are 1 second and 1.5 seconds respectively, i.e.,

Tinactive=1.5sTactive=1.0s⇒Voice Activity Factor=11+1.5=0.4

With frame duration of 10ms, the number of frames generated in 1 second is 100. Thus, the corresponding transition probabilities are as follows:

P(A|I)=1150P(I|A)=1100

Assumptions on overhead due to protocol headers need to be specified by the proponents.

Fig. 1	Markov model for vocoder with 2 states

Performance / QoS criteria

The performance / QoS criteria can be included in section 15 Fairness Criteria of the evaluation criteria document [1]. As VoIP belongs to the real-time, interactive traffic class, the following performance criteria are proposed based on the QoS performance data in TIA/EIA/IS-810A [4, 8],  ITU-T recommendations G.1010 [9] and 3GPP TS 22.105 [10].

For VoIP traffic type, the QoS criteria can be used as the fairness criteria. In the best effort traffic types, e.g. full-buffer, FTP, unfair situation can happen when a few heavy users consume all resources in the system such that other users could not have a fair share of the system resources. In contrast, each VoIP or similar conversational class traffic user may not need to consume a large amount of system resources typically, but the QoS requirements for the traffic type will need to be met.

Proposed text that can be included in section 15 is as follows. See Contribution on “Proposed text for Fairness/QoS criteria for 802.20 proposal evaluation” [11] for a more complete description.

For VoIP traffic type, the QoS criteria are listed as follows:

- Packet loss ratio < 2%

- One way Network Delay < 150 ms

References

- “IEEE 802.20 Evaluation Criteria, Ver. 14”, March 14, 2005.

- “VoIP Models for 802.20 System Performance evaluation”, C802.20-04/12, Jan 2004

- “VoIP models – update”, C802.20-04/37, May 2004

- “Comments on Traffic Models open issue”, C802.20-05/05, Jan 2005

- “Markov Service Option (MSO) for cdma2000 Spread Spectrum Systems”, TIA/EIA/IS-871, April 2001.

- “Enhanced Variable Rate Codec, Speech Service Option 3 for Wideband Spread Spectrum Digital Systems”, TIA-127A, May 2004.

- ITU-T, “Coding of speech at 8 kb/s using Conjugate-Structure Algebraic-code-Excited Linear-Prediction (CS-ACELP)”, G.729, March 1996.

  - Annex A: Reduced complexity 8 kb/s CS-ACELP speech codec, Nov, 1996

  - Annex B: A Silence compression scheme for G.729 optimized for terminals conforming to Recommendation V.70, Nov 1996.

- “Transmission requirements for Narrowband Voice over IP and Voice over PCM Digital Wireline Telephones”, TIA/EIA-810-A, Dec 10, 2000.

- “End-user multimedia QoS categories”, ITU-T G.1010.

- 3GPP TS 22.105, "Services and service capabilities“

- “Proposed text for Fairness/QoS criteria for 802.20 proposal evaluation”, C802.20-05/xx, March 2005.