| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | VoIP Evaluation for MBWA (Rev.1) |  |
| Date Submitted | 2005-09-19 |  |
| Source(s) | Kazuhiro Murakami 2-1-1 Kagahara, Tsuzuki-ku, Yokohama, KANAGAWA 224-8502, JAPAN Minako Kithara 2-1-1 Kagahara, Tsuzuki-ku, Yokohama, KANAGAWA 224-8502, JAPAN Radhakrishna Canchi 2480 N. First Street #280 San Jose, CA 95131 | Voice: +81 45 943 6113 Fax: +81 45 943 6175 Email: kazuhiro_murakami@csg.kyocera.co.jp Voice: +81 45 943 6102 Fax: +81 45 943 6175 Email: Minako_kitahara@csg.kyocera.co.jp Voice: +1-408-952-4701 Fax: +1-408-954-8709 Email: cradhak@ktrc-na.com |
| Re: | MBWA Call for Contributions for Evaluation Criteria for VoIP application |  |
| Abstract | This document proposes a method for the characterization and evaluation of VoIP for an MBWA |  |
| Purpose | This document addresses the open issue on VoIP traffic modeling and its Quality Evaluation in the IEEE802.20 Evaluation Criteria Document Version 16 (Eval_criteria_ver16_061005) |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

### Introduction

The 802.20-based air-interface (AI) shall be optimized for high-speed IP-based wireless data services. One of the applications that confirmed to open standards and protocols is VoIP.

Also, The QoS Features in the AI shall provide the required performance of latency, jitter, and packet loss needed to support the use of industry standard codecs applicable to mobile networks.

(System Requirements for IEEE 802.20 Mobile Broadband Wireless Access Systems-Version 14)

Currently, various kinds of VoIP algorithms are appeared and MBWA shall support the flexibility for them. To achieve this system requirement and current trends of VoIP, Evaluation criteria document shall defines how to evaluate AI feature for VoIP related performance with several characteristics; Simple, easy to unify parameter and environment condition, enable to compare with each system;

E-model (ITU-T G.107) , one of the Objective Quality Assessment methods matched to these needs would like to be proposed to adopt of basic measurement for VoIP quality in traffic model in MBWA

Quality Assessment on Voice over IP

For VoIP quality assessment, it is necessary to investigate the communication quality felt by the user between end to end environments. However, the assessment index must also follow an objective and common systematic approach because there are various kinds of factors reflected to the quality. When we consider the approach, we can clarify psychological quality that determine the communication quality and the physical factors of the network and terminals that affect them.  The following figure is the summary of the relationship;

Subjective

quality

assessment

Design	     		Physical quality　　　Psychological quality

Management

Parameter

Terminal

Codec			      Jitter buffer overflow		Distortion

Jitter buffer		      Coding distortion					Listening MOS

Packet size		      Jitter-buffer delay			Loudness

Acoustic characteristics    echo cancellation

Conventional MOS

IP-packet delay	      Network packet loss		   Delay

Delay jitter		      Network delay

IP-packet loss		      					    Echo

Objective

quality

assessment

Link utilization

Network

Fig. 1 Factors that determine the quality of VoIP service

According to the figure,  approaches for one way assessment  (listening MOS) and two way assessment (conventional MOS) with MOS value result  in the Absolute Category Rating(ACR) defined by ITU-T Recommendation P.800 are categorized to Subjective quality assessment.

However in order to maintain a stable assessment result for this MOS testing, it is necessary to have dedicated equipments which are manageable in audio environment and are satisfied to the reference condition.  In addition, a large number of people are also needed for the testing.

In 1998, E-model, which is one of the objective assessment methodologies, was standardized by ITU-T Recommendation G.107 . The G.107 was updated in March 2005 and is now adopted by ETSI, TIA, TTC and so on.  (G.107 (03/2005))

### E-Model

E-model is a parameter based algorism based on 20 parameters related to terminal factor, environment factor, Network factor and so on and result is calculated to Rating Value.

R = Ro – Is – Id – Ie-eff + A

Distortion

Advantage factor

A

Equipment impairment factor

Ie-eff=f4(Ie, Bpl, Ppl)

Delay and echo

Delay impairment factor

Loudness

Id=f3(T, Tr, Ta, RLR, STMR, TELR, WEPL)

Noisiness

Simultaneous impairment factor

Is=f2(Ro, SLR, RLR, STMR, TELR, qdu)　　　　　　　　　　　　　Fig.2

Basic signal-to-noise ratio

Ro=f1(Nc, SLR,  Ps, Ds, RLR, Pr,  LSTR)

According to C802.20-05/05, Jan 2005 “Comments on Traffic Models open issue”, E-model can estimate conventional MOS value using the R-value. The equation describing the relation between R and conventional MOS values is included in the Annex B of ITU-T Recommendation G107 (03/2005) and  ITU-T Recommendation.

Table 1. Relation between R-value and MOS

| R-value lower limit | MOS | Speech Transmission Quality Category | User Satisfaction |
| --- | --- | --- | --- |
| 90 | 4.34 | Best | Very satisfied |
| 80 | 4.03 | High | Satisfied |
| 70 | 3.6 | Medium | Some users dissatisfied |
| 60 | 3.1 | Low | Many users dissatisfied |
| 50 | 2.58 | Poor | Nearly all users dissatisfied |

Equation of R value calculation in detail are as follows (Refer to ITU-T G.107 (03/2005))

R=Ro - Is - Id – Ie-eff + A

(1) Ro=f1(Nc, SLR,  Ps, Ds, RLR, Pr,  LSTR)   :  Basic signal-to-noise ration

Ro represents in principle the basic signal-to-noise ration including noise sources such as

circuit noise and room noise. The definition is as follows;

Ro=15−1.5(SLR+No) 								(1-1)

No=10log10Nc10+10Nos10+10Nor10+10Nfo10					(1-2)

Nos =Ps−SLR−Ds−100+0.004(Ps−OLR−Ds−14)2

Nor =RLR−121+Pre+0.008(Pre−35)2

Nfo =Nfor+RLR

OLR=SLR+RLR

Pre=Pr+10log1+1010−LSTR10

Nc: 	The sum of all circuit noise powers in dBm0p

Ps: 	Room Noise at the Send Side

SLR: 	Send Loudness Rating

Ds: 	D-Value of Telephone, send side

RLR: 	Receive Loudness Rating

Pr: 	Room Noise at the Receive Side

LSTR: Side tone Masking Rating

(2) Is=f2(Ro, SLR, RLR, STMR, TELR, qdu)　:  A combination of all impairments

simultaneously

Is is the sum of all impairments which ocuure more or less simultaneously with voice signal.

The definition is as follows;

Is=IoIr+Ist+Iq

IoIr=201+xolr8818−xolr8

Ist=121+STMRo−136818−281+STMRo+119.435135−131+STMRo−33313113+29												(2-1)

lq=15log1+10Y+10Z

XoIr=OLR+0.264+No−RLR

STMRo=−10log10STMR10+eγ410TELR10

Y=Ro−10015+468.4−G9

Z=4630−G40

OLR=SLR+RLR

No : (1-2)

Ro : (1-1)

G=1.07+0.258Q+0.0602Q2

Q=37−15log(qdu)

RLR: 	Receive Loudness Rating

SLR: 	Send Loudness Rating

STMR: Side tone Masking Rating

TELR: Talker Echo Loudness Rating

qdu:     Number of Quantization distortion units

(3) Id=f3(T, Tr, Ta, RLR, STMR, TELR, WEPL) :  Impairment caused by delay

Id represents all impairment due to delay of voice signal. The definition is as follows;

Id=Idte+Idle+Idd

Idte:

For 9dB<STMR

Idte=Roe−ℜ2+Roe−ℜ24+100−11−e−γ

Roe=−1.5No−RLR

Re=80+2.5TERVs−14

TERVs=TELR−40log1+T101+T150+6e−0.3γ2+Ist2

For 9dB≤STMR≤20dB

Idte=Roe−ℜ2+Roe−ℜ24+100−11−e−γ

Roe=−1.5No−RLR

Re=80+2.5TERV−14

TERV=TELR−40log1+T101+T150+6e−0.3γ2

For STMR>20dB

Idte is replaced by Idtes

Idtes=Idte2+Ist2

Ist:  (2-1)

Idle=Ro−Rle2+Ro−Rle24+169

Rle=10.5WEPL+γTr+1−0.25

Idd:

For Ta≤100ms:

idd=0

For Ta>100ms:

idd=251+X616−31+X3616+2

X=logTa100log2

RLR: 	Receive Loudness Rating

STMR: Side tone Masking Rating

TELR: Talker Echo Loudness Rating

WEPL: Weight Echo Path Loss

T :	 Mean one-way delay of the echo path

Ta:	 Absolute delay in echo free connections

Tr:	Absolute delay in a 4-wire loop

(4) Ie-eff=f4(Ie, Bpl, Ppl)      : The packet-loss dependent Effictive Equipmet Impairment

Factor

Ie-eff represents impairtments cause by low bit-rate codec including impairment due to packet

losses of random distribution. The definition is as follows;

Ie−eff=Ie+95−IePplPplBurstR+Bpl

BurstR=Average length of observed bursts in an arrival sequenceAverage length of burst expected for tbe network under  ital Random
               loss

If Packet loss is random

BurstR=−1

If Packet loss is bursty

BurstR>1

Ie: 	Equipment impairment factor

Ppl: 	Packet-loss Probability

Bpl:	Packet-loss robustness factor

(5) A       : Advantage Factor

Advantage Factor A is specified by some provisional environment.

Conventional (Wire bound)  					A = 0

Mobility by cellular network in the building			A = 5

Mobility in a geographical area or moving in a vehicle	A = 10

Access to hard-to-reach location				A = 20

(e.g. via multi-hop satellite connections)

* Detail explanations of R value calculation can be refer to ITU-T G.107 (03/2005)

E-model R Value can be calculated with 20 parameters as follows. (ITU-T  G.107(03/05)

Out of the 20detail parameters, default values can be used for 15 which are the voice quality parameters in analog transmission and parameter for terminal design.

So, only 6 parameters which are related to need to be decided upon to calculate the R value by operation or simulation.

The parameters and default values are defined in ITU-T. G. 108 (09/99). The default values and the clarification are shown in the table below.

Table 2. Input parameters for E-model and the clarification

| Category | Clarify reason |  | Parameter |  | Default Value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters that use Default value | Parameter for Quality for Analog transmission |  | Circuit noise referred to 0 dBm-point | Nc | -70 | dBm0p |
|  |  |  | Noise floor at the review Side | Nfor | -64 | dBmp |
|  |  |  | Weight Echo Path Loss | WEPL | 110 | dB |
|  | Unclear use direction |  | Advantage Factor | A | 0,5,10*1 | - |
|  | Environment factor |  | Room noise at the send side | Ps | 35 | dB(A) |
|  |  |  | Room noise at the receive side | Pr | 35 | dB(A) |
|  | Terminal Design Parameter |  | Send Loudness Rating | SLR | 8 | dB |
|  |  |  | Receive Loudness Rating | RLR | 2 | dB |
|  |  |  | Side tone Masking Rating | STMR | 15 | dB |
|  |  |  | Listener Side tone Rating | LSTR | 18 | dB |
|  |  |  | D-value of telephone, send side | Ds | 3 | - |
|  |  |  | D-value of telephone, receive side | Dr | 3 | - |
|  |  |  | Talker Echo Loudness Rating (in Ext.NW) | TELR(1) | 65 | dB |
| Parameters that need simulation or operation for decide values | VoIP necessary parameter | Voice quality | Number of Quantization distortion units | qdu | 1 | - |
|  |  |  | Equipment impairment factor | Ie |  | - |
|  |  |  | Packet-loss robustness factor | Bpl |  | - |
|  |  |  | Packet-loss Probability | Ppl |  | % |
|  |  |  | Burst Ratio | BurstR |  | - |
|  |  | Echo | Talker Echo Loudness Rating (in terminal) | TELR(2) | 65 | dB |
|  |  | Delay | Absolute delay in echo free connections | Ta |  | ms |
|  |  |  | Mean one-way delay of the echo path | T |  | ms |
|  |  |  | Absolute delay in a 4-wire loop | Tr |  | ms |

*1 Advantage Factor will be decide by the type of communication systems

e.g.   Conventional Wirebound) 					A = 0

Mobility by cellular network in the building			A = 5

Mobility in a geographical area or moving in a vehicle	A = 10

In ITU-T G.107 (03/05) , All parameters for calculating R value are listed as a default value.

Ie	= 0

Bpl	= 1

Ppl	= 0

BurstR	= 1

Ta	= 0

T	= 0

Tr	= 0

A	= 0

The calculation using these vaules results very high quarity with a rating factor  of R = 93.2

At this calculation, each element  constructing  R value (R=Ro - Is - Id – Ie-eff + A) are as follows;

Ro 	= 94.77

Is   	=   1.41

Id   	=   0.15

Id-eff 	=   0.00

A  	=   0.00

On IEEE 802.20 Mobile Broadband Wireless Access Systems,  Factor Id, impairment caused by delay, and Id-eff, Effective Impairment Factor calculated with Ie, Bpl, Ppl, and Burst R parameters

are significant to system performance evaluation. On the other hand, value of  R0, Is and A calculated with default value described in ITU-T G107 (03/05) and  G.108(09/99).

Examples of Ie and Bpl value used to calculate Ie-eff based on several codecs are listed in the ITU-T G.113 (05/02).

These parameters are related to system performance of delay, Jitter, and Packet-loss and characteristics  of a used codec.

The major codecs used these days for voice communications are listed below.

Table 3.  Example of major codecs and the characteristics

| Standard | Algorithm | Sampling Rate(kHz) | Bit Rate (kbit/s) | Frame Length (msec) | Theoretical delay (msec) | Moss Score | Main Usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ITU-T G.711 | PCM | 8 | 64 | - | 0.125 | 4.1 | ISDN/VoIP |
| ITU-T G.723.1 | MP-MLQ/ ACELP | 8 | 6.3/5.3 | 30 | 37.5 | 3.9/3.65 | VoIP |
| ITU-T G.726 | ADPCM | 8 | 16/24/32/40 | - | 0.125 | 3.85 | PHS |
| ITU-T G.728 | LD-CELP | 8 | 16 | 0.625 | 0.625 | 3.85 |  |
| ITU-T G.729 | CS-ACELP | 8 | 8 | 10 | 15 | 3.92 | PDC |
| ITU-T G.729Annex A | CS-ACELP | 8 | 8 | 10 | 15 | 3.7 | VoIP |
| ARIB STD-27 | PSI-CELP | 8 | 3.45 | 40 | 45 |  | PDC |
| ARIB STD-27 | VCELP | 8 | 6.7 | 20 |  |  | PDC |
| ARIB STD-27 | ACELP | 8 | 6.7 | 20 | 25 |  | PDC |
| 3GPP GSM 06.10 | RPE-LTP | 8 | 13 | 20 |  | 3.5 | GSM FR |
| 3GPP GSM 06.20 | VSELP | 8 | 5.6 | 20 |  | 3.5 | GSM HR |
| 3GPP GSM 06.60 | ACELP | 8 | 12.2 | 20 | 25 |  | GSM EFR |
| TIA IS-54 | VSELP | 8 | 7.95 | 20 |  |  | D-AMPS |
| TIA IS-641 | ACELP | 8 | 7.4 | 20 | 25 |  | D-AMPS |
| 3GPP TS 26.071 | AMR | 8 | 4.75/5.15/5.9/6.7/7.4/7.95/10.2/12.2 | 20 | 20/25 |  | 3G |
| INMARSAT | IMBE | 8 | 4.15 | 20 |  |  | Vessel |
| DDVPC FS-1016 | CELP | 8 | 4.8 | 30 | 105 |  | Military |
| DDVPC FS-1015 | LPC-10e | 8 | 2.4 | 22.5 | 90 |  | Military |
| DDVPC | MELP | 8 | 2.4 | 22.5 | 45.5 |  | Military |
| ITU-T G.722 | SB-ADPCM | 16 | 48/56/64 | 0.125 | 1.5 |  |  |
| 3GPP TS 26.171 ITU-T G.722.2 | AMR-WB | 16 | 6.6/8.85/12.65/14.25/15.85/ 18.25/19.85/23.05/23.85 | 20 | 25 |  |  |

For using several codec algorithms for VoIP freely depends on user needs,  MBWA should not decide specified codec for the R Value calculation.

Instead of specifying the codec, representing the parameters needed to evaluate the R Value makes the assessment method more quantitative and global.

### Conclusion

1. Use the R Value of the E-model defined in the ITU-T G.107 (03/2005) for VoIP quality assessment,

2. Adopt the default parameters defined in ITU-T G.108 (09/1999) for the parameters not related to system or Codec in order to calculate the R Value for evaluation of  the VoIP loading performance under similar conditions as much as possible,

- Calculate the R Value using the detail parameters in R Value calculation without specifying the Codec (Various example of the detail parameters are proposed in ITU-T G.113 (05/2002)

- Investigate and specify significant parameters for R Value calculation  on Mobile Broadband

Wireless Access (e.g  Impairment factor due to delay, jitter, packet-loss )

### Reference

- “System Requirement Document for IEEE 802.20 Mobile Broadband Wireless Access Systems – Version 14”Models-Update”, IEEE C802.20-04/37, May 2004.

- Anna Tee , Anna Tee , Joseph Cleveland ,“Comments on Traffic Models open issue”, IEEE C802.20-05/05, Jan 2005

- ITU-T, Recommendation G. 107 “The E-model, a computational model for use in transmission Planning” Mar. 2005

- ITU-T, Recommendation G. 108 “Application of the E-model: A Planning Guide” Sep. 1999

- ITU-T, Recommendation G. 113 “Appendix I : Provisional planning values for the equipment impairment factor Ie and packet-loss robustness factor Bbl”

- Akira Takahashi, Hideaki Yoshino, Nobuhiko Kitawaki, “Quality Assessment Methodology for IP-Telephony Services”, IEICE Trans. Commun.., (Japanese Edition)

7.   ITU-T Recommendation P.800 “Method of Subjective determination of transmission quality