| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Compliance Table for the 802.20 Technology Selection Process (TSP) |  |
| Date Submitted | 2005-07-15 |  |
| Source(s) | Dan Gal 67 Whippany Road, Whippany, NJ 07981 | Voice: 973-428-7734 Fax: 973-386-4555 Email: dgal@lucent.com |
| Re: | MBWA Call for Contributions: Session # 15 (San Francisco, July 17-22, 2005) |  |
| Abstract | This contribution builds upon and improves on the initial Compliance Table proposed in contribution C802.20-05-19. Note: At this time only the 802.20 SRD is finalized and adopted. When the Evaluation Criteria and Channel Models documents are adopted, the appropriate parts of the Compliance Table in this contribution will be updated. |  |
| Purpose | Provide a requirements compliance assessment tool for the 802.20 technology selection process. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

IEEE 802.20 Technology Proposals -- Compliance Table

Part 1: System Requirements (SRD v14)

{Editor’s note: the table entries should be renumbered in the next version of this contribution}

| # | Requirement | SRD Section # | Requirement Type |  | Compliance |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | shall | should | Yes | Notes |
| 1 | PAR requirements | 1.3 | ● |  |  |  |
| # | Support open standards and protocols | 2 | ● |  |  |  |
| # | Support location-based services | 2 | ● |  |  |  |
| # | Support battery-life preservation | 2 | ● |  |  |  |
| 2 | VoIP Services | 2.1 | ● |  |  |  |
| 3 | Broadcast and Multicast services | 2.2 | ● |  |  |  |
| # | Cellular architecture, supporting Macro, Micro and Pico cells | 3.1 | ● |  |  |  |
| 4 | non-line of sight outdoor to indoor scenarios and indoor coverage | 3.1 | ● |  |  |  |
| 5 | layered architecture and separation of functionality between user, data and control | 3.1 | ● |  |  |  |
| # | Layer1- Layer2 reference model | 3.1.1 | ● |  |  |  |
| # | LLC | 3.1.1 | ● |  |  |  |
| # | Well-defined service interface between PHY and MAC | 3.1.1 | ● |  |  |  |
| # | MAC design for multiple PHY technologies | 3.1.1 |  | ● |  |  |
| # | Specify how 802.20 fits into the IEEE 802 L1&L2 protocol reference model | 3.1.1 | ● |  |  |  |
| 6 | Spectral efficiency – DL @ 3 km/hr: 2.0 b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 7 | Spectral efficiency – DL @ 120km/hr: 1.5 b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 8 | Spectral efficiency – UL @ 3km/hr: 1.0 b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 9 | Spectral efficiency – UL @ 120km/hr: 0.75 b/s/Hz/sector | 4.1.1 | ● |  |  |  |
| 10 | Block assignment size support | 4.1.2 | ● |  |  |  |
| 11 | AI support both FDD and TDD Duplexing. | 4.1.3 | ● |  |  |  |
| 12 | Support for Half Duplex FDD subscriber station. | 4.1.3 |  | ○ |  |  |
| 13 | Support for different mobility rates (3 km/hr to 250 km/hr) | 4.1.4 | ● |  |  |  |
| 14 | DL Aggregate data rates consistent with spectral efficiency specs | 4.1.5 | ● |  |  |  |
| 15 | UL Aggregate data rates consistent with spectral efficiency specs | 4.1.5 | ● |  |  |  |
| 18 | Peak User Data Rate (DL) of 4.5 Mbps in 1.25 MHz | 4.16 | ● |  |  |  |
| 19 | Peak User Data Rate (UL) of 2.25 Mbps in 1.25 MHz | 4.16 | ● |  |  |  |
| 20 | Peak User Data Rate (DL) of 18 Mbps in 5.0 MHz | 4.16 | ● |  |  |  |
| 21 | Peak User Data Rate (UL) of 9 Mbps in 5.0 MHz | 4.16 | ● |  |  |  |
| 22 | Number of simultaneous active sessions per sector (supported by the MAC) >100 (radio bearer to be available to user in less than 25 ms @ probability >0.9) | 4.1.7 |  | ○ |  |  |
| 23 | Link-level QoS to conform to an end-to-end QoS architecture | 4.1.8 | ● |  |  |  |
| # | Ability to enforce QoS authorization per user | 4.1.8 | ● |  |  |  |
| # | Support IPv4 and IPv6 QoS resolutions | 4.1.8 | ● |  |  |  |
| # | Radio resource management per QoS and policy requirements | 4.1.8 | ● |  |  |  |
| # | Support differentiated QoS delivery of packet flows | 4.1.8 | ● |  |  |  |
| # | Support IETF’s DiffServ QoS model defined in listed RFCs | 4.1.8 | ● |  |  |  |
| # | Service and QoS mapping: define a common set of parameters for all classes of service | 4.1.8 | ● |  |  |  |
| 24 | Support the configuration of a flexible set variety of traffic classes (see more detail in section 4.1.8.1) | 4.1.8.1 | ● |  |  |  |
| 25 | MAC/PHY features to support multi-antenna capabilities at the BS and, optionally, at the MS | 4.1.9 | ● |  |  |  |
| 26 | Base station antenna diversity | 4.1.10 |  | ○ |  |  |
| 27 | Support coverage enhancing technologies | 4.1.11 | ● |  |  |  |
| 28 | BS authentication | 4.1.12 | ● |  |  |  |
| 29 | MT authentication | 4.1.12 | ● |  |  |  |
| # | Link-level security to be part of an end-to-end solution | 4.1.12 | ● |  |  |  |
| 30 | Network and mobile terminal mutual authentication and support of session-key agreement protocol. | 4.1.12.1 | ● |  |  |  |
| 31 | Privacy and message integrity methods | 4.1.12.2 | ● |  |  |  |
| 32 | Support for all four Encryption- Message- Integrity combinations across the air interface. | 4.1.12.2 | ● |  |  |  |
| 33 | Protection from unauthorized disclosure of the device permanent identity to passive attackers. | 4.1.12.3 | ● |  |  |  |
| 34 | Protection against Denial of Service (DOS) attacks | 4.1.12.4 | ● |  |  |  |
| 35 | Mandatory support for AES encryption algorithm | 4.1.12.5 | ● |  |  |  |
| 36 | automatic selection of optimized user data rates that are consistent with the RF environment constraints and application requirements | 4.2.1 | ● |  |  |  |
| 37 | Graceful reduction or increase of user data rates, to maintain a given frame error-rate, on the downlink and uplink | 4.2.1 | ● |  |  |  |
| 38 | Link adaptation: support adaptive modulation & coding | 4.2.1 | ● |  |  |  |
| 39 | BS and MS to support transmit Power Control mechanisms and mutual exchange of control and monitoring information | 4.2.1 |  | ○ |  |  |
| 40 | Support channel models defined for various mobility and delay-spread scenarios applicable to dense urban, urban, suburban, rural, outdoor-indoor, pedestrian, and vehicular environments. | 4.2.2 | ● |  |  |  |
| 41 | Physical layer measurements and reporting by BS | 4.2.4 | ● |  |  |  |
| 42 | Physical layer measurements and reporting by MS | 4.2.4 | ● |  |  |  |
| # | Provide detailed specifications of key RF parameters defined in sections 4.2.5.1 through 4.2.5.3 | 4.2.5 | ● |  |  |  |
| # | Frequency-plan to identifies channel plans suitable for mobile service, below 3.5 GHz, in which deployment of FDD and TDD 802.20 systems may be shared with incumbent Cellular system. | 4.3 | ● |  |  |  |
| 43 | Design has to be readily extensible to wider channels. | 4.3 | ● |  |  |  |
| 44 | Mechanisms, interfaces and procedures for quality of service (QOS) configuration, control and monitoring. | 4.4.1 | ● |  |  |  |
| (included in 44) |  |  | ● |  |  |  |
| 46 | Support both IPv4 and IPv6. | 4.5 | ● |  |  |  |
| 47 | Support for Handoff at the inter-cell, inter-system, inter-carrier- frequency and inter IP sub-net levels. | 4.5.1 | ● |  |  |  |
| 48 | Support these IP-level mobility/handoff protocols:MobileIPv4, MobileIPv6 or SimpleIP | 4.5.1.1 | ● |  |  |  |
| 49 | Support OA&M operations with mechanism that enable: system control, provisioning and collection of performance metrics. | 4.5.2 | ● |  |  |  |
| 50 (not verifiable) |  |  | ● |  |  |  |
| 51 | Power conservation features to improve battery life for idle mobile terminals. | 4.7 | ● |  |  |  |

Part 2: Evaluation Criteria Requirements (ECD Version xx)

(Shown on the next page)

Part 2: Evaluation Criteria Requirements (ECD Version xx)

| # | ECD section # | Requirement | Requirement Type |  | Compliance |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Shall | Should | Yes | Notes |
| 1 | 2 | Carry out Link level and System level Analyses | ● |  |  |  |
| 2 | 3 | Perform Link level modeling | ● |  |  |  |
| 3 | 3.1 | Evaluate modulation/coding schemes in all channel environments | ● |  |  |  |
| 4 | 3.2 | Performance metrics. Provide Link-level simulation results: (1). FER vs. SINR curves (2). Throughput vs. SINR curves | ● |  |  |  |
|  | 4.3 | Traffic models: simulate the traffic using the defined models (Table 1): (1) (2) . . (15) | ● |  |  |  |
|  | 4.4 | Traffic mix | ● |  |  |  |
|  | 5 | System Level Modeling: system simulations in full-duplex mode (UL-DL) in the same run | ● |  |  |  |
|  | 5.1 | Cell layout: 19 tri-sector cell wrap-around model | ● |  |  |  |
|  | 5.1.1 | Uniform distribution of users in the cells | ● |  |  |  |
|  | 5.2 | Simulations to use TCP as the L4 transport protocol for all applications | ● |  |  |  |
|  | 5.3 | Backhaul Network Modeling | ● |  |  |  |
|  | 5.4 | Mobility Modeling |  |  |  |  |
|  | 5.5 | Control signaling modeling | ● |  |  |  |
|  | 6.1 | Phase 1 simulations: Channel models | ● |  |  |  |
|  | 6.2 | Phase 2 simulations: | ●? |  |  |  |
|  | 7 | Link-System Interface (LSI) | ● |  |  |  |
|  | 8 | System Simulation Calibration | ● |  |  |  |
|  | 9 | Channel Modeling | ● |  |  |  |
|  | 9.1 | Channel Mix | ● |  |  |  |
|  | 10 | RF Environment: | Ref |  |  |  |
|  | 10.1 | Provide radio transceiver characteristics | ● |  |  |  |
|  | 11 | Fill out the Link Budget template | ● |  |  |  |
|  | 12 | Provide equipment characteristics | ● |  |  |  |
|  | 12.1 | Provide antenna characteristics | ● |  |  |  |
|  | 12.2 | Provide hardware characteristics | ● |  |  |  |
|  | 12.3 | Provide deployment characteristics | ● |  |  |  |
|  | 13 | Provide system level performance metrics: | Ref |  |  |  |
|  | 13.1 | System capacity metrics: - CDF curves - Aggregate throughput plots - Number of active users in a cell vs BS separation - Sustained spectral efficiency for stated load opearting points | ● |  |  |  |
|  | 14 | Payload Based Evaluation: | Ref |  |  |  |
|  | 14.1 | Capacity performance evaluation parameters: - channel, modulation rate and MAC frame parametes - service-type specific payloads | ● |  |  |  |
|  | 14.2 | Payload transmission delay evaluation criteria | ● |  |  |  |
|  | 15 | Fairness Criteria | ● |  |  |  |
|  | 16 | Simulation and evaluation of various block assignments | ● |  |  |  |
|  | APP. A | 19 Cell Wrap-Around Implementation | ● |  |  |  |

Part 3: Channel Modeling Requirements (CMD Version xx)

{TBD}

| # | CMD section # | Requirement | Requirement Type |  | Compliance |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Shall | Should | Yes | Notes |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |

State what size block assignment supported.

State if FDD or TDD or both schemes are supported.

State which mobility rates are NOT supported.

Applies to 1.25 MHz FDD or 2.50 MHz TDD system

State any deviations from requirements in 4.1.8

State any deviation from requirements in 4.1.12.5.