| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for the Section on Correlation Matrix in the Channel Model Document |  |
| Date Submitted | 2005-07-17 |  |
| Source(s) | David Huo 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 428 7787 Fax: +1 973 386 4555 Email: dhuo@lucent.com |
| Re: | Channel Model |  |
| Abstract | This document proposes text for the channel model document |  |
| Purpose | Discuss and adopt |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group to accomplish the simulator calibration process. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Introduction

This contribution proposes text for [1] that sorts the logical relations between proposed ideas and re-emphasizes the idea of reducing the burden of irrelevant but complicated simulation work as proposed in [2], incorporating [3].

- Proposed Text

[based on the text of last meeting in 5.2005]

### Definition of Correlation Channel Matrices

In the correlation matrix approach, the channel from any of the N transmit antennas to the M receive antenna elements is generated from M independent channels from that transmit antenna to the M receive antennas. That is, for any given channel tap, we will have

Where is the channel vector from the i-th transmitting antenna to the M receive antennas, is the underlying  independent Gaussian channel vector  (i.e. it is the channel vector from the i-th transmitting antenna to the M receive antennas if the receive antennas were uncorrelated), and is the square root of the channel receive correlation matrix. Please note that the dimensions of , , and  are , , and  , respectively.  In addition, we note that each ITU channel profile defines a number of taps with a corresponding tap delay and average tap power.  The above description for the channel vector  is repeated for each channel tap. Moreover, please note that the underlying independent Gaussian channel vector  is completely different (i.e. independent) for each tap. Note that, when there is only one transmit antenna and one receive antenna,  is simply 1 and the above reduces to the scalar ITU channel model.

In a similar fashion, let us now consider the channel from the N transmit antennas to any of the M receive antennas.  The channel row-vector corresponding to the channels from all the N transmit antennas to the j-th receive antenna is related to the underlying independent Gaussian channel row-vector (i.e the channel row-vector from all N-transmit antennas to the j-th receive antenna if the transmit antennas were uncorrelated) by

- where   is the square root of the transmit array correlation matrix. We note that the dimensions of , , and are , , and   respectively . The special case N=1 and M=1 corresponds to the known model as used by ITU specification, when the channel component of  is generated by Jack’s model. For details see Appendix A2.

- There are various ways of obtaining the correlation matrix; two of them are explained in the appendix A.  When correlation matrix is used in the channel description, the proponent should provide the value of the correlation matrix used in their simulation as well as demonstration of the consistence between the values of the matrix and the method used to generate the claimed values.

….

Appendix A:

A.1.  Generating Correlation Matrix  using  Spatial Channel Model

The procedure for generating MIMO correlation matrix coefficients is shown in Figure 3.5.2-1.

Figure 3.5.2-1 Correlation Channel Modeling Procedure

The procedure is divided into two major phases.  In the first phase, a correlation matrix is generated for each mobile station (MS) and base station (BS) based on the number of antennas, antenna spacing, number of clusters, power azimuth spectrum (PAS), azimuth spread (AS), and angle of arrival (AoA).  These two correlation matrices are combined to create a spatial correlation matrix using the Kronecker product.  In the second phase, a correlated signal matrix is created using fading signals derived from various Doppler spectra and power delay profiles, and a symmetrical mapping matrix based on the spatial correlation matrix. Some of the parameters that can be used in the correlation channel model are shown in Table 3.5.2-1

A.2.  Generating Correlation  Matrix using the Jack’s Model

[to be inserted in word format]

5. Reference

[1] Channel Models for IEEE 802.20 MBWA System Simulations – Rev 08r1

[2] C802.20-05-09.pdf

[3] C802.20-05-32r1.pdf