| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for the Section on VoIP in the Evaluation Criteria Document |  |
| Date Submitted | 2005-07-17 |  |
| Source(s) | David Huo 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 428 7787 Fax: +1 973 386 4555 Email: dhuo@lucent.com |
| Re: | Evaluation Criteria |  |
| Abstract | This document proposes text for VoIP section in the evaluation criteria document. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group to accomplish the simulator calibration process. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Introduction

Document [1] pointed out  a possible way for the evaluation of VoIP via system level simulation. Following the meeting minute, this contribution proposes the text to be included in to the corresponding section in [2].

- Proposed Text

----------------------------------------------------------------------

### Voice (VoIP)

The voice traffic model will be implemented as voice over IP (VoIP).   The objective of the evaluation of VoIP is two fold:

Objective 1: The service availability and continuity

Objective 2: The speech quality during a given connection.

The  objective 1 deals with the statistic evaluation of the outcome from system level simulation which indicate whether and to which extent the voice service can be provided in the given network, e.g. the outage of the service for given number of users and the number of users for given outage. The objective 2 deals with testing the voice quality as experienced by individual users. Considering that both objectives depend on the vocoder used by the receiver,  AMR will be used as the default vocoder for the purpose of evaluation to allow comparison between proposals.  Moreover, an agreed-upon set of voice samples should be used for  the tests regarding both objectives.

Given a common vocoder and a speech sample, the following steps are required to achieve the two objectives, respectively:

For objective 1:  Derive the frame occupancy of the data stream, which would result in a file containing the time sequence of the occurance of each consecutive speech frame during a layer 2 packet stream. The system level simulation is then performed using this file.  In order to simplify the simulation and achieve the worst case evaluation, full buffer users could be assumed for the interferers during the system level simulation.

For objective 2: Perform the MOS test for the given speech sample and vocoder, using the error pattern file generated by the layer 2 outcome of the system level simulation.

3. Reference:

[1] IEEE C802.20-05/29

[2] 802.20 Evaluation Criteria V16.