| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for the Section on System-level Simulator Calibration in the Evaluation Criteria Document |  |
| Date Submitted | July 17, 2005 Based on 2005-3-14 (r5); 2005-3-8 (r4); 2005-2-22 (r3b) ; 2005-2-8 (r3a); 2005-1-9 (r2); 2004-11-9 (r1) |  |
| Source(s) | David Huo 67 Whippany Rd, Rm2A-384 Whippany, NJ 07981 | Voice : 973-428-7787 Email: dhuo@lucent.com |
|  | Arak Sutivong 5775 Morehouse Dr., San Diego, CA 92122 | Voice: (858) 651-6957 Fax: Email: araks@qualcomm.com |
| Re: | Contribution to IEEE 802.20 Evaluation |  |
| Abstract | This contribution proposes a possible approach for calibration of system-level simulation in the evaluation process of proposals for IEEE 802.20. |  |
| Purpose | For discussion & adoption into 802.20 evaluation criteria document. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Introduction

In the latest version of the IEEE 802.20 Evaluation Criteria Document [1], the section on calibration is still open. It is the objective of this contribution to propose a method in which simulators used by various technology proponents can be calibrated at the system level to enable a fair comparison of simulation results in the future. The calibration procedures/metrics specified in this contribution are intended to be as technology-independent as possible. As indicated in the Evaluation criteria document, calibration should be part of phase 1 of the evaluation process. The following text is proposed for adoption into section 8 “System Simulation Calibration".

Proposed Text

---------------------------------------------------------------------------------------------------

# System Simulation Calibration

The purpose of system-level calibration is to ensure that, under a set of common assumptions and models, the simulator platforms that will be used by various proponents can produce results that are reasonable and close to each other, so that confidence can be drawn with regard to the principle and coding of each platforms. The calibration procedures and metrics specified in this section are intended to be as technology-independent as possible.

Assumptions on the parameters:

The link budget in section 11 and the channel models adopted for phase 1 evaluation should be used in the calibration.

Assumption on the transmit power:

For  the down-link, each base station sector transmits at the maximum transmitter power to each of the users.

For the up-link, the user with the highest path and shadowing losses is required  to transmit at the maximum power, so that the corresponding long-term average received power at the base station can be  used as the target long-term average received power for all users in the same sector. That is, let PR be the target average received power. The transmit power of user k, PT,k, is given by PT,k = PR/Gk, where Gk is a long-term average channel gain (i.e., excluding short-term fading) of user k. This rule corresponds to an assumption of perfect power control.

- Step 1:  No Random Number

The purpose of deterministic calibration is to assure that the basic configuration and layout of the simulation environment is coded in accordance with the evaluation criteria. The configuration of the simulation scenario is characterized by the following parameters:

- Base Station (BS) to BS distance: 2.5 km

- Frequency reuse 1 is assumed

- Path loss model as specified in the channel model document for suburban macro, urban micro cells, etc

- A single antenna for BS and for MS, respectively.

- Mobiles are put in deterministic locations in each sector. For instance, 3 mobiles in each sector, located at (-60, R/2), (0,R/2) and (60, R), respectively, where (theta, r) refers to the polar coordinate system of the sector with the reference direction (theta=0) being the antenna main lobe direction and the maximum radius of the cell being (r=R).

Results of forward-link C/I, reverse-link C/I, and distance to each sector for each mobile are recorded in a spreadsheet, for which a common definition of numbering scheme need be set. Such a number scheme could be, for example: a=index of the sector (0,1,2…,56) , b=index for location within the sector (0,1,2 in counter clock-wise, in case of 3 mobiles per sector).

A forward-link carrier-to-interference ratio for mobile i, whose serving-sector is s(i) is defined asCIF,i=PT,BGi,s(i)m=0,m≠s(i)56PT,BGi,m+N, where PT,B is the (per-sector) base station maximum transmit power, Gi,m is the long-term channel gain (where the long-term channel gain is defined as the average path gain between the mobile and the base station, excluding the short-term fading) between mobile i and sector m, and N is the thermal noise power measured at the mobile. Note that this calculation assumes that each base station transmits at the maximum power using the entire bandwidth to a single user per sector at any given time. Define a reverse-link carrier-to-interference ratio for mobile i, whose serving-sector is s(i) as CIR,i=PT,iGi,s(i)m=0,m≠iMPT,mGm,s(i)+N, where PT,m is mobile m’s transmit power, Gm,s(i) is the long-term channel gain between mobile m and sector s(i) (i.e., mobile i’s serving sector), M is the total number of mobiles in the system, and N is the thermal noise power measured at the base station. Note that this calculation assumes that each user transmits using the entire bandwidth and entire time.

  - Step 2:  With Random Numbers

After the deterministic calibration succeeds, the second step is to perform simple stochastic calibration. The random parameters will be added incrementally into the parameters assumed in 6.1.2.1:

- Lognormal shadowing standard deviation as specified in the channel models document; the actual fading value is generated at the beginning of the simulation and remain constant during the simulation.

- At first 10 users/sector at fixed locations as specified in Appendix C. Then, 30 users/sector uniformly distributed over each sector of the 19 3-sectored cells (assuming the wrap-around model specified in Appendix A), where area within the minimum distance around the base station, as required by the path loss model used, is prohibited. The locations of the mobiles are fixed during the simulation.

- Each user selects the sector with the best downlink long-term channel gain as the serving sector. Note that in doing so, the eventual number of users per sector may be different across different sectors.

- Full-buffer traffic model, i.e. time static evaluation.

- Perfect slow power control on the uplink with the same target long-term average received power for all users.

- Simulation duration : 10 seconds

- Time interval for fast fading channel update & (C/I) data collection: 40 ms for MS speed ~3 km/h, 1 ms for MS speed ~120 km/h.

As output, the cumulative distribution function (CDF) and corresponding mean and standard deviation of user C/I, for users in all sectors and cells, and for both Downlink and Uplink, shall be provided for comparison. Separate CDF plots shall be provided for the results of fixed users case and the randomly distributed users case.

In this section, a forward-link carrier-to-interference ratio for mobile i, whose serving-sector is s(i) is defined asCIF,i=PT,BGi,s(i)m=0,m≠s(i)56PT,BGi,m+N, where PT,B is the (per-sector) base station maximum transmit power, Gi,m is the channel gain (including short-term fading) between mobile i and sector m, and N is the thermal noise power measured at the mobile. Note that this calculation assumes that each base station transmits at the maximum power using the entire bandwidth to a single user per sector at any given time. Define a reverse-link carrier-to-interference ratio for mobile i, whose serving-sector is s(i) as CIR,i=PT,iGi,s(i)m=0,m≠iMPT,mGm,s(i)+N, where PT,m is mobile m’s transmit power, Gm,s(i) is the channel gain (including the short-term fading) between mobile m and sector s(i) (i.e., mobile i’s serving sector), M is the total number of mobiles in the system, and N is the thermal noise power measured at the base station. Note that this calculation assumes that each user transmits using the entire bandwidth and entire time.

    - Outputs

The simulation results from the calibration should include the following:

- Cumulative distribution (CDF) plots and corresponding mean and standard deviation of user C/I, for both Downlink and Uplink, at time = 0 (i.e., excluding short-term fading).

- Cumulative distribution plots and corresponding mean and standard deviation of user’s distance from the serving sector. Due to log-normal shadowing, a user may end up being served by a sector that is not geographically closest to it.

- Spatial distribution of user C/I, i.e., a plot of C/I distribution as a function of users’ location in the 19 cells

- Cumulative distribution plots and corresponding mean and standard deviation of user C/I for both Downlink and Uplink, at the end of the simulation, including all the C/I samples collected over the simulation duration.

- Relative uplink inter-sector over intra-sector received power (f). Definef=IocIor , where Ior is the average received power from intra-sector users and Ioc is the average received power from users outside the sector. The contribution from thermal noise is excluded from the calculation. All users are given the same system resources on the uplink.

- As the evaluation is an evolving process, new quantities may be identified during the process as useful and effective for the characterization and comparison. New metrics identified as such should be included into the calibration to facilitate the process.

      - Further Issues

The traffic models should be calibrated by plotting and comparing the CDF of packet sizes, inter-packet arrival times and related traffic model parameters, before the start of evaluation based on those models.

Depending on the stage within the process of technology evaluation and selection, further calibration may be needed, e.g., calibration using a common set of link-level performance curves [2].

## Passing Criteria

For deterministic calibration, no deviation should be allowed in order to qualify for a correct result. For the non-deterministic calibration, the results produced by various simulators should be discussed by all proponents, and possibly repeated, until consensus is achieved.

Appendix C. 	Fixed user locations for system level calibration

References

- “IEEE 802.20 Evaluation Criteria Document Version 11r2”, Oct 24, 2004.

- 3GPP2/TSG-C.R1002, “1xEV-DV Evaluation Methodology (V14)”, June 2003.

- Channel Models for IEEE 802.20 MBWA System Simulations-Rev. 06, Oct 26, 2004.

Short-term fading is excluded from the calculation of the long-term average received power.

It follows that the received power at the base station is given by PT,k x Gk = PR/Gk x Gk = PR as desired.

MIMO calibration procedure is beyond the scope of this document.

Results of deterministic calibration between the simulators should be similar.