| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for a Method of generating Spatial Correlation Coefficients for MIMO Channel Modeling for MBWA |  |
| Date Submitted | 2005-07-20 |  |
| Source(s) | Radhakrishna Canchi Deepshikha Garg 2480 N. First Street #280 San Jose, CA 95131 Murakami Kazuhiro, Kitahara Minako 2-1-1 Kagahara, Tsuzuki-ku, Yokohama, KANAGAWA 224-8502, JAPAN Daniel Garcia-Alis DEPARTMENT OF EEE, University of Strathclyde, Glasgow G1 1XW, Scotland, UK | Voice: +1-408-952-4701 Fax: +1-408-954-8709 Email: cradhak@ktrc-na.com Voice: +81 45 943 6102 Fax: +81 45 943 6175 Email: kazuhiro_murakami@csg.kyocera.co.jp Voice: +44 141 548 2679 Email: dgarcia@eee.strath.ac.uk |
| Re: | MBWA Call for Contributions for Channel Model Document |  |
| Abstract | This document proposes text for “A Method of generating Spatial Correlation Coefficients for MIMO Channel Modeling for MBWA” in “ChannelModelsDocC80220-04-66r3_Version9r1.doc”: (Ref: Contributions IEEE C802.20-05/37 and IEEE C802.20-05/38) |  |
| Purpose | This document addresses the open issue of Spatial Correlation Coefficient generation in the IEEE802.20 Channel Model Document Version 09 (C802.20-04-66r3_Version9) for discussion and adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# A Method of Generating Spatial Correlation Coefficients for  MIMO Channels Generation of Spatial Correlation Coefficients for  MIMO Channels

Proposed Text in “ChannelModelsDocC80220-04-66r3_Version9r1.doc”:

### Generation of Spatial Correlation Coefficients [IEEE  C802.20-05/37 & IEEE  C802.20-05/38]

Performance of MIMO systems in realistic radio environments greatly depends on the spatial correlation due to the presence of scatterers in the propagation environments. In order to predict the performance of MBWA system in real environments by simulation, we develop the correlated MIMO fading channel model by adopting the available stochastic spatial parameters that have been identified as the best fit to the propagation measurements. Cross correlation between radio waves arriving at two different antenna elements is a function of PAS (Power Azimuth Spectrum), radiation pattern of antenna and antenna element spacing.

We assume mutipath signals are represented by channel taps each of which is represented a cluster of scatterers. MIMO channel matrix generated by on a tap by tap basis can be written as

Hi=Rr12HiRt12                                                    (19)

where Hi is an N×M  matrix of  iid complex Gaussian random variables with zero mean, unit variance. Rrand Rtare the correlation matrices at the receiver and transmitter.

Received signal at the mth array element at time t can be written as

rm(t)=s(t)1Pi=1Pgi(t)ejDmsin(φ0−φi)                                                (20)

where P is the number of sub-paths per channel for a given channel tap, s(t) is the complex envelope, g(t) the random fading coefficient for this sub-path, and D=2πdλnormalized distance between array elements.

Assuming the angles of arrival (AoA)s to be independent across different sub-paths, cross correlation between the mth and nth array element can be evaluated as

Rm,n=Ermrn=  Rm,n=EφejD(m−n)sin(φ0−φ)                                      (21)

Assuming antenna pattern of unity,

Rm,n=−π+πejD(m−n)sin(φ0−φ)Pφdφ                                           (22)

Stochastical Modeling of Experimental Test bed measurements of outdoor propagation environments concluded that Lapacain function accurately describes the estimated PAS [28]. We define truncated Laplacian distribution  of PAS is defined [31] as under:.

Pφ=c2σΑe−2φ−φ0σΑ−Δ+φ0φ≤Δ+φ0	                                 (23)

c is a normalization constant which guarantees that the integration Pφ over the range [−Δ+φ0,Δ+φ0]is unity.

An exact expression for Spatial Correlation coefficients is given [28] as sum of Bessel functions of the first kind,

ℜ[R(φ0,σφ)]m,n=J0Dm−n+2r=1∞J2rDm−n2σφ22+2r2cos2rφ02σφ2+exp−Δ2σφ22rsin2rΔ−2σφ2cos2rΔℑ[R(φ0,σφ)]m,n=2r=1∞J2r+1Dm−n2σφ22+2r+12sin2r+1φ02σφ2−exp−Δ2σφ22r+1sin2r+1Δ+2σφ2cos2r+1Δ(24)

Where φ0,Δ and σφ are the parameters for Laplacian distributed PAS as defined in (23).

In the above equation, φ represents path components (sub-rays) of the path power arriving at an AoA φ0.

When Δ is very small, Laplacian distribution is defined over the range (-π,π), spatial correlation between mth and nth antenna element at the same antenna array is approximated as [29,30],

[R(φ0,σφ)]m,n≈cejD(m−n)sin(φ0)[B(φ0,σφ)]m,n                                              (25)

where

[B(φ0,σφ)]m,n=11+σφ22[D(m−n)cosφ0]2

References

[28] Schumacher, L.; Pedersen, K.I.; Mogensen, P.E.; “From antenna spacings to theoretical capacities - guidelines for simulating MIMO systems”, The 13th IEEE International Symposium on Personal, Indoor and Mobile Radio Communications, 2002. Volume 2, 15-18 Sept. 2002 Page(s):587 - 592 vol.2

[29] A.Forenza, D.J.Love, and R.W.Heath Jr., “A low complexity algorithm to simulate the Spatial Covariance Matrix for clustered MIMO Channel Models, IEEE Vehicular Technology Conference, 2004.

[30] A.Forenza, D.J.Love, and R.W.Heath Jr., “Simulation of the Spatial Covariance Matrix”, doc.: IEEE 802. 11-03/925r0, 802.11 TGnChannel Model Special Committee,Nov.2003

[31] R.Canchi et.al., “PAS (Power Azimuth Spectrum) Model for Channel Model for MBWA”, IEEE C802.20-05/37

[32] R.Canchi et.al., Spatial Correlation Coefficients Generation for  MIMO Channel Modeling  for MBWA IEEE  C802.20-05/38