| `Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | BEST-WINE Technology Performance and Evaluation Crtieria Report 1 |  |
| Date Submitted | 2005-OCT-28 |  |
| Authors(s) | Radhakrishna Canchi 2480 N. First Street, Suite#280 San Jose, CA 95131 Kazuhiro Murakami 2-1-1 Kagahara, Tsuzuki-ku, Yokohama, KANAGAWA 224-8502, JAPAN Minako Kithara 2-1-1 Kagahara, Tsuzuki-ku, Yokohama, KANAGAWA 224-8502, JAPAN | Voice: +1-408-952-4701 Fax: +1-408-954-8709 Email: cradhak@ktrc-na.com Voice: +81 45 943 6130 Fax: +81 45 943 6175 Email: kazuhiro.murakami.xm@kyocerajp Voice: +81 45 943 6102 Fax: +81 45 943 6175 Email: minako.kitahara.nf@kyocera.jp |
| Re: | MBWA Call for Proposal |  |
| Abstract | This document presents the Technology Performance and Evaluation Crtieria Report 1 of the Tehcnlogy Proposal BEST-WINE for IEEE 802. 20 MBWA |  |
| Purpose | To disucus and Adopt BEST-WINE for Draft Specfications of IEEE802.20 MBWA |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

BEST-WINE

(Broadband MobilE SpaTial Wireless InterNet AcEess)
A Complete and Fully Compliant TDD Technology Proposal for MBWA

Technology Performance

and

Evaluation  Crtieria Report 1

# Executive Summary

This document BEST-WINE Technology Performance and Evaluation  Crtieria Report 1 reports  the performance of the BEST-WINE based on the evaluation methodology defined in IEEE802.20 Evalaution Criterai Doccument [5]. The Channel Models of Channel Model Document [4] were used.

Table of Contents

1	Executive Summary	3

2	References	8

3	Definitions	9

4	Abbreviations and acronyms	10

5	Introduction	12

5.1	Purpose of This Report	12

5.2	Key Technologies	12

5.3	System Model	12

5.3.1	Cell lay out	12

5.3.2	Base station  & Mobile station (User Terminals)	14

5.4	RF parameters	14

5.5	BEST-WINE System’s PHY and MAC Layer information	15

5.5.1	BEST-WINE System Basic PHY/MAC layer parametes Channel structure	15

5.5.2	BEST-WINE Modulation classes	16

5.6	Link Budget	17

5.6.1	Suburban Macro Test Environmnet – Pedestrain B (3KMPH)	17

5.6.2	Suburban Macro at 120 KMPH	18

6	Simulation environment	20

6.1	Link – System Interface	20

6.2	Link level simulation	21

6.2.1	Link Level simulation Parameters	21

6.2.2	Channel models used in Link Level Simulations	21

6.3	System level simulation environment	23

6.3.1	System level simulation target feature	23

6.3.2	System level simulation parameters	23

6.3.3	System level simulation channel model	23

6.3.4	Traffic Model System Level Simulation	24

7	Simulation Reuslts	24

7.1	Link level simulation	24

7.1.1	Uplink (VehicularB model) FER vs SINR Performance	24

7.1.2	Downlink  (Vehicular B model) FER vs SINR Performance	25

7.1.3	Uplink (PedestrianB model) FER vs SINR Performance	26

7.1.4	Downlink (PedestrianB model) FER vs SINR Performance	27

7.1.5	Uplink (Vehicular B model) Throughput vs SINR	28

7.1.6	Downlink( Vehicular B model) Throughput vs SINR	29

7.1.7	Uplink (Pedestrian B model) Throughput vs SINR	30

7.1.8	Downlink (Pedestrian B model) Throughput vs SINR	31

7.2	System level simulation calibration result	31

7.2.1	System level simulation calibration condition and parameters	31

7.3	System Level Simulation Results	32

7.3.1	User Date Rate CDF	32

7.3.2	120km/h　 User Date Rate CDF	32

7.3.3	Aggregated Throughput vs Base station Sepparation	34

7.4	Fairness criteria	37

8	Simulations Results Summary	39

9	Practical System Results	39

List of Figures

Figure 5-1 Cell definition in the Cartesian Coordination System and the Numbering of Cells	13

Figure 5-2 Base station Antenna pattern	13

Figure 5-3 UT positions	14

Figure 5-4 Channel Configuration in a Block Assignement of 2.5 MHz	16

Figure 6-1 Link level simulation channel models and Asscoiated Spatial Parameters	21

Figure 7-1 Uplink 120kmph FER vs received SINR	24

Figure 7-2 Downlink 120kmph FER vs received SINR	25

Figure 7-3 Uplink 3kmph FER vs received SINR	26

Figure 7-4 Downlink 3kmph FER vs received SINR	27

Figure 7-5 Uplink 120kmph Throughput vs received SINR	28

Figure 7-6  Downlink 120kmph Throughput vs received SINR	29

Figure 7-7 Uplink 3kmph Throughput vs received SINR	30

Figure 7-8 Downlink 3kmph Throughput vs received SINR	31

Figure 7-9 User Terminal Location Map for Deterministic calibration	32

Figure 7-10 120kmph  User Date Rate CDF	33

Figure 7-11  3km/h CDF of throughput vs SINR	34

Figure 7-12 Contours of constant Minimum Servcie Level at 3Kmph -Downlink	35

Figure 7-13 Contours of constant Minimum Servcie Level at 120Kmph -Downlink	35

Figure 7-14 Contours of constant Minimum Servcie Level at 3Kmph -Uplink	36

Figure 7-15 Contours of constant Minimum Servcie Level at 120Kmph -Uplink	37

Figure 9-1 25m height Tower,  Sydney MAP and Userterminal	40

Figure 7-13: Downlink Date Rate Results	41

Figure 7-14: Date Rate and Spectrum Efficency Test Results of HC-SDMA in Australia	41

List of Tables

Table 5-1 RF parameters	15

Table 5-2 BEST-WINE PHY/MAC Basic Parameters	16

Table 5-3 ModClasses and Maximum Data Rates	17

Table 5-4 Link Budget table (3km/h)	18

Table 5-5 Link Budget table (3km/h)	20

Table 6-1Sub-path Spatial parameters AoD and AoA offset	22

Table 6-2 Suburban Macro Environment Parameters	24

Table 7-9  120kmph  User Date Rate CDF	33

Table 7-10 3km/h CDF of throughput vs SINR	34

Table 8-1　Suburban  Pedestrian B Case	37

Table 8-2　 Suburban  Vehicular B Case	37

# References

- ATIS-PP-0700004*-2005, High Capacity-Spatial Division Multiple Access (HC-SDMA)

  - *The copyright of this document is owned by the Alliance for Telecommunications Industry Solutions. Any request to reproduce this document, or portion thereof, shall be directed to ATIS, 1200 G Street, NW, Suite 500, Washington, DC 20005.

  - *For Electornic Downloads, Paper Copy or CD-ROM please follow the link https://www.atis.org/atis/docstore/doc_display.asp?ID=3617

- IEEE 802.20 PD-2.doc: Mobile Broadband Wireless Access Systems: Approved PAR (02/12/11):

- IEEE 802.20 PD-06r1.doc: IEEE 802.20 System Requirement Document (V 1.0)

- IEEE_802.20-PD-08.doc: IEEE 802.20 Channel Models (V 1.0)

- IEEE_802.20-PD-09.doc: IEEE 802.20 Evaluation Criteria (V 1.0)

- IEEE_802.20-PD-10.doc:  IEEE 802.20 Techology Selection Process (V 1.0)

- X.P0011-001-D on 3gpp2 TSG-X specification

# Definitions

As defined in the References [1],[2],[3]

# Abbreviations and acronyms

| AA | Access Assignment |
| --- | --- |
| AAA | Adaptive Antenna Array |
| ACLPR | Adjacent Channel Leakage Power Ratio |
| ACS | Adjacent Channel Selectivity |
| AM | Acknowledged Mode |
| API | Application Programming Interface |
| ARQ | Automatic Repeat Request |
| BCH | Broadcast Channel |
| BS | Base Station |
| BSCC | Base Station Color Code |
| CA | Certificate Authority |
| CCH | Configuration Channel |
| CM | Configuration Message |
| CoS | Class of Service |
| CR | Configuration Request |
| CRC | Cyclic Redundancy Check |
| EUD | End User Device |
| FACCH | Fast Asscoiated Control Channel |
| FEC | Forward Error Control |
| FER | Frame Error Rate |
| GPS | Global Positioning System |
| HC-SDMA | High Capacity Spatial Division Multiple Access |
| i-HAP | Handshake and Authentication Protocol |
| IMSI | International Mobile Station Identifier |
| IPPR | Intermodulation Product Power Ratio |
| i-SEC | Secure Communications Protocol |
| i-TAP | Terminal Authentication Protocol |
| IWAN | Interconnection Wide Area Network |
| L2 | Layer 2 |
| L2TP | Layer 2 Tunneling Protocol |
| L3 | Layer 3 |
| L3 | CM L3 Connection Management |
| L3 | MMC L3 Mobility Management and Control |
| L3 | RM L3 Registration Management |
| LLC | Logical Link Control |
| LDAP | Lightweight Directory Access Protocol |
| LFSR | Linear Feedback Shift Register |
| LNA | Low Noise Amplifier |
| LNS | L2TP Network Server |
| LSB | Least Significant Bit |
| MAC | Medium Access Control |
| MBWA | Mobile Broadband Wireless Access |
| MSB | Most Significant Bit |
| PA | Power Amplifier |
| PAR | Project Authorization Requirments |
| PCH | Paging Channel |
| PDCL | Packet Data Conversion Layer |
| PHY | Physical Layer |
| PID | Paging Identity |
| PPM | Parts Per Million |
| PPP | Point to Point Protocol |
| PPPoE | PPP over Ethernet |
| PSS | Packet Services Switch |
| QoS | Quality of Service |
| RA | Request Access |
| RACH | Random Access Channel |
| RSA | Rivest, Shamir, Adleman |
| RF | Radio Frequency |
| RLC | Radio Link Control |
| RM | Registration Management |
| RMU | RLC Message Unit |
| RRC | Radio Resource Control or Root Raised Cosine |
| RSSI | Received Signal Strength Indicator |
| SDMA | Space Division Multiple Access |
| SDU | Service Data Unit |
| SINR | Signal-to-Interference plus Noise Ratio |
| SN | Slot Number |
| SNR | Signal to Noise Ratio |
| TCH | Traffic Channel |
| TDD | Time Division Duplex |
| TDMA | Time Division Multiple Access |
| TWAN | Transport Wide Area Network |
| UM | Unacknowledged Mode |
| UT | User Terminal |

# Introduction

By this document, Kyocera Team respectfully submits Technolgy Performance and Evaluation Report 1 for th Proposed  TDD Technology tilted BEST-WINE (Broadband MobilE SpaTial Wireless InterNet AcEess), which is an enhanced Air Interface based on “ATIS-PP-0700004-2005, High Capacity-Spatial Division Multiple Access (HC-SDMA), (Pre Published American National Standard)”[1].

This Evaluation Report 1 Report presents the both the system and linke level Technolgy Performance results obtaine by the Simulations by following the methodologies sepcficied in Evaluation Criteria Document [5] while using Suburban Macro Channel Model and related spatial parameters as defined in Channel Model Document [4].  Both the link level and System Level performaces are obtained at the User Terminal Mobility Speeds of 3Kmph and 120Kmph. Also this report provides Linkbudget calculation for each o

## Purpose of This Report

This Evalaution Report 1 serves as the basis for comparing with other technology Proposals and

## Key Technologies

Key technologies of the BEST-WINE system are

- Adaptive Anetenna Array Processing

- Spatial Division Multiple Access

- Link Adaptation With Modulation & Coding Classes

## System Model

### Cell lay out

19BS / 3sector

The systel layout consists of 19 Cell sites with each Cell split in 3 Sectors as shown in Figure 5-1. Inter BS separation is 1.73 Km and the Cell Radius is 1Km.

Each cell is divided into 3 sectors, characterized by the antenna direction of each sector. The number of sector is counter-clock wise with 0, 1 and 2, respectively, where the respective antenna direction is 0:  theta=0 degree, 1: theta=120 degree,

3: theta=240 degree, where theta is the local polar angle of the cell. By this convention, the first sector of the center cell has the index   (0, 0), while the last sector has the index (18,2).  Mobiles are uniformly dropped in each sector, where an area around the cell center with radius 35 meters are excluded for mobiles. The unit of distance is meter.

Figure 5-1 Cell definition in the Cartesian Coordination System and the Numbering of Cells

Each base station in each sector employs Antennas that has radiation patter as shown in Figure 5-2.

Figure 5-2 Base station Antenna pattern

### Base station  & Mobile station (User Terminals)

Inter BS separation is 1.73 Km.

Cell Radius is 1Km.

Load user number:  58users

User terminal: Rx 4antenna or 1antenna　/ Tx 1antenna

BS: Rx & Tx antenna elements 12antenna

Antenna separation 10λ

UTs are placed uniform distribution in the 19BS area.

UT’s have two states: Conneced State and Not connected state.

Figure 5-3 UT positions

## RF parameters

Table 1. RF parameter table

| # | RF Parameter TDD System BW | Base Value 1 MHz Channel | 2.5 MHz TDD - SYSTEM |
| --- | --- | --- | --- |
| 1 | Transmitter Power -- BS | 43 dBm/MHz | +44 dBm |
| 2 | Transmitter Power -- MS | 27 dBm | +27 dBm |
| 3 | Out of Band emission limits – BS and MS (emission measured in 1 MHz resolution bandwidth) | Attenuation of the transmit power P by: 43 +10 log(P) dB | -13 dBm |
| 4* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) – BS | 43 dB | 50.2 dB |
| 5* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) – MS | 36.8 dB | 35.4 dB |
| 6 | Receiver noise figure -- BS | 5 dB | 5 dB |
| 7 | Receiver noise figure -- MS | 10 dB | 10 dB |
| 8 | Receiver reference sensitivity (to be proposed by each technology) | Specify at BER of 0.1% | See linkbudget |
| 9* | Receiver Selectivity -- BS | Mod 1 /Mod 7 : 30 dB | 30 dB |
| 10* | Receiver Selectivity -- MS | Mod 1 : 30 dB Mod 8 : 27 dB | 30 dB 27 dB |
| # | Frequency offset for Blocking | 4MHz off | 5MHz off |
| 11[*] | Receiver Blocking – BS (level of same technology blocking signal at frequency offset of 2 times Channel BW) | Mod 1 : -49.6 dBm Mod 7 : -35.6 dBm | -49.6 dBm -35.6 dBm |
| 12[*] | Receiver Blocking – MS (level of same technology blocking signal at frequency offset of 2 times Channel BW) | Mod 1 : -57.5 dBm Mod 8 : - 41.6 dBm | -57.5dBm -41.6dBm |

Table 5-1 RF parameters

## BEST-WINE System’s PHY and MAC Layer information

### BEST-WINE System Basic PHY/MAC layer parametes Channel structure

| Items |  | Specification |
| --- | --- | --- |
| Duplexing |  | TDD |
| Mutiple Access |  | TDMA ・SDMA |
| Single Carrier Bandwidth |  | 625 kHz |
| Frame Lenght |  | 5 ms |
| symbol duration |  | 2usec |
| Uplink Time Slot | slots | 3 |
|  | Length | 545 us |
|  | Payload | 182 symbols |
| Downlink Time Slot | slots | 3 |
|  | Length | 1090 us |
|  | Payload | 494 symbols |
| Symbol rate |  | 500 ksps |

Table 5-2 BEST-WINE PHY/MAC Basic Parameters

For BEST-WINE TDD system Block Assignment Size of 2.5MHz has been assumed. So, this block assignment accomdates 4 Single Carriers of 625KHz width and its channel configuration is shown below.

2.5MHz

frequency

625kHz

Figure 5-4 Channel Configuration in a Block Assignement of 2.5 MHz

BEST-WINE System employs Adaptive Antenna Array Processing using MMSE Algorithm. Antenna Array Weights calculated for Uplink will be used for Downlink transmissions that follow.

### BEST-WINE Modulation classes

BEST-WINE system Suppports 10 Modulation Clasess, refereed as ModClass, which defines Modulation and Coding Scheme Combination. There are 10 ModClasses as mentioned in the following Table. The tiered Multilevel Modulation schemes facilitate Link Adaptation. There are 7 Modulation Schemse: BPSK,QPSK,8-PSK,12QAM,16QAM,24 QAM and 64 QAM. The moaximu data rates associated with these ModClasses are also defined in the Table

Down link 	: 1473Kbps x 4 Carrier aggregation = 5.89 Mbps(Max)

Up Link 	:   566Kbps x 4 Carrier aggregation = 2.26 Mbps(Max)

*1　24QAM+ perform error coding which has 3/4 coding rate .

*2    64QAM   perform error coding which has 5/6 coding rate.

Table 5-3 ModClasses and Maximum Data Rates

## Link Budget

### Suburban Macro Test Environmnet – Pedestrain B (3KMPH)

| id/ii | Item | Downlink | Uplink |
| --- | --- | --- | --- |
|  | Test environment | Suburban macro-cell | Suburban macro-cell |
|  | Operating frequency | 1.9GHz | 1.9GHz |
|  | Test service | full buffer | full buffer |
|  | Multipath channel class | Case III(Pedestrian B) | Cases III(Pedestrian B) |
| ii/id | (a0) Average transmitter power per traffic channel | 1 slot Tx : 32.3 dBm | 1 slot Tx : 17.1 dBm |
|  |  | 2 slot Tx : 35.3 dBm | 2 slot Tx : 20.1 dBm |
|  |  | 3 slot Tx : 37.0 dBm | 3 slot Tx : 21.9 dBm |
| Id | (a1) Maximum transmitter power per traffic channel dBm | 39 | 27 |
| Id | (a2) Maximum total transmitter power dBm/MHz | 43 | 27 |
| Ii | (b) Cable,connector,and combiner losses (enumerate sources) [dB] | 3 | 0 |
|  | Body Losses [dB] | 0 | 3 |
| Ii | (c) Transmitter antenna gain [dBi] | 17 | 0 |
| Id | (d1) Transmitter e.i.r.p. per traffic channel [ dBm] =a1-b+c | 53 | 27 |
| Id | (d2) Total transmitter e.i.r.p. =a2-b+c | 57 | 27 |
|  | Penetration Loss (Ref: 3GPP2) 10 dB (Vehicular) | 0 | 0 |
| Ii | (e) Receiver antenna gain [dBi] | 0 | 17 |
| Ii | (f) Cable and connector losses [dB] | 0 | 3 |
|  | Body Losses [dB] | 3 | 0 |
| Ii | (g) Receiver noise figure [ dB] | 10 | 5 |
| Ii | (h) Thermal noise density dBm/Hz | -174 | -174 |
|  | H) (linear unts) mW/Hz | 3.98 ´ 10–18 | 3.98 ´ 10–18 mW/Hz |
| Id | (i) Receiver interference density (NOTE 1) [dBm/Hz] | -120.9691001 | -120.9691001 |
|  | (I) (linear units) [mW/Hz] | 8E-13 | 8E-13 |
| Id | (j) Total effective noise plus interference density [dBm/z] | -120.968884 | 120.969018 |
|  | = 10 log (10((g + h)/10) + I) |  |  |
| Ii | (k) Information rate (10 log (Rb)) [dB(Hz)] |  |  |
|  | @mod0 | 53.98 | 53.98 |
|  | @mod1 | 55.25 | 55.25 |
|  | @mod2 | 56.99 | 56.99 |
|  | @mod3 | 58.75 | 58.75 |
|  | @mod4 | 60.00 | 60.00 |
|  | @mod5 | 60.97 | 60.97 |
|  | @mod6 | 61.76 | 61.76 |
|  | @mod7 | 62.43 | 62.43 |
|  | @mod8 | 63.01 |  |
| d | (l) Required Eb/(N0 + I0) at BER0.1% |  |  |
|  | @mod0 | 2.51 | 2.21 |
|  | @mod1 | 3.04 | 2.54 |
|  | @mod2 | 2.80 | 2.50 |
|  | @mod3 | 3.94 | 3.64 |
|  | @mod4 | 4.89 | 4.59 |
|  | @ mod5 | 6.12 | 5.92 |
|  | @ mod6 | 7.43 | 7.13 |
|  | @ mod7 | 8.06 | 7.76 |
|  | @mod8 | 9.38 |  |
| Id | (m) Receiver sensitivity = (j + k + l) @ BER0.1% |  |  |
|  | @mod0 | -121.47 | -121.77 |
|  | @mod1 | -119.67 | -120.17 |
|  | @mod2 | -118.17 | -118.47 |
|  | @mod3 | -115.27 | -115.57 |
|  | @mod4 | -113.07 | -113.37 |
|  | @mod5 | -110.87 | -111.07 |
|  | @mod6 | -108.77 | -109.07 |
|  | @mod7 | -107.47 | -107.77 |
|  | @mod8 | -105.57 |  |
| Id | (n) Hand-off gain[ dB] | 2 | 2 |
| Id | (o) Explicitdiversity gain [dB] | 6 | 0 |
| Id | (o¢) Other gain [dB] | 21.6 | 10.8 |
| Id | (p) Log-normal fade margin [dB] | 10.4 | 10.4 |
| Id | q) Maximumpatlos{d1 – m + (e – f) + o + n + o¢ – p} dB |  |  |
|  | @mod0 | 190.67 | 165.17 |
|  | @mod1 | 188.87 | 163.57 |
|  | @mod2 | 187.37 | 161.87 |
|  | @mod3 | 184.47 | 158.97 |
|  | @mod4 | 182.27 | 156.77 |
|  | @mod5 | 180.07 | 154.47 |
|  | @mod6 | 177.97 | 152.47 |
|  | @mod7 | 176.67 | 151.17 |
|  | @mod8 | 174.77 |  |
| Id | (r) Maximum range |  |  |
|  | @mod0 | 35292.50 | 6593.49 |
|  | @mod1 | 31351.18 | 5934.73 |
|  | @mod2 | 28405.12 | 5306.76 |
|  | @mod3 | 23471.43 | 4385.03 |
|  | @mod4 | 20308.71 | 3794.16 |
|  | @mod5 | 17572.16 | 3261.38 |
|  | @mod6 | 15304.72 | 2859.29 |
|  | @mod7 | 14050.20 | 2624.91 |
|  | @mod8 | 12399.29 |  |

Table 5-4 Link Budget table (3km/h)

### Suburban Macro at 120 KMPH

| Id/ii | Item | Downlink | Uplink |
| --- | --- | --- | --- |
|  | Test environment | Suburban macro-cell | Suburban macro-cell |
|  | Operating frequency | 1.9GHz | 1.9GHz |
|  | Test service | full buffer | full buffer |
|  | Multipath channel class | Case IV(Vehicular B) | Case IV(Vehicular B) |
| ii/id | (a0) Average transmitter power per traffic channel | 1 slot Tx : 32.3 dBm | 1 slot Tx : 17.1 dBm |
|  |  | 2 slot Tx : 35.3 dBm | 2 slot Tx : 20.1 dBm |
|  |  | 3 slot Tx : 37.0 dBm | 3 slot Tx : 21.9 dBm |
| id | (a1) Maximum transmitter power per traffic channel dBm | 39.0 dBm | 27 dBm |
| id | (a2) Maximum total transmitter power dBm/MHz | 43 | 27 |
| ii | (b) Cable,connector,and combiner losses (enumerate sources) [dB] | 3 | 0 |
|  | Body Losses [dB] | 0 | 3 |
| ii | (c) Transmitter antenna gain [dBi] | 17 | 0 |
| id | (d1) Transmitter e.i.r.p. per traffic channel [ dBm] =a1-b+c | 53 | 27 |
| id | (d2) Total transmitter e.i.r.p. =a2-b+c | 57 | 27 |
|  | Penetration Loss (Ref: 3GPP2) 10 dB (Vehicular) | 10 | 10 |
| ii | (e) Receiver antenna gain [dBi] | 0 | 17 |
| ii | (f) Cable and connector losses [dB] | 0 | 3 |
|  | Body Losses [dB] | 3 | 0 |
| ii | (g) Receiver noise figure [ dB] | 10 | 5 |
| ii | (h) Thermal noise density dBm/Hz | -174 | -174 |
|  | (H (linear unit) mW/Hz | 3.98 ´ 10–18 | 3.98 ´ 10–18 mW/Hz |
| id | (i) Receiver interference density (NOTE 1) [dBm/Hz] | -120.9691001 | -120.9691001 |
|  | (I) (linear units) [mW/Hz] | 8E-13 | 8E-13 |
| id | (j) Total effective noise plus interference density [dBm/Hz | -120.968884 | -20.969038 |
|  | = 10 log (10((g + h)/10) + I) |  |  |
| ii | (k) Information rate (10 log (Rb)) [dB(Hz)] |  |  |
|  | @mod0 | 53.98 | 53.98 |
|  | @mod1 | 55.25 | 55.25 |
|  | @mod2 | 56.99 | 56.99 |
|  | @mod3 | 58.75 | 58.75 |
|  | @mod4 | 60.00 | 60.00 |
|  | @mod5 | 60.97 | 60.97 |
|  | @mod6 | 61.76 | 61.76 |
|  | @mod7 | 62.43 | 62.43 |
|  | @mod8 | 63.01 |  |
| id | (l) Required Eb/(N0 + I0) |  |  |
|  | @mod0 | 2.51 | 2.21 |
|  | @mod1 | 3.04 | 2.54 |
|  | @mod2 | 2.80 | 2.50 |
|  | @mod3 | 3.94 | 3.64 |
|  | @mod4 | 4.89 | 4.59 |
|  | @ mod5 | 6.12 | 5.92 |
|  | @ mod6 | 7.43 | 7.13 |
|  | @ mod7 | 8.06 | 7.76 |
|  | @mod8 | 9.38 |  |
| Id | (m) Receiver sensitivity = (j + k + l) |  |  |
|  | @mod0 | -121.47 | -121.77 |
|  | @mod1 | -119.67 | -120.17 |
|  | @mod2 | -118.17 | -118.47 |
|  | @mod3 | -115.27 | -115.57 |
|  | @mod4 | -113.07 | -113.37 |
|  | @mod5 | -110.87 | -111.07 |
|  | @mod6 | -108.77 | -109.07 |
|  | @mod7 | -107.47 | -107.77 |
|  | @mod8 | -105.57 |  |
| id | (n) Hand-off gain[ dB] | 2 | 2 |
| Id | (o) Explicit divesity gain [dB] | 3.7 | 0 |
| Id | (o¢) Other gain [dB] | 18.7 | 10.8 |
| Id | (p) Log-normal fade margin [dB] | 10.4 | 1.4 |
| Id | q) Maximumpatrn loos {d1 – m + (e–f) + o + n + o¢ – p} dB |  |  |
|  |  @mod0 | 188.47 | 165.17 |
|  | @mod1 | 186.67 | 163.57 |
|  | @mod2 | 185.17 | 161.87 |
|  | @mod3 | 182.27 | 158.97 |
|  | @mod4 | 180.07 | 156.77 |
|  | @mod5 | 177.87 | 154.47 |
|  | @mod6 | 175.77 | 152.47 |
|  | @mod7 | 174.47 | 151.17 |
|  | @mod8 | 172.57 |  |
| id | (r) Maximum range |  |  |
|  | @mod0 | 30536.92 | 6593.49 |
|  | @mod1 | 27126.68 | 5934.73 |
|  | @mod2 | 24577.60 | 5306.76 |
|  | @mod3 | 20308.71 | 4385.03 |
|  | @mod4 | 17572.16 | 3794.16 |
|  | @mod5 | 15204.36 | 3261.38 |
|  | @mod6 | 13242.44 | 2859.29 |
|  | @mod7 | 12156.97 | 2624.91 |
|  | @mod8 | 10728.51 |  |

Table 5-5 Link Budget table (3km/h)

# Simulation environment

## Link – System Interface

The simulations were divided into two steps, link level simulatin and System level simulation.

The SINR versus  FER characteristic were obtained  by the link level simulation for all the modulation classes. Based on this link level simulation  results, System(Network Level) were carried out to obtain the performance of  BEST-WINE airinterface in the defined Suburban Macro radio test environment.

## Link level simulation

### Link Level simulation Parameters

- TDD /TDMA system

- 3 timeslot structure

- BS antenna number 12antennas

- UT antenna number 4antennas

- Tx 1antenna / Rx 4 or 1 antennas

- Adaptive Array Antenna (MMSE)

- Equalizer : Equalizer has been employed to over come multipath Distortion effects.

### Channel models used in Link Level Simulations

| Models |  | case-iii |  | case-Iv |  |
| --- | --- | --- | --- | --- | --- |
| PDP |  | Pedestrian-B (Phase I) |  | Vehicular-B (Phase I) |  |
| Number of Paths |  | 6 |  | 6 |  |
| Relative Path power (dB) | Delay (ns) | 0 | 0 | -2.5 | 0 |
|  |  | -0.9 | 200 | 0 | 300 |
|  |  | -4.9 | 800 | -12.8 | 8900 |
|  |  | -8.0 | 1200 | -10.0 | 12900 |
|  |  | -7.8 | 2300 | -25.2 | 17100 |
|  |  | -23.9 | 3700 | -16.0 | 20000 |
| Speed (km/h) |  | 3 |  | 120 |  |
| Mobile Station | Topology | 0.5λ |  | 0.5λ |  |
|  | PAS | RMS angle spread of 35 degrees per path with a Laplacian distribution |  | RMS angle spread of 35 degrees per path with a Laplacian distribution |  |
|  | DoT (degrees) | -22.5 |  | 22.5 |  |
|  | AoA (degrees) | 67.5 (all paths) |  | 67.5 (all paths) |  |
| Base Station | Topology | 10λ-spacing |  |  |  |
|  | PAS | Laplacian distribution with RMS angle spread of 2 degrees per path depending on AoA/AoD |  |  |  |
|  | AoD/AoA (degrees) | 50 for 2 RMS angle spread per path 20 for 5 RMS angle spread per path |  |  |  |

Figure 6-5 Link level simulation channel models and Asscoiated Spatial Parameters

| Sub-path # (m) | Offset for a 2 deg AS at BS (Macrocell) Δn,m,AoD (degrees) | Offset for a 35 deg AS at MS Δn,m,AoA(degrees) |
| --- | --- | --- |
| 1, 2 | ±0.0894 | ±1.5649 |
| 3, 4 | ±0.2826 | ±4.9447 |
| 5, 6 | ±0.4984 | ±8.7224 |
| 7, 8 | ±0.7431 | ±13.0045 |
| 9, 10 | ±1.0257 | ±17.9492 |
| 11, 12 | ±1.3594 | ±23.7899 |
| 13, 14 | ±1.7688 | ±30.9538 |
| 15, 16 | ±2.2961 | ±40.1824 |
| 17, 18 | ±3.0389 | ±53.1816 |
| 19, 20 | ±4.3101 | ±75.4274 |

Table 6-6Sub-path Spatial parameters AoD and AoA offset

## System level simulation environment

### System level simulation target feature

- TDD system

- 3 timeslot structure

- Spatial Divison multiple Access feature (Max. 4 )

- Power control

- Link adaptation

### System level simulation parameters

- BS antenna number 12antennas (10λ)

- UT antenna number 4antennas(0.5λ)

- 19BS 3sector

- BS max Tx power 39dBm/12ant

- UT max Tx power 27dBm

- BS antenna gain 17dBi

- UT antenna gain 0dBi

- BS NF 5dB

- UT NF 10dB

- Temperature 15℃

- BS cable loss 3dB

- UT body loss 3dB

- 1carrier (625KHz). Simulation ( 1.25MHz BW= 625kHz ×4carrier)

### System level simulation channel model

.

| Channel Scenario | Suburban Macro |
| --- | --- |
| Number of paths (N) | 6 |
| Number of sub-paths (M) per-path | 20 |
| Mean AS at BS | E[σAS] =50 |
| AS at BS as a lognormal RV | μAS= 0.69 εAS= 0.13 |
| rAS=σAoDσAS | 1.2 |
| Per-path AS at BS (Fixed) | 20 |
| BS per-path AoD Distribution standard distribution | η(0,σAoD2)where σAoD=rASσAS |
| Mean AS at MS | E[AS, MS] = 680 |
| Per-path AS at MS (fixed) | 350 |
| MS Per-path AoA Distribution | η(0,σAoA2(Pr)) |
| Delay spread as a lognormal RV | DS = - 6.80 DS = 0.288 |
| Mean total RMS Delay Spread | E[σDS] = 0.17 s |
| rDS=σdelaysσDS | 1.4 |
| Distribution for path delays |  |
| Lognormal shadowing standard deviation | 10dB |
| Pathloss model (dB), d is in meters | 31.5 + 35log10(d) |

Table 6-7 Suburban Macro Environment Parameters

### Traffic Model System Level Simulation

The simulation was carried out by full-duplex mode (UL-DL) in the same run and by assuming full buffers (Infinite backlog) model. No ARQ is employed.

# Simulation Reuslts

## Link level simulation

The following Sections report the FER versus SINR perfroance and LInklevel throughput curves for the 7 ModClasses in Uplink and 8 ModClasses in Downlinj

### Uplink (VehicularB model) FER vs SINR Performance

Figure 7-6 Uplink 120kmph FER vs received SINR

### Downlink  (Vehicular B model) FER vs SINR Performance

Figure 7-7 Downlink 120kmph FER vs received SINR

### Uplink (PedestrianB model) FER vs SINR Performance

Figure 7-8 Uplink 3kmph FER vs received SINR

### Downlink (PedestrianB model) FER vs SINR Performance

Figure 7-9 Downlink 3kmph FER vs received SINR

### Uplink (Vehicular B model) Throughput vs SINR

.

Figure 7-10 Uplink 120kmph Throughput vs received SINR

### Downlink( Vehicular B model) Throughput vs SINR

The Downlink characteristic of the reception SINR and Throughput in 120kmph is shown in a figure.

Figure 7-11  Downlink 120kmph Throughput vs received SINR

### Uplink (Pedestrian B model) Throughput vs SINR

Figure 7-12 Uplink 3kmph Throughput vs received SINR

### Downlink (Pedestrian B model) Throughput vs SINR

Figure 7-13 Downlink 3kmph Throughput vs received SINR

## System level simulation calibration result

### System level simulation calibration condition and parameters

- Path number1

- Environment: Suburban macro

- BS & MS antenna number 1

- Inter BS separation 2.5km

- 1carrier(625kHz).

- 1 user/timeslot @sector

user1 (-60, R/2) @timeslot1

user2 (0,R/2)@timeslot2

user3 (60, R)@ timeslot3

(degree, distance from the BS)

System level simulation calibration result is in the attached XLS data: Calibration_data.xls.

Figure 7-14 User Terminal Location Map for Deterministic calibration

## System Level Simulation Results

### User Date Rate CDF

In this section we present system level simulation results as per the output metrics suggested by the Evalaution Criteria Document [5].  In the following User Data rate CDFs

### 120km/h　 User Date Rate CDF

CDF (Uplink and downlink) of User data rate in 120 kmph is shown in Figure.

Blue line shows downlink throughput CDF.

Red line shows uplink throughput CDF.

According to the result of Gnuplot in 120km/h environment,  Uplink result shows higher performance about higher distribution in user data rate.  On the otherhand, Downlink result shows normal distribution in user data rate. However, it can maintain higer spectram efficiency  in 120km/h  environment.

Table 7-8  120kmph  User Date Rate CDF

Figure 7-15 120kmph  User Date Rate CDF

#### 3km/h Uplink and Downlink

CDF (Uplink and downlink) of User data rate in 3 kmph is shown in Figure.

Blue line shows downlink throughput CDF.

Red line shows uplink throughput CDF.

According to the result of Gnuplot in 3km/h environment,  Uplink and Down link  results show higher performance about higher distribution in high user data rate.  Also, many user can keep higher data rate stably  so that user can  enjoy stable higher  broadband  access in 3 km/h environment.. Theee results also have possibility to make higer spectral efficiency in other simulation results.

Figure 7-16  3km/h CDF of throughput vs SINR

### Aggregated Throughput vs Base station Sepparation

The minimum service level is computed by the simulation, by changing inter BS Separation and the number of load users.

In measurement of Minimum service level, the degradation by the increase in load user number is larger compared with degradation by the increase in Base station separation.

The maximum SC number required of a base station is 4[SC] @4Carrier BS=48use.

In the 120 km/h simulation[vehicular B], aggravation of frequency efficiency was seen compared with 3 km/h [Pedestrian B].

#### Downlink

Followings are the contour plots of constant uplink minimum service levels against full buffer users per sector versus basestation separation.

The following figures are the plots by 3 time slot aggregation per 1BS and 1 career.

##### 3 Kmph

Figure 7-17 Contours of constant Minimum Servcie Level at 3Kmph -Downlink

##### 120 Kmph

Figure 7-18 Contours of constant Minimum Servcie Level at 120Kmph -Downlink

#### Uplink

Followings are the contour plots of constant downlink minimum service levels against full buffer users per sector versus basestation separation.

The following figures are the plots by 3 time slot aggregation per 1BS and 1 career.

##### 3 Kmph

Figure 7-19 Contours of constant Minimum Servcie Level at 3Kmph -Uplink

##### 120 Kmph

Figure 7-20 Contours of constant Minimum Servcie Level at 120Kmph -Uplink

## Fairness criteria

The CDF of the Normalized throughput with Respect to average user throughput was deatermined for Cell radius 1km.

Table 8-9　Suburban  Pedestrian B Case

| Normalized throughput with Respect to average user throughput | Uplink | Downlink |
| --- | --- | --- |
| <0.1 | 0.142% | 0.0% |
| <0.2 | 0.285% | 0.142% |
| <0.5 | 0.285% | 0.142% |

Table 8-10　 Suburban  Vehicular B Case

| Normalized throughput with Respect to average user throughput | Uplink | Downlink |
| --- | --- | --- |
| <0.1 | 0% | 0 |
| <0.2 | 0% | 0 |
| <0.5 | 0.432% | 0.519% |

# Simulations Results Summary

- Downlink average spectral efficiency　＝

Downlink aggregate total throughput in 57 sectors 57 sectors×(3slot×1090μsec/5msec)×2.5MHz×2324

- Uplink average spectral efficiency　＝

Uplink aggregate total throughput in 57 sectors 57 sectors×(3slot×545μsec/5msec)×2.5MHz×2324

*Downlink time slot time: 1090 μsec, Downlink time slot time: 545 μsec

*One frame time : 5msec

|  | Spectral Efficiency @pedestrian B | Spectral Efficiency @Vehicular B |
| --- | --- | --- |
| Uplink | 3.155 | 2.592 |
| Downlink | 4.248 | 1.776 |

# Practical System Results

The proposed BEST-WINE’s Base systems HC-SDMA [1] has been implemented and tested in some countries. The section illustrates the performance results spacial channels in delivering high spectrum efficiency in Australia.

HC-SDMA [1] implemented spacial channel testing  in Nov 7th 2003 at North Sydney (North ride). Basestation had 12 dipole antennas on 25m height  tower and terminals are of PCMCIA type as shown in Figure 9-21 . Total 5MHz bandwidth (625KHz x 8Carrier) was used for performance testing. The basestation performs 8 Carrier communication including  BCH and 3 spacial channels simultaneously. User terminal needs to use 625KHz bandwidths for 1Mbps communication. Therefore, Basestation delivered the following data rates to a total  of24 (3 × 8) User simultaneously.

User terminal data rate without BCH	: 	Uplink    1,061 kbps

Downlink  346 kbps

User terminal data rate with BCH	: 	Uplink       707 kbps

:Downlink  231 kbps

So, Aggregate data rate (1,061 kbps ＋ 346 kbps) × 7 Carrier × 3 Spacial

+ (707 kbps + 231 kbps) × 1 Carrier × 3 Spacial @ 32.4 Mbps.

HC-SDMA system could deliver Aggregate data rate @ 32.4 Mbps. The corresponding Maximum Spectrum efficiency that is achieved in 5MHz band is  32.4 Mbps / 5 MHz@ 6.5 bit/sec/Hz/cell

Figure 9-21 25m height Tower,  Sydney MAP and Userterminal

The test result for 24 terminals communication performed total 29.6Mbps.Table 6 show throughput result for downlink performance. The logical maximum data rate is 32.4Mbps as already stated. From this result, Spacial efficiency is computed as 29.6Mbps / 32.4 Mbps @ 0.913      91.3%

Furthermore, about the Spectrum efficiency is computed as 29.6 Mbps / 5 MHz @ 5.9 bit/sec/Hz/cell

1Mbps 21Users

700Kbps 3Users

Figure 7-22: Downlink Date Rate Results

| Data Flow Direction | Typical/Terminal | Total Data Rates/Basestation | Spectrum Efficiency (bit/sec/Hz/sector) |
| --- | --- | --- | --- |
| Downlink | 942kbps | 22.6Mbps | 6.8 |
| Uplink | 290kbps | 7.0Mbps | 4.2 |
| Uplink | 1,232kbps | 29.6Mbps | 5.9 |

Figure 7-23: Date Rate and Spectrum Efficency Test Results of HC-SDMA in Australia