| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Systematic Problems with the Current Draft |  |
| Date Submitted | 2006-05-05 |  |
| Source(s) | Alan Jette Motorola, Inc James F. Mollenauer Technical Strategy Associates | Voice: (847) 632-4201 Mobile: (847) 421-8931 Email: A.Jette@Motorola.com Voice: 617-244-0077 Fax: 617-244-0077 Email: jmollenauer@technicalstrategy.com |
| Re: | IEEE 802.20 Call for Contributions |  |
| Abstract | There are major systematic problems with the scope of the current 802.20 document, and with the use of text copied from other organizations. Remedies are proposed. |  |
| Purpose | To facilitate discussion of the 802.20 draft |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Summary

The current 802.20 draft exhibits several systemic problems, which must be corrected before the draft is ready for submission to sponsor ballot.  These problems include exceeding the scope of IEEE 802 generally (let alone the scope of the 802.20 PAR), utilizing another standard by copying the text rather than by reference, and failure to show a distinct identity.

These problems are distinct from detailed technical problems, which are the subject of other comments on the letter ballot.

Scope Problems

IEEE 802, since its inception in 1980, has been restricted to standardizing the lower two layers of the OSI Reference Model, the physical layer and the medium access control layer.  The 802.20 draft goes well beyond this, including material from the session layer, three layers above the normal scope of IEEE 802 standards.

Other standards bodies do not necessarily have the same restriction and commonly write standards that involve higher layers.   IEEE 802 is not permitted to do so.

The 802.20 PAR clearly states the project is:  “Specification of physical and medium access control layers of an air interface for interoperable mobile broadband wireless access systems, operating in licensed bands below 3.5 GHz, optimized for IP-data transport, with peak data rates per user in excess of 1 Mbps.”  [Our underlining.]

The Physical and Medium Access Control layers are defined in  http://www.leapforum.org/published/internetworkMobility/split/node12.html:

Physical Layer:

The physical layer functions include all physical aspects of communicating between two directly-connected physical entities. Typically these physical properties include electromechanical characteristics of the medium or link between the communicating physical entities such as connectors, voltages, transmission frequencies, etc. This layer summarizes the physics which underlie the communication path.

The MAC Layer:

The medium access control (MAC) layer is closely associated with the physical layer and defines the means by which the physical channel (medium) may be accessed. It coordinates the attempts to seize a shared channel by multiple MAC entities, much as a school teacher must arbitrate between pupils' conflicting desires to speak. The MAC layer commonly provides a limited form of error control, especially for any header information which defines the MAC-level destination and higher-layer access mechanism

Sections of the 802.20 Letter Ballot draft are clearly upper layer functions that reside above the MAC layer and should be removed, as they exceed the scope of the current Project Authorization.

Much of the text in the 802.20 document is similar or identical to text in C.S0024-A from 3GPP2.  In that document, the applicability to the session layer is clearly spelled out:

### Layers

Figure 1.4.1-1 describes the layering architecture for the air interface. Each layer consists of one or more protocols that perform the layer’s functionality. Each of these protocols can be individually negotiated.

- Figure 1.4.1-1. Air Interface Layering Architecture

The protocols and layers specified in Figure 1.4.1-1 are:

Application Layer: The Application Layer provides multiple applications. It provides the Default Signaling Application for transporting air interface protocol messages. The Default Signaling Application is defined in Chapter 2. It also provides the Default Packet Application for transporting user data. The Default Packet Application is defined in Chapter 3.

Stream Layer: The Stream Layer provides multiplexing of distinct application streams. The Default Stream Protocol provides four streams. Stream 0 is dedicated to signaling and defaults to the Default Signaling Application (see Chapter 2). Stream 1, Stream 2, and Stream 3 are not used by default. The Stream Layer is defined in Chapter 4. The Generic Virtual Stream Protocol provides 255 virtual streams to which applications may be bound.

Session Layer: The Session Layer provides address management, protocol negotiation, protocol configuration and state maintenance services. The Session Layer is defined in Chapter 7.

Connection Layer: The Connection Layer provides air link connection establishment and maintenance services. The Connection Layer is defined in Chapter 8.

Security Layer: The Security Layer provides authentication and encryption services. The Security Layer is defined in Chapter 9.

MAC Layer: The Medium Access Control (MAC) Layer defines the procedures used to receive and to transmit over the Physical Layer. The MAC Layer is defined in Chapter 10.

Physical Layer: The Physical Layer provides the channel structure, frequency, power output, modulation, and encoding specifications for the Forward and Reverse Channels. The Physical Layer is defined in Chapters 11, 12, and 13.

Clearly, 3GPP2 recognizes the Security Layer, Connection Layer, Session Layer, Stream Layer, and Application Layer as ABOVE the Physical and MAC Layer.

Also, referencing 3GPP2 C.S0024-A, Figure 1.6.6-1 shows the different protocol layers and how they are structured:

IEEE 802.20 Section 2, Session Control Sublayer

Note:  Section 2 of IEEE 802.20 Letter Ballot draft appears to be very similar to Section 7.2 of C.S0024-A (ref:  Ballot Comment #119 from Al Wieczorek).  The Ballot Resolution Committee appears to agree with this comment and their response is:  “Copyright clearance or a ‘fair use’ statement for specific sections that have been included in the draft has been requested in writing from the organizations owning the copyright. Responses are anticipated prior to final recirculation.”

Section 7 of C.S0024-A is for the Session Layer – which according to 3GPP2 is clearly above the MAC Layer (see figure 1.4.1-1 above).

Also, the 802.20 Letter Ballot document states: “The Session Control Sublayer contains protocols used to negotiate a session between the access terminal and the access network”.  Referencing the OSI 7 Layer model, the Session Layer is typically Layer 5, which is well above the MAC layer.

The following (extractred from the 802.20 Letter Ballot) are functions of the Session Control Sublayer (Section 2.1.1):

The Session Control Sublayer contains the following protocols:

- Session Management Protocol: Provides the means to control the activation of the other Session Control Sublayer protocols. In addition, this protocol ensures that the session is still valid and manages closing of the session.

- Address Management Protocol: Specifies procedures for the initial UATI assignment and maintains the access terminal addresses.

- Session Configuration Protocol: Provides the means to negotiate the SessionConfigurationToken’s used during the session.

- Capabilities Discovery Protocol: Provides the means for the access network to discover the capabilities of the access terminal.

- Inter RAT Protocol: Provides the means to send messages for other radio access technologies.

Again, clearly these functions are well outside the scope of a MAC layer.  This layer includes functionality that is typically part of the OSI Session Layer.

IEEE 802.20 Section 3.2, Default Signaling Transport

Note: Section 3.2 of IEEE 802.20 Letter Ballot appears to be very similar to Section 2 of C.S0024-A (ref:  Ballot Comment #146 from Al Wieczorek).  The Ballot Resolution Committee appears to agree with this comment and their response is:  “Copyright clearance or a "fair use" statement for specific sections that have been included in the draft has been requested in writing from the organizations owning the copyright. Responses are anticipated prior to final recirculation.”

Section 2 of C.S0024 is for Default Signaling Application – which according to 3GPP2 is clearly above the MAC Layer (see figure 1.4.1-1 above and text from C.S0024-A on the Application Layer).

Also, the 802.20 Letter Ballot document states includes the following functionality for the Default Signaling Transport:

SLP: Signaling Link Protocol requirements. The sender of the message shall send the message only using the SLP in the mode(s) indicated by this information field. Values are:

- Best Effort: the message is sent once and is subject to erasure, and

- Reliable: erasures are detected and the message is retransmitted one or more times, if necessary.

Clearly this is QoS and is outside the scope of the MAC functionality.

Security: Security modes for the message. The sender of the message shall send the message only with a security type(s) indicated by this information field. Values are:

- Required: if SecurityEnabled public data of the Security Protocol is set to ‘1’, then the message shall be sent with IsSecure field of the Lower MAC header set to ‘1’.  Any message received when SecurityEnabled public data of the Security Protocol is set to ‘1’ and the IsSecure field of the Lower MAC header is set to ‘0’ shall be discarded, and

- Optional: the message is always processed.

Clearly a Security Protocol is outside the scope of MAC functionality.

IEEE 802.20 Section 3.3, Default Data Transport

Note: Section 3.3 of IEEE 802.20 Letter Ballot appears to be similar to Section 4 of C.S0024-A (ref:  Ballot Comment #152 from Al Wieczorek).  The Ballot Resolution Committee appears to agree with this comment and their response is:  “Copyright clearance or a "fair use" statement for specific sections that have been included in the draft has been requested in writing from the organizations owning the copyright. Responses are anticipated prior to final recirculation.”

Section 4 of C.S0024 is for Multi-Flow Packet Application – which is part of their Stream Layer, according to 3GPP2 is clearly above the MAC Layer (see figure 1.4.1-1 above and text from C.S0024-A on the Stream Layer).

Also, the 802.20 Letter Ballot document states the following functionality for the Default Signaling Transport:

“The Default Data Transport provides multiple packet streams that can be used to carry packets between the access terminal and the access network.”

The Default Data Transport provides:

- The Route Selection Protocol, which routes Flow Protocol PDUs over either Route A or Route B of a Link Flow.

- The Radio Link Protocol (RLP), which provides retransmission (if needed) and duplicate detection of higher layer packets transmitted on each route.

- The Flow Control Protocol, which provides flow control for the Default Data Transport.

- The ability to negotiate protocol parameters for all protocols in the Default Data Transport.

Clearly the above functionality is outside the scope of a MAC layer.

In short, the 802.20 letter ballot proposal has taken layers that are outside the scope of Physical and MAC – when using the OSI model – and are calling them enhanced sub-layers of the MAC.

Material from Other Standards Development Organizations

There are many reasons not to incorporate text from other organizations.  These include market confusion, design confusion, and implementation confusion.  Inclusion by reference is considered preferable, and rightly so.

In the communications world, there are many technologies and many standards.  Usually it is clear to informed individuals what does what.  For example, Ethernet is easily distinguished from Token Ring, TDMA is distinct from CDMA, and so on.  But when one standard copies from another, distinctions become blurred.

What is IEEE 802.20?   What is 3GPP2  - C.S0024-A?

IEEE standards policies clearly deprecate copying material from another standards body.  See for example the IEEE Standards Guidlines.

Design confusion can occur also, because the original standard and the copy can diverge from each other over time.  This is especially true of 802.20 since we are still in the comment phase of development  The result is that designers can miss differences between the two versions.

We can also expect implementation confusion.  Can hardware and software components be recycled from one to the other?

Lack of distinct identiy

A related issue is that taking material from another standard violates the principle of distinct identity.  If the technology being standardized has already been standardized elsewhere, why should we go to the effort of doing another just like it?  The cost in time and travel expenses for the participants invariably reaches many millions of dollars.  Hence the Five Criteria for a new PAR include distinct identity as a requirement.

What to do

To rectify the situation we must do several things:

Remove copied text.

Replace the text with reference to 3GPP2 document(s) such as C.S0024-A

Concentrate the 802.20 document on the physical layer.

Add messages to the 3GPP2 MAC layer if needed by the new physical layer

Explicitly indicate what parts of 3GPP2 standard are not used.

These may be left out of products at the implementer's option.

The consequences of not taking appropriate action

Given that the present draft violates IEEE policies, it is unlikely to survive review by the IEEE Standards Board even if it passes the working group and sponsor ballots.  Rather than be turned back months from now, we need to correct the situation before more time passes.