| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | IEEE 802.20 Evaluation Criteria (Ver 11) |  |
| Date Submitted | 2004-09-12 |  |
| Source(s) | Farooq Khan 1301 E Lookout Dr. Richardson, TX 75082 | Voice: +1 972 761 7929 Fax: +1 972 761 7909 Email: fkhan@sta.samsung.com |
| Re: | MBWA Call for Contributions: Session # 10 – September 12-16, 2004 |  |
| Abstract | This document is a draft of the evaluation criteria document. In final form, it will reflect the consensus opinion of the evaluation criteria correspondence group. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

IEEE P 802.20™/PD<insert PD Number>/V<insert version number>

Date: <September 9, 2004>

# Draft 802.20 Permanent Document

# <802.20 Evaluation Criteria – Ver 11>

This document is a Draft Permanent Document of IEEE Working Group 802.20. Permanent Documents (PD) are used in facilitating the work of the WG and contain information that provides guidance for the development of 802.20 standards.  This document is work in progress and is subject to change.

Contents

1	Overview	5

1.1	Scope	5

1.2	Purpose	5

1.3	Organization of the Document	5

2	Link level and System Level Analysis	5

3	Link level Modeling	6

3.1	Modeling assumptions	6

3.2	Performance metrics	6

3.3	Link level simulation model to include the effects of adjacent channel interference	6

4	Traffic Models for 802.20 System Simulations	8

4.1	Introduction	8

4.2	Context and Scope	8

4.3	Traffic Models	10

4.4	Traffic Mix	15

5	System Level Modeling	16

5.1	Cell layout	16

5.2	Fading Models	17

5.3	Higher Layer Protocol Modeling	17

5.4	Backhaul Network Modeling	24

5.5	Mobility Modeling	25

5.6	Control signaling modeling	26

6	Phased Approach for Technology Evaluation	26

6.1	Channel models for Phase 1 of the simulations	26

7	Link-System Interface (LSI)	26

8	System Simulation Calibration	27

9	Channel Modeling	27

9.1	Channel Mix	27

9.2	Channel Models	27

10	RF Environment	27

10.1	Radio Transceiver Characteristics	27

11	Link Budget	29

12	Equipment Characteristics	32

12.1	Antenna Characteristics	32

12.2	Hardware Characteristics	32

12.3	Deployment Characteristics	32

13	Output Metrics	33

13.1	System Capacity Metrics	33

14	Payload Based Evaluation	37

14.1	Capacity performance evaluation criteria	38

14.2	Payload transmission delay evaluation criteria	38

15	Fairness Criteria	38

16	Simulation and evaluation of various channel bandwidths	39

17	Appendix A: Definition of terms	40

17.1	Number of Active Users Per Cell	40

17.2	Inter-basestation separation	40

17.3	One-Way Internet packet delay	40

18	References	40

Appendix A: 19 Cell Wrap-Around Implementation	42

# <802.20 Evaluation Criteria>

# Overview

## Scope

This document describes the evaluation criteria used by the IEEE 802.20 working group to evaluate different candidate air interface proposals for the IEEE 802.20 standard.  This document and the IEEE 802.20 requirements document form the basis for decisions.

Although the IEEE 802.20 standard defines operations at the Link and Physical layer of the ISO Model, many of the criteria in this document extend to other ISO layers.  The evaluation criteria based on other ISO layers are for information use only.  Informational areas of this document are used when other methods are insufficient to determine an alternative.

## Purpose

This document presents the criteria used for the evaluation of air interface (i.e. combined MAC/PHY) proposals for the future 802.20 standard. As such, the evaluation criteria emphasize the MAC/PHY dependent IP performance of an 802.20 system.

An “802.20 system” constitutes an 802.20 MAC/PHY airlink and the interfaces to external networks for the purpose of transporting broadband IP services.

## Organization of the Document

# Link level and System Level Analysis

A great deal can be learned about an air interface by analyzing its airlink to a single user.  For example, a link-level analysis can reveal the system’s noise-limited range, peak data rate, maximum throughput, and the maximum number of active users.  Extension of the link-level analysis to a multi-user single-cell setting is generally straightforward and provides a mechanism for initial understanding of the multiple-access (MAC) characteristics of the system.   Ultimately, however, quantifying the network-level performance of a system, i.e. system level performance, although difficult, carries with it the reward of producing results that are more indicative of the viability of the system and its expected worth to a service provider.

Since system level results vary considerably with the propagation environment, the number and spatial distribution of users loading the network, and many other fixed and stochastic factors, the assumptions and parameters used must be reported carefully lest the quoted network-level performance be misleading.

Given the charter of 802.20 as a mobile broadband wide area system, it is important to understand the system’s performance in a network setting where multiple base stations serve a large mobile customer base.  In a macro-cellular deployment as required by the PAR, multiple base stations are required to cover a geographic region.  In practice, cell radii may range from 0.5 km to 15 km. The proposed systems must cope with the considerable effects of intra-cell and inter-cell interference that arise in network deployments.

Ultimately, the system level performance is the key metric that will drive much of the system level economics. For example, while the per-user peak data rate is an important service metric, a more important one is the achievable service level as a function of the network loading. While link-level performance quantifies what is possible, system level performance quantifies what is likely.

# Link level Modeling

Single user link-level analysis is an analysis of the performance of a single user terminal (UT) in an assumed propagation environment.  This is an important metric for understanding the air interface and yields important information about the system including:

- the effectiveness of link-adaptation and power control,

- the noise-limited range,

- the SNR requirements to support various classes of service,

- the tolerance to multipath and fading, and so on.

However, it should be clear that relying solely on link-level performance can lead the working group to drawing erroneous conclusions.  Due to variability in the propagation environment and inter-cell interference, single-user link-level analysis cannot be directly extrapolated to network-level performance.

## Modeling assumptions

Modulation and coding schemes are simulated for all channel models described in section 9.

## Performance metrics

FER vs. SINR is the product of link-level simulations.  Systems with adaptive modulation should produce a set of curve (one curve per modulation and coding class).  A second family of curves is the link-level throughput vs. SINR.  This is derived by combining the FER from the first curve with the number of bits/symbol for each of the modulation classes at a fixed FER of 1 percent.

## Link level simulation model to include the effects of adjacent channel interference

When multiple frequency channels are deployed in an assigned channel block, the effect of adjacent channel interference can be modeled and included in the link level simulation as shown in Figure 1.  A non-linearity model that can be used in the simulation is shown in Figure 2 [TBD]. Link performance of the desired user under the effect of ACI can be obtained through the computation of error probabilities at the receiver. The result of link performance should then be incorporated into the system level simulation.

The desired signal is generated by the transmitter model with carrier frequency at fc. Two interfering signals, which are generated by similar transmitter models, each of which represents an adjacent channel centered at f on each side of the desired channel that is centered about fc. f is the required channel spacing for the specific proposed technology.

A typical and the worst case scenarios should both be evaluated. The typical scenario is the one in which all three channels are transmitting at the same power.  The worst case scenario happens when the adjacent channels are transmitting at their maximum power, while the desired channel is transmitting at the minimum power.

The set of link-level simulation results that need to be incorporated into the system simulation should reflect the performance degradation caused by ACI, based on this simulation methodology.

For the channels that are located at the two edges of a frequency block, interference from only one adjacent channel needs to be considered.

In the simplified power amplifier model for the AM/AM characteristics, PTx is the average transmitter power, with values indicated in Table 10-1. The maximum transmitter power Pmax should be provided by the proposal.

Figure 1	  Link Simulation Model to include the effects of Adjacent Channel Interference

Figure 2	  Power amplifier model (TBD)

# Traffic Models for 802.20 System Simulations

## Introduction

The Mobile Broadband Wireless Access (MBWA) systems will be designed to provide a broadband, IP-oriented connection to a wireless user that is comparable to wired broadband connections that are in use today. It is expected that there will be a mix of user applications, not unlike that of such wired systems. Further, the traffic characteristics and system requirements of the various applications can vary widely. The performance of such MBWA systems is thus very much dependant on the details of the applications and their traffic models. This is in contrast to cellular wireless voice systems where the performance studies focused on physical and link layer performance with a relatively simple traffic generation model. The purpose of this document is to provide detailed statistical traffic models that can be used as an input to generate packets in a simulation study of a MBWA system.

## Context and Scope

### User scenarios

[Editor’s Note: It was discussed over the 12/2 conference call if we need to consider all the user scenarios (Laptop, PDA, Smart phone, machine-to-machine) or only a subset of the user scenarios can be considered. In order to capture different user scenarios, parameters values of some traffic models (e.g. web browsing) would be adapted to the user scenario (e.g. heavy, medium or light web browsing application).]

There can be various different user scenarios for MBWA systems, some of which we cannot foresee at this time. For purposes of illustration, we include some candidate scenarios to frame the context of our work. [Editor’s note: These descriptions need to be discussed]. In all cases, the MBWA modem can either be built-in or supplied through a card or a peripheral device.

- Laptop user: The large and rich display capabilities can be expected to generate graphics-rich and multimedia-rich applications. In general, laptop users will provide the highest data volume demands due to the storage and battery capabilities of laptops. They can provide a full range of applications with perhaps less emphasis on voice and WAP applications. Except for special cases, they tend to be stationary during use.

- PDA user: The display, battery, and storage capabilities are less than that of laptops, and so they are expected to have somewhat less traffic volume. They can be very portable. They are typically used for Web browsing, e-mail, synchronization, video, and voice applications.

- Smartphone user: These devices are very portable and very constrained display and storage capabilities. It is expected that they will be oriented towards voice, WAP, and light video.

d)	Machine to machine (telematics, remote cameras etc.): These usage scenarios can have a wide range of characteristics. In some remote monitoring/control applications driven by specific events, the traffic is bursty. For remote surveillance using continuous video feeds, the traffic is more like streaming. This can be a potentially significant usage scenario for 802.20 systems, but the relevant traffic characteristics may not have received as much study as a applications with human users.

Since the various devices can have very distinct traffic characteristics, we will create multiple traffic models for different usage scenarios of an application.

For example, web browsing is likely to have different statistical characteristics for laptop and PDA scenarios. Rather than tie the models specifically to device types such as laptop and PDA, we will adopt multiple versions of a traffic model with generic names, e.g. Web Browsing A & Web Browsing B, or Web Browsing Heavy & Web Browsing Light. These could have different statistical functions, or different parameters for the same function.

### Basis for Traffic Models

Most traffic modeling work is based on measurements of real traffic, which are analyzed to generate usable statistical descriptions. These are typically used in computer simulations, but can also be used to generate packet traffic for a real system under test. Since MBWA is a future service that is similar to some existing wired systems, a lot of the basis of this document is the traffic modeling work done for wired systems. These provide a reasonable and realistic description of the potential user. Our approach is to use statistical models that can be used to generate a stream of packets that need to be transmitted over the system.

We realize that characteristics of user applications keep changing. At best, one can develop a reasonable consensus model that is useful for bringing some uniformity in comparisons of systems. In particular, it is known that user traffic patterns change as the network performance changes. Traffic modeling work has attempted to adjust to this trend. For example, some of the traffic models such as Web and FTP try to capture the essence of the user applications by describing the amount of data work the user is trying to retrieve rather than specifying a packet stream.

We specifically do not use the trace-based approach where a real recorded stream of packets is played back for simulation. While traces can capture sophisticated details, such traces have details that are often very dependant on the system from which they were recorded, and do not provide flexibility for computer simulation work.

### Adaptive applications

[Editor’s note: This needs to be discussed]. Certain applications such as audio streaming sense the available bit rate of the channel and then adjust the amount of traffic that is transmitted. Certain multi-media sessions may employ content-adaptation of images or video based on network conditions. This directly changes the amount of data that is transmitted. The adaptive nature of applications can be incorporated into the traffic model. We do not perceive a strong need for the adaptive nature of an application to be incorporated as a dynamic feature of the traffic model. Such adaptive behavior can be addressed by using traffic models with different parameters and switching between them in an appropriate manner. Thus, adaptation of traffic characteristics based on network/device conditions is outside the scope of this modeling.

## Traffic Models

This section describes the traffic models in detail. Sections 4.3.1 and 4.3.2 clarify some aspects of the modeling approach and the remaining sections provide detailed models for traffic type listed in Table 1.

(Editor's note: This list of traffic types is finalized except for additions triggered by the Requirements. The details of the table are up for discussion. We will focus efforts on traffic types with higher priority).

Table 1 Characteristics of 802.20 Traffic Types

| Application | Traffic Category | Priority for Evaln. Group | Availability of suitable traffic model(s) | Different versions needed |
| --- | --- | --- | --- | --- |
| VoIP | Real-time | High | High | High-rate, low-rate |
| Web Browsing | Interactive | High | High | Heavy, Medium, Light |
| WAP | Interactive | High | High |  |
| FTP (File transfer) | Best-effort | High | Medium | Fixed/deterministic (for testing), Heavy, Light |
| Video-conference | Real-time | Medium | High | Heavy, Light |
| E-mail | Interactive/ Best-effort | Medium | Low | Heavy, Medium, Light, Non-interactive mode |
| Multimedia Messaging | Interactive | Medium | Medium |  |
| Instant Messaging | Interactive | Medium | Medium |  |
| Gaming | Interactive | Medium | Low |  |
| Audio streaming | Streaming | Medium | Low | High-rate, low-rate |
| Video streaming | Streaming | Medium | Medium | High-rate, low-rate |
| PDA remote synch | Best-effort | Medium | Low |  |
| File-sharing | Best-effort | Low | Low |  |
| Broadcast/multicast | Best-effort | Low | Low | High-rate, low-rate |
| Telematics | Best-effort/ Real-time | Low | Low |  |

### User/Traffic Modeling Approach

One of the objectives of a modeling and simulation exercise is to determine the number of users a MBWA system can support. The proposed approach here is to have traffic models for a user who is maintaining a session with transmission activity. These can be used to determine the number of such registered users that can be supported. This document does not address the arrival process of such registered users, i.e. it does not address the statistics of subscribers that register and become active.

Modeling of an aggregated load from a number of user nodes for background loading purposes may not be feasible for a wireless network. Such an abstraction is particularly difficult with adaptive antenna technologies and systems with complex channel dependencies. So, our traffic models apply to one user terminal.

### Packet Generation

In some of the traffic models, there is a statistical description of the workload or the content of the application rather than the actual packet stream. This is consistent with the state of the art in evaluation of multi-service data systems. For example, the Web browsing model describes the Web pages and the timing between the Web pages. Depending on the details of the underlying TCP model (e.g. MTU size, max receive window) and the HTTP (HTTP v1.0 v. HTTPv1.1), the actual stream of packets will change. In some cases, as in the Voice models, the model may describe the packet stream more directly.

### Web Browsing

Web browsing is the dominant application for broadband data systems, and has been studied extensively. See references [5- 7-8-13-]

The parameters for the web browsing traffic are as follows:

SM: Size of the main object in a page

SE: Size of an embedded object in a page

Nd: Number of embedded objects in a page

Dpc: Reading time

Tp: Parsing time for the main page

Table 2 HTTP Traffic Model Parameters

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Main object size (SM) | Truncated Lognormal | Mean = 10710 bytes Std. dev. = 25032 bytes Minimum = 100 bytes Maximum = 2 Mbytes | fx=12πσxexp−lnx−μ22σ2,x≥0σ=1.37,μ=8.35 |
| Embedded object size (SE) | Truncated Lognormal | Mean = 7758 bytes Std. dev. = 126168 bytes Minimum = 50 bytes Maximum = 2 Mbytes | fx=12πσxexp−lnx−μ22σ2,x≥0σ=2.36,μ=6.17 |
| Number of embedded objects per page (Nd) | Truncated Pareto | Mean = 5.64 Max. = 53 | fx=αkαxα+1,k≤x<mfx=kmα,x=mα=1.1,k=2,m=55Note: Subtract k from the generated random value to obtain Nd |
| Reading time (Dpc) | Exponential | Mean = 30 sec | fx=λe−λx,x≥0λ=0.033 |
| Parsing time (Tp) | Exponential | Mean = 0.13 sec | fx=λe−λx,x≥0λ=7.69 |

Note: When generating a random sample from a truncated distribution, discard the random sample when it is outside the valid interval and regenerate another random sample.

### FTP

In FTP applications, a session consists of a sequence of file transfers, separated by reading times.  The two main parameters of an FTP session are:

: the size of a file to be transferred

: reading time, i.e., the time interval between end of download of the previous file and the user request for the next file.

The underlying transport protocol for FTP is TCP. The parameters for the FTP application session are described in Table 3.

Table 3 FTP Traffic Model Parameters

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| File size (S) | Truncated Lognormal | Mean = 2Mbytes Std. Dev. = 0.722 Mbytes Maximum = 5 Mbytes | fx=12πσxexp−lnx−μ22σ2,x≥0σ=0.35,μ=14.45 |
| Reading time (Dpc) | Exponential | Mean = 180 sec. | fx=λe−λx,x≥0λ=0.006 |

### Voice (VoIP)

The voice traffic model will be implemented as voice over IP (VoIP).  Voice will in general follow a Markov source model with different rates (full rate, half rate, etc) with a corresponding set of transition probabilities between different rates.

### Video (Videotelephony/Videoconferencing)

### Audio streaming

This can be an important class of traffic. It has received relatively less attention in the modeling community.  (See [14-])

### Video streaming

The following section describes a model for streaming video traffic on the forward link.  Figure 3 describes the steady state of video streaming traffic from the network as seen by the base station.  Latency of starting up the call is not considered in this steady state model.

Figure 3 Near Real-Time Video Traffic Model

A video streaming session is defined as the entire video streaming call time, which is equal to the simulation time for this model.

Each frame of video data arrives at a regular interval T determined by the number of frames per second (fps).  Each frame is decomposed into a fixed number of slices, each transmitted as a single packet.  The size of these packets/slices is distributed as a truncated Pareto.  Encoding delay, Dc, at the video encoder introduces delay intervals between the packets of a frame.  These intervals are modeled by a truncated Pareto distribution. The parameter TB is the length (in seconds) of the de-jitter buffer window in the mobile station used to guarantee a continuous display of video streaming data.  This parameter is not relevant for generating the traffic distribution but is useful for identifying periods when the real-time constraint of this service is not met.  At the beginning of the simulation, it is assumed that the mobile station de-jitter buffer is full with (TB x source video data rate) bits of data.  Over the simulation time, data is “leaked” out of this buffer at the source video data rate and “filled” as forward link traffic reaches the mobile station.  As a performance criterion, the mobile station can record the length of time, if any, during which the de-jitter buffer runs dry.  The de-jitter buffer window for the video streaming service is 5 seconds.

Using a source video rate of 32 kbps, the video traffic model parameters are defined Table 4.

Table 4 Near Real-Time Video Traffic Model Parameters

| Information types | Inter-arrival time between the beginning of each frame | Number of packets (slices) in a frame | Packet (slice) size | Inter-arrival time between packets (slices) in a frame |
| --- | --- | --- | --- | --- |
| Distribution | Deterministic (Based on 10fps) | Deterministic | Truncated Pareto (Mean= 50bytes, Max= 125bytes) | Truncated Pareto (Mean= 6ms, Max= 12.5ms) |
| Distribution Parameters | 100ms | 8 | K = 20bytes  = 1.2 | K = 2.5ms  = 1.2 |

### Wireless Multi-Party Gaming Traffic

[Note: It was noted over the 12/2 conference call that wireless gaming is an important application that needs to be considered in 802.20 system evaluation. Input required on mobile wireless gaming models.]

Some types of multi-player games may have demanding requirements on response times.

### Full buffers (Infinite backlog) model

Full buffers (Infinite backlog) model will be used in phase 1 of the simulations.

## Traffic Mix

A MBWA system is expected to have mix of traffic types. There can be different types of usage scenarios (multi-service v. single-type), different types of devices (laptops v. PDAs), different levels of use (intense v. light)., and different demands on response times (real-time v. best-effort). This document is primarily concerned with the traffic models for each of the potential traffic type. As discussed in the previous section, these are based on statistical analysis of measured traffic to extract some invariant patterns that are not very dependant on the specific system. It is more difficult to describe a similar invariant mix of traffic types since these tend to depend more heavily on the type of system and the mix of device/user types.

In the context of a system evaluation using traffic models, the specific mix of traffic types will emphasize different aspects of the system performance, e.g. sustained throughput for file downloads v. faster response times for interactive applications. [Editor’s note: This needs to be discussed] While we may discuss some candidate traffic mixes as pre-mixed recipes for consideration,

Table 5 Traffic mix: percentage of different Traffic Types

| Application | Percentage |
| --- | --- |
| VoIP |  |
| Web Browsing |  |
| WAP |  |
| FTP (File transfer) |  |
| Video-conference |  |
| E-mail |  |
| Multimedia Messaging |  |
| Instant Messaging |  |
| Gaming |  |
| Audio streaming |  |
| Video streaming |  |
| PDA remote synch |  |
| File-sharing |  |
| Broadcast/multicast |  |
| Telematics |  |

# System Level Modeling

In order to accurately model the traffic, physical and MAC layer dependencies between the uplink and the downlink, the system simulations include both UL and the DL in a fully duplex fashion in the same simulation run.

[Note: This issue can be revisited later on as more details on the evaluation methodology, channel models, traffic models and proposals become available. At that point, if the full-duplex simulations are determined to be infeasible due to complexity, a simplex approach can be adopted.]

## Cell layout

The system consists of 19 cells, each with an imaginary hexagonal coverage area.   The sectorization details are TBD.  Mobile stations are uniformly dropped into the 19-cell system.

All 19 cells are simulated using a cell wrap-around technique (See Appendix A) and the statistics are collected from all the cells.

### Distribution of users

Most users of wireless systems experience very good link-quality near the base station.  For this reason, the distribution of users throughout the network is integral to the quoting of network-level performance results.  Absent the desire to highlight specific abilities of an air interface, users should be distributed uniformly throughout each cell of the network.

### User usage model

The following user terminal usage parameters must be specified:

- distribution of indoor vs. outdoor users

- mobility profile across the user base

## Fading Models

### Slow Fading Model

<Shadow Fading standard deviation and correlation between cell sites etc.>

### Fast Fading Model

<Rayleigh and Rician Fading Models etc.>

## Higher Layer Protocol Modeling

<Models for protocols other than MAC/PHY. For example, HTTP and TCP models>

### HTTP Model

### TCP Model

Many Internet applications including Web browsing and FTP use TCP as the transport protocol.  Therefore, a TCP model is introduced to more accurately represent the distribution of TCP packets from these applications.

#### TCP Connection Set-up and Release Procedure

The TCP connection set-up and release protocols use a three-way handshake mechanism as described in Figure 4 and Figure 5. The connection set-up process is described below:

- The transmitter sends a 40-byte SYNC control segment and wait for ACK from remote server.

- The receiver, after receiving the SYNC packet, sends a 40-byte SYNC/ACK control segment.

- The transmitter, after receiving the SYNC/ACK control segment starts TCP in slow-start mode (the ACK flag is set in the first TCP segment).

The procedure for releasing a TCP connection is as follows:

- The transmitter sets the FIN flag in the last TCP segment sent.

- The receiver, after receiving the last TCP segment with FIN flag set, sends a 40-byte FIN/ACK control segment.

- The transmitter, after receiving the FIN/ACK segment, terminates the TCP session.

Figure 4: TCP connection establishment and release for Uplink data transfer

Figure 5: TCP connection establishment and release for Downlink data transfer

#### TCP slow start Model

The amount of outstanding data that can be sent without receiving an acknowledgement (ACK) is determined by the minimum of the congestion window size of the transmitter and the receiver window size.  After the connection establishment is completed, the transfer of data starts in slow-start mode with an initial congestion window size of 1 segment.  The congestion window increases by one segment for each ACK packet received by the sender regardless of whether the packet is correctly received or not, and regardless of whether the packet is out of order or not.  This results in exponential growth of the congestion window.

##### UL slow start model

This UL slow start process is illustrated in Figure 6. The round-trip time in Figure 6, rt, consists of two components:

rt  = u + l

where u = the sum of the time taken by a TCP data segment to travel from the base station router to the server plus the time taken by an ACK packet to travel from the server to the client; l = the transmission time of a TCP data segment over the access link from the client to the base station router. u is further divided into two components; 2 = the time taken by a TCP data segment to travel from the base station router to the server plus the time taken by an ACK packet to travel from the server back to the base station router and 3 = the time taken by the ACK packet to travel from the base station router to the client.

Figure 6: TCP Flow Control During Slow-Start; l = Transmission Time over the Access Link (UL); rt = Roundtrip Time

Table 6 Delay components in the TCP model for the UL upload traffic

| Delay component | Symbol | Value |
| --- | --- | --- |
| The transmission time of a TCP data segment over the access link from the client to the base station router. | 1 | Determined by the access link throughput |
| The sum of the time taken by a TCP data segment to travel from the base station router to the server and the time taken by an ACK packet to travel from the server to the base station router. | 2 | See 5.4.1 |
| The time taken by a TCP ACK packet to travel from the base station router to the client. | 3 | See 5.4.1 |

##### DL slow start model

This DL slow start process is illustrated in Figure 7. The round-trip time in Figure 7, rt, consists of two components:

rt  = d + 4

where d = the sum of the time taken by an ACK packet to travel from the client to the server and the time taken by a TCP data segment to travel from the server to the base station router; 4 = the transmission time of a TCP data segment over the access link from the base station router to the client. d is further divided into two components; 5 = the time taken by a TCP ACK to travel from the base station router to the server plus the time taken by a TCP packet to travel from the server back to the base station router and 3 = the time taken by the TCP packet to travel from the base station router to the client.

Figure 7 TCP Flow Control During Slow-Start; l = Transmission Time over the DL; rt = Roundtrip Time

Table 7 Delay components in the TCP model for the DL traffic

| Delay component | Symbol | Value |
| --- | --- | --- |
| The transmission time of a TCP data segment over the access link from the base station router to the client. | 4 | Determined by the access link throughput |
| The sum of the time taken by a TCP ACK to travel from the base station router to the server and the time taken by TCP data packet to travel from the server to the base station router. | 5 | See 5.4.1 |
| The time taken by a TCP data segment to travel from the base station router to the client. | 6 | See 5.4.1 |

From Figure 6 and Figure 7, it can be observed that, during the slow-start process, for every ACK packet received by the sender two data segments are generated and sent back to back.  Thus, at the mobile station (base station), after a packet is successfully transmitted, two segments arrive back-to-back after an interval u = 2 + 3 ( d = 5 + 6).  Based on this observation, the packet arrival process at the mobile station for the upload of a file is shown in Figure 8.  It is described as follows:

- Let S = size of the file in bytes. Compute the number of packets in the file, N = S/(MTU-40). Let W = size of the initial congestion window of TCP. The MTU size is fixed at 1500 bytes

- If N>W, then W packets are put into the queue for transmission; otherwise, all packets of the file are put into the queue for transmission in FIFO order. Let P=the number of packets remaining to be transmitted beside the W packets in the window. If P=0, go to step 6

- Wait until a packet of the file in the queue is transmitted over the access link

- Schedule arrival of next two packets (or the last packet if P=1) of the file after the packet is successfully ACKed.  If P=1, then P=0, else P=P-2

- If P>0 go to step 3

- End.

Figure 8 Packet Arrival Process at the mobile station (base station) for the upload (download) of a File Using TCP

## Backhaul Network Modeling

### Network Delay model

The one-way Internet packet delay is modeled using a shifted Gamma distribution [6-] with the parameters shown in Table 8. The packet delay is independent from packet to packet.

Table 8 Parameters for the shifted Gamma Distribution

| Scale parameter () | 1 |
| --- | --- |
| Shape parameter () | 2.5 |
| Probability density function (PDF) | f(x)=(xα)β−1e−xaα⋅Γ(β) (.) is the gamma function |
| Mean |  |
| Variance |  |
| Shift | See Table 9 |

Two values, 7.5ms and 107.5ms are used for the shift parameter in order to model the domestic routes and the International routes respectively.  The users’ routes are selected randomly at the time of drop with the distribution shown in Table 9.

Table 9 Shift parameter for the Domestic and International IP routes

| IP Route Type | Percentage of users | Shift parameter | Mean one-way IP packet delay |
| --- | --- | --- | --- |
| Domestic | 80% | 7.5ms | 10ms |
| International | 20% | 107.5ms | 110ms |

### Network Loss model

The transmission of IP packets between the base station (server) and the server (base station) is assumed error free.

Table 10 Internet Loss Model

| IP packet error rate | 0% (lossless packet transmission) |
| --- | --- |

## Mobility Modeling

<For example, Handoff modeling>

## Control signaling modeling

### DL signaling models

<For example, models for MAC state transition messages and scheduling grant transmission etc.>

### UL signaling models

<For example, models for access channel, ACK and channel quality Feedback etc.>

# Phased Approach for Technology Evaluation

[Note: The group agreed in principle to adopt a phased approach for technology evaluation e.g. start with lowest complexity simulations and incrementally add more details (3-4 phases may be specified). The details of the various phases are TBD.]

The 802.20 evaluation will be structured with multiple phases with each phase progressively adding more complexity. The evaluation work for each proposal may then be compared at each phase to ensure a progressive "apples to apples" comparison of proposals. This structured approach will also provide performance metrics for the physical and link layer performance early rather than later in the evaluation process.

Phase 1 of the evaluation will consist of:

- Items/issues/criteria that are required for the calibration of simulations

- Items/issues/criteria that will draw out the important differences between the various proposals that cannot be otherwise inferred.

The goals at the end of phase 1 are, first, to achieve confidence that different simulation models are calibrated and, two, to present fundamental performance metrics for the physical and link layer of various proposals.

## Channel models for Phase 1 of the simulations

Current Recommendation is to use suburban macro, 3 Km/h pedestrian B and 120Km/h Vehicular B models.

# Link-System Interface (LSI)

An interface between link and system simulations is required because the link and system simulations are performed separately (the simulation complexity would be very high if joint link and system simulations are required).  [Editor’s Note: The current understanding on LSI is that the actual link curves is the default methodology for the link-system interface i.e. the link curves can always be used even if an agreement on a common methodology is reached. The group is trying to define a common methodology for the link-system interface. If a common methodology is used then no justification is required from the proponent. In the absence of a common methodology, a technology specific methodology can be used if provided with full verification subject to the satisfaction of the group.]

# System Simulation Calibration

[Note: The evaluation criteria would specify a system simulation calibration process. However, it is not clear, at this stage, to what level of detail different simulations need to be calibrated. The group is open to proposals to nail down the calibration specifications.]

# Channel Modeling

## Channel Mix

<Percentage of different Channel types>

## Channel Models

<Input from Traffic and Channel Models Correspondence Group>

# RF Environment

## Radio Transceiver Characteristics

The RF environment for the Evaluation Criteria (EC) model is defined by a set of RF parameters that should be considered as fixed constrains (that apply to all technologies) and by technology-specific performance specifications.

Table 10-1 defines the transmitter and receiver parameters for the base station (BS) and the mobile station (MS) radios. Note that some parameters vary with channel bandwidth. Base values were chosen for a 5 MHz channel and adjustments were made for 1.25 MHz and 10 MHz channels. The adjustment for other channel bandwidths can easily be calculated by factoring the channel BW ratio to the 5 MHz channel.

### Transmitter

The maximum base station (BS) transmit power is specified here as peak power density per 1 MHz and is +43 dBm. The mobile station (MS) maximum transmit power shall be +27 dBm for all channel bandwidths.

The out-of-band emission limits were chosen to be those specified by the FCC for the PCS band. Note that this requirement shall apply only to band/block edge channels but not to mid-band channels. The spectral shape of the transmitted modulated carrier is informative to the evaluation criteria and should be submitted. It is required that the occupied bandwidth (OB) of the transmit channel (identified by a center frequency) shall be that which contains 99% of the transmit power.(TBD)

In the case of a multi-carrier technology, the transmit power is the total power of all carriers transmitted simultaneously and intended to be received by one receiver simultaneously.

It shall be further required that in the evaluation criteria model, any band-edge guard bands that are used in order to meet the out of band emission limits should be specified. (TBD).

Note that when applying the FCC emission attenuation formula (see Table 10-1, item 3, where P is the transmit power in Watts), the maximum permissible out-of-band power is -13dBm for all transmit powers and all channel bandwidths. Also note that the resolution-bandwidth for out-of-band power measurements is 1 MHz.

### Receiver

Noise Figure:

To achieve a fair performance comparison, different technologies should be constrained by similar environmental interference conditions and have identical receiver noise figure specifications. Thus, for the evaluation purposes it is required that all technology proposals use the same receiver noise figure (NF) specs: 10 dB for the MS and 5 dB for the BS.

Receiver Sensitivity:

The theoretical receiver sensitivity is expected to vary from one technology to another, but again, for the sake of fair comparison, it is required that the receiver sensitivity be specified for raw data bit error rate (BER) of 0.1%.

The receiver sensitivity (in dBm) shall be calculated using the following formula:

Sensitivity = (-174.5 dBm) + NF (in dB) + 10 log (channel-BW in Hz) + C/N min for 0.1% BER).

ACLR, Selectivity and Blocking:

In addition to co-channel interference, the RF environment in which a receiver operates includes interference from adjacent and second-adjacent channels (of same or different technologies). The contribution of the latter signals to the overall C/N level can be calculated from the ACLR, Selectivity and Blocking specifications. The total interference power present in the wanted channel band consists of the aggregate interference power caused by co-channel, adjacent channel and second-adjacent channel signals.

For the evaluation model purposes, items 4, 5, 9 and 10 in Table 10-1 are the recommended values for ACLR and Selectivity. Individual proposals may specify different values and must commit their entire performance analysis and simulations to their chosen specifications. The Blocking signal levels are mandatory values.

The power level of interference from adjacent channels and from blocking signals shall be calculated by applying the ACLR and Selectivity figures from table 10-1 or from the individual proposal’s technical specifications.

Note1: Typically the interference effect of a blocking signal is measured when the level of the wanted signal is 3 dB above receiver sensitivity (at BER 0.1%). When applying the blocking signal to the receiver input (in addition to the wanted signal) no change in the BER should be detected.

Note 2: Contribution # C802.20-04-58 [2] proposes an alternative method (to ACLR) for analyzing the impact on performance of adjacent channel interference. Proposals may choose to use either method.

Table 10-11:  Evaluation Criteria RF Parameters

| # | RF Parameter | Base Value | 1.25 MH Ch BW | 5 MHz Ch BW | 10 MHz Ch BW |
| --- | --- | --- | --- | --- | --- |
| 1 | Transmitter Power -- BS | 43 dBm/MHz | +44 dBm | +50 dBm | +53 dBm |
| 2 | Transmitter Power -- MS | 27 dBm | +27 dBm | +27 dBm | +27 dBm |
| 3 | Out of Band emission limits – BS and MS (emission measured in 1 MHz resolution bandwidth) | Attenuation of the transmit power P by: 43 +10 log(P) dB | -13 dBm | -13 dBm | -13 dBm |
| 4* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) -- BS | 45 dB | 39 dB | 45 dB | 48 dB |
| 5* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) -- MS | 33 dB | 27 dB | 33 dB | 36 dB |
| 6 | Receiver noise figure -- BS | 5 dB | 5 dB | 5 dB | 5 dB |
| 7 | Receiver noise figure -- MS | 10 dB | 10 dB | 10 dB | 10 dB |
| 8 | Receiver reference sensitivity (to be proposed by each technology) | Specify at BER of 0.1% | value 1 (proposal specific) | value 2 (proposal specific) | value 3 (proposal specific) |
| 9* | Receiver Selectivity -- BS | 63 dB | 63 dB | 63 dB | 63 dB |
| 10* | Receiver Selectivity -- MS | 33 dB | 33 dB | 33 dB | 33 dB |
| 11* | Receiver Blocking – BS (level of same technology blocking signal at frequency offset of 2*Channel BW) | -40 dBm | -40 dBm | -40 dBm | -40 dBm |
| 12* | Receiver Blocking – MS (level of same technology blocking signal at frequency offset of 2*Channel BW) | -56 dBm | -56 dBm | -56 dBm | -56 dBm |

* Recommended values. Proposals may choose (and commit to) different values.

# Link Budget

[Open items: Should maximum range (link budget) be used as a performance metric for proposal comparison or not? Also need to determine how to use the building/vehicular penetration loss numbers for different environments]

| id/ii | Item | Downlink | Uplink |
| --- | --- | --- | --- |
|  | Test environment | Suburban/urban macro-cell, micro-cell, indoor pico-cell | Suburban/urban macro-cell, micro-cell, indoor pico-cell |
|  | Operating frequency | 1.9GHz | 1.9GHz |
|  | Test service |  |  |
|  | Multipath channel class | Cases I-V | Cases I-V |
| ii/id | (a0) Average transmitter power per traffic channel (NOTE 1) | dBm | dBm |
| id | (a1) Maximum transmitter power per traffic channel | dBm | dBm |
| id | (a2) Maximum total transmitter power | 43 dBm/MHz | 27dBm |
| ii | (b) Cable, connector, and combiner losses (enumerate sources) | 3 dB | 0 dB |
|  | Body Losses | 0 dB | 3 dB |
| ii | (c) Transmitter antenna gain | 17 dBi | 0 dBi |
| id | (d1) Transmitter e.i.r.p. per traffic channel  (a1 – b  c) | dBm | dBm |
| id | (d2) Total transmitter e.i.r.p.  (a2 – b  c) | 57 dBm | 27 dBm |
|  | Penetration Loss (Ref: 3GPP2) [Determine how to use these numbers for different environments, revisit if 20dB is a reasonable value for building penetration)] | 20 dB (Building) 10 dB (Vehicular) | 20 dB (Building) 10 dB (Vehicular) |
| ii | (e) Receiver antenna gain | 0 dBi | 17 dBi |
| ii | (f) Cable and connector losses | 0 dB | 3 dB |
|  | Body Losses | 3 dB | 0 dB |
| ii | (g) Receiver noise figure | 10 dB | 5 dB |
| ii | (h) Thermal noise density (H) (linear units) | –174 dBm/Hz 3.98  10–18 mW/Hz | –174 dBm/Hz 3.98  10–18 mW/Hz |
| id | (i) Receiver interference density (NOTE 2) (I) (linear units) | dBm/Hz mW/Hz | dBm/Hz mW/Hz |
| id | (j) Total effective noise plus interference density  10 log (10((g  h)/10)  I) | dBm/Hz | dBm/Hz |
| ii | (k) Information rate (10 log (Rb)) | dB(Hz) | dB(Hz) |
| id | (l) Required Eb/(N0  I0) | dB | dB |
| id | (m) Receiver sensitivity = (j  k  l) |  |  |
| id | (n) Hand-off gain | dB | dB |
| id | (o) Explicit diversity gain | dB | dB |
| id | (o) Other gain | dB | dB |
| id | (p) Log-normal fade margin | dB | dB |
| id | (q) Maximum path loss  {d1 – m  (e – f)  o  n  o – p} | dB | dB |
| id | (r) Maximum range | m | m |

# Equipment Characteristics

## Antenna Characteristics

<antenna pattern, number of antennas, antenna array geometry (if applicable), orientation, number of sectors>

## Hardware Characteristics

The assumed hardware parameters of both the basestation and the user terminals are necessary to interpret the quoted results.  For example, differences in specification (both BS and UT) significantly affect performance results:

- maximum output power

- noise figures

- antenna gain, pattern, and height

- cable loss (if applicable).

## Deployment Characteristics

Relevant system-level parameters used for an 802.20 deployment include:

- number of carriers

- total spectral bandwidth

- system frequency allocation

- sectorization (if applicable)

# Output Metrics

<For example, spectral efficiency, number of users supported per sector, per user throughput and system capacity etc.>

Two good criteria for evaluating the network-level performance of an MBWA system are its ability to cover the worst served users and the aggregate throughput that can be delivered within the cell.  In this section, statistics for quantifying these aspects of network-level performance are described.

## System Capacity Metrics

This section presents several metrics for evaluating system capacity.  Specifically, respondents are required to provide:

- User data rate CDF for specified load and basestation separation (Section 13.1.1: Fixed load/coverage operating point: Service Distribution)

- Plot of aggregate throughput vs. basestation separation for stated minimum service levels. (Section 13.1.2: Aggregate Throughput)

- Plot of number of active users per cell vs. basestation separation for stated minimum service levels (Section 13.1.3: Network performance under Varying Load/Coverage)

- Spectral Efficiency for stated load coverage operating points (Section 13.1.4: Computing Sustained Spectral Efficiency)

The results presented for the uplink and downlink capacity should be achievable simultaneously by the system.  If the results for uplink and downlink cannot be achieved simultaneously by the system, the respondent should indicate so.

### Fixed load/coverage operating point: Service Distribution

Let the load/coverage point be fixed at (Nu,S), where (by definition) the number of active users per cell (Nu), and the (common) inter-basestation separation (S) for a hexagonal tessellation of Nc cells is specified.  This operating point implies a distributionD(Nu,S)of data rates for each user that the system is able to deliver within the cell area.  We propose that the distributionD(Nu,S) be sampled separately in uplink and downlink directions (Monte-Carlo simulation) with statistics gathered only from the interior cells of the network.

Figure 9 shows a qualitative example of a cumulative distribution function (CDF) of the distribution of downlink data ratesD(Nu,S) in the interior cells of a network for a specified load/coverage operating point (Nu,S).  This graph shows the distribution of data rates on the ensemble of random placements of NUactive users in each cell of the network and all other stochastic input parameters.  The CDF is not complete without specification of the assumed probability distribution of user placement.

Figure 9:  Service Distribution for a fixed load/coverage operating point

#### Minimum Service Level

From a service integrity standpoint, the lower tail of the resulting service CDF contains important information.  Continuing the example of Figure 9, 90% of the active users will be served with a minimum service level of 566 kbits/sec at the load/coverage operating point(Nu,S).  The notation TDL(Nu,S) emphasizes that the minimum service level is a function of the load/coverage operating point.

### Aggregate Throughput

For each placement of users, the aggregate throughput is the sum of the data rates delivered to the Nuactive users in a cell.  The per-user data rate is computed by dividing the total number of information bits received by the time-duration of the simulation. The respondent should provide a graph of the aggregate throughput vs. basestation separation for constant minimum service levels (See Section: 13.1.3) .  This graph would be of the same for as Figure 10 with the vertical axis being aggregate through put instead of number of users.

### Network performance under Varying Load/Coverage

The CDF of Figure 9 characterizes the ability of the system to serve active users at a fixed load/coverage operating point.  Studying the behavior of the system with varying network load gives additional insight.  One interesting approach is to compute the minimum service level TDL(Nu,S)on a grid of points in the load-coverage (NU,S) plane.  Sample contours of constant minimum service level are shown in Figure 2.  This example (synthetically produced for illustrative purposes), reveals the tradeoff between the basestation separation (S) and the number of active users per cell (Nu).

For example, to guarantee an expected minimum service rate of, say, 1024 kbits/sec across 90% of the cell area, few active users (less than 5) can be supported per cell at the noise-limited inter-basestation separation of 6 km.  Conversely, many active users per cell (more than 20) can be supported in the interference-limited case when the basestations are closely spaced.

Figure 10: Contours of constant minimum service level

### Computing Sustained Spectral Efficiency

In the present setting, the sustained spectral efficiency (η) can be computed in a meaningful and straightforward manner.  A moment’s reflection will reveal that rather than being a single number, spectral efficiency is a family of numbers parameterized by the load/coverage operating point (Section13.1.1) and the assumed minimum service level.

For a specified operating point (Nu,S)and a minimum service level, the expected aggregate throughput (A) is defined as the expected sum of the data rates delivered to the NUactive users in the cell.  For example, in the downlink direction, the expected aggregate throughput (per-cell) is defined

ADL=Ek=1NuRDL,k

where RDL,k is the downlink rate to the kthuser and E is the statistical expectation.  A similarly defined statistic AUL applies in the uplink direction.  The total expected aggregate throughput is the sum of uplink and downlink: AT=AUL+ADL.

The sustained (total) spectral efficiency is computed

ηT=ATBWTbits/sec/Hz/cell

where BWT is the total system bandwidth.  Similarly, the spectral efficiency is computed in the uplink direction as

ηUL=AULBWULbits/sec/Hz/cell

where BWUL is the (effective) bandwidth reserved for uplink traffic.  The spectral efficiency in the downlink direction is similarly defined.

# Payload Based Evaluation

[Need further discussion on how we are going to use payload based evaluation criteria]

The payload-based evaluation method for MAC-Modem-Coding capacity and delay performance assessment is described below.

## Capacity performance evaluation criteria

In order to evaluate the different proposals capacity performance, it is useful to define evaluation scenarios. The evaluation parameters are:

- Channel spacing: 1.25MHz and 5MHz

- Modem rate (max rate & minimum coding, medium rate & medium coding, minimum rate & maximum coding);

- MAC frame duration: 5ms

For capacity evaluation, the payloads associated with every type service are:

- 30 bytes for VoIP, G.729 codec, 30ms period

- 1518 bytes for long IP packets;

- 64 bytes for short IPv4 packets;

- 40 bytes for video-conference, 64kb/s (64kb/s*5ms/8 =40bytes)

- 240 bytes for video-conference, 384kb/s

- T.B.C. bytes for multi-media streaming.

The computation shall take into account the influence of the MAC overheads, MAC granularity, interleaver, coding block, etc.

In order to simplify the procedure, only one type of traffic is assumed for all the Base Station subscribers. For every type of traffic shall be calculated the subscriber number, separately for up-link and down-link

## Payload transmission delay evaluation criteria

The delay is an important factor for real-time services.

The payload transmission delay shall be evaluated according to the same procedure and parameters, as specified for capacity evaluation. The computation shall take into account the influence of the MAC granularity, interleaver, coding block, etc.

The delay will be calculated between the moment in which the payload enters the MAC and the moment in which the payload exits the MAC, on the other side of the wireless link. The processing power of the implied devices will not be taken into account.

The calculation shall be done separately for up-link and down-link, assuming the number of subscribers resulted from capacity calculation.

# Fairness Criteria

In the evaluation of spectral efficiency and in order to make a fair comparison of different proposals, it is important that all mobile users be provided with a minimal level of throughput.  The fairness for best effort traffic (HTTP, FTP and full buffers) is evaluated by determining the normalized cumulative distribution function (CDF) of the user throughput, which meets a predetermined function given in Table 12. For applications other than best effort, application specific outage criteria are defined.

The proposals will also provide additional fairness metrics. The details of the additional fairness metrics are TBD (see for example IEEE C802.20-04/05).

Let Tput[k] be the throughput for user k.  The normalized throughput with respect to the average user throughput for user k, Tput[k] is given by:

Tput[k]=Tput[k]avgi Tput[i].

The CDF of the normalized throughput with respect to the average user throughput is determined.  The CDF shall lie to the right of the curve given by the points in Table 12.

Table 12 Fairness Criterion CDF

| Normalized Throughput w.r.t average user throughput | CDF |
| --- | --- |
| 0.1 | 0.1 |
| 0.2 | 0.2 |
| 0.5 | 0.5 |

# Simulation and evaluation of various channel bandwidths

Two sets of spectrum allocations (over which the results are quoted) are used in the evaluation:

- 2X5 MHz (total 10 MHz) and

- 2X15 MHz (total 30 MHz)

The individual technology proposals may split the total spectrum into a given number of channels and specify their reuse factor and channel bandwidth. For example, if 2X15MHz is used as the spectrum allocation, then individual technology proposals can perform simulations for 2X5 MHz and then scale the simulation output data to 2X15MHz.

[The issue that needs to be further addressed is how the spectral mask requirement would apply to the spectrum allocation used for the evaluation process. One possibility is that proposals specify both its channel bandwidth and its "necessary bandwidth” and justify the ability to support their specified number of carriers within the spectrum allocation specified. In this case, proposals with multiple carriers within the spectrum allocation used for the evaluation process may have to simulate the inter-carrier leakage in order to justify the number of carriers used within the allocation.

In order to accommodate cases where a proposal choose to simulate only a single spectrum allocation, a scaling between the 2 sets of spectrum allocation needs to be defined.

# Appendix A: Definition of terms

## Number of Active Users Per Cell

For the purposes of this analysis, an active user is a terminal that is registered with a cell and is seeking to use air link resources to receive and/or transmit data within the simulation interval.  Evaluating service quality as a function of the well-defined concept of the number of active users per cell is a natural way of comparing how well disparate MBWA systems behave under increasing network load.

## Inter-basestation separation

For the purposes of defining network load, it is natural to treat inter-basestation distance as a parameter.   Closely spaced deployments will stress the interference-limited performance of the network while widely spaced deployments will stress the range-limited performance.  In any case, users of an 802.20 system will likely experience different link quality at locations throughout the cell that depend both on the distance from the basestation and the inter-basestation separation.  Thus, we include inter-basestation separation in our definition of the load/coverage operating point.

## One-Way Internet packet delay

One-way Internet packet delay is defined as the time it takes for an IP packet to travel from the base station (server) to the server (base station).

# References

- IEEE C802.20-03/32, Selected Topics on Mobile System Requirements and Evaluation Criteria.

- IEEE C802.20-03/33r1, Criteria for Network Capacity.

- IEEE C802.20-03/35, Evaluation Methodology for 802.20 MBWA.

- IEEE C802.20-03/43, 802.20 Evaluation Methodology Strawman - 00.

- 3GPP2/TSG-C.R1002, “1xEV-DV Evaluation Methodology (V14)”, June 2003.

- A Corlett, D.I. Pullin and S. Sargood, “Statistics of One-Way Internet Packet Delays,” 53rd IETF, Minneapolis, March 2002.

- C802.20-03/43, “802.20 Evaluation Methodology Strawman (03/57 is ppt)”, IEEE 802.20 May 2003 Session.

- C802.20-03/13r1, “User Data Models for an IP-based Cellular  Network,” IEEE 802.20 March 2003 Session.

- C802.20-03/35, “Evaluation Methodology for MBWA”, IEEE 802.20 May 2003 Session.

- C802.20-03/53, “Operator Systems Requirements for MBWA”, IEEE 802.20 May 2003 Session.

- C802.20-03/46r1, “Channel Requirements For MBWA (Rev 1)S”, IEEE 802.20 May 2003 Session.

- IEEE 802.20 Requirements Document v2.

- D. Staehle et al, “Source Traffic Modeling of Wireless Applications,” Research Report, June 2000. available at: http://www3.informatik.uni-wuerzburg.de/TR/tr261.pdf

- A. Mena and J. Heidemann, “An empirical study of real audio traffic,” INFOCOM 2000. Proceedings. IEEE , volume: 1 , 26-30 March 2000. pp 101 -110 vol.1

- R. A. Bangun and E. Dutkiewicz, “Modelling multi-player games traffic,” Information Technology: Coding and Computing, 2000. Proceedings. International Conference on , 27-29 March 2000. pp: 228 –233

- C. Heyaime-Duverge and V. K. Prabhu, “Modeling action and strategy Internet-games traffic  
Vehicular Technology Conference, 2002. VTC Spring 2002., Vol. 3 , 6-9 May 2002, pp: 1405 -1409 vol.3

# Appendix A: 19 Cell Wrap-Around Implementation

The cell layout is wrap-around to form a toroidal surface to enable faster simulation run times [5-].  A toroidal surface is chosen because it can be easily formed from a rhombus by joining the opposing edges. To illustrate the cyclic nature of the wrap-around cell structure, this set of 19 cells is repeated 8 times at rhombus lattice vertices as shown in Figure 11. Note that the original cell set remains in the center while the 8 sets evenly surround this center set. From the figure, it is clear that by first cutting along the blue lines to obtain a rhombus and then joining the opposing edges of the rhombus can form a toroid. Furthermore, since the toroid is a continuous surface, there are an infinite number of rhombus lattice vertices but only a select few have been shown to illustrate the cyclic nature.

Figure 11 Wrap-around with ’9’ sets of 19 cells showing the toroidal nature of the wrap-around surface.

An example of the antenna orientations in case of a sectorized system is defined in Figure 12. For simplicity, the clusters in blue from Figure 11 have been deleted in this Figure. The distance from any MS to any base station can be obtained from the following algorithm: Define a coordinate system such that the center of cell 1 is at (0,0).  The path distance and angle used to compute the path loss and antenna gain of a MS at (x,y) to a BS at (a,b) is the minimum of the following:

a.	Distance between (x,y) and (a,b);

b.	Distance between (x,y) and (a+3R,b+83R2);

c.	Distance between (x,y) and (a−3R,b−83R2);

d.	Distance between (x,y) and (a+4.5R,b−73R2);

e.	Distance between (x,y) and (a−4.5R,b+73R2);

f.	Distance between (x,y) and (a+7.5R,b+3R2);

g.	Distance between (x,y) and ,

where R is the radius of a circle which connects the six vertices of the hexagon.

Figure 12: An example of the antenna orientations for a sectorized system to be used in the wrap-around simulation. The arrows in the Figure show the directions that the antennas are pointing.

Source: 3GPP TR25.892 v1.1.0

The actual coverage areas are determined by propagation, fading, antenna patterns, and other factors.

See Section 17.1 for definition of active users

See definition of spectrum allocation from the Terminology Annex of Requirements Document.

See definition of channel bandwidths from the Terminology Annex of Requirements Document.

Note that the set of 19 cells are only repeated for illustrating the cyclic nature of the wrap-around cell structure. The simulation only contains 19 cells and not 9 sets of 19 cells.