| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | IEEE 802.20 Evaluation Criteria - Version 16 |  |
| Date Submitted | 2005-6-10 |  |
| Source(s) | Jerry Upton Chair 802.20 | Voice: +1 847-692-2497 Email: jerry.upton@ieee.org |
| Re: | Updated Version of Evaluation Criteria document based upon Editor’s clean up of the document and agreements from Session #14, May 17-19, 2005 |  |
| Abstract | This document is a draft of the evaluation criteria document. In final form, it will reflect the consensus opinion of the evaluation criteria correspondence group. |  |
| Purpose | To provide an up-to-date version of the evaluation criteria document draft |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

IEEE P 802.20™/PD<insert PD Number>/V<insert version number>

Date: <June 10, 2005>

# Draft 802.20 Permanent Document

# <802.20 Evaluation Criteria – Ver 16>

This document is a Draft Permanent Document of IEEE Working Group 802.20. Permanent Documents (PD) are used in facilitating the work of the WG and contain information that provides guidance for the development of 802.20 standards.  This document is work in progress and is subject to change.

Contents

1	Overview	7

1.1	Scope	7

1.2	Purpose	7

1.3	Organization of the Document	7

2	Link level and System Level Analysis	9

3	Link level Modeling	9

3.1	Modeling assumptions	9

3.2	Performance metrics	9

4	Traffic Models for 802.20 System Simulations	10

4.1	Introduction	10

4.2	Context and Scope	10

4.3	Traffic Models	11

4.4	Traffic Mix	20

5	System Level Modeling	22

5.1	Cell layout	22

5.2	Higher Layer Protocol Modeling	23

5.3	Backhaul Network Modeling	30

5.4	Mobility Modeling	32

5.5	Control signaling modeling	35

6	Phased Approach for Technology Evaluation	35

6.1	Phase 1	36

6.2	Phase 2	36

7	Link-System Interface (LSI)	37

8	System Simulation Calibration	38

9	Channel Modeling	40

9.1	Channel Mix	40

10	RF Environment [Editor’s Note: Updated based on C802.20-04/64R4]	44

10.1	Radio Transceiver Characteristics	44

11	Link Budget	46

12	Equipment Characteristics	52

12.1	Antenna Characteristics	52

12.2	Hardware Characteristics	52

12.3	Deployment Characteristics	52

13	Output Metrics	52

13.1	System Capacity Metrics	52

14	Payload Based Evaluation	59

14.1	Capacity performance evaluation criteria	59

14.2	Payload transmission delay evaluation criteria	60

15	Fairness Criteria	60

16	Simulation and evaluation of various block assignments	61

17	References	62

18	Appendix A: Definition of terms	63

19	Appendix B: 19 Cell Wrap-Around Implementation	63

20	Appendix C: Fixed user locations for system level calibration	64

# <802.20 Evaluation Criteria>

# Overview

## Scope

This document describes the evaluation criteria used by the IEEE 802.20 working group [1-] to evaluate different candidate air interface proposals for the IEEE 802.20 standard.  This document and the IEEE 802.20 requirements document [2-] form the basis for decisions.

Although the IEEE 802.20 standard defines operations at the Link and Physical layer of the ISO Model, many of the criteria in this document extend to other ISO layers.  The evaluation criteria based on other ISO layers are for information use only.  Informational areas of this document are used when other methods are insufficient to determine an alternative.

## Purpose

The purpose of this document is to establish performance criteria and a framework in which candidate IEEE 802.20 technology-proposals should be evaluated.

## Organization of the Document

The organization of this document is depicted in the following block diagram. The evaluation task consists of two major activities, modeling and simulation. The entities that define the evaluation modeling are shown as functional blocks on the left hand side of the diagram. The corresponding section numbers are indicated within each block. The functional blocks on the right side of the diagram define the simulation activities.

Figure 1	Organization of the Evaluation Criteria Document

# Link level and System Level Analysis

A great deal can be learned about an air interface by analyzing its fundamental performance in a so-called Link-level settings consisting of one base station and one mobile user station.  The analysis can provide information on the system’s fundamental performance metrics such as: noise-limited range, peak data rate, maximum throughput, etc. The actual performance, in real-world settings, where multiple base stations are deployed in a service area and in the presence of a large number of active mobile users, can only be evaluated in a System-level analysis.   The extension of the link-level analysis methods to a system-level analysis may start with adding multiple users in a single-cell setting. This technique is generally straightforward and provides a mechanism for initial understanding of the multiple-access characteristics of the system. Ultimately, however, quantifying the system level performance, although difficult, carries with it the reward of producing results that are more indicative of the system performance.

Since system level results vary considerably with different propagation and interference environments as well as with the number and distribution of users within the cells. In addition, a system-level analysis should typically evaluate the impact on performance of many other fixed and stochastic factors. It is, thus, important that the assumptions and parameters used in the analysis be reported carefully lest the quoted network-level performance be misleading.

This document specifies detailed requirements for both the link-level and the system-level analyses.

# Link level Modeling

Single user link-level analysis is an analysis of the performance of a single user terminal mobile station in an assumed propagation and interference environments. As discussed in section 2, this is an important performance assessment tool for understanding the air interface that yields important information about the system including:

- the effectiveness of link-adaptation and power control,

- the noise-limited range,

- the SNR requirements to support various classes of service,

- the   sensitivity to multipath and fading.

Again, it should be emphasized that due to the variability and complexity of the propagation environment and the inter-cell interference, a single-user link-level analysis cannot be directly extrapolated to a system-level to determine the actual system performance.

## Modeling assumptions

The performance of modulation and coding schemes is to be evaluated using all channel environments associated with the channel models described in this document.

## Performance metrics

FER vs. SINR is generated by the product of link-level simulations.  Systems with adaptive modulation and coding should produce a set of curves (one curve per modulation and coding class).  A second family of curves is the link-level throughput vs. SINR.  The link-level throughput is derived by using the FER as given in:

Throughput=n×1−FERT

where “T” is the frame duration in seconds, and “n” is the number of information bits/frame supported by a modulation coding class.

# Traffic Models for 802.20 System Simulations

## Introduction

The Mobile Broadband Wireless Access (MBWA) systems will be designed to provide a broadband, IP-oriented connection to a wireless user that is comparable to wired broadband connections that are in use today. It is expected that there will be a mix of user applications, not unlike that of such wired systems. Further, the traffic characteristics and system requirements of the various applications can vary widely. The performance of such MBWA systems is thus very much dependant on the details of the applications and their traffic models. This is in contrast to cellular wireless voice systems where the performance studies focused on physical and link layer performance with a relatively simple traffic generation model. The purpose of this section is to provide detailed statistical traffic models that can be used as an input to generate packets in a simulation study of a MBWA system.

## Context and Scope

### User Device Scenarios – For Information Only

There can be various different user scenarios for MBWA systems, some of which we cannot foresee at this time. For purposes of illustration, we include some candidate scenarios to frame the context of our work. In all cases, the MBWA modem can either be built-in or supplied through a card or a peripheral device.

- Laptop user: The large and rich display capabilities can be expected to generate graphics-rich and multimedia-rich applications. In general, laptop users will provide the highest data volume demands due to the storage and battery capabilities of laptops. They can provide a full range of applications with perhaps less emphasis on voice and WAP applications. Except for special cases, they tend to be stationary during use.

- PDA user: The display, battery, and storage capabilities are less than that of laptops, and so they are expected to have somewhat less traffic volume. They can be very portable. They are typically used for Web browsing, e-mail, synchronization, video, and voice applications.

- Smartphone user: These devices are very portable and very constrained display and storage capabilities. It is expected that they will be oriented towards voice, WAP, and light video.

d)	Machine to machine (telematics, remote cameras etc.): These usage scenarios can have a wide range of characteristics. In some remote monitoring/control applications driven by specific events, the traffic is bursty. For remote surveillance using continuous video feeds, the traffic is more like streaming. This can be a potentially significant usage scenario for 802.20 systems, but the relevant traffic characteristics may not have received as much study as a applications with human users.

Since the various devices can have very distinct traffic characteristics, we will create multiple traffic models for different usage scenarios of an application.

For example, web browsing is likely to have different statistical characteristics for laptop and PDA scenarios. Rather than tie the models specifically to device types such as laptop and PDA, we will adopt multiple versions of a traffic model with generic names, e.g. Web Browsing A & Web Browsing B, or Web Browsing Heavy & Web Browsing Light. These could have different statistical functions, or different parameters for the same function.

### Basis for Traffic Models

Most traffic modeling work is based on measurements of real traffic, which are analyzed to generate usable statistical descriptions. These are typically used in computer simulations, but can also be used to generate packet traffic for a real system under test. Since MBWA is a future service that is similar to some existing wired systems, a lot of the basis of this section is the traffic modeling work done for wired systems. These provide a reasonable and realistic description of the potential user. Our approach is to use statistical models that can be used to generate a stream of packets that need to be transmitted over the system.

We realize that characteristics of user applications keep changing. At best, one can develop a reasonable consensus model that is useful for bringing some uniformity in comparisons of systems. In particular, it is known that user traffic patterns change as the network performance changes. Traffic modeling work has attempted to adjust to this trend. For example, some of the traffic models such as Web and FTP try to capture the essence of the user applications by describing the amount of data the user is trying to retrieve rather than specifying a packet stream.

We specifically do not use the trace-based approach where a real recorded stream of packets is played back for simulation. While traces can capture sophisticated details, such traces have details that are often very dependant on the system from which they were recorded, and do not provide flexibility for computer simulation work.

### Adaptive applications

Certain applications such as audio streaming sense the available bit rate of the channel and then adjust the amount of traffic that is transmitted. Certain multi-media sessions may employ content-adaptation of images or video based on network conditions. This directly changes the amount of data that is transmitted. The adaptive nature of applications can be incorporated into the traffic model. We do not perceive a strong need for the adaptive nature of an application to be incorporated as a dynamic feature of the traffic model. Such adaptive behavior can be addressed by using traffic models with different parameters and switching between them in an appropriate manner. Thus, adaptation of traffic characteristics based on network/device conditions is outside the scope of this modeling.

## Traffic Models

This section describes the traffic models in detail. Sections 4.3.1 and 4.3.2 clarify some aspects of the modeling approach and the remaining sections provide detailed models for traffic type listed in Table 1.

OPTION #1

Table 1 Characteristics of 802.20 Traffic Types

| Application | Traffic Category | Priority for Evaln. Group | Availability of suitable traffic model(s) | Different versions needed |
| --- | --- | --- | --- | --- |
| VoIP | Real-time | High | High | High-rate, low-rate |
| Web Browsing | Interactive | High | High | Heavy, Medium, Light |
| WAP | Interactive | High | High |  |
| FTP (File transfer) | Best-effort | High | Medium | Fixed/deterministic (for testing), Heavy, Light |
| Video-conference | Real-time | Medium | High | Heavy, Light |
| E-mail | Interactive/ Best-effort | Medium | Low | Heavy, Medium, Light, Non-interactive mode |
| Multimedia Messaging | Interactive | Medium | Medium |  |
| Instant Messaging | Interactive | Medium | Medium |  |
| Gaming | Interactive | Medium | Low |  |
| Audio streaming | Streaming | Medium | Low | High-rate, low-rate |
| Video streaming | Streaming | Medium | Medium | High-rate, low-rate |
| PDA remote synch | Best-effort | Medium | Low |  |
| File-sharing | Best-effort | Low | Low |  |
| Broadcast/multicast | Best-effort | Low | Low | High-rate, low-rate |
| Telematics | Best-effort/ Real-time | Low | Low |  |

OPTION #2

From Dan Gal’s contribution C802.20-05/25

Table 2 Characteristics of 802.20 Traffic Types

{D. Gal’s proposed changes: 1. sort the table rows by Traffic-Category 2. reduce the number of simulated applications (a short-list is shown in bold font) }

| # | Application | Traffic Category | Priority for Evaluation | Availability of suitable traffic models | Testing Variants |
| --- | --- | --- | --- | --- | --- |
| 1 | FTP | Best-effort | High | Medium | Fixed /deterministic Heavy, Light |
| 2 | PDA remote synch | Best-effort | Low | Low |  |
| 3 | File-sharing | Best-effort | Low | Low |  |
| 4 | Broadcast / Multicast | Best-effort | Low | Low | High-rate, low-rate |
| 5 | Telematics | Best-effort/ Real-time | Low | Low |  |
| 6 | E-mail | Interactive/ Best-effort | Medium | Low | Heavy, Medium, Light, Non-interactive mode |
| 7 | Web Browsing | Interactive | High | High | Heavy, Medium, Light |
| 8 | WAP | Interactive | High | High |  |
| 9 | Multimedia Messaging | Interactive | Medium | Medium |  |
| 10 | Instant Messaging | Interactive | Medium | Medium |  |
| 11 | Gaming | Interactive | Medium | Low |  |
| 12 | Audio Streaming | Streaming | Medium | Low | High-rate, low-rate |
| 13 | Video Streaming | Streaming | Medium | Medium | High-rate, low-rate |
| 14 | VoIP | Real-time | High | High | High-rate, low-rate |
| 15 | Video Telephony | Real-time | Medium | High | Heavy, Light |

### User/Traffic Modeling Approach

One of the objectives of a modeling and simulation exercise is to determine the number of users a MBWA system can support. The proposed approach here is to have traffic models for a user who is maintaining a session with transmission activity. These can be used to determine the number of such registered users that can be supported. This document does not address the arrival process of such registered users, i.e. it does not address the statistics of subscribers that register and become active.

Modeling of an aggregated load from a number of user nodes for background loading purposes may not be feasible for a wireless network. Such an abstraction is particularly difficult with adaptive antenna technologies and systems with complex channel dependencies. So, our traffic models apply to one user terminal.

### Packet Generation

In some of the traffic models, there is a statistical description of the workload or the content of the application rather than the actual packet stream. This is consistent with the state of the art in evaluation of multi-service data systems. For example, the Web browsing model describes the Web pages and the timing between the Web pages. Depending on the details of the underlying TCP model (e.g. MTU size, max receive window) and the HTTP protocol version (HTTP v1.0 versus HTTPv1.1 versus HTTPv2.0), the actual stream of packets will change. In some cases, as in the Voice models, the model may describe the packet stream more directly.

### Web Browsing

Web browsing is the dominant application for broadband data systems, and has been studied extensively. See references [7- 9-10-14-]

The parameters for the web browsing traffic are as follows:

SM: Size of the main object in a page

SE: Size of an embedded object in a page

Nd: Number of embedded objects in a page

Dpc: Reading time

Tp: Parsing time for the main page

Table 3 HTTP Traffic Model Parameters

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Main object size (SM) | Truncated Lognormal | Mean = 10710 bytes Std. dev. = 25032 bytes Minimum = 100 bytes Maximum = 2 Mbytes | fx=12πσxexp−lnx−μ22σ2,x≥0σ=1.37,μ=8.35 |
| Embedded object size (SE) | Truncated Lognormal | Mean = 7758 bytes Std. dev. = 126168 bytes Minimum = 50 bytes Maximum = 2 Mbytes | fx=12πσxexp−lnx−μ22σ2,x≥0σ=2.36,μ=6.17 |
| Number of embedded objects per page (Nd) | Truncated Pareto | Mean = 5.64 Max. = 53 | fx=αkαxα+1,k≤x<mfx=kmα,x=mα=1.1,k=2,m=55Note: Subtract k from the generated random value to obtain Nd |
| Reading time (Dpc) | Exponential | Mean = 30 sec | fx=λe−λx,x≥0λ=0.033 |
| Parsing time (Tp) | Exponential | Mean = 0.13 sec | fx=λe−λx,x≥0λ=7.69 |

Note: When generating a random sample from a truncated distribution, discard the random sample when it is outside the valid interval and regenerate another random sample.

### FTP

In FTP applications, a session consists of a sequence of file transfers, separated by reading times.  The two main parameters of an FTP session are:

: the size of a file to be transferred

: reading time, i.e., the time interval between end of download of the previous file and the user request for the next file.

The underlying transport protocol for FTP is TCP. The parameters for the FTP application session are described in Table 4.

Table 4 FTP Traffic Model Parameters

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| File size (S) | Truncated Lognormal | Mean = 2Mbytes Std. Dev. = 0.722 Mbytes Maximum = 5 Mbytes | fx=12πσxexp−lnx−μ22σ2,x≥0σ=0.35,μ=14.45 |
| Reading time (Dpc) | Exponential | Mean = 180 sec. | fx=λe−λx,x≥0λ=0.006 |

### Voice (VoIP)

The voice traffic model will be implemented as voice over IP (VoIP).  Voice will in general follow a Markov source model with different rates (full rate, half rate, etc) with a corresponding set of transition probabilities between different rates.

Editor’s Notes:

Proponents with other Options need to supply text for this section.

Contribution C802.20-05/29 proposes voice quality testing.

One voice codec, AMR, for simulations was proposed referencing IETF RFC 3267.

Overall resolution for this section is required.

### Video (Videotelephony/Videoconferencing)

Editor’s Note: Need a Contribution proposing Text OR Delete the section

### Audio streaming

This can be an important class of traffic. It has received relatively less attention in the modeling community.  (See [15-])

Editor’s Note: Need a Contribution proposing Text OR Delete the section

### Video streaming

The following section describes a model for streaming video traffic on the forward link.  Figure 2 describes the steady state of video streaming traffic from the network as seen by the base station.  Latency of starting up the call is not considered in this steady state model.

Figure 2 Near Real-Time Video Traffic Model

A video streaming session is defined as the entire video and associated audio streaming call time, which is equal to the simulation time for this model.

Each frame of video data arrives at a regular interval T determined by the number of frames per second (fps).  Each frame is decomposed into a fixed number of slices, each transmitted as a single packet.  The size of these packets/slices is distributed as a truncated Pareto.  Encoding delay, Dc, at the video encoder introduces delay intervals between the packets of a frame.  These intervals are modeled by a truncated Pareto distribution. The parameter TB is the length (in seconds) of the de-jitter buffer window in the mobile station used to guarantee a continuous display of video streaming data.  This parameter is not relevant for generating the traffic distribution but is useful for identifying periods when the real-time constraint of this service is not met.  At the beginning of the simulation, it is assumed that the mobile station de-jitter buffer is full with (TB x source video data rate) bits of data.  Over the simulation time, data is “leaked” out of this buffer at the source video data rate and “filled” as forward link traffic reaches the mobile station.  As a performance criterion, the simulation shall record the length of time, if any, during which the de-jitter buffer runs dry.

Option 1:

The de-jitter buffer window for the video streaming service is 5 seconds.

Option 2:

The de-jitter buffer window for the video streaming service is a maximum of 5 seconds.

[Note: Need to confirm if the de-jitter buffer window size of 5 seconds needs to be changed for the higher data rate]

Using a source rate of 64 kbps, the video traffic model parameters are defined Table 5.

Table 5 Near Real-Time Video Traffic Model Parameters

| Information types | Inter-arrival time between the beginning of each frame | Number of packets (slices) in a frame | Packet (slice) size | Inter-arrival time between packets (slices) in a frame |
| --- | --- | --- | --- | --- |
| Distribution | Deterministic (Based on 10fps) | Deterministic | Truncated Pareto (Mean= 50bytes, Max= 125bytes) | Truncated Pareto (Mean= 6ms, Max= 12.5ms) |
| Distribution Parameters | 100ms | 8 | K = 20bytes  = 1.2 | K = 2.5ms  = 1.2 |

### Wireless Multi-Party Gaming Traffic

Wireless gaming is an important application that should be considered in 802.20 system evaluation. Therefore inputs are required on mobile wireless gaming models Some types of multi-player games may have demanding requirements on response times.

Note 1: Clarification is required - - Should the traffic Model address Single Party Gaming or Multi-player Gaming.

Note 2: Options from contribution C802.20-04/86 and C802.20-05/06 are included below.

OPTION 1: Modify 3GPP2 model, to include DL characteristics as in Faber [2002]:

This section describes a model for mobile network gaming traffic on the forward link and reverse link.  This model is a combination of a standardized reverse link model (see cdma2000 Evaluation Methodology, C.P1002, Version 0.3, July 2004) and a forward link model developed from the research literature.

#### Reverse Link

Table 6 describes the parameters for the mobile network gaming traffic on the reverse link.

Table 6 Mobile Reverse Link network gaming traffic model parameters

| Component | Distribution | PDF and generation method |
| --- | --- | --- |
| Initial packet arrival | Uniform (a=0, b=40ms) | f(x)=1b−aa≤x≤b |
| Packet arrival | Deterministic (40ms) |  |
| Packet size | Extreme (a=45 bytes, b = 5.7) | f(x)=1be−x−abe−e−x−ab,b>0 X=a−bln−lnY, Y∈U(0,1) Because packet size has to be integer number of bytes, the largest integer less than or equal to X is used as the actual packet size. |
| UDP header | Deterministic (2bytes) |  |

This model uses Largest Extreme Value distribution for the packet size. For cellular system simulation, 2-byte UDP header (after header compression) should be added to the packet size X. Because the packet size has to be an integer number of bytes, the largest integer less than or equal to X is used as the actual packet size. To simulate the random timing relationship between client traffic packet arrival and reverse link frame boundary, the starting time of a network gaming mobile is uniformly distributed within [0, 40ms].

A maximum delay of 160ms is applied to all reverse link packets, i.e., a packet is dropped by the mobile station if any part of the packet have not started physical layer transmission, including HARQ operation, 160ms after entering the mobile station buffer.. A packet can start physical layer transmission at the 160ms time instant. Packet dropping should be the last operation of mobile station buffer management, if any, at any time instant. The packet delay of a dropped packet is counted as 180ms.

A mobile network gaming user is in outage if the average packet delay is greater than 60ms. The average delay is the average of the delay of all packets, including the delay of packets delivered and the delay of packets dropped.

#### Forward Link Model

Table 7 describes the parameters for the mobile network gaming traffic on the forward link.

Table 7 Forward Link network gaming traffic model parameters

| Component | Distribution | PDF and generation method |
| --- | --- | --- |
| Initial packet arrival | Uniform (a=0, b=40ms) | f(x)=1b−aa≤x≤b |
| Packet arrival | Extreme (a=55, b=6) |  |
| Packet size | Extreme (a=120 bytes, b = 36) | f(x)=1be−x−abe−e−x−ab,b>0 X=a−bln−lnY, Y∈U(0,1) Because packet size has to be integer number of bytes, the largest integer less than or equal to X is used as the actual packet size. |
| UDP header | Deterministic (2bytes) |  |

This model uses Largest Extreme Value distribution for the packet size. For cellular system simulation, a 2-byte UDP header (after header compression) should be added to the packet size X. Because the packet size has to be an integer number of bytes, the largest integer less than or equal to X is used as the actual packet size. To simulate the random timing relationship between client traffic packet arrival and reverse link frame boundary, the starting time of a network gaming mobile is uniformly distributed within [0, 40ms].

A maximum delay of 160ms is applied to all reverse link packets, i.e., a packet is dropped by the mobile station if any part of the packet have not started physical layer transmission, including HARQ operation, 160ms after entering the mobile station buffer.. A packet can start physical layer transmission at the 160ms time instant. Packet dropping should be the last operation of base station buffer management, if any, at any time instant. The packet delay of a dropped packet is counted as 180ms.

A mobile network gaming user is in outage if the average packet delay is greater than 60ms. The average delay is the average of the delay of all packets, including the delay of packets delivered and the delay of packets dropped.

OPTION 2: Adopt or modify 3GPP model

OPTION 3: Combine the best of the two models

OPTION 4: Develop an 802.20 model based on more recent literature

### Full buffers (Infinite backlog) model

In the full buffers (Infinite backlog) user traffic model, all the users in the system always have data to send or receive. In other words, there is always a constant amount of data that needs to be transferred, in contrast to bursts of data that follow an arrival process. This model allows the assessment of the spectral efficiency of the system independent of actual user traffic distribution type.

## Traffic Mix

OPTION #1

A MBWA system is expected to have mix of traffic types. There can be different types of usage scenarios (multi-service v. single-type), different types of devices (laptops v. PDAs), different levels of use (intense v. light)., and different demands on response times (real-time v. best-effort). The previous sections are primarily concerned with the traffic models for each of the potential traffic types. As discussed in the previous section, these are based on statistical analysis of measured traffic to extract some invariant patterns that are not very dependant on the specific system. It is more difficult to describe a similar invariant mix of traffic types since these tend to depend more heavily on the type of system and the mix of device/user types.

In the context of a system evaluation using traffic models, the specific mix of traffic types will emphasize different aspects of the system performance, e.g. sustained throughput for file downloads v. faster response times for interactive applications.

Table 8 Traffic mix: percentage of different Traffic Types

| Application | Percentage |
| --- | --- |
| VoIP | TBD |
| Web Browsing | TBD |
| WAP | TBD |
| FTP (File transfer) | TBD |
| Video-conference | TBD |
| E-mail | TBD |
| Multimedia Messaging | TBD |
| Instant Messaging | TBD |
| Gaming | TBD |
| Audio streaming | TBD |
| Video streaming | TBD |
| PDA remote synch | TBD |
| File-sharing | TBD |
| Broadcast/multicast | TBD |
| Telematics | TBD |

OPTION #2 (Dan Gal Contribution C802.20-05/25)

A MBWA system is expected to support a mix of simultaneous traffic types. There can be different types of usage scenarios (multi-service v. single-type), different types of devices (laptops v. PDAs), different usage levels (intense v. light) and different delay/latency requirements (real-time v. best-effort).

The previous sections are primarily concerned with the traffic models for each of the potential traffic types. As discussed in the previous section, these models are based on statistical analysis of measured traffic that yielded some invariant patterns that are not very dependant on the specific system. It is more difficult to describe a similar invariant mix of traffic types since these tend to depend more heavily on the type of system and the actual deployment mix of user device types.

In the context of system performance evaluation, using traffic models, the specific traffic-mix should emphasize different aspects of the system performance, e.g. sustained throughput for file downloads v. faster response times for interactive applications.

A short list of representative applications and their corresponding percentage in a simulated system-wide traffic mix is shown in Table 7.

Table 9 Traffic mix: percentage of different Traffic Types

| Traffic Category | Application | Percentage ( % ) |
| --- | --- | --- |
| Best Effort | FTP | 10 |
|  | E-mail | 10 |
| Interactive | Web browsing | 20 |
|  | Instant Messaging | 5 |
|  | Gaming | 5 |
| Streaming | Video streaming | 10 |
| Real-time | VoIP | 25 |
|  | Video Telephony | 15 |

Editor’s Note:

The Definition of Traffic Mix needs clarification.

Proposed Definition from Session 14:

For Simulation purposes, “traffic mix” refers to the percentage of users in the system generating a particular type of traffic. In this context, each user is assumed to be generating only one type of traffic, recognizing that in an actual network a single user’s terminal could support multiple applications and generate several types of traffic simultaneously.

# System Level Modeling

In order to accurately model the traffic, physical and MAC layer dependencies between the uplink (UL) and the downlink (DL), the system simulations include both UL and the DL in a fully duplex fashion in the same simulation run.

## Cell layout

For evaluation purposes, the system consists of 19 tri-sector cells, each with an imaginary hexagonal coverage area. Mobile stations are uniformly dropped into the 19-cell system.

All 19 cells are simulated using a cell wrap-around technique (See Appendix B) and the statistics are collected from all the cells.

### Distribution of users

Most users of wireless systems experience very good link-quality near the base station.  For this reason, the distribution of users throughout the network is integral to the quoting of network-level performance results.  Absent the desire to highlight specific abilities of an air interface, users should be distributed uniformly throughout each cell of the network.

### User usage model

[Note from CC on April 19: (i) Distributions need to be the same for the evaluation of various proposals; (ii) As a reference, similar distributions have been proposed in section 9.2: Channel mix]

Editor’s Note: Clarification of this section needed or Delete.

The following user terminal usage parameters must be specified:

- distribution of indoor vs. outdoor users

- mobility profile across the user base

## Higher Layer Protocol Modeling

Different applications potentially use different higher layer protocols such as HTTP, RTP, TCP and UDP. In the 802.20 system evaluation we only consider TCP due to the following two major reasons. First, a large number of applications use TCP as the transport protocol. In fact, a large fraction of the Internet traffic consists of TCP traffic. Second, the TCP can have major impact on system performance due to its slow start and congestion control mechanisms.

### TCP Model

Many Internet applications including Web browsing and FTP use TCP as the transport protocol.  Therefore, a TCP model is introduced to more accurately represent the distribution of TCP packets from these applications.

#### TCP Connection Set-up and Release Procedure

The TCP connection set-up and release protocols use a three-way handshake mechanism as described in Figure 3 and Figure 4. The connection set-up process is described below:

- The transmitter sends a 40-byte SYNC control segment and wait for ACK from remote server.

- The receiver, after receiving the SYNC packet, sends a 40-byte SYNC/ACK control segment.

- The transmitter, after receiving the SYNC/ACK control segment starts TCP in slow-start mode (the ACK flag is set in the first TCP segment).

The procedure for releasing a TCP connection is as follows:

- The transmitter sets the FIN flag in the last TCP segment sent.

- The receiver, after receiving the last TCP segment with FIN flag set, sends a 40-byte FIN/ACK control segment.

- The transmitter, after receiving the FIN/ACK segment, terminates the TCP session.

Figure 3: TCP connection establishment and release for Uplink data transfer

Figure 4: TCP connection establishment and release for Downlink data transfer

#### TCP slow start Model

The amount of outstanding data that can be sent without receiving an acknowledgement (ACK) is determined by the minimum of the congestion window size of the transmitter and the receiver window size.  After the connection establishment is completed, the transfer of data starts in slow-start mode with an initial congestion window size of 1 segment.  The congestion window increases by one segment for each ACK packet received by the sender regardless of whether the packet is correctly received or not, and regardless of whether the packet is out of order or not.  This results in exponential growth of the congestion window i.e. after n RTTs (Round Trip Times), the congestion window size is 2n. segments

##### UL (Uplink) slow start model

This UL slow start process is illustrated in Figure 5. The round-trip time in Figure 5, rt, consists of two components, see Table 8:

rt  = u + l

where u = the sum of the time taken by a TCP data segment to travel from the base station router to the server plus the time taken by an ACK packet to travel from the server to the client; l = the transmission time of a TCP data segment over the access link from the client to the base station router. u is further divided into two components; 2 = the time taken by a TCP data segment to travel from the base station router to the server plus the time taken by an ACK packet to travel from the server back to the base station router and 3 = the time taken by the ACK packet to travel from the base station router to the client.

Figure 5: TCP Flow Control During Slow-Start; l = Transmission Time over the Access Link (UL); rt = Roundtrip Time

Table 10 Delay components in the TCP model for the UL upload traffic

| Delay component | Symbol | Value |
| --- | --- | --- |
| The transmission time of a TCP data segment over the access link from the client to the base station router. | 1 | Determined by the access link throughput |
| The sum of the time taken by a TCP data segment to travel from the base station router to the server and the time taken by an ACK packet to travel from the server to the base station router. | 2 | See 5.3.1 |
| The time taken by a TCP ACK packet to travel from the base station router to the client. | 3 | See 5.3.1 |

##### DL (Downlink) slow start model

This DL slow start process is illustrated in Figure 6. The round-trip time in Figure 6, rt, consists of two components, see Table 9:

rt  = d + 4

where d = the sum of the time taken by an ACK packet to travel from the client to the server and the time taken by a TCP data segment to travel from the server to the base station router; 4 = the transmission time of a TCP data segment over the access link from the base station router to the client. d is further divided into two components; 5 = the time taken by a TCP ACK to travel from the base station router to the server plus the time taken by a TCP packet to travel from the server back to the base station router and 3 = the time taken by the TCP packet to travel from the base station router to the client.

Figure 6 TCP Flow Control During Slow-Start; l = Transmission Time over the DL; rt = Roundtrip Time

Table 11 Delay components in the TCP model for the DL traffic

| Delay component | Symbol | Value |
| --- | --- | --- |
| The transmission time of a TCP data segment over the access link from the base station router to the client. | 4 | Determined by the access link throughput |
| The sum of the time taken by a TCP ACK to travel from the base station router to the server and the time taken by TCP data packet to travel from the server to the base station router. | 5 | See 5.3.1 |
| The time taken by a TCP data segment to travel from the base station router to the client. | 6 | See 5.3.1 |

From Figure 5 and Figure 6, it can be observed that, during the slow-start process, for every ACK packet received by the sender two data segments are generated and sent back to back.  Thus, at the mobile station (base station), after a packet is successfully transmitted, two segments arrive back-to-back after an interval u = 2 + 3 ( d = 5 + 6).  Based on this observation, the packet arrival process at the mobile station for the upload of a file is shown in Figure 7.  It is described as follows:

- Let S = size of the file in bytes. Compute the number of packets in the file, N = S/(MTU-40). Let W = size of the initial congestion window of TCP. The MTU size is fixed at 1500 bytes

- If N>W, then W packets are put into the queue for transmission; otherwise, all packets of the file are put into the queue for transmission in FIFO order. Let P=the number of packets remaining to be transmitted beside the W packets in the window. If P=0, go to step 6

- Wait until a packet of the file in the queue is transmitted over the access link

- Schedule arrival of next two packets (or the last packet if P=1) of the file after the packet is successfully ACKed.  If P=1, then P=0, else P=P-2

- If P>0 go to step 3

- End.

Figure 7 Packet Arrival Process at the mobile station (base station) for the upload (download) of a File Using TCP

## Backhaul Network Modeling

### Network Delay model

The one-way Internet packet delay is modeled using a shifted Gamma distribution [8-] with the parameters shown in Table 12. The packet delay is independent from packet to packet.

Table 12 Parameters for the shifted Gamma Distribution

| Scale parameter () | 1 |
| --- | --- |
| Shape parameter () | 2.5 |
| Probability density function (PDF) | f(x)=(xα)β−1e−xaα⋅Γ(β) (.) is the gamma function |
| Mean |  |
| Variance |  |
| Shift | See Table 13 |

Two values, 7.5ms and 107.5ms are used for the shift parameter in order to model the domestic routes and the International routes respectively.  The users’ routes are selected randomly at the time of drop with the distribution shown in Table 13.

Table 13 Shift parameter for the Domestic and International IP routes

| IP Route Type | Percentage of users | Shift parameter | Mean one-way IP packet delay |
| --- | --- | --- | --- |
| Domestic | 80% | 7.5ms | 10ms |
| International | 20% | 107.5ms | 110ms |

### Network Loss model

The transmission of IP packets between the base station (server) and the server (base station) is assumed error free.

Table 14 Internet Loss Model

| IP packet error rate | 0% (lossless packet transmission) |
| --- | --- |

## Mobility Modeling

### Editor’s Note: Adopted from Contribution 05/10r2 and the contributor agreed to check action items, though not noted in this document.

All terminals except one are fixed.  The mobility related performance metrics are computed only for this mobile terminal.

Mobility Model: The movement of the single mobile terminal is constrained to one of the following paths.

- Path 1: Move from A to B along line joining the cells

- Path 2: Move from A to B with “around the corner” effect that causes rapid signal loss from A, signal gain to B. (built into the propagation model)

- Path 3: Move along cell edge

Cells A and B are two cells in the center of the simulation region (cells 1 and 2 of the cell layout in the appendix).

Figure 8	Path of Mobile in models 1, 2 and 3

The propagation seen in each of the models is shown in the following figures.  The curved lines in the figures include shadow fading, while the straight lines include only path loss. Mobility models 1 and 3 are computed using the path loss and shadowing parameters defined in other parts of the document. Mobility model 2 assumes that there is a sudden propagation loss of EdgeLoss dB as the terminal moves across the cell boundary. This stringent model is useful to test the robustness of handoff signaling.

Figure 9	Propagation for Mobility Path 1

Figure 10	Propagation for Mobility Path 2

Figure 11	Propagation for Mobility Path 3

Table 15	Parameters for the Mobility Model

| Parameter Name | Interpretation | Value |
| --- | --- | --- |
| R | Distance between A and B | As in system sim. |
| EdgeLoss | Sudden propagation loss at cell edge | 3, 6, 9 dB |
| V | Mobile Velocity | 5, 30, 120 kmph |
| Dcorr | Shadow Fading Corr. Distance | 30 m |

Simulation Procedure: For parameters such as cell size, terminal density and channel models, the simulation follows the simulation methodology defined elsewhere in the document.

For all channels relevant to the mobility scenario, a realistic channel load shall be simulated. For example,

- Common channels (e.g. paging): If paging performance is being measured, and pages are carried on a separate channel (say a FDM channel), then all sectors shall be assumed to transmit at a certain paging rate. Thus pages from the sector of interest will be subject to realistic interference from other sectors. Further, if a proposal requires that the paging channel and some other channel (say pilot channel) are not orthogonal, then the other channel load must be simulated in association with the paging channel.

- Unicast channels: For messages such as pilot reports and handoff direction, the message may experience interference from other sectors or users. To obtain a realistic model for this interference, all users in the system shall be modeled as full buffer users on both the uplink and downlink.

Depending on whether the channels being simulated used separate coding or modulation schemes, the simulation results may be based on separate link curve results for each separate channel (e.g., paging channel, pilot channel etc).

Simulation Realizations and Averaging: The simulation shall construct realizations using the following rules

- Each realization shall consist of independent positions and channels for the stationary terminals

- Each realization shall consist of independent channel realizations along the mobile terminal’s path.

- Internal random variables (if any) that govern any signaling event (such as exponential backoff) shall be drawn independently for each realization.

The performance metrics shall be obtained by averaging across each realization.

Mobile Channel Modeling: The channel for the mobile terminal shall be modeled by adding a fast fading model on top of a first order auto-regressive shadow fading model along the mobile terminal’s path. The fast fading model shall use the mobile velocity being studied.

Results: Results shall be presented in the form of values of the Connected State Handoff Metrics, and the Power Save State Handoff metrics, as defined in the beginning of this section. Separate metric values shall be given for the paths 1, 2 and 3. Further, for path 3, results shall be given for all EdgeLoss values given in the table above.

## Control signaling modeling

Editor’s Note: This section needs contributions or a decision to delete.

### DL signaling models

<For example, models for MAC state transition messages and scheduling grant transmission etc.>

### UL signaling models

<For example, models for access channel, ACK and channel quality Feedback etc.>

# Phased Approach for Technology Evaluation

The 802.20 evaluation will be structured in two phases with each phase progressively adding more complexity. The evaluation work for each proposal may then be compared at each phase to ensure a progressive "apples to apples" comparison of proposals. This structured approach will also provide performance metrics for the physical and link layer performance early rather than later in the evaluation process. A summary of items simulated in phase 1 and phase 2 is provided in Table 16. The items marked with a ‘X’ are included in the corresponding phase. The details of phase 1 and phase 2 are described below.

## Phase 1

Phase 1 of the evaluation will consist of:

- Items/issues/criteria that are required for the calibration of simulations

- Items/issues/criteria that will draw out the important differences between the various proposals that cannot be otherwise inferred.

The goals at the end of phase 1 are, first, to achieve confidence that different simulation models are calibrated and, second, to present fundamental performance metrics for the physical and link layer of various proposals.

6.1.2  Phase 1 Calibration

The system level calibration shall follow the procedures described in section 8.

Editor’s Note: Also see Contributions C802.20-04/86r6 and C802.20-05/30

## Phase 2

Table 16 Phased Approach

| Items |  | Phase I | Phase II |
| --- | --- | --- | --- |
| Full-Duplex system simulations with 19 tri-sector cells layout |  | X | X |
| System Simulation calibration |  | X | TBD |
| Applications | Full Buffers | X | TBD |
|  | VoIP (RTP) |  | X |
|  | Web Browsing (TCP) |  | X |
|  | WAP |  | TBD |
|  | FTP (File transfer) (TCP) |  | X |
|  | Video-conference |  | TBD |
|  | E-mail |  | TBD |
|  | Multimedia Messaging |  | TBD |
|  | Instant Messaging |  | TBD |
|  | Gaming (TCP) |  | X |
|  | Audio streaming |  | TBD |
|  | Video streaming (RTP) |  | X |
|  | PDA remote synchronization |  | TBD |
|  | File-sharing |  | TBD |
|  | Broadcast/multicast (RTP/UDP) |  | X (TBD) |
|  | Telematics |  | TBD |
| Channel Models | Suburban macro, 3 Km/h pedestrian B, 100% | X | TBD |
|  | Suburban macro, 120Km/h Vehicular B, 100% | X | TBD |
|  | Other channel models |  | X (TBD) |
| Transmission Control Protocol (TCP) modeling |  |  | X |
| Network delay and loss model |  |  | TBD |
| Mobility (i.e. Handoff) model |  |  | (TBD) |
| Control signaling models |  |  | X (TBD) |
| RF characteristics |  | X | X |
| Link Budget |  | X | X |

# Link-System Interface (LSI)

An interface between link and system simulations is required because the link and system simulations are performed separately (the simulation complexity would be very high if joint link and system simulations are required).

Using the actual link curves is the default methodology for the link-system interface. The link curves can always be used even if an agreement on a common methodology is reached.  If a common methodology is defined, then no justification is required from the proponent. In the absence of a common methodology, a technology specific methodology can be used if provided with full verification subject to the satisfaction of the group

# System Simulation Calibration

Editor’s Note: To be updated with the agreed upon version of the proposed text in contribution C802.20-04/83rX. Also see contribution C802.20-05/30.

The purpose of system-level calibration is to ensure that, under a set of common assumptions and models, the simulator platforms that will be used by various proponents can produce results that are similar. The calibration procedures and metrics specified in this section are intended to be as technology-independent as possible.

Simulation assumptions:

The link budget in section 11 and the channel models adopted for phase 1 evaluation should be used in the calibration.

- Step 1:  Deterministic Calibration

The purpose of deterministic calibration is to assure that the basic configuration and layout of the simulation environment is coded in accordance with the evaluation criteria. The configuration of the simulation scenario is characterized by the following parameters:

- Base Station (BS) to BS distance: 2.5 km

- Path loss model as specified in the channel model document for suburban macro, urban micro cells etc

- For the forward link, maximum C/I = 30 dB , where C/I is defined as the ratio of carrier traffic power to the total interference and noise power at the receiver

- Mobiles are put in deterministic locations in each sector. For instance, 3 mobiles in each sector, located at (-60, R/2), (0,R/2) and (60, R), respectively, where (theta, r) refers to the polar coordination system of the sector with the reference direction (theta=0) being the antenna main lobe direction and the maximum radius of the cell being (r=R).

- A single antenna for BS and for MS, respectively.

Results of C/I and propagation time delay for each mobile are recorded in a spreadsheet, for which a common definition of numbering scheme need be set. Such a number scheme could be, for example: a=index of the sector (0,1,2…,18) , b=index for location within the sector (0,1,2 in counter clock-wise, in case of 3 mobiles per sector)

  - Step 2:  Monte Carlo Simulation

After the deterministic calibration succeeds, the second step is to perform simple stochastic calibration. The random parameters will be added incrementally into the parameters assumed in 6.1.2.1:

- Lognormal shadowing standard deviation as specified in the channel models document; the actual fading value is generated at the beginning of the simulation and remain constant during the simulation.

- At first 10 users/sector at fixed locations as specified in Appendix C. Then, 30 users/sector uniformly distributed over each sector of the 19 3-sectored cells (assuming the wrap-around model specified in Appendix A), where area within the minimum distance around the base station, as required by the path loss model used, is prohibited. The locations of the mobiles are fixed during the simulation.

- No admission control

- Each user selects the sector with the best downlink long-term channel gain (i.e., excluding short-term fading) as the serving sector.

- Full-buffer traffic model

- Maximum C/I = 30 dB , where C/I is defined as the ratio of carrier traffic power to the total interference and noise power at the receiver

- No soft handoff  for the calibration

- No power control on the downlink

- Perfect slow power control on the uplink with the same target long-term average received power for all users. The user with the highest path and shadowing losses is allowed to transmit at the maximum power. The corresponding long-term average received power at the base station is used as the target long-term average received power for all users in the same sector. That is, let PR be the target average received power. The transmit power of user k, PT,k, is given by PT,k = PR/Gk, where Gk is a long-term average channel gain (i.e., excluding short-term fading) of user k.

- Simulation duration : 10 seconds

- Time interval for fast fading channel update & (C/I) data collection: 40 ms for MS speed ~3 km/h, 1 ms for MS speed ~120 km/h.

As output, the cumulative distribution (CDF) plots of user C/I, for users in all sectors and cells, and for both Downlink and Uplink, shall be provided for comparison. Separate CDF plots shall be provided for the results of fixed users case and the randomly distributed users case.

    - Outputs for calibration

The simulation results from the calibration should include the following:

- Cumulative distribution (CDF) plots of user C/I, for both Downlink and Uplink, at time = 0 (i.e., excluding short-term fading).

- Cumulative distribution plots of user’s distance from the serving sector. Due to log-normal shadowing, a user may end up being served by a sector that is not geographically closest to it.

- Spatial distribution of user C/I, i.e., a plot of C/I distribution as a function of users’ location in the 19 cells

- Cumulative distribution plots of user C/I for both Downlink and Uplink, at the end of the simulation, including all the C/I samples collected over the simulation duration.

- Relative uplink inter-sector over intra-sector received power (f). Definef=IocIor , where Ior is the average received power from intra-sector users and Ioc is the average received power from users outside the sector. The contribution from thermal noise is excluded from the calculation. All users are given the same system resources on the uplink.

      - Further calibration

The traffic models should be calibrated by plotting and comparing the CDF of packet sizes, inter-packet arrival times and related traffic model parameters, before the start of evaluation based on those models.

Depending on the process of technology evaluation and selection, further calibration may be defined during the evaluation process, e.g., by using a common set of link-level performance curves, see Section 3 of this document.

# Channel Modeling

The channel models, associated parameters and parameter values, used to describe channel environment to be used in performance evaluation simulations are described in the IEEE 802.20 Channel Models Document [18-], which is incorporated herein by reference.

## Channel Mix

<Percentage of different Channel types>

OPTION # 1: Delete the section  {Editor’s Note: Straw Poll at May Interim was 10 Yes to delete and 1 No}

OPTION # 2:

Based on the above four scenarios of channel environment, each simulated mobile user in the cell sites can be assigned a channel model with parameter values that are representative of the corresponding channel environment as described in Table 3.1 of [Error: Reference source not found].

In the original document for Spatial Channel Model [19-], which is the basis of the IEEE 802.20 MIMO channel models document, the path delays for Macro cells are modeled as exponential distributions whereas those for the Micro cells are modeled as uniform distributions.

- Suburban Macro cell

According to the descriptions in Section 3.4 of [18-] and Section 3.3 of [19-], each user will be assigned a channel model which is generated based on the probability distribution with the parameters shown in Table 3.1 of [18-]. Thus, a channel mix can be obtained by specifying a percentage mix of mobile user speeds. Fig. 1 shows a distribution of mobile user speeds that has been used in the evaluation of 3GPP HSDPA technology [20-]. The distribution can be further simplified by a coarser quantization of the user speeds. The number of paths is 6 for this scenario, although it is noted that more than 6 paths may be necessary for signal bandwidths greater than 5MHz [19-].

Fig. 1	Probability Distribution of Mobile User Speed [20-]

- Urban Macro cell

The number of paths in this case is either 6 or 11 as shown in Table 3.1 of [18-]. So a mix channel model scenario should include a specific distribution of number of users with 6 or 11 paths, e.g., Table 17, in addition to the user speed distribution as shown in Fig. 1.

| Number of Paths | 6 | 11 |
| --- | --- | --- |
| Fraction of simulated users | 0.5 | 0.5 |

Table 17	Distribution of Number of Paths for users in an Urban Macro Cell

- Urban Micro cell

In this scenario, the number of paths for a simulated user can be either 6 or 11. The user can be in line-of-sight (LoS) or non-line-of-sight (NLOS) link with a base station. Different shadowing standard deviation and pathloss model are used for LOS and NLOS. A possible channel mix for this scenario is shown in Table 18. The same speed distribution as in Fig. 1 may be used.

Alternatively, a uniform distribution of mobile speeds between a minimum and maximum limits can be used.

| Number of Paths | 6, LOS | 11, NLOS |
| --- | --- | --- |
| Fraction of simulated users | 0.3 | 0.7 |

Table 18	Distribution of simulated user channel characteristics in Urban Micro cells

- Indoor Pico cell

In the case of indoor pico cells, the number of paths is either 6 or 12. A simulated user may either be in LOS or NLOS link with the base station. A possible distribution can be found in Table 19. As a simulated user in the indoor scenario has a relatively high probability of low mobility or stationary (except in a gymnasium), a possible method is to simulate for Doppler frequency uniformly distributed between 0 to 10Hz.

| Number of Paths | 6, LOS | 12, NLOS |
| --- | --- | --- |
| Fraction of simulated users | 0.5 | 0.5 |

Table 19	Distribution of simulated user channel characteristics in Indoor Pico cells

OPTION # 3:

At the link level, the channel models that have been defined in the channel model document [18-] include the following non-spatial-varying parameters:

Case-I: Modified Pedestrian A

- LOS - speed: 3 km/h; 4+1 paths

- NLOS - speed: 30, 120 km/h; 4 paths

Case-II: Vehicular A

- Speed: 30, 120, 250 km/h; 6 paths

Case-III: Pedestrian B

- Speed: 3, 30, 120 km/h; 6 paths

Case-IV: Typical Urban

- Speed: 3, 30, 120 km/h; 11 paths

Case-V: Vehicular B

- Speed: 30, 120 km/h; 6 paths

In this set of link level channel models, the path delays and the relative path power are set to fixed values, as compared to the exponential or uniformly distributed path delays in the system level channel models.

In the phase I evaluation as described in Section 6, channel models for Case-III and V have been adopted or speed at 3 km/h and 120 km/h respectively. The simplistic simulation scenario in phase I assumed all users in the 19 cells has the same channel model and speed. While this may be beneficial for the purpose of calibration between the simulators used by different proponents, the scenario does not reflect a typical deployment environment.

The following channel mix based on the link-level channel models with fixed path delays can be a compromise between the simple but unrealistic simulation scenario, and the more complex simulation scenario with random path delays for each mobile user as described in Option 2.

Two scenarios: Suburban macro cell and urban micro cell are used to represent the two typical extremes of deployment environment. The assumptions on user speed distribution, quantized to 3, 30, 120 and 250 km/h, for each scenario are shown in

The probability of users for each channel power delay profile corresponding to each of the above speeds is then equally distributed.

| User speed (km/h) | 3 | 30 | 120 | 250 |
| --- | --- | --- | --- | --- |
| Suburban macro cell | 0.08 | 0.30 | 0.6 | 0.02 |
| Urban micro cell | 0.6 | 0.36 | 0.04 | 0 |

Table 20	Assumption on mobile user speed for option 2 (TBD)

Scenario 1: Suburban Macro cells:

| Channel PDP Models | I | II |  |  | III | IV | V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User speed (km/h) | 3 | 30 | 120 | 250 | 3 | 0 | 30 | 120 |
| Probability | 0.04 | 0.15 | 0.3 | 0.02 | 0.04 | 0 | 0.15 | 0.3 |

Scenario 2: Urban Micro cells:

| Channel PDP Models | I | II |  |  | III | IV |  | V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User speed (km/h) | 3 | 30 | 120 | 250 | 3 | 3 | 30 | 30 | 120 |
| Probability | 0.2 | 0.12 | 0.02 | 0 | 0.2 | 0.2 | 0.12 | 0.12 | 0.02 |

# RF Environment [Editor’s Note: Updated based on C802.20-04/64R4]

## Radio Transceiver Characteristics

The RF environment for the EC model is defined by a set of RF parameters that should be considered as fixed constrains (that apply to all technologies) and by technology-specific performance specifications

Table 12 defines the transmitter and receiver parameters for the base station (BS) and the mobile station (MS) radios. Note that some parameters vary with channel bandwidth. Base values were chosen for a 1 MHz reference channel and adjustments were made for 1.25 MHz, 5MHz and 10 MHz channels. The adjustment for other channel bandwidths can be easily calculated by factoring the channel BW ratio to the 1 MHz channel.

10.1.1	Transmitter

For the evaluation purposes, the maximum base station (BS) transmit power is specified here as peak power per 1 MHz and is +43 dBm/MHz. Likewise, the mobile station (MS) maximum transmit power shall be +27 dBm for all channel bandwidths.

For the evaluation purposes, the out-of-band emission limit (item 3 in table 12) shall be that which is specified for the block edge, by the FCC for the PCS band. The spectral shape of the transmitted modulated carrier is material to the evaluation criteria and shall  be submitted.

In the case of a multi-carrier technology, the transmit power is the total power of all carriers transmitted simultaneously and intended to be received by one receiver simultaneously.

It shall be further required that in the evaluation criteria model, any band-edge guard bands that are used in order to meet the out of band emission limits shall be specified..

Note that when applying the FCC emission attenuation formula (see Table 12, item 3, where P is the transmit power in Watts), {editor note: verify correctness} the maximum permissible out-of-band power is -13dBm for all transmit powers and all channel bandwidths. Also note that the resolution-bandwidth for out-of-band power measurements is 1 MHz.

    - Receiver

Noise Figure:

To achieve a fair performance comparison, different technologies should be constrained by similar environmental interference conditions and have identical receiver noise figure specifications. Thus, for the evaluation purposes it is required that all technology proposals use the same receiver noise figure (NF) specs: 10 dB for the MS and 5 dB for the BS.

Receiver Sensitivity:

The theoretical receiver sensitivity is expected to vary from one technology to another, but again, for the sake of fair comparison, it is required that the receiver sensitivity be specified for raw data bit error rate (BER) of 0.1%.

The receiver sensitivity (in dBm) shall be calculated using the following formula:

Sensitivity = (-174.5 dBm) + NF (in dB) + 10 log (channel-BW in Hz) + C/N min for 0.1% BER).

ACLR, Selectivity and Blocking:

In addition to co-channel interference, the RF environment in which a receiver operates includes interference from adjacent and second-adjacent channels (of same or different technologies). The contribution of the latter signals to the overall C/N level can be calculated from the ACLR, Selectivity and Blocking specifications. The total interference power present in the wanted channel band consists of the aggregate interference power caused by co-channel, adjacent channel and second-adjacent channel signals.

For the evaluation model purposes, items 4, 5, 9 and 10 in Table 12 are the recommended values for ACLR and Selectivity. Individual proposals may specify different values and must commit their entire performance analysis and simulations to their chosen specifications. The Blocking signal levels are mandatory values.

The power level of interference from adjacent channels and from blocking signals shall be calculated by applying the ACLR and Selectivity figures from table 10-1 or from the individual proposal’s technical specifications.

Note1: 
Typically the interference effect of a blocking signal is measured when the level of the wanted signal is 3 dB above receiver sensitivity (at BER 0.1%). When applying the blocking signal to the receiver input (in addition to the wanted signal) no change in the BER should be detected.

Table 12:  Evaluation Criteria RF Parameters

| # | RF Parameter | Base Value 1 MHz Channel | 1.25 MH Ch BW | 5 MHz Ch BW | 10 MHz Ch BW |
| --- | --- | --- | --- | --- | --- |
| 1 | Transmitter Power -- BS | 43 dBm/MHz | +44 dBm | +50 dBm | +53 dBm |
| 2 | Transmitter Power -- MS | 27 dBm | +27 dBm | +27 dBm | +27 dBm |
| 3 | Out of Band emission limits – BS and MS (emission measured in 1 MHz resolution bandwidth) | Attenuation of the transmit power P by: 43 +10 log(P) dB | -13 dBm | -13 dBm | -13 dBm |
| 4* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) – BS | 45 dB | 39 dB | 45 dB | 48 dB |
| 5* | ACLR - Attenuation of emissions into an adjacent channel (same Ch BW) – MS | 33 dB | 27 dB | 33 dB | 36 dB |
| 6 | Receiver noise figure -- BS | 5 dB | 5 dB | 5 dB | 5 dB |
| 7 | Receiver noise figure -- MS | 10 dB | 10 dB | 10 dB | 10 dB |
| 8 | Receiver reference sensitivity (to be proposed by each technology) | Specify at BER of 0.1% | value 1 (proposal specific) | value 2 (proposal specific) | value 3 (proposal specific) |
| 9* | Receiver Selectivity -- BS | 63 dB | 63 dB | 63 dB | 63 dB |
| 10* | Receiver Selectivity -- MS | 33 dB | 33 dB | 33 dB | 33 dB |
| 11[*] | Receiver Blocking – BS (level of same technology blocking signal at frequency offset of 2 times Channel BW) | -40 dBm | -40 dBm | -40 dBm | -40 dBm |
| 12[*] | Receiver Blocking – MS (level of same technology blocking signal at frequency offset of 2 times Channel BW) | -56 dBm | -56 dBm | -56 dBm | -56 dBm |

* Recommended values. Proposals may choose (and commit to) different values.

# Link Budget

[Prev. Editor’s note: Open items: Should maximum range (link budget) be used as a performance metric for proposal comparison or not? Also need to determine how to use the building/vehicular penetration loss numbers for different environments]

Option 1:

The link budget template as adopted from ITU-R M.1225 with slight modifications in Table 11.1 should be used for computing the link budget under the following cellular test environments:

- Suburban Macro

- Urban Macro

- Urban Micro

- Indoor Pico

The corresponding path loss model as described in the 802.20 channel models document should be used for path loss computation.

The maximum information data rate at the cell boundary that is supported by the proposed technology is used in the link budget computation to determine the corresponding maximum range or cell size that can be achieved by the technology.

Required Eb/(No+Io) for the corresponding information data rate at the target BER should be taken from the simulation results using the channel models case I-IV, as specified in the 802.20 channel models document.

The target BER used in the link budget evaluation is stated in section 10.1.2.

Option 2:  (Alternate Text contributed by: Mike Youssefmir via email reflector, Feb 22, 2005)

The link budget template as adopted from ITU-R M.1225 with slight modifications is given in Table 11.1. Entries that have explicit numerical values in the table (such as power levels, cable losses, etc) shall be used by proponents in their respective system simulations. Proponents may provide informative values for other entries in the table (such as diversity gain, soft handoff gain etc) as they may pertain to their respective technology.

Option 3: Delete the paragraph: “The maximum information…by the technology” in option 1.

Option 4:

The link budget template as adopted from ITU-R M.1225 with slight modifications in Table 11.1 shall be used for computing the link budget under the following cellular test environments:

- Suburban Macro, data rate: x

- Urban Macro, data rate: xx

- Urban Micro , data rate: xxx

- Indoor Pico, data rate: xxxx

The corresponding path loss model as described in the 802.20 channel models document shall be used for path loss computation.

Entries that have explicit numerical values in the table (such as power levels, cable losses, etc) shall be used by proponents in their respective system simulations. Proponents shall provide values for other entries in the table (such as diversity gain, soft handoff gain etc) as they pertain to their respective technology.

Required Eb/(No+Io) for the corresponding information data rate at the target BER should be taken from the simulation results using the channel models case I-IV, as specified in the 802.20 channel models document, for each environment. The target BER used in the link budget evaluation is stated in section 10.1.2.

| id/ii | Item | Downlink | Uplink |
| --- | --- | --- | --- |
|  | Test environment | Suburban/urban macro-cell, micro-cell, indoor pico-cell | Suburban/urban macro-cell, micro-cell, indoor pico-cell |
|  | Operating frequency | 1.9GHz | 1.9GHz |
|  | Test service |  |  |
|  | Multipath channel class | Cases I-IV | Cases I-IV |
| ii/id | (a0) Average transmitter power per traffic channel | dBm | dBm |
| id | (a1) Maximum transmitter power per traffic channel | dBm | dBm |
| id | (a2) Maximum total transmitter power | 43 dBm/MHz | 27dBm |
| ii | (b) Cable, connector, and combiner losses (enumerate sources) | 3 dB | 0 dB |
|  | Body Losses | 0 dB | 3 dB |
| ii | (c) Transmitter antenna gain | 17 dBi | 0 dBi |
| id | (d1) Transmitter e.i.r.p. per traffic channel  (a1 – b  c) | dBm | dBm |
| id | (d2) Total transmitter e.i.r.p.  (a2 – b  c) | 57 dBm | 27 dBm |
|  | Penetration Loss (Ref: 3GPP2) [Determine how to use these numbers for different environments, revisit if 20dB is a reasonable value for building penetration)] | 20 dB (Building) 10 dB (Vehicular) | 20 dB (Building) 10 dB (Vehicular) |
| ii | (e) Receiver antenna gain | 0 dBi | 17 dBi |
| ii | (f) Cable and connector losses | 0 dB | 3 dB |
|  | Body Losses | 3 dB | 0 dB |
| ii | (g) Receiver noise figure | 10 dB | 5 dB |
| ii | (h) Thermal noise density (H) (linear units) | –174 dBm/Hz 3.98  10–18 mW/Hz | –174 dBm/Hz 3.98  10–18 mW/Hz |
| id | (i) Receiver interference density (NOTE 1) (I) (linear units) | dBm/Hz mW/Hz | dBm/Hz mW/Hz |
| id | (j) Total effective noise plus interference density  10 log (10((g  h)/10)  I) | dBm/Hz | dBm/Hz |
| ii | (k) Information rate (10 log (Rb)) | dB(Hz) | dB(Hz) |
| id | (l) Required Eb/(N0  I0) | dB | dB |
| id | (m) Receiver sensitivity = (j  k  l) |  |  |
| id | (n) Hand-off gain | dB | dB |
| id | (o) Explicit diversity gain | dB | dB |
| id | (o) Other gain | dB | dB |
| id | (p) Log-normal fade margin | dB | dB |
| id | (q) Maximum path loss  {d1 – m  (e – f)  o  n  o – p} | dB | dB |
| id | (r) Maximum range | m | m |

Note: Peak power is equivalent to maximum power according to ITU ITURM1225, see A.3.2.2.1. For definition of maximum power and average power refer to ITURM1225-9709  pp 30-31;i.e.

NOTES to Table 11.1:

NOTE 1 – Since the significance and method of calculating this value will vary from RTT to RTT, the proponent must give a detailed explanation of their method for calculating this value and its significance in determining capacity and coverage of the RTT. In particular, the proponent must state explicitly what frequency reuse ratio and traffic loading per sector are assumed in determining this quantity. Interference has to be evaluated for the specified low traffic level given for each test environment. The following sections provide descriptions of the individual link budget template items. Descriptions apply to both forward and reverse links unless specifically stated otherwise. For the forward link the base station is the transmitter and the mobile station the receiver. For the reverse link the mobile station is the transmitter and the base station the receiver. id: Implementation dependent ii: Implementation independent (a0) Average transmitter power per traffic channel (dBm) The average transmitter power per traffic channel is defined as the mean of the total transmitted power over an entire transmission cycle with maximum transmitted power when transmitting. (a1) Maximum transmitter power per traffic channel (dBm) Maximum transmitter power per traffic channel is defined as the total power at the transmitter output for a single traffic channel. A traffic channel is defined as a communication path between a mobile station and a base station used for user and signalling traffic. The term traffic channel implies a forward traffic channel and reverse traffic channel pair. (a2) Maximum total transmitter power (dBm) Maximum total transmit power is the aggregate maximum transmit power of all channels. (b) Cable, connector, and combiner losses (transmitter) (dB) These are the combined losses of all transmission system components between the transmitter output and the antenna input (all losses in positive dB values). The value is fixed in the template. (c) Transmitter antenna gain (dBi) Transmitter antenna gain is the maximum gain of the transmitter antenna in the horizontal plane (specified as dB relative to an isotropic radiator). The value is fixed in the template. (d1) Transmitter e.i.r.p. per traffic channel (dBm) This is the summation of transmitter power output per traffic channel (dBm), transmission system losses (–dB), and the transmitter antenna gain (dBi), in the direction of maximum radiation. d2) Transmitter e.i.r.p. (dBm) This is the summation of the total transmitter power (dBm), transmission system losses (–dB), and the transmitter antenna gain (dBi). (e) Receiver antenna gain (dBi) Receiver antenna gain is the maximum gain of the receiver antenna in the horizontal plane (specified as dB relative to an isotropic radiator). (f) Cable, connector, and splitter losses (receiver) (dB) These are the combined losses of all transmission system components between the receiving antenna output and the receiver input (all losses in positive dB values). The value is fixed in the template. (g) Receiver noise figure (dB) Receiver noise figure is the noise figure of the receiving system referenced to the receiver input. The value is fixed in the template. (h), (H) Thermal noise density, N0 (dB(m/Hz)) Thermal noise density, N0, is defined as the noise power per Hertz at the receiver input. Note that (h) is logarithmic units and (H) is linear units. The value is fixed in the template.

(i), (I)	Receiver interference density  I0 (dBm/Hz)

Receiver interference density is the interference power per Hertz at the receiver front end. This is the in-band interference power divided by the system bandwidth. The in-band interference power consists of both co-channel interference as well as adjacent channel interference. Thus, the receiver and transmitter spectrum masks must be taken into account. Note that (i) is logarithmic units and (I) is linear units. Receiver interference density I0 for forward link is the interference power per Hertz at the mobile station receiver located at the edge of coverage, in an interior cell.

(j)	Total effective noise plus interference density (dBm/Hz)

Total effective noise plus interference density (dBm/Hz) is the logarithmic sum of the receiver noise density and the receiver noise figure and the arithmetic sum with the receiver interference density, i.e:

j    10 log (10((g  h)/10)  I)

(k)	Information rate (10 log Rb) (dB(Hz))

Information rate is the channel bit rate in (dB(Hz)); the choice of Rb must be consistent with the Eb assumptions.

(l)	Required Eb /(N0  I0) (dB)

The ratio between the received energy per information bit to the total effective noise and interference power density needed to satisfy the quality (BER) objectives specified in section 10.1.2 under condition of channel model cases I-IV. Power control should not exceed the ceiling established by the sum of the log-normal fade margin plus hand-off gain. Diversity gains included in the Eb /(N0  I0) requirement should be specified here to avoid double counting. The translation of the threshold error performance to Eb /(N0  I0) performance depends on the particular multipath conditions assumed.

(m)	Receiver sensitivity (j  k  l) (dBm)

This is the signal level needed at the receiver input that just satisfies the required Eb /(N0  I0).

(n)	Hand-off gain/loss (dB)

This is the gain/loss factor ( or –) brought by hand-off to maintain specified reliability at the boundary. Assume equal average loss to each of the two cells. The hand-off gain/loss shall be calculated for 50% shadowing correlation. The proponent must state explicitly the other assumptions made about hand-off in determining the hand-off gain.

(o)	Explicit diversity gain (dB)

This is the effective gain achieved using diversity techniques. It should be assumed that the correlation coefficient is zero between received paths. Note that the diversity gain should not be double counted. For example, if the diversity gain is included in the Eb /(N0  I0) specification, it should not be included here.

(o)	Other gain (dB)

An additional gain may be achieved due to future technologies. For instance, space diversity multiple access (SDMA) may provide an excess antenna gain. Assumptions made to derive this gain must be given by the proponent.

(p)	Log-normal fade margin (dB)

The log-normal fade margin is defined at the cell boundary for isolated cells. This is the margin required to provide a specified coverage availability over the individual cells.

(q)	Maximum path loss (dB)

This is the maximum loss that permits minimum RTT performance at the cell boundary:

Maximum path loss  d1 – m  (e – f)  o  o  n – p

(r)	Maximum range (km)

The maximum range is computed for each deployment scenario. Maximum range, Rmax, is given by the range associated with the maximum path loss. The equations to determine path loss are given in the 802.20 channel models document.

# Equipment Characteristics

## Antenna Characteristics

Each proposal will specify its antenna characteristics, e.g. antenna pattern, number of antennas, antenna array geometry (if applicable), orientation, number of sectors.

## Hardware Characteristics

The assumed hardware parameters of both the basestation and the user terminals are necessary to interpret the quoted results.  For example, differences in specification (both BS and UT) significantly affect performance results:

- maximum output power

- noise figures

- antenna gain, pattern, and height

- cable loss (if applicable).

A proposal shall include detailed information regarding the amplifier/s used in the simulation.  The information shall be sufficiently detailed such that the claimed simulation results can be verified by others and that the practicality of the proposed amplifier arrangement is justified.

## Deployment Characteristics

Information such as values of system-level parameters shall be provided to allow evaluation of the proposed technology in a typical deployment scenario. Relevant system-level parameters used for an 802.20 deployment include:

- number of carriers

- total spectral bandwidth

- system frequency allocation

- sectorization

# Output Metrics

In this section, statistics for quantifying the aspects of network-level performance are described.

## System Capacity Metrics

This section presents several metrics for evaluating system capacity.  Specifically, proponents are required to provide:

- User data rate CDF for specified load and basestation separation (Section 13.1.1: Fixed load/coverage operating point: Service Distribution)

- Plot of aggregate throughput vs. basestation separation for stated minimum service levels. (Section 13.1.2: Aggregate Throughput)

- Plot of number of active users per cell vs. basestation separation for stated minimum service levels (Section 13.1.3: Network performance under Varying Load/Coverage)

- Spectral Efficiency for stated load coverage operating points (Section 13.1.4: Computing Sustained Spectral Efficiency)

The results presented for the uplink and downlink capacity should be achievable simultaneously by the system.  If the results for uplink and downlink cannot be achieved simultaneously by the system, the  proponent should indicate so.

### Fixed load/coverage operating point: Service Distribution

Let the load/coverage point be fixed at (Nu,S), where (by definition) the number of active users per cell (Nu), and the (common) inter-basestation separation (S) for a hexagonal tessellation of Nc cells is specified.  This operating point implies a distributionD(Nu,S)of data rates for each user that the system is able to deliver within the cell area.  The distributionD(Nu,S) is to be sampled separately in uplink and downlink directions (Monte-Carlo simulation) with statistics gathered only from the interior cells of the network.

Figure 12 shows a qualitative example of a cumulative distribution function (CDF) of the distribution of downlink data ratesD(Nu,S) in the interior cells of a network for a specified load/coverage operating point (Nu,S).  This graph shows the distribution of data rates on the ensemble of random placements of NUactive users in each cell of the network and all other stochastic input parameters.  The CDF is not complete without specification of the assumed probability distribution of user placement.

Figure 12:  Service Distribution for a fixed load/coverage operating point

#### Minimum Service Level

From a service integrity standpoint, the lower tail of the resulting service CDF contains important information.  Continuing the example of Figure 12, 90% of the active users will be served with a minimum service level of 566 kbits/sec at the load/coverage operating point(Nu,S).  The notation TDL(Nu,S) emphasizes that the minimum service level is a function of the load/coverage operating point.

### Aggregate Throughput

For each placement of users, the aggregate throughput is the sum of the data rates delivered to the Nuactive users in a cell.  The per-user data rate is computed by dividing the total number of information bits received by the time-duration of the simulation. The proponent should provide a graph of the aggregate throughput vs. basestation separation for constant minimum service levels (See Section: 13.1.3). This graph would be the same as Figure 13 with the vertical axis being aggregate throughput instead of number of users.

Figure 13: Contours of constant minimum service level

### Network performance under Varying Load/Coverage

The CDF of Figure 12 characterizes the ability of the system to serve active users at a fixed load/coverage operating point.  Studying the behavior of the system with varying network load gives additional insight.  One interesting approach is to compute the minimum service level TDL(Nu,S)on a grid of points in the load-coverage (NU,S) plane.  Sample contours of constant minimum service level are shown in Figure 13.  This example (synthetically produced for illustrative purposes), reveals the tradeoff between the basestation separation (S) and the number of active users per cell (Nu).

For example, to guarantee an expected minimum service rate of, say, 1024 kbits/sec across 90% of the cell area, few active users (less than 5) can be supported per cell at the noise-limited inter-base station separation of 6 km.  Conversely, many active users per cell (more than 20) can be supported in the interference-limited case when the base stations are closely spaced.

### Computing Sustained Spectral Efficiency

### [Editor’s Note: Consistency needs to be checked with respect to the Requirement Document]

In the present setting, the sustained spectral efficiency (η) can be computed in a meaningful and straightforward manner.  A moment’s reflection will reveal that rather than being a single number, spectral efficiency is a family of numbers parameterized by the load/coverage operating point (Section13.1.1) and the assumed minimum service level.

For a specified operating point (Nu,S)and a minimum service level, the expected aggregate throughput (A) is defined as the expected sum of the data rates delivered to the NUactive users in the cell.  For example, in the downlink direction, the expected aggregate throughput (per-cell) is defined

ADL=Ek=1NuRDL,k

where RDL,k is the downlink rate to the kthuser and E is the statistical expectation.  A similarly defined statistic AUL applies in the uplink direction.  The total expected aggregate throughput is the sum of uplink and downlink: AT=AUL+ADL.

The sustained   spectral efficiency is computed

ηT=ATBWTbits/sec/Hz/cell

where BWT is the total system bandwidth.  Similarly, the spectral efficiency is computed in the uplink direction as

ηUL=AULBWULbits/sec/Hz/cell

where BWUL is the (effective) bandwidth reserved for uplink traffic.  The spectral efficiency in the downlink direction is similarly defined.

The definition of system spectral efficiency as adopted in the systems requirements document [2-] has been quoted in Appendix A. The description on the computation of system spectral efficiency for proposal evaluation is quoted as follows:

“For proposal evaluation purposes, the System Spectral Efficiency of the 802.20 air interface shall be quoted for the case of a three sector baseline configuration and an agreed-upon block assignment size. It shall be computed in a loaded multi-cellular network setting, which shall be simulated based on the methodology established by the 802.20 evaluation criteria group. It shall consider, among other factors, a minimum expected data rate/user and/or other fairness criteria, QoS, and percentage of throughput due to duplicated information flow.”

As the performance requirement stated in the systems requirements document specifies the downlink and uplink spectral efficiency in bps/Hz/sector respectively, the computation should be performed on the downlink and uplink simulation results respectively.

[Prev. Editor’s note: The following 3 options are included corresponding to the options proposed for section 11: Link Budget, as proposed in ECCG CC contribution: “Eval_LinkBudget1r1.doc”.]

Option 1:

    - Coverage & Cell boundary data rate

The maximum data rate that can be supported at the cell boundary at 90% coverage, as indicated by the appropriate log-normal fade margin, should be provided together with the corresponding maximum range.

Option 2:

[No need for the new section based on the alternative text as proposed by Mike for section 11: link budget.]

Option 3:

    - Maximum range

The maximum range corresponding to a data rate of 128 kbps (TBD) should be computed using the link budget template for the various cellular test environments: suburban macro, urban macro and micro, and indoor pico, using channel models as described in the IEEE 802.20 channel models document.

Option 4:

[No need for the new section]

# Payload Based Evaluation

[Editor’s Note: Need further discussion on how we are going to use payload based evaluation criteria]

The payload-based evaluation method for MAC-Modem-Coding capacity and delay performance assessment is described below.

## Capacity performance evaluation criteria

In order to evaluate the different proposals capacity performance, it is useful to define evaluation scenarios. The evaluation parameters are:

- Channel spacing: 1.25MHz and 5MHz [Prev. Editor Note: Example of  inconsistency, see Section 16 below]

- Modem rate (max rate & minimum coding, medium rate & medium coding, minimum rate & maximum coding);

- MAC frame duration: 5ms

For capacity evaluation, the payloads associated with every type service are:

- 30 bytes for VoIP, G.729 codec, 30ms period

- 1518 bytes for long IP packets;

- 64 bytes for short IPv4 packets;

- 40 bytes for video-conference, 64kb/s (64kb/s*5ms/8 =40bytes)

- 240 bytes for video-conference, 384kb/s

- T.B.C. bytes for multi-media streaming.

The computation shall take into account the influence of the MAC overheads, MAC granularity, interleaver, coding block, etc.

In order to simplify the procedure, only one type of traffic is assumed for all the Base Station subscribers. For every type of traffic shall be calculated the subscriber number, separately for up-link and down-link

[NOTE the Alternate text for section 14.1was proposed by prev. editor as follows:]

In order to evaluate and interpret the capacity performance of different proposals, the proponents should provide sufficient information on the system parameters, including but not limited to the following:

- Set of modulation schemes, coding rates and the corresponding information bit rates supported

- MAC  frame duration supported, and those included in the simulation

The evaluation scenario and simulation models have been described in various sections of this document. In summary, the evaluation parameters and assumptions are:

- Deployment of technology in channel block assignments as described in Section 16

- For phase 1, full-buffer (infinite backlog) traffic type is used for evaluation, as described in Section 6.1

- For phase 2, a percentage mix of traffic types as specified in Table 16 is used for evaluation, as described in Section 6.2.

The computation shall take into account the influence of non-payload traffic, e.g., PHY and MAC overheads, MAC granularity, interleaving and de-interleaving delay, and re-transmission of error packets etc.

## Payload transmission delay evaluation criteria

The delay is an important factor for real-time services.

The payload transmission delay shall be evaluated according to the same procedure and parameters, as specified for capacity evaluation. The computation shall take into account the influence of the MAC granularity, interleaver, coding block, etc.

The delay is calculated between the moment in which the payload enters the MAC and the moment in which the payload exits the MAC, on the other side of the wireless link. The processing power of the implied devices will not be taken into account.

The calculation shall be done separately for up-link and down-link, assuming the number of subscribers resulted from capacity calculation.

# Fairness Criteria

In the evaluation of spectral efficiency and in order to make a fair comparison of different proposals, it is important that all mobile users be provided with a minimal level of throughput.  The fairness for best effort traffic (HTTP, FTP and full buffers) is evaluated by determining the normalized cumulative distribution function (CDF) of the user throughput, which meets a predetermined function given in Table 21. For applications other than best effort, application specific outage criteria are defined. Editor’s Note: If no contributions are received addressing the text/notes that follow in Brackets then we should delete the text.[

The proposals will also be evaluated on the basis of additional fairness metrics.  The details of the additional fairness metrics are TBD (Editor’s note: for example IEEE C802.20-04/05 for other traffic types.)].

Let Tput[k] be the throughput for user k.  The normalized throughput with respect to the average user throughput for user k, Tput[k] is given by:

Tput[k]=Tput[k]avgi Tput[i].

The CDF of the normalized throughput with respect to the average user throughput is determined.  The CDF shall lie to the right of the curve given by the points in Table 21.

Table 21 Fairness Criterion CDF

| Normalized Throughput w.r.t average user throughput | CDF |
| --- | --- |
| 0.1 | 0.1 |
| 0.2 | 0.2 |
| 0.5 | 0.5 |

# Simulation and evaluation of various block assignments

[Prev.Editor’s note: text needs to be provided for unpaired block assignments]

Two sets of spectrum allocations (over which the results are quoted) are used in the comparative evaluation:

- 2X5 MHz (total 10 MHz) and

- 2X15 MHz (total 30 MHz)

The individual technology proposals may split the total spectrum into a given number of channels and specify their reuse factor and channel bandwidth. For example, if 2X15MHz is used as the spectrum allocation, then individual technology proposals can perform simulations for 2X5 MHz and then scale the simulation output data to 2X15MHz.

A proposal should specify the channel spacing and justify the ability to support their specified number of carriers within the spectrum allocation specified. In this case, proposals with multiple carriers within the spectrum allocation used for the evaluation process have to validate that the number of carriers used within the allocation and the channel spacing do not cause a violation of the out-of-band emission limits.

Note the Prev. Editor added this text and it has not been accepted:

In order to accommodate cases where a proposal chooses to simulate only a single spectrum allocation, a scaling between the 2 sets of spectrum allocation needs to be defined.

# References

- IEEE C802.20-PD-02, Mobile Broadband Wireless Access Systems: Approved PAR, Dec 11, 02.

- 802.20 - PD-06r, IEEE 802.20 System Requirement Document (V 1.0).

- IEEE C802.20-03/32, Selected Topics on Mobile System Requirements and Evaluation Criteria.

- IEEE C802.20-03/33r1, Criteria for Network Capacity.

- IEEE C802.20-03/35, Evaluation Methodology for 802.20 MBWA.

- IEEE C802.20-03/43, 802.20 Evaluation Methodology Strawman - 00.

- 3GPP2/TSG-C.R1002, “1xEV-DV Evaluation Methodology (V14)”, June 2003.

- A Corlett, D.I. Pullin and S. Sargood, “Statistics of One-Way Internet Packet Delays,” 53rd IETF, Minneapolis, March 2002.

- C802.20-03/43, “802.20 Evaluation Methodology Strawman (03/57 is ppt)”, IEEE 802.20 May 2003 Session.

- C802.20-03/13r1, “User Data Models for an IP-based Cellular  Network,” IEEE 802.20 March 2003 Session.

- C802.20-03/35, “Evaluation Methodology for MBWA”, IEEE 802.20 May 2003 Session.

- C802.20-03/53, “Operator Systems Requirements for MBWA”, IEEE 802.20 May 2003 Session.

- C802.20-03/46r1, “Channel Requirements For MBWA (Rev 1)S”, IEEE 802.20 May 2003 Session.

- D. Staehle et al, “Source Traffic Modeling of Wireless Applications,” Research Report, June 2000. available at: http://www3.informatik.uni-wuerzburg.de/TR/tr261.pdf

- A. Mena and J. Heidemann, “An empirical study of real audio traffic,” INFOCOM 2000. Proceedings. IEEE , volume: 1 , 26-30 March 2000. pp 101 -110 vol.1

- R. A. Bangun and E. Dutkiewicz, “Modelling multi-player games traffic,” Information Technology: Coding and Computing, 2000. Proceedings. International Conference on , 27-29 March 2000. pp: 228 –233

- C. Heyaime-Duverge and V. K. Prabhu, “Modeling action and strategy Internet-games traffic  
Vehicular Technology Conference, 2002. VTC Spring 2002., Vol. 3 , 6-9 May 2002, pp: 1405 -1409 vol.3

- 802.20 - PD-xx, IEEE 802.20 Channel Models.

- “Spatial Channel Model Text Description, V7.0 (SCM-135)”, 3GPP/3GPP2 ad hoc, Jun, 2003.

- “3GPP TR 25.848 V4.0.0, Physical Layer Aspects of UTRA High Speed Downlink Packet Access”, March 2001.

# Appendix A: Definition of terms

Refer to the System Requirements Document [2-].

Number of Active Users Per Cell

For the purposes of this analysis, an active user is a terminal that is registered with a cell and is using or seeking to use air link resources to receive and/or transmit data within the simulation interval.  Evaluating service quality as a function of the well-defined concept of the number of active users per cell is a natural way of comparing how well disparate MBWA systems behave under increasing network load.

Inter-basestation separation

For the purposes of defining network load, it is natural to treat inter-basestation distance as a parameter.   Closely spaced deployments will stress the interference-limited performance of the network while widely spaced deployments will stress the range-limited performance.  In any case, users of an 802.20 system will likely experience different link quality at locations throughout the cell that depend both on the distance from the basestation and the inter-basestation separation.  Thus, we include inter-basestation separation in our definition of the load/coverage operating point.

One-Way Internet packet delay

One-way Internet packet delay is defined as the time it takes for an IP packet to travel from the base station (server) to the server (base station).

System Spectral Efficiency

System Spectral Efficiency is defined in the context of a full block assignment deployment and is calculated as the average aggregate throughput per sector (in bps/sector), divided by the spectrum block assignment size (in Hz) (excluding all PHY/MAC layer overhead).

# Appendix B: 19 Cell Wrap-Around Implementation

In order to allow for data collection in all cells within the hexagonal network, it is necessary to extend the network to a cluster of network consisting of 7 copies of the original hexagonal network, with the original hexagonal network in the middle while the other 6 copies are attached to it symmetrically on 6 sides, as shown in Figure 14.  The cluster can be thought of as 6 displacements of the original hexagon. There is a one-to-one mapping between cells/sectors of the center hexagon and cells/sectors of each copy, so that every cell in the extended network is identified with one of the cells in the central (original) hexagonal network. Those corresponding cells have thus the same antenna configuration, traffic, fading etc. except the location.  The correspondence of those cells/sectors is illustrated in Figure 14.

Editor’s Note: All remaining text and diagrams in this section may not be correct. The Editor’s Notes on agreement in Session 13 are not clear.

An example of the antenna orientations in case of a sectorized system is defined in Figure 14. The distance from any MS to any base station can be obtained from the following algorithm: Define a coordinate system such that the center of cell 1 is at (0,0).  The path distance and angle used to compute the path loss and antenna gain of a MS at (x,y) to a BS at (a,b) is the minimum of the following:

a.	Distance between (x,y) and (a,b);

b.	Distance between (x,y) and (a+3R,b+83R2);

c.	Distance between (x,y) and (a−3R,b−83R2);

d.	Distance between (x,y) and (a+4.5R,b−73R2);

e.	Distance between (x,y) and (a−4.5R,b+73R2);

f.	Distance between (x,y) and (a+7.5R,b+3R2);

g.	Distance between (x,y) and ,

where R is the radius of a circle which connects the six vertices of the hexagon.

Figure 14: An example of the antenna orientations for a sectorized system to be used in the wrap-around simulation. The arrows in the Figure show the directions that the antennas are pointing.

# Appendix C: Fixed user locations for system level calibration

In order to assure a fair and accurate comparison of technical proposals, it was proposed to calibrate the simulation tools, starting from a deterministic configuration [8.1]. For the deterministic simulation, it was proposed to use fixed but random dropped mobiles. An assignment to the author is to provide a list of such mobile locations. Locations for 10 mobiles per sector are generated for 19 cell sites, each with 3 sectors, and shown in the attached file below. The followings are some due explanations to the data.

C.1 	Cell/Sector Locations

The inter-cell distance is 2.5 km and the cells are located as shown in Figure 15:

Figure 15: Cell definition in the Cartesian Coordination System and the Numbering of Cells

The rules used for numbering the cells are the following, in the given order:

- Sector-wise starting from the sector one, which is the center cell. The sector of cells is numbered counter-clock wise, where a sector of cells is defined as those cells that are confined within an area between two radiation lines from the original with an angle of 60 degree.

- Row-wise from inner to outer rows within each sector

- Cell-wise from right to left on each row of cells.

Each cell is divided into 3 sectors, characterized by the antenna direction of each sector. The number of sector is counter-clock wise with 0, 1 and 2, respectively, where the respective antenna direction is

0:  theta=0 degree,

1: theta=120 degree,

3: theta=240 degree,

where theta is the local polar angle of the cell. By this convention, the first sector of the center cell has the index   (0, 0), while the last sector has the index (18,2).  Mobiles are uniformly dropped in each sector, where an area around the cell center with radius 35 meters are excluded for mobiles. The unit of distance is meter.

C.2 	Location Data

The generated locations are shown in the attached spreadsheet, where the names have the following meaning:

bs.id=index of the base stations as given above

l.sc.id=local sector identifier=sector index as given above

g.sc.id=global sector identifier=3*bs.id+l.sc.id

l.ms.id=local mobile station identifier

bs.loc.x=x-coordinate in Cartesian Coordination System of base station

bs.loc.y=y-coordinate in Cartesian Coordination System of base station

ms.loc.x=x-coordinate in Cartesian Coordination System of mobile station

ms.loc.y=y-coordinate in Cartesian Coordination System of base station

Random locations of 10 mobiles per sector for 57 sectors can be found in the spreadsheet below:

Figure 16: MS Locations

The actual coverage areas are determined by propagation, fading, antenna patterns, and other factors.

Link curves should be provided for the considered channel models

Channels models for Phase II simulations to be determined based on the outcome of channel models CG.

The use of maximum range as a metric is TBD, as noted in the Evaluation criteria document Ver.11r1.

Results of deterministic calibration between the simulators should be similar.

Short-term fading is excluded from the calculation of the long-term average received power.

It follows that the received power at the base station is given by PT,k x Gk = PR/Gk x Gk = PR as desired.

See Appendix A for definition of active users

Since the base configuration is only required for the purpose of comparing system spectral efficiency, proposals may submit deployment models over and beyond the base configuration.

See definition of spectrum allocation from the Terminology Annex of Requirements Document.

See definition of channel bandwidths from the Terminology Annex of Requirements Document.