# 802.20 Channel Models Set

- Pedestrian A

- Vehicular A

- Pedestrian B

- Typical Urban

- Vehicular B

- Indoor A

- Indoor B

# 802.20 Test Environments Set

- Suburban macro

- Urban macro

- Urban micro

- Indoor pico

# Channel Models and Test Environments for Phase-I Evaluation

The following assumptions are proposed for phase-I system level simulations:

- Vehicular A

- Suburban macro environment defined by the channel model document,

# General Requirements for System Simulation Set-up

Each technical proposal should provide

- General transmission scheme and block diagram,

- General receiver structure

# System Simulation Parameters – An example

## System-level parameters

Environment type						Suburban macro,

Distance between base stations		3000 m,

BS antenna height						35 m

Base station configuration				3-sectors

BS antenna type							ULA with 10λspacing,

MS antenna pattern						omni directional with an antenna gain of -1 dBi,

MS antenna type							ULA with 0.5λspacing,

Mean angular spread at BS			8 degrees,

Path-loss model in dB					suburban macro34.5+35logd, d expressed in meters,

Inter BS shadowing correlation		0.5,

Log normal shadowing std			8 dB.

The handsets are distributed uniformly in the middle of the network in order to avoid border effects. The base stations antenna pattern is given by:

Aθ=−min12θ702,20dBi

## Link level parameters

Number of paths														6

Number of sub-paths per path									20

Scheduler type														round robin (random)

User speed																1 - 120 km/h

Correlation between angular spread and shadowing		-0.6

Correlation between delay spread and shadowing			-0.6

Correlation between delay spread and angular spread		+0.5

Feedback delay														TBD

Feedback error														perfect

The channel varies from slot to slot according to the spatial channel model based on the mobile speed.