| id/ii | Item | Forward link | Reverse link |
| --- | --- | --- | --- |
|  | Test environment |  |  |
|  | Test service |  |  |
|  | Multipath channel class | A, B (TBR) | A, B (TBR) |
| ii/id | (a0) Average transmitter power per traffic channel (NOTE 1) | dBm | dBm |
| id | (a1) Maximum transmitter power per traffic channel | dBm | dBm |
| id | (a2) Maximum total transmitter power | dBm | dBm |
| ii | (b) Cable, connector, and combiner losses (enumerate sources) | 3 dB | 0 dB |
|  | Body Losses | 0 dB | 3 dB |
| ii | (c) Transmitter antenna gain | 2 dBi (indoor) (TBR For 1800-1900 MHz and higher frequencies 17 dBi For 800-900 MHz 14 dBi | 0 dBi (TBR) |
| id | (d1) Transmitter e.i.r.p. per traffic channel  (a1 – b  c) | dBm | dBm |
| id | (d2) Total transmitter e.i.r.p.  (a2 – b  c) | dBm | dBm |
| ii | (e) Receiver antenna gain | 0 dBi (TBR) | 2 dBi (indoor) (TBR) For 1800-1900 MHz and higher frequencies 17 dBi For 800-900 MHz 14 dBi |
| ii | (f) Cable and connector losses With no tower mounted amplifier (TMA) With tower mounted amplifier (TMA) | 0 dB 0 dB | 3 dB 0 dB |
|  | Body Losses | 3 dB | 0 dB |
| ii | (g) Receiver noise figure | 10 dB | 5 dB |
| ii | (h) Thermal noise density (H) (linear units) | –174 dBm/Hz 3.98  10–18 mW/Hz | –174 dBm/Hz 3.98  10–18 mW/Hz |
| id | (i) Receiver interference density (NOTE 2) (I) (linear units) | dBm/Hz mW/Hz | dBm/Hz mW/Hz |
| id | (j) Total effective noise plus interference density  10 log (10((g  h)/10)  I) | dBm/Hz | dBm/Hz |
| ii | (k) Information rate (10 log (Rb)) | dB(Hz) | dB(Hz) |
| id | (l) Required Eb/(N0  I0) | dB | dB |
| id | (m) Receiver sensitivity = (j  k  l) |  |  |
| id | (n) Hand-off gain | dB | dB |
| id | (o) Explicit diversity gain | dB | dB |
| id | (o) Other gain | dB | dB |
| id | (p) Log-normal fade margin | dB | dB |
| id | (q) Maximum path loss  {d1 – m  (e – f)  o  n  o – p} | dB | dB |
| id | (r) Maximum range | m | m |

Table 1	ITU-R M.1225 Link Budget Template

Notes:

Body Losses:  body losses are generally included for mobile phone (handset) applications.  If the target is only PCMCIA cards then this may not represent actual losses due to a user’s body or head but this should still be included since the directionality and polarization of the antenna on a laptop PC will usually not be optimum.

Antenna Gain:  for 1800 MHz and higher the base station antenna gain is 17 dBi, for 800-900 MHz the base station antenna gain is 14 dBi; this is assuming that the cell sites are configured in 3-sector

Noise figure:  the base station receiver noise figure is generally assumed to be 4 or 5 dB; the mobile station receiver noise figure is generally assumed to be 8 to 10 dB

Maximum transmit power:  Regulations in the US are specific to the frequency band being used; for 1900 MHz the mobile device EIRP is limited to 2 Watts.  For the newly approved 1700/2100 MHz band this limit has been reduced to 1 Watt EIRP.  The base station power is generally limited to 100 Watts with a limit for EIRP of 1640 Watts.