Section 4.1.1 – System Gain

# [blank]

# Proposed Deleted text

N/A

# Proposed New text

The 802.20 air interface specification is required to provide appropriate means to enable future implementations of 802.20 to maximize their system gain as defined below. This can be achieved through a combination of factors including receiver threshold for specific modulation schemes at specified bit error probability. It is expected that numerical values for system gain and related parameters be provided in the air interface evaluation criteria process.

The system gain is defined as the difference, in dB, between transmitter power output at the base station and the receiver threshold (sensitivity) at the mobile terminal.

Rationale

Defining system gain through maximum allowable path loss (a link budget term), as Neka provided, has the problem of becoming deployment specific since it includes antenna gains and cable losses, etc. That’s the reason why we decided not to have a section on link budget but only define system gain. The definition provided here makes it only dependent on the transmitter power and the receiver design for specific modulation, specific Eb/No requirement and specific bit error rate, all of which are part of the evaluation criteria for comparing air interface proposals. It is clear that one should not expect the same system gain for QPSK and 64QAM. Also, it is not favorable to set the requirement for only one scenario (e.g., lowest order modulation, or average rates, etc.). Consequently, the functional requirements document should only ask for the maximization of system gain and leave the actual numbers to the proposal evaluation process.