3.1.1  MBWA System Reference Architecture

Adopting current communications systems specification principles, 802.20 MBWA systems will be specified using a layered architecture. The 802.20 standards, in conjunction with other 802 standards, will specify the services to be delivered by layers 1 and 2  to an IP based layer 3 or a switching layer, e.g. PPP, MPLS.  To facilitate a layered approach, the 802.20 specification shall incorporate a reference partitioning model consisting of the MAC and PHY. This layered approach shall be generally consistent with other IEEE 802 standards and shall remain generally within the scope of other IEEE 802 standards as shown in figures 1 & 2.   The 802.20 standards shall also address the needs of logical link control and how and when the 802.2 LLC functionality is used. The 802.20 standards include PHY and MAC layer specifications with a well-defined service interface between the PHY and MAC layer.  To provide the best possible performance, the MAC layer design is optimized for the specific characteristics of the air interface PHY. Figure 2 shows the relationship of various 802 PHY and MAC layer standards to other 802 architectural components. The 802.20 standards shall clarify how 802.20 fits into this architecture.