## PAR Summary (Closure Proposed)

The scope of the PAR (listed in Item 12) is as follows:

“Specification of physical and medium access control layers of an air interface for interoperable mobile broadband wireless access systems, operating in licensed bands below 3.5 GHz<PHY>, optimized for IP-data transport, with peak data rates per user in excess of 1 Mbps<MAC>. It supports various vehicular mobility classes up to 250 Km/h<Goal> in a MAN environment and targets spectral efficiencies, sustained user data rates and numbers of active users that are all significantly higher than achieved by existing mobile systems.”

In addition, a table (provided in Item 18) lists “additional information on air interface characteristics and performance targets that are expected to be achieved.”

| Characteristic | Target Value |
| --- | --- |
| Mobility | Vehicular mobility classes up to 250 km/hr (as defined in ITU-R M.1034-1)<GOAL> |
| Sustained spectral efficiency | > 1 b/s/Hz/cell <MAC> |
| Peak user data rate (Downlink (DL)) | > 1 Mbps* <MAC> |
| Peak user data rate (Uplink (UL)) | > 300 kbps* <MAC> |
| Peak aggregate data rate per cell (DL) | > 4 Mbps* <MAC> |
| Peak aggregate data rate per cell (UL) | > 800 kbps* <MAC> |
| Airlink MAC frame RTT | < 10 ms <PHY> |
| Bandwidth | e.g., 1.25 MHz, 5 MHz <Goal> |
| Cell Sizes | Appropriate for ubiquitous metropolitan area networks and capable of reusing existing infrastructure. <Goal> |
| Spectrum (Maximum operating frequency) | < 3.5 GHz <PHY> |
| Spectrum (Frequency Arrangements) | Supports FDD (Frequency Division Duplexing) and TDD (Time Division Duplexing) frequency arrangements <PHY> |
| Spectrum Allocations | Licensed spectrum allocated to the Mobile Service <Goal> |
| Security Support | AES (Advanced Encryption Standard) <MAC> |

* Targets for 1.25 MHz channel bandwidth. This represents 2 x 1.25 MHz (paired) channels for FDD and a 2.5 MHz (unpaired) channel for TDD. For other bandwidths, the data rates may change.