- European Telecommunications Standards Institute 	BRAN28d031r1

- BRAN#28

- 15th to 19th April 2002

- Sophia Antipolis, France

- Source:		Siemens AG

- Title:		Generic Authentication Transport (GST) operation & use

- Date:		11th April 2002

- Document for:	Information

- Agenda item:	3G Interworking

Abstract:

This document describes how GST (Generic Signalling Transport, previously GST) is used to implement EAPOH for user authentication within HIPERLAN/2 networks.

Change History:

See section 1.1 (page 3).

# CONTENTS

1.	General Information	3

1.1	Document History	3

1.2	References and Related Documents	3

1.3	Abbreviations	3

2.	Introduction	4

2.1	Scope of GST	4

3.	GST Configuration	4

3.1	Use of GST	4

3.2	Use of EAPOH	4

3.3	Authentication Protocol Identifier	6

4.	EAPOH Message Transfer	7

4.1.1	GST Configuration	7

4.1.2	Arbitrary EAP Dialogue	7

FIGURES & TABLES

Table 1.  Document History	3

Figure 1.  RLC Authentication using GST	6

Figure 2.  GST Configuration Message	7

Figure 3.  GST Configuration acknowledgement	8

Figure 4.  RLC_GST EAP Message Example: First segment	9

Figure 5.  RLC_GST EAP Message Example: Second segment	9

Figure 6.  RLC_GST EAP Message Example: Third segment	9

Figure 7.  RLC_GST EAP Message Example: Fourth segment	10

Figure 8.  RLC_GST_ACK Example: first three segments received	10

Figure 9  RLC_GST_ACK Example: all segments received	11

Figure 10  RLC_GST_ACK Example: retransmit request (1)	11

Figure 11.  RLC_GST_ACK Example: retransmit request (2)	12

Total no. of pages in document: 10

# General Information

## Document History

| Issue | Date | Who? | Reason for changes |
| --- | --- | --- | --- |
| – | 23 April 2002 | DAF | First draft |
| 1.0 | 10 April 2002 | SM | Addition of ETSI leader page. Addition of some explanatory text |
| 1.1 | 16 April 2002 | SM | Changed GST to GST, plus correction of typos |

Table 1.  Document History

## References and Related Documents

[1]	ETSI TS 101 761-2: Broadband Radio Access Networks (BRAN); HIPERLAN Type 2; Data Link Control (DLC) Layer; Part 2: Radio Link Control (RLC) sublayer

[2]	ETSI BRAN25d092 EAPOH RLC Packet Transport, October 2001

[3]	ETSI TS 101 761-2 CR096: Addition of generic authentication message transport mechanism, November 2001

[4]	BRAN28d022 “Transparent mode using EAP”, Thomson Multimedia, April 2002

## Abbreviations

AAA	Authentication, Authorisation and Accounting

AAAH	Home AAA

AAAL	Local AAA

AKA	Authentication and Key Agreement

AP	Access Point

AVP	Attribute Value Pair

BRAN	Broadband Radio Access Networks

CMS	Cryptographic Message Syntax

CRC	Cyclic Redundancy Check

DLC	Data Link Control

DES	Data Encryption Standard

EAP	Extensible Authentication Protocol

EAPOH	EAP over HIPERLAN

EAPOL	EAP over LANs

ETSI	European Telecommunications Standards Institute

GST	Generic Authentication Transport

GST	Generic Signalling Transport

HIPERLAN	High Performance Radio Local Area Network

HIPERLAN/2	HIPERLAN Type 2

IEEE	Institute of Electrical and Electronic Engineers

IETF	Internet Engineering Task Force

IP	Internet Protocol

LAN	Local Area Network

LCH	Long Transport Channel

MT	Mobile Terminal

NAI	Network Access Identifier

NAS	Network Access Server

NASREQ	Network Access Server Requirements

PDU	Protocol Data Unit

PKCS	Public-Key Cryptography Standards

PPP	Point-to-Point Protocol

RADIUS	Remote Authentication Dial In User Service

RBCH	RLC Broadcast Channel

RFC	Request For Comment

RLC	Radio Link Control

RSA	Rivest Shamir and Adleman

SCH	Short Transport Channel

SCTP	Stream Control Transmission Protocol

SIM	Subscriber Identity Module

TBD	To Be Determined

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

UICC	UMTS Integrated Circuit Card

UMTS	Universal Mobile Telecommunications System

USIM	Universal Subscriber Identity Module

3G	3rd Generation (Cellular System)

# Introduction

This contribution describes how GST is used to perform authentication within a HIPERLAN/2 interworked network.  Previous work has not covered the usage of this container protocol and this document fills that requirement.  GST may also be used to carry other non-authentication signalling messages, for example service advertising/discovery.

## Scope of GST

At the time of writing, EAP is the only authentication scheme that has been selected for possible use, although it must be pointed out, that it is not necessarily the only one.  GST must be considered as a simple protocol within the HIPERLAN/2 RLC layer that merely facilitates the transportation of higher layer authentication protocols, of which EAP is an example.  Other possible authentication schemes may also be used, which are considered to be closer to UMTS/GSM, such as GMM, although in this case a new Diameter GMM application would also have to be standardised within the IETF.

To facilitate the division of EAP messages into suitable GST payload chunks (together with extra padding packets and flags) EAPOH has been created to allow EAP messages to be transported by GST.  This then corresponds to EAPOL used within the Ethernet based IEEE 802.11.

Section 3 begins by describing how GST and, within GST, EAPOH are selected.

Section 4 gives some example EAPOH message transfers.

# GST Configuration

This section reviews how authentication is configured to use GST, and how GST may be configured to use EAPOH [2] for user authentication.

## Use of GST

Use of GST for user authentication is negotiated during the Link Capability Negotiation phase of the association procedure (q.v. [1] as modified by [3]).

## Use of EAPOH

Once GST has been chosen, user authentication proceeds by the Authentication_GST procedure outlined in [3].  This is shown in Figure 1.

Figure 1.  RLC Authentication using GST

Note that GST is intended to support authentication by mechanisms other the EAP.  Therefore the first exchange during the GST authentication procedure must negotiate the exact authentication procedure required.  To do this, the first message transmitted by the MT is a GST_CONFIGURATION message.  In reply, the AP returns a GST_CONFIGURATION_ACK message.  Both these messages are formally identified as RLC_GST messages with the following format.

Figure 2.  GST Configuration Message

Note that the GST Configuration messages always contain a single GST payload segment (hence the segment number is always zero).  Additionally, the GST payload segment has two initial zero octets followed by (up to) 42 octets of configuration data (exact format to be decided).  Part of the configuration data of the GST_CONFIGURATION message specifies the authentication mechanisms supported by the MT; correspondingly, part of the configuration data of the GST_CONFIGURATION_ACK message specifies which (if any) of the mechanisms has been chosen by the AP as the one actually to use.

## Authentication Protocol Identifier

The ‘configuration data’ section of the GST_CONFIGURATION_ACK, will indicate which authentication protocol is to be used.  Although this has not been detailed within this document, it is likely that the AP will supply a list of supported protocols (e.g. EAP, GMM) and then the MT chooses one.  The number space for these identifiers should be defined within the Interworking Technical Standard, currently referred to as the DTS.  A suitable suggestion for a number scheme is present within section 5.1 of [4].

If no mechanisms are specified in the GST_CONFIGURATION_ACK message, either the authentication procedure must fail, or the MT must issue another GST_CONFIGURATION message proposing alternative authentication mechanisms.

Note that the only authentication mechanism currently defined is EAPOH.

Note that the message format shown in Figure is compatible with the EAPOH format described in [2], with the observation that an EAPOH payload padding packet can never logically appear as the first packet within the EAPOH PDU payload.

# EAPOH Message Transfer

This section explicitly shows how GST is used to transport EAP messages.

### GST Configuration

Note that the GST_CONFIGURATION and GST_CONFIGURATION_ACK messages have already been fully described.  Note that since both of these messages have only a single payload segment, and the responses to the messages can be immediately constructed, strictly RLC_GST_ACK messages are not required to verify their transfer.  The appropriate values of the RLC_GST_ACK message fields required to verify transfer are shown in Figure 3.

| Field | Value | Note |
| --- | --- | --- |
| gst-ack-flags: S1 | 1 | all indicated segments received correctly |
| gst-ack-flags: SL | 1 | acknowledges receipt of complete payload |
| gst-ack-initial-segment-number | any | field ignored since SL = 1 |
| gst-ack-sequence-number | seq_no | copied from RLC_GST message |
| gst-ack-final-segment-number | any | field ignored since S1 = 1 |
| gst-ack-received-segment-map | any | field ignored since S1 = 1 |

Figure 3.  GST Configuration acknowledgement

### Arbitrary EAP Dialogue

The authentication procedure at some stage will require the exchange of arbitrary EAP messages.  For illustrative purposes, we’ll show the transfer of a 132 octet EAP packet (1024 bits data plus 4 octets EAP header).  The payload of the corresponding RLC_GST (EAPOH) message contains the EAP packet.  A padding packet of 24 octets is added to bring the total RLC_GST message payload to 176 octets.  The message payload is transferred in four segments, as shown in the following figures.

| Field | Value | Note |
| --- | --- | --- |
| gst-flags: S1 | 1 | this is the first segment |
| gst-flags: SL | 0 | this isn’t the last segment |
| gst-flags: P1 | 1 | first payload packet is EAP |
| gst-flags: P2 | 0 | second payload packet isn’t EAP |
| gst-segment-number | 3 | number of last segment (since S1 = 1) |
| gst-sequence-number | seq_no | message sequence number (same for all segments) |
| gst-payload-segment | data | first 44 octets of payload |

Figure 4.  RLC_GST EAP Message Example: First segment

| Field | Value | Note |
| --- | --- | --- |
| gst-flags: S1 | 0 | this isn’t the first segment |
| gst-flags: SL | 0 | this isn’t the last segment |
| gst-flags: P1 | 1 | first payload packet is EAP |
| gst-flags: P2 | 0 | second payload packet isn’t EAP |
| gst-segment-number | 1 | number of this segment (since S1 = 0) |
| gst-sequence-number | seq_no | message sequence number (same for all segments) |
| gst-payload-segment | data | second 44 octets of payload |

Figure 5.  RLC_GST EAP Message Example: Second segment

| Field | Value | Note |
| --- | --- | --- |
| gst-flags: S1 | 0 | this isn’t the first segment |
| gst-flags: SL | 0 | this isn’t the last segment |
| gst-flags: P1 | 1 | first payload packet is EAP |
| gst-flags: P2 | 0 | second payload packet isn’t EAP |
| gst-segment-number | 2 | number of this segment (since S1 = 0) |
| gst-sequence-number | seq_no | message sequence number (same for all segments) |
| gst-payload-segment | data | third 44 octets of payload |

Figure 6.  RLC_GST EAP Message Example: Third segment

| Field | Value | Note |
| --- | --- | --- |
| gst-flags: S1 | 0 | this isn’t the first segment |
| gst-flags: SL | 1 | this is the last segment |
| gst-flags: P1 | 1 | first payload packet is EAP |
| gst-flags: P2 | 0 | second payload packet isn’t EAP |
| gst-segment-number | 3 | number of this segment (since S1 = 0) |
| gst-sequence-number | seq_no | message sequence number (same for all segments) |
| gst-payload-segment | data | fourth 44 octets of payload |

Figure 7.  RLC_GST EAP Message Example: Fourth segment

While the protocol does not require a RLC_GST_ACK message for every RLC_ACK, we’ll assume that an RLC_GST_ACK is issued to acknowledge correct receipt of the first three segments of the EAPOH message.  This is done using the message shown in Figure 8.

| Field | Value | Note |
| --- | --- | --- |
| gst-ack-flags: S1 | 1 | all indicated segments received correctly |
| gst-ack-flags: SL | 0 | complete payload not received |
| gst-ack-initial-segment-number | 3 | first segment not received |
| gst-ack-sequence-number | seq_no | copied from RLC_GST |
| gst-ack-final-segment-number | any | field ignored since S1 = 1 |
| gst-ack-received-segment-map | any | field ignored since S1 = 1 |

Figure 8.  RLC_GST_ACK Example: first three segments received

On correct receipt of the final segment, the RLC_GST_ACK shown in Figure 9 would be used.

| Field | Value | Note |
| --- | --- | --- |
| gst-ack-flags: S1 | 1 | all indicated segments received correctly |
| gst-ack-flags: SL | 1 | acknowledges receipt of complete payload |
| gst-ack-initial-segment-number | any | field ignored since SL = 1 |
| gst-ack-sequence-number | seq_no | copied from RLC_GST |
| gst-ack-final-segment-number | any | field ignored since S1 = 1 |
| gst-ack-received-segment-map | any | field ignored since S1 = 1 |

Figure 9  RLC_GST_ACK Example: all segments received

Alternatively, in the presence of errors, the first RLC_GST_ACK could indicate that only the first and third segments of the message payload had been received.  This would result in the message in Figure 10

| Field | Value | Note |
| --- | --- | --- |
| gst-ack-flags: S1 | 0 | intermediate segments not received correctly |
| gst-ack-flags: SL | 0 | complete payload not received correctly |
| gst-ack-initial-segment-number | 1 | lowest segment number not received |
| gst-ack-sequence-number | seq_no | copied from RLC_GST |
| gst-ack-final-segment-number | 2 | highest segment number received |
| gst-ack-request-segment-map | map | first bit is 1, (indicates third segment is received) |

Figure 10  RLC_GST_ACK Example: retransmit request (1)

If the final payload segment was received, but the second segment were still missing, the message in Figure 11 would be used.

| Field | Value | Note |
| --- | --- | --- |
| gst-ack-flags: S1 | 0 | intermediate segments not received correctly |
| gst-ack-flags: SL | 0 | complete payload not received correctly |
| gst-ack-initial-segment-number | 1 | lowest segment number not received |
| gst-ack-sequence-number | seq_no | copied from RLC_GST |
| gst-ack-final-segment-number | 3 | highest segment number received |
| gst-ack-received-segment-map | map | first two bits are 1, 1 (indicates third and fourth segments have been received) |

Figure 11.  RLC_GST_ACK Example: retransmit request (2)

Subsequent correct receipt of the second segment would allow the RLC_GST_ACK message shown in Figure 9to be transmitted to acknowledge receipt of the complete payload.

Note that if the sender does not receive any RLC_GST_ACK message within the time period T_gst_ack, the sender should begin retransmission of all unacknowledged message segments.  If this situation persists, it is an indication that the radio link has failed.

Note also that the value T_gst_ack, together with how frequently the sender manages to transmit segments of a payload, determines after how many segments a RLC_GST_ACK message will be generated by the receiver.