| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER |  |
| Date Submitted | January, 2005 |  |
| Source(s) | Joint Contribution |  |
| Re: |  |  |
| Abstract | Draft Text for MEDIA INDEPENDENT HANDOVER Specification |  |
| Purpose | Initial Proposal for 802.21 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

List of Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Vivek Gupta | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503 712 1754 | 503 264 4230 | vivek.g.gupta@intel.com |
| David Johnston | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503 264 3855 | 503 264 4230 | dj.johnston@intel.com |
| Prakash Iyer | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503 264 1815 | 503 264 4230 | prakash.iyer@intel.com |
| Xiaoyu Liu | Samsung AIT | Mt.14-1,Nongseo-Ri,Giheung-Eup,Yongin-Si,Gyeonggi-Do,Korea 449-712 | +82-31-280-9615 | +82-31-280-9569 | xiaoyu.liu@samsung.com |
| Stephen McCann | Siemens Roke Manor | Roke Manor, Romsey, Hampshire, SO51 )ZN, UK | +44 1794-833341 | +44 1794 833434 | stephen.mccann@roke.co.uk |
| Mani Mahalingam | Avaya | 1001 Murphy Ranch Road, Milpitas, CA 95035 | 408 321 4840 |  | mmani@avaya.com |

1 Overview	5

1.1 Scope	5

1.2 Purpose	5

1.3 Design Guidelines	5

1.4 Media Independence	5

1.5 Media Dependence	5

2 References	6

2.1 Normative References	6

2.2 Other References	6

3 Definitions	6

4 Acronyms and Abbreviations	6

5 General Architecture	8

5.1 Introduction	8

5.2 Media Independent Handover Reference Models	8

5.3 Media Independent Event Service	9

5.4 Media Independent Information Service	10

5.5 Media Independent Handover Function	10

5.6 MIH Protocol	10

5.7 Service Access Points (SAPs)	11

6 Media Independent Event Service	12

6.1 Introduction	12

6.2 Event Flow Model	12

6.3 Event Primitives	13

6.3.1 MIH_Event_Register.request	13

6.3.2 MIH_Event_Register.response	14

6.3.3 MIH_Event.indication	15

6.4 Event Indications	15

6.4.1 Link_Up	15

6.4.2 Link_Down	16

6.4.3 Link_Going_Down	16

6.4.4 Link_Going_Up	17

6.4.5 Link_Event_Rollback	17

6.4.6 Link_Available	18

6.4.7 Link_Parameters_Change	18

7 Media Independent Information Service	20

7.1 Introduction	20

7.2 Information Flow	20

7.3 Primitives	20

7.3.1 MIH_Information.request	20

7.3.2 MIH_Information.response	21

7.4 Information Elements	22

7.5 Information Format	25

8 Media Independent Handover Function	26

8.1 Introduction	26

8.2 MIH Signaling	26

8.3 Primitives	26

9 Media Independent Handover Protocol	26

9.1 Introduction	26

9.2 Description	26

9.2.1 New ethertype	26

9.2.2 Messages	26

10 Appendix	26

10.1 Handover Procedures	26

10.1.1 MIP based Handover Sequence Diagrams	26

10.2 Actions by Higher layers on receiving Events	30

10.3 Media Independent Information Service	30

10.3.1 Development and Deployment	30

10.4 Media Specific Amendments	30

# Overview

## Scope

The scope of the IEEE 802.21 (Media Independent Handover) standard is to develop a specification to optimize layer 3 and above handovers between heterogeneous media. This includes both wired and wireless media in the 802 family as well as cellular wireless media.

## Purpose

The purpose of the IEEE 802.21 standard is to enhance user experience of mobile devices by supporting seamless handovers between heterogeneous networks.

This document describes a proposal to satisfy requirements as outlined in [1]. The proposal can support handovers both for mobile as well as stationary users. For the mobile users handovers may occur due to change in wireless coverage as the user moves from coverage area of one network to another. For the stationary user handovers may become imminent when the environment around the user changes making one network more attractive than another. Specifically the described proposal:

- Describes the overall MIH Reference model for handovers between different heterogeneous networks.

- Defines the various Service Access Points (SAPs) that need to be supported in the above reference model.

- Defines the various procedures and primitives to support mechanisms that optimize layer 3 and above handovers. These are as follows.

  - Media Independent Event service which helps to optimize detection of useable attachment point to a network through a set of events and triggers.

  - Media Independent Information service which permits information exchange between mobile terminals and/or networks to enable mobile terminals and/or networks to make more effective handover decisions.

  - Media independent transport so that the mobile terminal can access information in a standardized way from a network attachment point on which to base good and timely handoff decisions.

## Design Guidelines

TBD

## Media Independence

TBD

## Media Dependence

TBD

# References

## Normative References

TBD

## Other References

- 21-04-0087-12-0000 IEEE 802.21 Media Independent Handover Service Draft Technical Requirements, Sept. 21, 2004.

# Definitions

# Acronyms and Abbreviations

3G				Third Generation

3GPP				3G Partnership Project

3GPP2				3G Partnership Project 2

AAA				Authentication, Authorization, and Accounting

AN				Access Network

AP				Access Point

AR				Access Router

BS				Base Station

BSC				Base Station Controller

BSSID   			Basic Service Set Identifier

BTS				Base Transceiver Station

ESS        			Extended Service Set

CN				Core Network

FA				Foreign Agent

FMIP				Fast Handover Mobile IP

GGSN				Gateway GPRS Support Node

GPRS				General Packet Radio Service

GSM				Global System for Mobile Communication

HA				Home Agent

HMIP				Hierarchical Mobile IP

IEEE				Institute of Electrical and Electronics Engineers

IETF				Internet Engineering Task Force

ICMP				Internet Control Message Protocol

IP				Internet Protocol

ISP				Internet Service Provider

ITU				International Telecommunications Union

L3MP				Layer 3 mobility protocol

L1                			Physical Layer (PHY)

L2			         	Medium Access Control (MAC)

LAN				Local Area Network

LLC            			Logical Link Control

MAC				Medium Access Control

MIH	          			Media Independent Handover

MIHO        			Media Independent Handover

MIHS        			Media Independent Handover Services

MIP				Mobile IP

MIIS				Media Indépendant Information Service

MLME        			MAC Layer Management Entity

MN				Mobile Node

MS				Mobile Station

MT		   		Mobile Terminal

NME				Network management entity

PHY            			Physical Layer

PLMN				Public Land Mobile Network

PLME				PHY Layer Management Entity

QoS				Quality of Service

RFC				Request for Comment

RNC				Radio Network Controller

SAP             			Service Access Point

SGSN				Serving GPRS Support Node

SNR				Signal Noise Ratio

SME				Station management entity

STA				Station

TCP				Transmission Control Protocol

UDP				User Datagram Protocol

UMTS				Universal Mobile Telecommunications System

WLAN				Wireless Local Area Network

WPAN				Wireless Personal Area Network

WMAN				Wireless Metropolitan Area Network

# General Architecture

## Introduction

The IEEE 802.21 specification defines services that enhance handovers between heterogeneous media. This is achieved through:

- A Media Independent Event service which provides access to MAC and PHY state changes and other network management events. These are described in section 6.

- A Media Independent Information service which includes an information model and a database. These provide information useful for making effective handover decisions. This is described in section 7.

- A Media Independent Handover (MIH) convergence layer above the LLC that crosses heterogeneous interface types, enabling policy enforcement, network selection, QoS parameter mapping and handover signaling between heterogeneous interfaces in multi interfaced systems. This is described in section 8.

- A layer 2 protocol operating over 802 style MSDUs, enabling the transport of events, handover related information and network detection between pairs of attached 802 and other systems. This is described in section 9.

A standard communication stack includes MAC and PHY layers. Depending on the media type, there may also be a management plane with MLME and PLME layers. An unspecified management function, the SME (in 802.11) or NME (in 802.16) provides coordinated management of the layers.

## Media Independent Handover Reference Models

Figure 1: MIH Reference Model for 802.11

Figure 2: MIH Reference Model for 802.16

## Media Independent Event Service

The Media Independent Event service supports both local and remote events. In case of local events information propagates upwards from the MAC to the layer 3 mobility protocol (L3MP) within a local stack. In case of remote events information may propagate from a L3MP in one stack to a L3MP in another stack connected directly through the medium. Events can indicate state changes or predict state changes of the MAC and PHY layers. Events can also be used to indicate management actions on par of network or some such management entity.

The event service will typically be used to optimize handover signaling within the L3MP. For example an indication that the link will cease to carry MSDUs at some point in the near future may be used by the L3MP to prepare a new point of attachment ahead of the current point of attachment ceasing to carry frames. This has the potential to reduce the time taken to hand over between attachment points.

In an asymmetric link, such as a STA-AP link in 802.11 or an SS-BS link in 802.16, the AP or BS might make a management decision, based on load conditions to require an attached node to handoff elsewhere to some other more favorable attachment point at some point in the future, allowing the node to prepare a layer 3 handover. For this reason, events may originate not only within the MACs or PHYs of the link, but also from the L3MP.

Events defined include Link Available, Link Up, Link Going Up, Link Down, Link Going Down and Event Rollback.

Events can originate from the MAC, the PHY or the MIH_SAP. The destination of any event may be the MIH_SAP within the local stack, the MIH_SAP of the remote stack at the other end of the link, or both. The destination of events is established dynamically with a registration mechanism that enables an endpoint to register its interest in particular event types.

Events may carry additional context data. For example a Link_Up event may carry an ARID. This enables a mobile station to determine whether or not a handover has necessitated a layer 3 handover immediate at the time of attachment. This can serve to improve the performance of layer 3 handovers.

## Media Independent Information Service

The Media Independent Information service primarily provides a query/response type of mechanism for information transfer. This contrasts with the asynchronous push model of information transfer for the event service. The information may be stored within the MIH layer or maybe present in some information server from where the MIH layer can access it. The information is primarily made available at L2 though it could be made available at L3 as well. Information can be made available at L2 through both a secure and an insecure port. Information available through insecure port would typically be less sensitive and would allow the mobile terminal to make a quick handover decision before incurring the overhead of authentication and establishing a secure L2 connection with the network.

The Information service typically provides access to static information such as neighbor maps. Such information helps in network discovery. The service may also provide access to information which may optimize link layer connectivity with different networks. This could include link layer parameters such as channel information, MAC addresses, security information, etc. Information about available services in a network may also help in more effective handover decision making before actually attaching to any particular network.

The information provided by Information service conforms to structure and semantics specified within 802.21. The Media Independent Information service specifies a common (or media independent) way of representing this information across different technologies by using a format such as ASN.1. Further the MIIS provides ability to access this information about all heterogeneous networks in a geographical area from any single L2 network. MIIS uses network media specific transports and security mechanisms to provide access to the information. How this information is developed and deployed is outside the scope of the standard.

## Media Independent Handover Function

In the case of a system with multiple network interfaces of arbitrary type, the MIH or L3MP can use the event service to determine the state of the underlying interfaces and select between them appropriately. The information service helps the MIH in network discovery and selection and thus helps in making effective handover decisions. In a system containing heterogeneous network interfaces of both 802 and non 802 types, the Media Independent Handover Function layer may contain procedures to couple services across heterogeneous network interfaces. Such procedures may include a signaling mechanism between MIH Function layer across different entities to query and reserve resources required for a handover operation between heterogeneous networks.

## MIH Protocol

The IEEE 802.21 standard supports the Media Independent Event service and Media Independent Information service. The packet payload for these services may be carried over management frames or data frames. Wireless networks such as 802.11 and 802.16 have a management plane and support management frames which could be suitably enhanced for carrying the above payloads. However the wired Ethernet network does not have management plane and can carry the above payloads only in data frames.

As such we need to define a L2 protocol with a new ethertype which can carry the above payload over normal data frames. The MIH ethertype is encoded in the SNAP header. At the same time 802 networks don’t support data frames in unauthenticated state. Hence the MIH Protocol cannot be used in this case. Only management frames can be used to transport information in this case.

The IEEE 802.21 standard should define the packet format and payloads in media independent manner in standard TLV format. Thereafter these packets can be encapsulated in a L2 MIH Protocol using MIH ethertype when the payload needs to be sent over normal data frames as in case of ethernet. In other cases the TLV based messages and payload can be directly encapsulated in media specific management frames.

## Service Access Points (SAPs)

Figure 1 and 2 shows the MIH Reference model for 802.11 and 802.16 networks. The key MIH related SAPs are as follows:

- MIH_MGMT_SAP: This defines the interface between the MIH Function layer and the management plane of different networks. This SAP can be used for sending MIH messages between peer MIH entities. Messages based on management frames can be sent in the unauthenticated state as well. The MIH_MGMT_SAP defines primitives for MIES and MIIS.

- MIH_ME_SAP: This SAP defines the interface between MIH Function layer and the SME or NMS. This SAP can be the same as MIH_MGMT_SAP as well.

- MIH_L3_SAP: This SAP defines the interface between MIH and a L3MP. The interface between MIH and L3MP is a local interface which may depend on the choice of L3MP as well. The methods and other messages defined by this SAP result in local communication between peer layer entities. Messages defined by this SAP shall not travel the across the medium. Definition of this SAP is outside the scope of 802.21.

- MIH_MAC_SAP: This SAP defines the interface between MIH and media specific MAC. The interfaces defined by this SAP are primarily used to transfer data (MSDUs) between peer entities. No new interfaces and primitives need to be defined for this SAP. However the interfaces defined by this SAP can be used to deliver MIH protocol based payloads to peer MIH entities.

- MIH_PHY_SAP: This SAP defines the interface between MIH and media specific PHY. The MIH shall communicate with different media specific PHYs using the media specific MACs. As such no new interfaces or primitives need to be defined for this SAP as well.

- LSAP: This defines the LLC interface to MIH. The MIH can directly use the LLC interface to initiate connections and communicate with peer LLC entities and thus establish data path for sending MSDUs across different links. As such no new interfaces or primitives need to be defined for this SAP as well.

# Media Independent Event Service

## Introduction

The Media Independent Event service can support several types of events.

- State Change events

These events correspond to changes in MAC and PHY state. These events correspond to definite changes in state. For example Link_Up event is a case of a state change event.

- Predictive events

Predictive events express the likelihood of change in properties in future based on past and present conditions. For example, decay in signal strength of WLAN network may indicate loss of link connectivity in near future. Since they attempt to predict the future, they may be incorrect and hence there is a need to retract predictive events. Predictive events may carry predictive information including a time bound, specifying the time interval in which the event is expected to occur and a level of confidence that the event shall occur in the specified time bound.

- Management events

Management events help with overall management of the network. For example a BS or AP may force a MT to attach to a new attachment point when load balancing the network.

The following set of events is defined.

| Event Id | Event Class | Event Name |
| --- | --- | --- |
| 1 | State Change | Link Up |
| 2 | State Change | Link Down |
| 3 | Predictive | Link Going Down |
| 4 | Predictive | Link Going Up |
| 5 | Management | Link Event Rollback |
| 6 | Management | Link Available |
| 7 | State Change | Link Parameters Change |
| 8+ | Reserved | Reserved |

## Event Flow Model

The figure above shows the event flow model.

Decision to request Event Registration from peer STA

MIH

MLME

MLME

MIH

STA (Client)

AP

MIH_Event_Register.request

Event Registration REQUEST FRAME

REMOTE EVENTS

MIH_Event_Register.confirm

MIH_EVENT.indication

Actual Event Indication from local or remote MLME

Decision to request Event Deregistration from peer STA

MIH_Event_Deregister.request

Event Deregistration REQUEST FRAME

MIH_Event_Deregister.confirm

Event Registration RESPONSE FRAME

Event Deregistration RESPONSE FRAME

MLME-EVENT.request

MLME-EVENT.confirm

Remote Event Generation

LOCAL EVENTS

## Event Primitives

### MIH_Event_Register.request

#### Function

This primitive is used by a L3MP (the registrant) to register an interest in a particular or group of event types from a particular event origination point. The response indicates which of the requested event types were successfully registered. Event types not successfully registered will not be delivered to the registrant.

#### Semantics of service primitive

MIH_Event_Register.request

(

EventSource,

EventDestination,

RequestedEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | N/A | The origination point from where events are requested. |
| EventDestination | EVENT_LAYER_TYPE | N/A | The destination point where events need to be delivered. |
| RequestedEventList | Set of Event IDs | 1-7 | List of events that the requested endpoint would like to receive indications for from the EventSource |

EVENT_LAYER_TYPE is defined as follows.

{ Local PHY, Local MAC, Local MIH, Remote PHY, Remote MAC, Remote MIH}.

#### When generated

This primitive is generated by a registrant L3MP that is seeking to receive event primitives from a point in the protocol stacks terminating either end of the link.

#### Effect on receipt

The recipient responds immediately with MIH_Event_Register.response primitive.

### MIH_Event_Register.response

#### Function

This primitive returns the result of the registration request.

#### Semantics of service primitive

MIH_Event_Register.response

(

EventSource,

EventDestination,

ResponseEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | N/A | The origination point from where events are requested. |
| EventDestination | EVENT_LAYER_TYPE | N/A | The destination point where events need to be delivered. |
| ResponseEventList | Set of Event IDs and their Registration Status | N/A | List of events along with their registration status |

#### When generated

This primitive is generated in response to a MIH_Event_Register.request primitive.

#### Effect on receipt

The recipient may examine the returned event list and learn about the registration status of different events.

### MIH_Event.indication

#### Function

This primitive delivers event notification to a layered entity that had previously registered to receive such indication.

#### Semantics of service primitive

MIH_Event.indication

(

EventId,

EventSpecificParameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventId | Event IDs | 1-7 | Identification of event for which notification is being delivered. |
| EventSpecificParameters |  | N/A | Other event specific attributes and parameters. |

#### When generated

This primitive is generated when an event notification is to be delivered.

#### Effect on receipt

The recipient may examine the event specific parameters and take event specific action.

## Event Indications

The following are the event indications delivered by the event service.

### Link_Up

This notification is delivered when a layer 2 connection is established on the specified link interface and when L3MP and other upper layers can send layer 3 packets. All layer 2 activities in configuring the link are expected to be completed at this point of time.

Event Type:	State Change

Event Source:	{Local MAC, Remote MAC}

#### Parameters:

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router |
| NetworkIdentifer | Media Specific | Unique network identifier that can be used to detect change in subnet. (SSID+BSSID) serves as unique network identifier for 802.11 networks |

### Link_Down

This notification is delivered when a layer 2 connection is broken on the specified interface, and when no more layer 3 packets can be sent on the specified link

Event Type:	State Change

Event Source:	{Local MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |
| ReasonCode |  | Reason for why the link went down |

### Link_Going_Down

This notification is delivered when a Layer 2 connection is expected (predicted) to go down (Link_Down) within a certain time interval.

Link_Going_Down event may be the signal to initiate handoff procedures. Different applications may have different requirements with respect to the time interval required to anticipate in advance a link going down. Hence the L3MP may actually configure the time interval in which a Link_Going_Down event may be delivered. This would allow the flexibility for the time interval to be in msecs, 100s of msecs or even seconds.

Event Type:	Predictive

Event Source:	{Local MAC, Remote MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |
| TimeInterval | Time in msecs | Time Interval in which the link is expected to go down |
| ConfidenceLevel | % | The confidence level for link to go down within the specified time interval |
| UniqueEventIdentifer |  | To be used in case of event rollback |

### Link_Going_Up

This notification is delivered when a Layer 2 connection is expected (predicted) to go up (Link_Up) within a certain time interval. Link_Going_Up is used in cases wherein the network takes a long time to initialize. In such cases the pending availability of a particular type of network may influence decisions related to Network Discovery and Selection at Layer 3.

Event Type:	Predictive

Event Source:	{Local MAC, Remote MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |
| TimeInterval | Time in msecs | Time Interval in which the link is expected to be up |
| ConfidenceLevel | % | The confidence level for link to be up within the specified time interval |
| UniqueEventIdentifer |  | To be used in case of event rollback |

### Link_Event_Rollback

Link_Event_Rollback is used in conjunction with Link_Going_Up and Link_Going_Down. In case of Link_Going_Down if the link is no longer expected to go down in the specified time interval, then a Link_Event_Rollback message is sent to the Event destination. Similarly in case of Link_Going_Up if the link is no longer expected to go up in the specified interval then a Link_Event_Rollback message is sent to the Event destination. The destination should disregard or rollback the changes associated with the event identifier in such cases.

Event Type:	Management

Event Source:	{Local MAC, Remote MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |
| UniqueEventIdentifer |  | Used to identify the event which needs to be rolled back. |

### Link_Available

Link_Available indicates a link is available for use. If the link indicated for use is the same as one being previously used then, a better network attachment point may be available than the one currently being used.

Link_Available specifies that the Mobile Node may have the possibility of attaching to another AP or Base Station which may have better link quality than the AP or Base Station to which the Mobile Node is currently attached. In some cases the mobile device may actively procure “Scan list” or “Neighborhood list” to figure out the list of neighbor APs or base stations. However the Link_Available trigger helps in notifying the higher layers the event when a specific AP or base station offers significantly better link quality connection than the current one used by the mobile device. There may be situations wherein layer 3 and above may need to make AP or base station selection for handoff on other factors, such as cost , availability of a particular service, etc. as well.

Event Type:	Management

Event Source:	{Local MAC, Remote MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |

### Link_Parameters_Change

Link_Parameters_Change indicates changes in link parameters have crossed specified threshold levels. This may include link layer parameters such as speed of the link, QoS, security settings, etc. The threshold level for each such parameter may have to be configured through a separate call.

Event Type:	State Change

Event Source:	{Local MAC, Remote MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | EVENT_LAYER_TYPE | The origination point from where the event is generated. |
| EventDestination | EVENT_LAYER_TYPE | The destination point where events need to be delivered. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacAccessRouter | MAC Address | MAC Address of current Access Router (if any) |
| oldValueOfLinkParameter |  | Specify the old value of link parameter |
| newValueOfLinkParameter |  | Specify the new value of link parameter |

# Media Independent Information Service

## Introduction

Some networks such as the cellular networks already have an existing means of detecting a list of neighborhood base stations within the vicinity of an area via the broadcast control channel. Other IEEE groups define similar means of detecting a list of neighborhood access points within the vicinity of an area via beaconing.

The Media Independent Information service provides a similar framework across the heterogeneous network environment to facilitate the discovery and selection of multiple types of networks existing within a geographical area. In the larger scope, the macro objective is to acquire a global view of the heterogeneous networks to facilitate seamless handover when roaming across these networks.

## Information Flow

Decision to request Information from peer STA

MIH

MLME

MLME

MIH

STA (Client)

AP

MIH_Info.request

Information REQUEST FRAME

MIH_Info.confirm

Neighbour Map Report Availability Confirmation

Information RESPONSE FRAME

MIH_Info.indication

MIH_Info.response

Neighbour Map Report Generation

## Primitives

### MIH_Information.request

#### Function

This primitive is used by a L3MP or MIH to request information using a specific network interface. The network interface is not explicitly specified in the primitive, but is rather implicitly selected based on the specific MIH_MGMT SAP that is selected.

#### Semantics of service primitive

MIH_Information.request

(

InfoQueryFilter,

InfoQueryParameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryFilter | Combination of QueryFilterType flags | N/A | A set of query filters which control the type and amount of information that is retrieved. |
| InfoQueryParameters |  | N/A | Query filter specific parameters which indicate the type of information the client may be interested in. |
| QueryId | Integer | N/A | A unique QueryId to help match requests with responses. |

QueryFilterType:

- FILTER_INFO_ALL_NETWORKS			Get Info about all networks

- FILTER_INFO_802_NETWORK			Get info about all 802 networks

- FILTER_INFO_802_11_NETWORK			Get info about only 802_11 network

- FILTER_INFO_802_16_NETWORK			Get info about only 802_16 network

- FILTER_INFO_GSM_NETWORK			Get info about only GSM network

- FILTER_INFO_CDMA_NETWORK

- ……

- …..

- FILTER_INFO_ALL_ELEMENTS			Get info about all elements

- FILTER_INFO_ELEMENT_MAC_ADDRESS		Get info about only the MAC Address

- FILTER_INFO_ELEMENT_SECURITY		Get info about only the security

- ….

- ….

#### When generated

This primitive is generated by a L3MP or MIH that is seeking to retrieve information.

#### Effect on receipt

The recipient tries to interpret the query request and retrieve the specified information. Once the information is retrieved the recipient responds with MIH_Information.response primitive.

### MIH_Information.response

#### Function

This primitive is used by MIH to respond to a previous MIH_Information.request primitive.

#### Semantics of service primitive

MIH_Information.response

(

InfoQueryFilter,

MIH_REPORT,

status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryFilter | Combination of QueryFilterType flags | N/A | A set of query filters which control the type and amount of information that is retrieved. |
| MIH_REPORT |  | N/A | Report consisting of information requested by the L3MP or MIH |
| QueryId | Integer | N/A | Used to match response with requests |
| Status | Success/Failure | N/A | Specifies whether the information was successfully retrieved or not. |

#### When generated

This primitive is generated by MIH that is responding to a previous query retrieve information.

#### Effect on receipt

The L3MP or MIH tries to interpret the MIH_REPORT and take suitable action.

## Information Elements

This section lists the information elements of the information service for the various types of network (e.g. IEEE802.11, IEEE802.16, and GSM).

Network

Network_List ::= ENUMERATED {802.11, 802.16, GSM, GPRS, W-CDMA, cdma2000}

802.11

802.11 ::= SEQUENCE{

- Location_LatLong ::= REAL

- Standards ::= ENUMERATED {802.11a, 802.11b, 802.11g}

- SSID_Network_Name ::= String (SIZE(1..32))

- BSSID ::= NumericString (SIZE(8))

- Channel ::= INTEGER

- Phy ::= ENUMERATED {CCK, DSSS, OFDM}

- Data_Rates ::= INTEGER

- Service_Provider_Code ::= String

- Security ::= ENUMERATED {802.1x, WPA, WPA2, WEP, open, MAC Address}

- 802.1x_WPA_WPA2 ::= ENUMERATED {EAP-PEAP-MSCHAPv2, EAP-TTLS-PAP, EAP-TTLS-CHAP, EAP-TLS, EAP-SIM, EAP-AKA}*

- Quality_of_Service :: = ENUMERATED {802.11e}

}

(1) This Information Element (IE) provides the accurate location of the AP in determining the proximity of the APs in the neighborhood.

(2), (6) This IE provides the ‘handover decision maker’ advanced knowledge of the type of wireless chip/physical layer supported at the APs in the neighbourhood. For example, if the client only supports 802.11a wireless standards, the ‘handover decision maker’ will only shortlist neighborhood APs with the 802.11a support.

(3) This IE provides the ‘handover decision maker’ the network that the client/user wishes to connect to.

(4) This IE provides the ‘handover decision maker’ the AP that the client/user wishes to connect to. This is relevant in the case of multiple BSSIDs configured on the AP to support services which offer QoS, authentication type, etc. For example, if user is on a voice conversation, the BSSIDs to associate with may be that configured for higher priority traffic for voice to provide QoS.

(5) This IE provides the client advanced knowledge of the new channel of the AP to associate prior to handover.

(7) This IE provides the ‘handover decision maker’ advanced knowledge of the data rates supported at the AP, e.g. 108 Mbps in 802.11a turbo mode, 11, 5.5, 2, 1 Mbps in 802.11b, etc. Based on certain pre-defined threshold of data rates, the ‘handover decision maker’ may select the appropriate AP for handover.

(8)  This IE provides the ‘handover decision maker’ if there is roaming occurred in handover to an AP belonging to a different service provider, region, or country and whether this is allowed.

(9) This IE provides the ‘handover decision maker’ advanced knowledge of the type pf security implemented at the APs. For example, if the client requires WPA authentication, the ‘handover decision maker’ will only shortlist neighbourhood APs with WPA support.

(10) This IE extends information on the 802.1x/WPA/WPA2 IE, and provides the ‘handover decision maker’ advanced knowledge of the type of EAP authentication supported at the backend network (Radius server), if the 802.1x framework is supported at the AP. This may be helpful as the client may support the 802.1x authentication framework but only specific types of EAP methods. For example, if the client only supports EAP-PEAP-MSCHAPv2, the ‘handover decision maker’ will only shortlist APs that are connected to the Radius servers with this support.

(11) This IE provides the ‘handset decision maker’ advanced knowledge if the AP supports Quality of Service for time-sensitive information or high priority traffic.

802.16

802.16 ::= SEQUENCE {

- Location_LatLong ::= REAL

- Standards ::= ENUMERATED {802.16a, 802.16d, 802.16e}

- Channel ::= INTEGER

- Cell_Identity_CID ::= INTEGER

- Phy ::= ENUMERATED {OFDM}

- Data_Rates ::= INTEGER

- Service_Provider_Code ::= String

- 802.1x_PKM ::= ENUMERATED {EAP-PEAP-MSCHAPv2, EAP-TTLS-PAP, EAP-TTLS-CHAP, EAP-TLS, EAP-SIM, EAP-AKA}*

}

(1) This IE provides the accurate location of the BS in determining the proximity of the BSs in the neighborhood.

(2), (5) This IE provides the ‘handover decision maker’ advanced knowledge of the type of wireless chip/physical layer supported at the BSs in the neighbourhood. For example, if the client only supports 802.16d wireless standards, the ‘handover decision maker’ will only shortlist neighborhood BSs with the 802.16d support.

(3) This IE provides the client advanced knowledge of the new channel of the BS to associate prior to handover.

(4) This IE allows the ‘handover decision maker’ to identify the base station and obtain advanced information about the base station via the Serving_Cell_Channel (BCCH).

(6) This IE provides the ‘handover decision maker’ advanced knowledge of the data rates supported at the BS. Based on certain pre-defined threshold of data rates, the ‘handover decision maker’ may select the appropriate BS for handover.

(7)  This IE provides the ‘handover decision maker’ if there is roaming occurred in handover to an BS belonging to a different service provider, region, or country and whether this is allowed.

(8) This IE provides the ‘handover decision maker’ advanced knowledge of the type of security implemented at the backend network (e.g. EAP type supported at the backend network). For example, if the client only supports EAP-PEAP-MSCHAPv2, the ‘handover decision maker’ will only shortlist BSs that are connected to the Radius servers with this support.

GSM

GSM ::= SEQUENCE {

- Frequency_Band ::= ENUMERATED {900, 1800, 1900}

- Operator_Code_MNC ::= INTEGER

- Location_Area_Code_LAC ::= INTEGER

- Cell_Identity_CID ::= INTEGER

- BSIC ::= INTEGER

- BCCH ::= REAL

- Location_LatLong ::= REAL

}

(1)  This IE provides the ‘handover decision maker’ advanced knowledge of the frequency band supported by the BSs in the neighbourhood. For example, if the handset only supports 1900MHz frequency band, the ‘handover decision maker’ will only shortlist the corresponding neigbourhood BSs.

(2)  This IE provides the ‘handover decision maker’ if there is roaming occurred in handover to a BS belonging to a different service provider, region, or country and whether this is allowed.

(3), (4), (5), (6) This IE allows the ‘handover decision maker’ to identify the base station and obtain advanced information about the base station via the Serving_Cell_Channel (BCCH).

(7) This IE provides the accurate location of the BS in determining the proximity of the BSs in the neighborhood.

## Information Format

The information can be represented in various different formats such as XML, TLVs, Variants, ASN.1 etc. Other IEEE groups use ASN.1 based encoding and hence this particular information format is preferred.

# Media Independent Handover Function

## Introduction

TBD

## MIH Signaling

TBD

## Primitives

TBD

# Media Independent Handover Protocol

## Introduction

TBD

## Description

TBD

### New ethertype

TBD

### Messages

TBD

# Appendix

## Handover Procedures

### MIP based Handover Sequence Diagrams

#### 802.x  802.y Handovers

In this scenario handover between 802.x and 802.y type network are covered. This could for example be a handover from an 802.16 network to 802.11 type network, e.g. user walking from the company campus to inside a building The scenario covers key aspects of Media Independent Information service and Media Independent Event service. The MIIS is used to discover presence of new networks and act as a trigger to initiate handover.

- This represents the steady state connection between MT and the original network. At this point the Mobile terminal uses the MIH Information service and retrieves information about available networks. This includes information about neighbor graphs and other associated information which helps in network discovery and selection.

- This shows the basic flow to retrieve information using the MIH Information service.

- Information can be retrieved in multiple steps using specific query filters etc. if required. The MT can first retrieve information about only the available networks, and then can retrieve details only about selected networks. Such details may include information such as MAC Addresses of different attachment points, link layer parameters for establishing connections, services available on the networks, etc. By selectively retrieving the details the terminal can manage the amount of information it chooses to retrieve. The terminal may choose to retrieve this information on a periodic basis in background. Depending on terminal mobility and network coverage area the terminal can continue to discover information about new networks.

- Once information about a new network is available, the terminal may do a handover evaluation and link selection using specified user policies and properties of different available networks.

- If the handover policy results in selection of a new link, the terminal may actually query the new link about availability of resources to support the current connection on the terminal. The MT in this case may send a MIH_Query to the MIH function of the existing network which in turn may forward the query to the MIH Function of the selected link. The terminal may in this way get a priori advance information about availability of resources at the selected network before actually connecting to it.

- Once resources are available at the new link, the terminal may actually setup a L2 connection using the new link as part of a make before break operation. The terminal may already know the link layer parameters that can be used to establish the connection and this can reduce the time spent in media sense and discovery operations such as probing/scanning etc.

- However if the information specified by Information service is inaccurate, then the terminal may have to resort to more conventional methods of establishing connection. In this case once the terminal has identified appropriate link layer parameters for connection a Link_Available trigger event may be sent to the MIH Function.

- Once the link layer parameters have been identified the terminal may attach to the new link using appropriate link attachment procedures. Once all the link attachment steps are completed and once the link is ready for sending L3 packets, a Link_Up trigger event may be sent to the MIH function both on the local and remote link.

- Once the link has been appropriately setup based on connection requirements the MIH function may actually commit to handover.

- The higher layer protocol FMIPv6 in this case can now prepare to initiate handover, obtain new Care of Address and carry out it’s binding update procedures.

- FMIPv6 may actually send out a router solicitation to the old access router get a new CoA for the new Access Router. Once the new CoA is available since IPv6 is stateless, the old AR may actually send a Handover Initiate message immediately to the new AR. The terminal may also indicate it’s movement from the old AR by sending the Fast Binding Update message. On receiving the Binding Acknowledge, the tunnel between the old AR and new AR is established and the old AR can now start forwarding packets to the new AR.

- The terminal may now disconnect the old link, which may result in the Link_Down event being delivered to the MIH Function on the terminal.

- The terminal at this point is connected to the new Access Router which is also receiving packets from the old AR. Thus the end to end connection is established again and normal packet delivery resumes.

L3  Protocol (FMIPv6)

MAC

MIH

OldAR

MIH

MAC Orig.

MAC New

MAC

MIH

NewAR

Mobile Terminal

Original Network

New Network

Entity

Layer

1.

2.

3.

4.

5.

6.

7.

8.

9.

Query List of Available Networks

Response List of Available Networks

Information Server

Query More Info on 802.y Networks

Response Info on 802.y Networks

MIH_Info.Request

MIH_Info.Indication

MIH_Info.Response

MIH_Info.Confirm

Information Request

Information Response

New Link Available

Handover Evaluation  & Link Selection

Steady state connection between MT and Original Network

Obtain Neighbor Reports and Network Information

Authentication

Association

Link Up

Set up new L2 Configuration.

Use device driver to enable new link. Use link parameters obtained above to connect directly to new link. If parameters not available/incorrect, use Active/Passive Scanning

Link Up (Remote Event)

Handover Commit

MIH_Resource.Query

MIH Query

Query new link for availability of resources to support L3 connection

MIH_Resource.Response

Fast Neighbor Advertisement

Packet delivery to/from New AR

Packet Forwarding

Fast Binding Update

Handover Initiate

Handover Acknowledge

Fast Binding Acknowledge

Link Down

Fast Binding Acknowledge

Disconnect  old AR/link

Connect new AR/link

Router Solicitation for Proxy

Proxy Router Advertisement

Initiate Handover

Obtain CoA

Binding Update

Initiate Handover and Update MIP Signaling.

MIP Signaling

MIH Information Service

MIH Events

L2 Connection

MIH Signaling

10.

11.

12.

13.

L3MP Operation

MIH Operation

Figure 3: MT Initiated FMIPv6 based Handovers between 802.x and 802.y

## Actions by Higher layers on receiving Events

## Media Independent Information Service

### Development and Deployment

## Media Specific Amendments