| 115Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER Functions and Services Specification |  |
| Date Submitted | March, 7th, 2004 |  |
| Source(s) | Alan Carlton, Marian Rudolf, Juan-Carlos Zuniga, Ulises Olvera |  |
| Re: |  |  |
| Abstract | MEDIA INDEPENDENT HANDOVER Functions and Services Specification |  |
| Purpose | Initial Proposal for 802.21 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1 References	4

2 Acronyms and Definitions	4

3 IEEE 802.21 Proposal	5

3.1 Scope of InterDigital 802.21 Contribution	5

3.2 Design Principles	6

3.3 Proposed Logical Network Architecture	7

3.3.1 IEEE 802 Technologies	8

3.4 Proposed Protocol Architecture	8

3.4.1 MIH Reference Model	8

3.4.2 Co-Existence with Technology Specific HO functions	9

3.4.3 MIH Plane Description	10

3.4.4 IEEE 802 Technologies	11

3.4.5 MIH State Machine	12

3.5 Service Access Points (SAPs)	13

3.5.1 Use of Triggers	14

3.5.2 Supported Remote Transport Options	15

3.5.3 Triggers	15

MIH_HO_PREPARE	39

MIH_HO_COMMIT	39

Link_Down	39

MIH_HO_COMMIT	39

3.6 Procedures	40

3.6.2 Intra and Inter 802 Technologies Handover Procedures	62

3.7 Analysis of Interaction with Mobile IP v6 Enhancements	74

3.7.1 Optimal mechanisms for routing to a new location	74

3.7.2 Neighbor discovery	75

3.7.3 Minimum Handover Latency	75

3.7.4 Proprietary signaling	75

3.7.5 Conclusion	75

3.7.6 Implementation with various MIPv6 Enhancements	76

List of Figures

Figure 1 802.21 Logical Network Architecture	7

Figure 2 Generic 802 Network Architecture	8

Figure 3: MIH Reference Model	9

Figure 4: MIH Management Plane	10

Figure 5 Generic Protocol Stack for Handover within 802 Technologies	11

Figure 6 MIH State Machine Definition	12

Figure 7: External Higher Layer SAPs	13

Figure 8: External Lower Layer SAPs	13

Figure 9 Illustration of Use of Triggers	14

Figure 10 System Access, 802.X and 3GPP inter-working (1/2)	40

Figure 11 System Access, 802.X and 3GPP inter-working (2/2)	41

Figure 12 MS Initiated and MS Controlled Handover from 802.X to 3GPP (1/3)	51

Figure 13 MS Initiated and MS Controlled Handover from 802.X to 3GPP (2/3)	52

Figure 14 MS Initiated and MS Controlled Handover from 802.X to 3GPP (3/3)	53

Figure 15 MS Initiated Handover from 3GPP to 802.11	56

Figure 16 MS Initiated Handover from 3GPP to 802.11 (2/4)	57

Figure 17 MS Initiated Handover from 3GPP to 802.11 (3/4)	58

Figure 18 MS Initiated Handover from 3GPP to 802.11 (4/4)	59

Figure 19 MS Initiated 802.X to 802.3 Handover (1/2)	62

Figure 20 MS Initiated 802.X to 802.3 Handover (2/2)	63

Figure 21 MS Initiated 802.3 to 802.X Handover (1/2)	65

Figure 22 MS Initiated 802.3 to 802.X Handover (2/2)	67

Figure 23 MS Initiated MS Controlled Inter-802 Wireless Handover (Mobile IP v4) – Part 1	69

Figure 24 MS Initiated MS Controlled Inter-802 Wireless Handover (Mobile IP v4) – Part 2	70

Figure 25 Network Initiated Network Controlled Inter-802 Handover (Mobile IP v4) – Part 1	72

Figure 26 Network Initiated Network Controlled Inter-802 Handover (Mobile IP v4) – Part 2	73

Figure 27 Inter-802 Fast Handover Message Flow (MS initiated)	78

Figure 28: Inter-802 HMIPv6 Message Flow (MS initiated)	80

# References

- 21-04-0087-12-0000 IEEE P802.21 Media Independent Handover Service Draft Technical Requirements, Sept. 21, 2004.

- TS 23.002 "Network Architecture"

- TS 23.060 "General Packet Radio Service (GPRS)"

- http://www.umatechnology.org

- TR 43.901 “Feasibility Study on Generic Access to A/Gb Interface (Release 6)”

- TR23.234 “3GPP system to Wireless Local Area Network (WLAN) interworking; System description”

- Low latency Handoffs in Mobile IPv4, Submitted: 3-Jun-04

- Hierarchical Mobile IPv6 mobility management (HMIPv6): Submitted: June 15, 2004.

- Fast Handovers for Mobile IPv6, Submitted: 15-Jul-04

- RFC3115: Mobile IP Vendor/Organization-Specific Extensions: Proposed Std. Submitted April 2001

- Fast Handover for Hierarchical MIPv6 (F-HMIPv6): Submitted June 2004

# Acronyms and Definitions

3G				Third Generation

3GPP				3G Partnership Project

3GPP2				3G Partnership Project 2

AAA				Authentication, Authorization, and Accounting

AG				Access Gateway

AN				Access Network

AP				Access Point

AR				Access Router

BS				Base Station

BSC				Base Station Controller

BSSID   			Basic Service Set Identifier

BTS				Base Transceiver Station

BU				Binding Update

ESS        			Extended Service Set

CoA				Care of Address

CoN 				Correspondent Node

CN				Core Network

CVSE 				Critical Vendor/Organization Specific Extensions

ESSID				Extended Service Set ID

FA				Foreign Agent

FBU				Fast-Binding Update

F-HMIP 			Fast Handover for Hierarchical Mobile IP

FMIP				Fast Handover Mobile IP

FNA				Fast Neighbor Advertisement

GGSN				Gateway GPRS Support Node

GPRS				General Packet Radio Service

GSM				Global System for Mobile Communication

HLCF				Higher Layer Convergence Function

HA				Home Agent

HAck				Handover Acknowledge

HI				Handover Initiate

HMIP				Hierarchical Mobile IP

HO				Handover

IEEE				Institute of Electrical and Electronics Engineers

IETF				Internet Engineering Task Force

ICMP				Internet Control Message Protocol

IP				Internet Protocol

ISP				Internet Service Provider

ITU				International Telecommunications Union

L1                			Physical Layer (PHY)

L2			         	Medium Access Control (MAC)

LAN				Local Area Network

LCoA				On-Link Care of Address

LLC            			Logical Link Control

LLCF				Lower Layer Convergence Function

MA				Media Access

MAC				Medium Access Control

MAP				Mobility Anchor Point

MIH	          			Media Independent Handover

MIHO        			Media Independent Handover

MIHS        			Media Independent Handover Services

MIP				Mobile IP

MLME        			MAC Layer Management Entity

MN				Mobile Node

MS				Mobile Station

MT		   		Mobile Terminal

NVSE 				Normal Vendor/Organization Specific Extensions

PDG				Packet Data Gateway

PHY            			Physical Layer

PLMN				Public Land Mobile Network

QoS				Quality of Service

RCoA				Regional Care of Address

RFC				Request for Comment

RNC				Radio Network Controller

SAP             			Service Access Point

SGSN				Serving GPRS Support Node

SNR				Signal Noise Ratio

STA				Station

TCP				Transmission Control Protocol

UDP				User Datagram Protocol

UMTS				Universal Mobile Telecommunications System

WAG				Wireless Access Gateway

WLAN				Wireless Local Area Network

WPAN				Wireless Personal Area Network

WMAN				Wireless Metropolitan Area Network

# IEEE 802.21 Proposal

## Scope of InterDigital 802.21 Contribution

According to [1],

"The purpose of 802.21 is to enhance user experience of mobile devices by supporting seamless handover between heterogeneous networks"

Our solution will support service continuity and will enable seamless handover.  Based on this main requirement the scope of our work will be:

- Define an architecture that enables transparent service continuity across heterogeneous networks.

- Define required functions and their allocation within the protocol stack and network.

  - Information services (network discovery and optimal system access)

  - Event indication (handover triggers classification)

  - Media Independent Handover (MIH) functions (handover services)

## Design Principles

The current MIH solution has been conceived with the following design principles in mind:

- The Media Independent Handover Functions (MIH) are provided on a Management Plane parallel to the User Plane similar to what it is considered in the 802.16g Task Group (802.16 Management Plane Procedures and Services) PAR. The core of the MIH function is located in a sub-layer called MIH Handover Functions (MIH HO).

- Specific convergence layers are provided for interfacing with both, media access technologies at the lower layers and service applications at the higher layers

- Accordingly, a Mobile IP (MIP) higher layer convergence function provides coupling and support for inter-working with other technologies

- Cellular inter-working support is provided with a dedicated convergence function for 3GPP technologies, and through MIP for 3GPP2

- The current proposal supports both network-initiated and mobile station-initiated handover

- In order to support service continuity and to enable for seamless handover, the handover procedure is performed in a make-before-break fashion

- Coexistence and harmonization with technology-specific handover functions is considered and a co-ordination strategy is proposed

- Transparency is envisioned when inter-working with legacy equipment; so 802.21-compatible devices should be able to co-exist with non-802.21 systems

## Proposed Logical Network Architecture

Figure 1 802.21 Logical Network Architecture

### IEEE 802 Technologies

Figure 2 Generic 802 Network Architecture

In Figure 2, an 802 instantiation of the 802.21 network with two handover scenarios is depicted. The first scenario shows the mobility between two different 802 Access Networks (ANs), in which case a Mobile IP handover is required. The second scenario shows a handover from one 802 Media Access (MA) to another, but within the same Access Network. In this case, since mobility can be handled below layer 3, Mobile IP is not required. However, 802.21 still provides the hooks for handing over between the two different 802 access technologies.

## Proposed Protocol Architecture

### MIH Reference Model

In order to provide correct Media Independent Handover (MIH) operation, an MIH Handover entity (MIH HO) shall be present. The MIH HO is a layer-independent entity that may be viewed as residing in a separate management plane or as residing “off to the side.” This management plane is referred to as MIH Management Plane and is parallel to the User Plane.

The MIH reference model is designed to be generic. It is able to work independently as the sole handover management entity, or coordinate with existing handover entities specific to different technologies.  The technology specific handover function is optional and shown in dotted line.

When no other HO functions exist, the HO triggers from PHY and MAC are sent to MIH LLCF directly. The MIH LLCF has interfaces with MAC and PHY. The MIH functions will handle handover related tasks.

When the MIH functions co-exist with technology specific HO functions, a two-tier handover method is used as shown in the figure. The MIH HLCF interfaces with higher layer HO functions.

The procedures required for performing MIH HO functions are defined in this standard. Determination of the need for a handover, selection of the system to which the MS should handover, and determination of when to perform the handover are all outside the scope of this project.

The MIH HLCF and MIH LLCF interfaces are implementation specific and therefore are not defined in the standard. Any description of MIH HLCF and LLCF in this document is informative only.

Figure 3: MIH Reference Model

### Co-Existence with Technology Specific HO functions

As illustrated in Figure 3, the MIH management can co-exist with other intra-technology handover management functions, for example, 802.11r intra-ESS fast handover and 802.16Netman mobility functions. The MIH reference model enables the co-existence and co-ordination with intra-technology handover functions. MIH triggers have been defined to facilitate the negotiation and co-ordination procedures.

Multiple scenarios can happen when the MIH management co-exists with the intra-technology handover function, for example, one handover management can take control of the handover procedures, or a combination of functions from both can be achieved. The realization is implementation specific.

### MIH Plane Description

The MIH HO entity interfaces with the system at both higher and lower layers through Convergence Functions. These functions are system-specific and a multiple of them could be present in order to support all system-specific features. The different entities within this model interact in various ways. The convergence layer functions are implementation specific and not defined in the standard. The description here is informative.

There are three main Higher Layer Convergence Functions (HLCF), one for interfacing with the Cellular system, one for Mobile IP interactions, and one for handover within the same IP subnet.

Similarly, there are Lower Layer Convergence Functions (LLCF) for each one of the access technologies covered in this standard, being Cellular, IEEE 802.3 (LAN), IEEE 802.11 (WLAN), IEEE 802.15 (WPAN), and IEEE 802.16 (WMAN). Figure 4 depicts the relationship among the different entities.

Figure 4: MIH Management Plane

#### Brief Function Descriptions

| Functional Entity | Description |
| --- | --- |
| MIH HLCF | MIH Higher Layer Convergence Function. This entity provides an interface between the MIH Handover entity and the mobility management entity in a particular technology. E.g., it provides the interface to various mobility management protocols. |
| MIH HO | MIH Handover Function. This function collects handover events from MIH LLCF and determines whether a handover is required based on certain criteria, for example, Link Quality, Service, and Subscription. |
| MIH LLCF | Media-Independent Handover Lower Layer Convergence Function. This function provides an event service that compiles Physical Layer and MAC Layer events specific to a particular technology. The event service can be configured in order to determine the set of MAC and physical layer measurements that need to be collected. When certain events or a collection of them meet certain configured criteria (e.g., SNR threshold) an event indication is generated for further processing. |

### IEEE 802 Technologies

The following figures depict the characteristics of the proposed multi-mode stack at the MS side and how it fits within a heterogeneous 802 network.

Figure 5 Generic Protocol Stack for Handover within 802 Technologies

### MIH State Machine

Figure 6 MIH State Machine Definition

Figure 6 shows a generic MIH state machine that can be applied to both network and mobile sides and whether the handover is network initiated or MS initiated. It consists of five states, namely, Init, Network Discovery/Update, MIH Steady State, MIH HO Prepare and MIH Handover Commit.

Init State: Handover configuration parameters are initialized in the init state and then make a transition to the Network Discovery Update.

Network Discovery/Update: After initialization, MIH layer gathers information about system conditions and network topology including neighbor lists from different technology. During operation, MIH performs network updates to get the latest system conditions.

MIH Steady State: MIH steady state represents the state when link condition is good and there is no need to perform handover. However, network discovery can be performed in the background to get the latest neighbor list conditions.

MIH HO Prepare: Upon receiving MIH handover request, if there is no intra-technology handover in progress, MIH handover will prepare a new link without releasing the established link (make before break). If MIH HO setup is aborted MIH will go back to MIH steady state. However, if MIH HO preparation is accomplished properly the MIH will make a transition to MIH HO Commit as long as there is no intra-technology handover happening in the background.

MIH Handover Commit: If the MIH handover succeeded the MIH will make a transition back to MIH Steady State, but if it failed it will make a transition back to MIH HO Prepare.

## Service Access Points (SAPs)

In a layered protocol system, the information flow across the boundaries between the layers can be described in terms of Service Access Points (SAPs), which are defined by primitives that represent different items of information and cause actions to take place. Only External Primitives (i.e. between the MIH and User Planes) are discussed here.

Since there are several sublayers, SAPs are divided into those providing MIH HO services to the Convergence Functions above the MIH (i.e. MIH Higher Layer Convergence Functions), the ones providing MIH HO services to the Convergence Functions below the MIH (i.e. MIH Lower Layer Convergence Functions), and a set of services provided to the external (non-IEEE Std 802.21) layers by each one of the specific Convergence Functions. The external SAPs are shown in Figure 7 and Figure 8.

Figure 7: External Higher Layer SAPs

Figure 8: External Lower Layer SAPs

### Use of Triggers

The use of triggers to provide internal, external and peer communication is shown in Figure 9. These triggers do not appear as such on the medium (e.g. the access interface), but serve to define more clearly the relationships between the different layers and planes.

In Figure 9 three groups of examples are shown for the use of triggers. They are identified as A, B, and C. The number illustrates the sequence of the triggers within the same group.

Group A illustrates a peer-to-peer communication scenario. The initial request for service from a layer (i.e. MIH HO) to the convergence function is provided by the “request” trigger. When this request is sent across the link to the peer convergence function, it generates an “indication” trigger to inform the peer MIH HO function of the request; the MIH HO entity responds with a “response” to the convergence function. Again this is sent across the link to the convergence function on the originating side, which sends a “confirmation” trigger to the original requesting MIH HO entity.

When the information is transported from the higher level entity to the lower level entity within the same management plane and within the same node, a pair of “request” and “confirmation” is used as shown by 1A and 4A.

Group B illustrates the scenario when the information is transported from the lower level entity to the higher level entity within the same management plane and within the same node. A pair of “indication” and “response” is used as shown by “1B” and 2B”.

Group C illustrates the scenario when the information is exchanged between the MIH management plane and the user plane, for example, between MIH HLCF and Mobile IP. A pair of “indication” and “response” is used as shown by “1C” and 2C”.

Figure 9 Illustration of Use of Triggers

### Supported Remote Transport Options

This proposal supports the following options for transport mechanisms.

Option 1) MIH sends generic messages that are passed as primitives to L2 (e.g. MAC) specific layers, so that dedicated (management) messages can be used to exchange the information and be delivered (as SAP primitives) at the other side (i.e. peer MIH function).

Option 2) MIH generates and exchanges messages through MIP vendor-specific extensions

Option 3) Ether type frames can be used, similarly to 802.1X, to exchange information between management planes.

Option 4) A hybrid approach can be used, on which different convergence layers implement different transport mechanisms.

### Triggers

The following triggers are supported at the MIH SAP. Internal triggers are those triggers within MIH functions. External triggers are those between MIH management plane and user plane protocols.

Table 1 External Triggers

| Triggers | Source | Destination | Local / Remote | Description |
| --- | --- | --- | --- | --- |
| MIH_PHY.set | MIH LLCF | PHY | both | MIH LLCF configures PHY for information services |
| MIH_PHY.get | MIH LLCF | PHY | both | MIH LLCF enquires configuration info |
| MIH_PHY.reset | MIH LLCF | PHY | both | MIH LLCF resets PHY configuration |
| MIH_MAC.set | MIH LLCF | MAC | both | MIH LLCF configures MAC for information services |
| MIH_MAC.get | MIH LLCF | MAC | both | MIH LLCF enquires configuration info |
| MIH_MAC.reset | MIH LLCF | MAC | both | MIH LLCF resets MAC configuration |
| MIH_MACINFO.indication | MAC | MIH LLCF | local | MAC sends system information |
| MIH_MACINFO.response | MIH LLCF | MAC | local | Response for the above indication. |
| MIH_INFO.indication | MIH LLCF MIH HLCF | L2, L3 handover | local | MIH entities indicate the existence of MIH functions |
| MIH_INFO.response | L2, L3 handover | MIH LLCF MIH HLCF | local | Response for the above indication. |
| MIH_PHYEVENT.indication | PHY | MIH LLCF | both | PHY measurement reports |
| MIH_PHYEVENT.response | MIH LLCF | PHY | both | Response for the above indication. |
| MIH_MACEVENT.indication | MAC | MIH LLCF | both | MAC measurement reports |
| MIH_MACEVENT.response | MIH LLCF | MAC | both | Response for the above indication. |
| MIH_MACORDER.request | MIH LLCF | MAC | local | MIH LLCF request HO related MAC actions |
| MIH_MACORDER.confirmation | MAC | MIH LLCF | local | MAC informs MIH LLCF if the action is accomplished |
| MIH_HANDOVER_PREPARE.indication | MIH HLCF MIH LLCF | MIP MAC | local | MIH HLCF and LLCF inform external entities to prepare for handover |
| MIH_HANDOVER_PREPARE.response | MIP MAC | MIH HLCF MIH LLCF | local | Response for the above indication. |
| MIH_HANDOVER_COMMIT.indication | MIH HLCF MIH LLCF | MIP MAC | local | MIH HLCF and LLCF inform external entities to execute for handover |
| MIH_HANDOVER_COMMIT.response | MIP MAC | MIH HLCF MIH LLCF | local | Response for the above indication. |

Table 2 Internal Triggers

| Triggers | Source | Destination | Local / Remote | Description |
| --- | --- | --- | --- | --- |
| MIH_SYSINFO.indication | MIH LLCF | MIH HO | local | MIH LLCF informs HO of any system information update |
| MIH_SYSINFO.response | MIH HO | MIH LLCF | local | Response for the above indication. |
| MIH_MOBILITY.request | MIH HO | MIH LLCF | both | MIH HO sends handover decision to MIH LLCF |
| MIH_MOBILITY.indication | MIH HO | MIH HLCF | both | MIH HO informs MIH HLCF and peer MIH HO of handover decision |
| MIH_MOBILITY.response | MIH HLCF | MIH HO | both | Response of the above indication |
| MIH_MOBILITY.confirmation | MIH LLCF | MIH HO | both | Confirmation of the MIH HO mobility request |
| MIH_REPORT.indication | MIH LLCF | MIH HO | local | MIH LLCF sends MIH triggers to MIH HO |
| MIH_REPORT.response | MIH HO | MIH LLCF | local | Response of the above indication |
| MIH_MACRELEASE.request | MIH HO | MIH LLCF | local | When handover procedures finished, MIH HO informs MIH LLCF |
| MIH_MACRELEASE.confirmation | MIH LLCF | MIH HO | local | Confirmation of the above request |

#### External Triggers

##### MIH_PHY.set

###### Function

MIH LLCF uses this trigger to request activation of the information service within the relevant 802 physical layers. The trigger conveys inter-technology configuration information that might be used by physical layers to determine when an event should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_PHY.set

(

event type

reporting period

inter-technology measurement channel numbers, frequency bands pair

technology specific performance thresholds

)

The request id provides a unique identifier that can be used to match this request with the associated confirmation message.

The event type is used to activate an event or a collection of events belonging to a predetermined category. E.g., events can be categorized as data throughput events or link quality related events.

The reporting period is used to specify how often an event should be reported, The reporting period can also be used to specify how often an event can be reported when a threshold has been crossed and the condition that triggered the event still prevails.

Technology specific performance thresholds are used to specify the minimum or maximum value that a measured system condition can take in order for an event notification to be triggered.

###### When generated

MIH LLCF generates this trigger, when it configures and activates the event service at the physical layer.

###### Effect of receipt

The physical layer configures and activates the event service according to the system parameter provided in the trigger.

##### MIH_PHY.get

###### Function

MIH LLCF uses this trigger to request information on the current configuration of the information service within the physical layer.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_PHY.get

(

event type info

)

The event type info is used to request information on a particular event. In addition, it is possible to request information for all event types currently configured. E.g., events can be categorized as data throughput events or link quality related events.

###### When generated

MIH LLCF generates this trigger, when configuration information for a particular event is required.

###### Effect of receipt

The physical layer provides information on the event or events specified by the event type information element.

##### MIH_PHY.reset

###### Function

MIH LLCF uses this trigger to initialize the configuration information service for a particular event of collection of them.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_PHY.reset

(

event type info

)

The event type info is used to request initialization of configuration data for a particular event or collection of them. E.g., events can be categorized as data throughput events or link quality related events.

###### When generated

MIH LLCF generates this trigger, when configuration information for a particular event or collection of them needs to be initialized.

###### Effect of receipt

The physical layer initializes configuration information on the event or events specified by the event type information element.

##### MIH_MAC.set

###### Function

MIH LLCF uses this trigger to request activation of the information service within the relevant 802 MAC layers. The trigger conveys inter-technology configuration information that might be used by MAC layers to determine when an event should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MAC.set

(

event type

reporting period

inter-technology system operator parameters

measurement channel technology

measurement channel numbers and frequency bands

technology specific header compression parameters

technology specific performance thresholds

)

The event type is used to configure and activate an event or a collection of events belonging to a predetermined category. E.g., events can be categorized as data throughput events or link quality related events.

The reporting period is used to specify how often an event should be reported. The reporting period can also be used to specify how often an event can be reported when a threshold has been crossed and the condition that triggered the event still prevails.

Inter-technology system operator parameters might be used to specify e.g., the cellular system operator code or system id belonging to other access technologies available within the current network.

Measurement channel technology is used to specify the technology that a channel requiring inter-technology measurements belongs to.

Measurement channel numbers and frequency bands are used to specify the frequency band and channel numbers that a channel requiring inter-technology measurements belongs to.

Technology specific header compression parameter are used to configure a particular header compression scheme.

Technology specific performance thresholds are used to specify the minimum or maximum value that a measured system condition can take in order for an event notification to be triggered.

###### When generated

MIH LLCF generates this trigger, when it configures and activates the event service at the MAC layer.

###### Effect of receipt

The MAC layer configures and activates the event service according to the system parameter provided in the trigger. This parameter might be set as an extension to the capability information field within a beacon or probe response message

##### MIH_MAC.get

###### Function

MIH LLCF uses this trigger to request information on the current configuration of the information service within the MAC layer.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MAC.get

(

event type info

)

The event type info is used to request information on a particular event. In addition, it is possible to request information for all event types currently configured. E.g., events can be categorized as data throughput events, link quality event or service related events.

###### When generated

MIH LLCF generates this trigger, when configuration information for a particular event is required.

###### Effect of receipt

The MAC layer provides information on the event or events specified by the event type information element.

##### MIH_MAC.reset

###### Function

MIH LLCF uses this trigger to initialize the configuration information service for a particular event of collection of them.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MAC.set

(

event type info

)

The event type info is used to request initialization of configuration data for a particular event or collection of them. E.g., events can be categorized as data throughput events, service events or link quality related events.

###### When generated

MIH LLCF generates this trigger, when configuration information for a particular event or collection of them needs to be initialized.

###### Effect of receipt

The MAC layer initializes configuration information on the event or events specified by the event type information element.

##### MIH_MACINFO.indication

###### Function

The MAC layer uses this trigger to transfer MIH system information from the infrastructure information service. The trigger conveys inter-technology configuration information that that might be used by the MIH LLCF to configure the specific event within the information service.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACINFO.indication

(

indication id

event type

inter-technology system operator parameters

technology specific header compression parameters

inter-technology measurement channel numbers, frequency bands pair

technology specific performance thresholds

)

The indication id provides a unique identifier that can be used to match this indication with the associated response.

The event type is used to activate an event or a collection of events belonging to a predetermined category. E.g., events can be categorized as data throughput events or link quality related events.

Inter-technology system operator parameters might be used to specify e.g., the cellular system operator code or system id belonging to other access technologies available within the current network.

Technology specific header compression parameter are used to configure a particular header compression scheme.

Inter-technology measurement channel numbers and frequency bands are used to specify the frequency band and channel numbers that a channel requiring inter-technology measurements belongs to

Technology specific performance thresholds are used to specify the minimum or maximum value that a measured system condition can take in order for an event notification to be triggered.

###### When generated

The MAC layer generates this trigger upon receipt of MIH information service detected within a Beacon or Probe response message.

###### Effect of receipt

MIH LLCF categorized the event type and generates the relevant invent information trigger towards the MIH HO function.

##### MIH_MACINFO.response

###### Function

The MIH LLCF entity uses this trigger to indicate the result of a previously received indication message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACINFO.response

(

indication id

)

The indication id identifies the indication message for which the response is being issued.

###### When generated

The MIH LLCF generates this trigger as a response to the correspondent indication.

###### Effect of receipt

The MAC layer might use the indication id to determine whether or not an indication message has been acted upon.

##### MIH_INFO.indication

###### Function

The MIH function uses this trigger to inform the existence of the MIH plane, and coordinate with other intra-technology handover functions.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_INFO.indication

(

indication id

co-existence type

)

The indication id provides a unique identifier that can be used to match this indication with the associated response.

The co-existence type defines the co-existence mechanism between MIH and the technology specific handover mechanism.

###### When generated

The MIH LLCF or HLCF generates the trigger as configuration message.

###### Effect of receipt

L2 or L3 forwards the information to the technology specific handover management entities, and generates a response message.

##### MIH_INFO.response

###### Function

The L2 or L3 entity uses this trigger to indicate the result of a previously received indication message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_INFO.response

(

indication id

)

The indication id identifies the indication message for which the response is being issued.

###### When generated

The L2 or L3 generates this trigger as a response to the correspondent indication.

###### Effect of receipt

The MIH LLCF or HLCF might use the indication id to determine whether or not an indication message has been acted upon.

##### MIH_PHYEVENT.indication

###### Function

The PHY entity uses this trigger to provide current measurement reports that can be used by the MIH entities to decide if a handover is necessary.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_PHYEVENT.indication

(

indication Id

event type

inter-technology channel measurement report

technology specific performance report

)

The indication id provides a unique identifier that can be used to match this indication with the associated response.

The event type is used to indicate nature of the event or a collection of events that triggered the event notification. E.g., data throughput performance threshold has been crossed.

The inter-technology channel measurement report provides an indication of the received signal strength on relevant channels belonging to one or several inter-technology systems within the neighborhood of the current serving network.

The technology specific performance report provides system measurements specific to an access technology.

###### When generated

The relevant PHY layer entity generates this trigger when system performance thresholds have been crossed. E.g., a RSSI measurement is received below specified threshold.

###### Effect of receipt

The MIH LLCF filters and categorizes the information received in this trigger and it determines whether a mobility indication trigger is warranted.

##### MIH_PHYEVENT.response

###### Function

The MIH LLCF entity uses this trigger to indicate the result of a previously received indication message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_PHYEVENT.response

(

indication id

inter-technology channel resource

)

The indication id identifies the indication message for which the response is being issued.

The inter-technology channel resource information element provides technology specific channel resource information such as channel number, frequency band and channel synchronization information.

###### When generated

The MIH LLCF generates this trigger as a response to the correspondent indication. The trigger might carry potential mobility management a resource management information that can be used to execute handovers across inter technology boundaries.

###### Effect of receipt

The PHY layer might use the indication id to determine whether or not an indication message has been acted upon.

##### MIH_MACEVENT.indication

###### Function

The MAC entity uses this trigger to provide system information that can be used in the selection of a new network. The trigger conveys system performance information that is used to determine when a mobility indication event should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACEVENT.indication

(

event id

event type

technology specific performance report

technology specific header compression context parameters

)

The event id provides a unique identifier that can be used to match this indication with the associated response message.

The event type is used to activate an event or a collection of events belonging to a predetermined category. E.g., events can be categorized as data throughput events or link quality related events.

Technology specific header compression context information specifies the values of the header compression packet stream that requires to be transferred

Technology specific performance report specifies the values of the performance parameters that trigger the event. E.g., Retransmission counter has reached its limit.

###### When generated

The MAC layer generates this trigger when a thresholds requiring event notification have been crossed.

###### Effect of receipt

The MIH LLCF filters and categorizes the information received in this trigger and it determines whether a mobility indication request is warranted.

##### MIH_MACEVENT.response

###### Function

The MIH LLCF entity uses this trigger to indicate the result of a previously received indication message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACEVENT.response

(

indication id

inter-technology channel resource vector

mobile ip routing

)

The indication id identifies the indication message for which the response is being issued.

The inter-technology channel resource vector provides technology specific channel resource information such as channel number, frequency band and channel synchronization information.

The mobile ip routing information element provides mobile ip information such as default router or routers and associated CoA list.

###### When generated

The MIH LLCF generates this trigger as a response to the correspondent indication. The trigger might carry potential mobility management a resource management information that can be used to execute handovers across inter technology boundaries.

###### Effect of receipt

The MAC layer might use the indication id to determine whether or not an indication message has been acted upon. The MAC layer transfers the received information that might be used to generate handover targets.

##### MIH_MACORDER.request

###### Function

The MIH LLCF entity uses this trigger to request handover operation from L2.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACORDER.request

(

request id

MIH request

inter-technology channel resource

)

The inter-technology channel resource information element provides technology specific channel resource information such as channel number, frequency band and channel synchronization information.

###### When generated

The MIH LLCF generates this trigger when MAC actions are required to accomplish the MIH HO decision.

###### Effect of receipt

The MAC layer acts upon the MIH trigger for HO related L2 procedures.

##### MIH_MACORDER.confirmation

###### Function

The MAC layer uses this trigger to indicate the result of a previously received request message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACORDER.confirmation

(

request id

)

The request id identifies the request message for which the response is being issued.

###### When generated

The MAC layer generates this trigger when the MIH request has been processed.

###### Effect of receipt

The MIH LLCF layer might use the request id to determine whether or not a request message has been acted upon.

###### MIH_HANDOVER_PREPARE.request

###### Function

The upper layer uses this trigger to indicate the availability of an alternate link or requirement for a new link.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_HANDOVER_PREPARE.request

(

TriggerType

Destination

Receiver

Source

Sender

ServiceReferenceIdList

)

TriggerType identifies the type of trigger conveyed with this indication

Destination identifies the entity that is ultimately processing the trigger.

Receiver identifies the entity that is receiving the trigger indication

Source identifies the entity originating the trigger

Sender identifies the entity that sending the trigger to the next layer

ServiceReferenceIDList provides a list of unique identifier that describe the services supported through the current interface

###### When generated

The upper layer generates this trigger when it determines that a new link is required or to signal the availability of a new link.

###### Effect of receipt

MIH entity receiving this trigger determines whether a handover toward a new link can be initiated or a new link can be establish in order to relief the current link.

###### MIH_HANDOVER_PREPARE.indication

###### Function

MIH uses this trigger to provide upper layers with an indication regarding the conditions prevailing at the link layer supporting the current interface.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_HANDOVER_PREPARE.indication

(

TriggerType

Destination

Receiver

Source

Sender

ServiceReferenceId

)

TriggerType identifies the type of trigger conveyed with this indication

Destination identifies the entity that is ultimately processing the trigger.

Receiver identifies the entity that is receiving the trigger indication

Source identifies the entity originating the trigger

Sender identifies the entity that sending the trigger to the next layer

ServiceReferenceIDList provides a list of unique identifier that describe the services supported through the current interface

###### When generated

MIH generates this trigger when it determines that the current link interface is no longer suitable to satisfy the carried service.

###### Effect of receipt

The upper layer entity receiving this trigger might use this information to optimized handover procedures in accordance to it internal policies.

###### MIH_HANDOVER_PREPARE.response

###### Function

The upper layer entity delivers this primitive in reply to the associated indication as specified by the corresponding ServiceReferenceId. It provides information on the actions taken by the upper layer in response to the associated indication trigger.

###### Semantics of the service trigger

The parameters of the primitive are as follows:

MIH_HANDOVER_PREPARE.response

(

Destination

Receiver

Source

Sender

ServiceReferenceId

Result

)

Destination identifies the entity that is ultimately processing the trigger.

Receiver identifies the entity that is receiving the trigger indication

Source identifies the entity originating the trigger

Sender identifies the entity that sending the trigger to the next layer

ServiceReferenceID provides a list of unique identifier that describe the services supported through the current interface

Result specifies the outcome of the actions triggered by the associated indication message.

###### When generated

The upper layer entity generates this primitive based on the type of trigger specified within the correspondent request. The outcome of the request might also be provided.

###### Effect of receipt

MIH layer uses the result provided by the primitive to determine the availability of an alternate link.

###### MIH_HANDOVER_PREPARE.confirm

###### Function

MIH uses this trigger to confirm the receipt of a request to provide an alternate link or acknowledgement that a new link is available

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_HANDOVER_PREPARE.confirm

(

TriggerType

Destination

Receiver

Source

Sender

ServiceReferenceId

)

TriggerType identifies the type of trigger conveyed with this indication

Destination identifies the entity that is ultimately processing the trigger.

Receiver identifies the entity that is receiving the trigger indication

Source identifies the entity originating the trigger

Sender identifies the entity that sending the trigger to the next layer

ServiceReferenceID provides a list of unique identifier that describe the services supported through the current interface

###### When generated

The upper layer generates this trigger when it determines that a new link is required or to signal the availability of a new link.

###### Effect of receipt

MIH entity receiving this trigger determines whether a handover toward a new link can be initiated or a new link can be establish in order to relief the current link

###### MIH_HANDOVER_COMMIT.indication

###### Function

MIH uses this trigger to provide upper layers with an indication regarding the availability of a link previously reported as unsuitable to provide the relevant service.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_HANDOVER_COMMIT.indication

(

TriggerType

Destination

Receiver

Source

Sender

ServiceReferenceId

)

TriggerType identifies the type of trigger conveyed with this indication

Destination identifies the entity that is ultimately processing the trigger.

Receiver identifies the entity that is receiving the trigger indication

Source identifies the entity originating the trigger

Sender identifies the entity that sending the trigger to the next layer

ServiceReferenceID provides a list of unique identifier that describe the services supported through the current interface

###### When generated

MIH layer generates this trigger to indicate whether the current link is no longer supporting the requested service.

###### Effect of receipt

The upper layer might use this information to modified the outcome of a L3 handover in accordance to its internal policies.

#### Internal Triggers

##### MIH_SYSINFO.indication

###### Function

MIH LLCF uses this trigger to request transfer system information as configured by the information service. The trigger conveys inter-technology configuration information that might be used by physical layers to determine when an event should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_SYSINFO.indication

(

event type

reporting period

technology specific performance thresholds

inter-technology system operator parameters

measurement channel technology

technology specific header compression parameters

inter-technology measurement channel numbers, frequency bands pair

)

The event type is used to activate an event or a collection of events belonging to a predetermined category. E.g., events can be categorized as data throughput events or link quality related events.

The reporting period is used to specify how often an event should be reported, The reporting period can also be used to specify how often an event can be reported when a threshold has been crossed and the condition that triggered the event still prevails.

Technology specific performance thresholds are used to specify the minimum or maximum value that a measured system condition can take in order for an event notification to be triggered.

Inter-technology system operator parameters might be used to specify e.g., the cellular system operator code or system id belonging to other access technologies available within the current network.

Measurement channel technology is used to specify the technology that a channel requiring inter-technology measurements belongs to.

Technology specific header compression parameters are used to configure a particular header compression scheme.

Measurement channel numbers and frequency bands are used to specify the frequency band and channel numbers that a channel requiring inter-technology measurements belongs to

###### When generated

MIH LLCF generates this trigger on receipt of new or modified system information that might trigger a mobility event.

###### Effect of receipt

MIH HO uses this information to set triggering parameter values and to determine whether a new system access is required.

##### MIH_SYSINFO.response

###### Function

The MIH HO entity uses this trigger to indicate the result of a previously received request message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_SYSINFO.response

(

indication id

)

The request id identifies the request message for which the response is being issued.

###### When generated

The MIH HO generates this trigger as a response to the correspondent request.

###### Effect of receipt

The MIH LLCF layer might use the request id to determine whether or not a request message has been acted upon

##### MIH_MOBILITY.request

###### Function

MIH HO uses this trigger to provide system information that might be used by the peer MIH HO to determine whether a handover should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MOBILITY.request

(

inter-technology channel measurement report

MIP routing

technology specific performance report

technology specific header compression context parameters

)

The inter-technology channel measurement report provides an indication of the received signal strength on relevant channels belonging to one or several inter-technology systems within the neighborhood of the current serving network.

The MIP routing information element provides details regarding current MIP routing configuration parameters. E.g., CoA list.

The technology specific performance report provides system measurements specific to an access technology specific header compression context information specifies the values of the header compression packet stream that requires to be transferred.

###### When generated

MIH HO generates this trigger when a compile mobility report message, made up of one or several event types, is deemed to be required based on predefined thresholds.

###### Effect of receipt

MIH LLCF relays the content of the trigger to its peer.

##### MIH_MOBILITY.indication

###### Function

MIH LLCF uses this trigger to provide system information that might be used to determine whether a handover should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MOBILITY.indication

(

inter-technology channel measurement report

MIP routing

technology specific performance report

technology specific header compression context parameters

)

The inter-technology channel measurement report provides an indication of the received signal strength on relevant channels belonging to one or several inter-technology systems within the neighborhood of the current serving network.

The MIP routing information element provides details regarding current MIP routing configuration parameters. E.g., CoA list

The technology specific performance report provides system measurements specific to an access technology

Technology specific header compression context information specifies the values of the header compression packet stream that requires to be transferred

###### When generated

The MIH LLCF layer relays this trigger when it is required to transfer system information from the MAC layer to the MIH HO.

###### Effect of receipt

The MIH HO process the full content of the trigger in order to determine whether a handover shall be triggered towards a better point of attachment.

##### MIH_MOBILITY.response

###### Function

The MIH HO entity uses this trigger to provide details regarding the selected resource that shall constitute the new point of attachment.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MOBILITY.response

(

indication result

inter-technology channel resource

mobile ip routing

)

The inter-technology channel resource information element provides technology specific channel resource information such as channel number, frequency band and channel synchronization information.

The mobile ip routing information element provides mobile ip information such as default router or routers and associated CoA list.

###### When generated

The MIH HO generates this trigger to provide details regarding the selected resource that shall constitute the new point of attachment. In addition, the trigger provides information regarding associated IP routing information. E.g., CoA associated to selected resource

###### Effect of receipt

The MIH LLCF layer transfers this information to the peer for further processing.

##### MIH_MOBILITY.confirmation

###### Function

The MIH HO entity uses this trigger to provide details regarding the selected resource that shall constitute the new point of attachment.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MOBILITY.confirmation

(

indication result

inter-technology channel resource

mobile ip routing

)

The inter-technology channel resource information element provides technology specific channel resource information such as channel number, frequency band and channel synchronization information.

The mobile ip routing information element provides mobile ip information such as default router or routers and associated CoA list.

###### When generated

The MIH HO generates this trigger to provide details regarding the selected resource that shall constitute the new point of attachment. In addition, the trigger provides information regarding associated IP routing information. E.g., CoA associated to selected resource

###### Effect of receipt

The MIH HO layer generates a HANDOVER orders to request packet stream relocation (if not already executed) and relocation towards a new point of attachment as determined by the provided channel resource.

##### MIH_REPORT.indication

###### Function

MIH LLCF uses this trigger to provide system information that might be used by MIH HO to determine whether a handover should be triggered.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_REPORT.indication

(

inter-technology channel measurement report

technology specific performance report

technology specific header compression context parameters

)

The inter-technology channel measurement report provides an indication of the received signal strength on relevant channels belonging to one or several inter-technology systems within the neighborhood of the current serving network.

The technology specific performance report provides system measurements specific to an access technology

Technology specific header compression context information specifies the values of the header compression packet stream that requires to be transferred

###### When generated

MIH LLCF generates this trigger when a compile mobility report message made up of one or several event types is deemed to be required based on predefined thresholds.

###### Effect of receipt

MIH HO uses this information to determine whether a handover towards a new access network shall take place

##### MIH_REPORT.response

###### Function

The MIH HO entity uses this trigger to indicate the result of a previously received request message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_REPORT.response

(

indication id

)

The request id identifies the request message for which the response is being issued.

###### When generated

The MIH HO generates this trigger as a response to the correspondent request.

##### MIH_MACRELEASE.request

###### Function

The MIH LLCF entity uses this trigger to request release of old layer 2 connection.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACRELEASE.request

(

request id

MIH request

inter-technology channel resource

)

The inter-technology channel resource information element provides technology specific channel resource information such as channel number, frequency band and channel synchronization information.

###### When generated

The MIH LLCF generates this trigger when MAC actions are required to release layer 2 connection.

###### Effect of receipt

The MAC layer acts upon the MIH trigger for HO related L2 procedures.

##### MIH_MACRELEASE.confirmation

###### Function

The MAC layer uses this trigger to indicate the result of a previously received request message.

###### Semantics of the service trigger

The parameters of the trigger are as follows:

MIH_MACRELEASE.confirmation

(

request id

)

The request id identifies the request message for which the response is being issued.

###### When generated

The MAC layer generates this trigger when the MIH request has been processed.

###### Effect of receipt

The MIH LLCF layer might use the request id to determine whether or not a request message has been acted upon.

| Trigger Designation | Trigger Type | Registrar | Originator | Receiver | Description |
| --- | --- | --- | --- | --- | --- |
| MIH_HO_PREPARE | Link_Down | L3MM, MIP | MIH | L3 MM, MIP | Trigger is fired when L2 is not longer able to provide any service to L3 |
|  | Link_Available | L3MM, MIP | MIH, L3MM | L3MM MIP, MIH | Trigger is fired to indicate the establishment or the availability of a L2 connection |
|  | Link_Required | L3MM, MIP | MIH | L3MM, MIP | Trigger is fired when the requirements established on a L2 link cannot longer be met |
| MIH_HO_COMMIT | LinkChange_Complete | L3MM, MIP | MIH | L3MM, MIP | Trigger is fired to indicate successful completion of a L2 handover |
|  | LinkChange_Abort |  |  |  | Trigger is fired when L2 handover is aborted. L2 HO might be aborted either due to unsuccessful HO or recovery of link condition on original link |

Figure 10 Layer 3 Triggers

## Procedures

#### System Access,802.X and 3GPP inter-working

Figure 11 System Access, 802.X and 3GPP inter-working (1/2)

Figure 12 System Access, 802.X and 3GPP inter-working (2/2)

MIHHO Function receives system info via Beacon (MIH management in Network Discovery State)

- At power up or system reselection the MS executes the scanning procedures (active or passive) to find a WLAN network. When beacon frames are detected the MS first identifies whether MIH information is supported and if so, the MS read its content. MIH specific information is set/updated either manually or dynamically by the AN MIH function.

Any MIH information found within a beacon frame (e.g., system operator identity, W-APN, neighboring maps and system capabilities) is passed to the MIH function through a MIH_MACINFO message.

- The information is processed and the MS determines that WLAN system is a suitable candidate for system access. As a result MIH orders WLAN authentication and association through a MIH_MACORDER message.

- WLAN specific authentication and associating procedures are executed on the chosen WLAN system.

MIH HO informs 3GPP side that handover is imminent (MIH management in HO Prepare State)

- The AG MIH function triggers WLAN 3GPP authentication and authorization using EAP-AKA protocol. In addition MS MIH uses the MIH_SYS_INFO message to provides information with regards to the PDGs  (W-APN) supported by the WLAN system and their capabilities (e.g., SMS, IMS VOIP support).

- The 3GPP component uses its assigned NAI (Network Access Id) to indicate the AG its associated 3GPP AAA server. Successful routing results in the establishment of an IPsec tunnel that carries EAP-AKA messages.

- Upon successful authentication and authorization the MS obtains a local IP address from the local DHCP server.

- Using the selected PDG (-APN) the MS derives a Fully Qualified Domain Name (FQDN). And it uses it to determine the IP address of the relevant PDG.

- The MS uses the FDQN determine the IP address of the relevant using the local DNS server.

- Once the PDG IP address is obtained, a tunnel can be established towards the PDG. Note that tunnel establishment can be done in three different way: 1) the MS established a tunnel directly to the PDG. 2) The MS establishes a tunnel to the WLAN Access Gateway (WAG) and a tunnel from the WAG to the PDG is further established. 3) The AG establishes a tunnel to the WAG and then a tunnel from the WAG to the PDG is further established. 4) The AG established a tunnel directly to the PDG.

- Once a tunnel is established, the MS either receives an Agent Advertisement message from the PDG (acting as Foreign Agent) or it requests it using an Agent Solicitation message in accordance to RFC2002.

- The MS uses the PGA router address to construct its CoA.

- The MS registers its CoA with its Home Agent (HA).

- Data destined for the MS will now be routed via HA through a new tunnel established between the HA and the FA based on the supplied CoA.

#### System Access,802.X and 3GPP inter-working, Failure Case

Figure 13 System Access, 802.X and 3GPP inter-working, Failure Case (1/3)

Figure 14 System Access, 802.X and 3GPP inter-working, Failure Case (2/3)

Figure 15 System Access, 802.X and 3GPP inter-working, Failure Case (3/3)

MIHHO Function receives system info via Beacon (MIH management in Network Discovery State)

- At power up or system reselection the MS executes the scanning procedures (active or passive) to find a WLAN network. When beacon frames are detected the MS first identifies whether MIH information is supported and if so, the MS read its content. MIH specific information is set/updated either manually  (through a management system) or dynamically by the AG MIH function.

- Any MIH information found within a beacon frame (e.g., system operator identity, W-APN, neighboring maps and system capabilities) is passed to the MIH function through a MIH_MACINFO message. The MIH function determines that one or more values provided within the system information parameters does not satisfy the necessary condition for system access. E.g., the system operator is bared, the QoS is not adequate or there is a better candidate identified within a potential neighboring set provided in the message. This scenario represents the first failure case. This is depicted in Figure 12 with an encircled “1”.

- If the MIH function determines that the parameters provided by the information service do not satisfy internal configured requirements, then the MIH function orders the MAC layer to return to the scanning phase.

- Otherwise the MIH function triggers WEP authentication toward its MAC layer. Note that in order to determine whether the user requires further EAP-AKA authentication that will allow access to special services (e.g., 3GPP IMS), the MS might use a specific WEP default key. The AG might use a specific default key to determine whether it shall proceed further with EAPOL authentication or basic Internet access can be granted.

- The MS is authenticated according to current 802.11 WEP procedures.

- If WEP authentication fails, system access is denied. The MS might return to the scanning phase. This scenario represents the second failure case. This is depicted in Figure 12 with an encircled “2”.

- MAC provides the MIH function with information regarding the key that was used for the WEP procedure. This allows MIH to determine, e.g., based on the default key used during WEP authentication, whether further authentication procedures are warranted. Note that WEP is not considered a secured authentication procedure. It this context it used primarily to identified specific users that require further authentication.

- MIHHO uses MIH_SYSCAP to trigger EAPOL authentication procedures.

- The AG executes EAPOL procedures. The AAA AG component will act as an authenticator between the supplicant (MS) and the authentication server (AAA). The AG uses the NAI provided during the initial message exchange in order to determine the AAA server that shall execute the authentication procedure. If the AG is not able to route the authentication request, it responds indicating the available AAA servers where request can be routed. If the MS determines that none of them is suitable, it might decide to return the scanning phase. This is depicted in Figure 13 with an encircled “3”.

- The finds a suitable server using the NAI realm provided by the MS and it relays authentication messages between the MS and the relevant authentication server.

- If the MS fails the cellular authentication procedure, access to special services, such as 3GPP services, might be denied.  Alternatively, access altogether might be denied. The MS might return to the scanning phase. This scenario represents the forth failure case. This is depicted in Figure 13 with an encircled “4”.

- Otherwise if the cellular AAA server MS successfully authenticates the MS, the MSproceeds to obtain a local IP address. E.g., from the local DHCP.

- Using the W-APN, the MS constructs a Fully Qualified Domain Name (FQDN) and it attempts it request IP address resolution. If the DNS server cannot resolve the FQDN to any IP address, the MS cannot access a PDG within the existing WLAN network. The MS might choose to return the scanning phase or to settle for local WLAN services. This scenario represents the fifth failure case. This is depicted in Figure 14 with an encircled “5”.

- Otherwise if the DNS returns a valid PDG, the MS established a tunnel towards the PDG (E.g., a L2TP tunnel) and listens for Agent Advertisement messages from the PDG. If no Agent Advertisement messages are received the MS send Agent Solicitation.

- If no response is received, E.g., MIP not supported, the MS might use its local IP for transparent access to the Internet for basic ISP services.

- This scenario does not provide seamless mobility beyond the PDG domain and it represent the sixth failure case. This is depicted in Figure 14 with an encircled “6”.

#### Handover from 802.X to 3GPP

Figure 16 MS Initiated and MS Controlled Handover from 802.X to 3GPP (1/3)

Figure 17 MS Initiated and MS Controlled Handover from 802.X to 3GPP (2/3)

Figure 18 MS Initiated and MS Controlled Handover from 802.X to 3GPP (3/3)

MIHHO Function Receives measurements (MIH management in Steady State)

- The physical layer detects that set performance thresholds have been crossed and it generated an event indication towards the MIH HO function.

- The MAC layer detects that set performance thresholds have been crossed and it generated an event indication towards the MIH HO function.

- MIH processes and filters MAC and PHY layers measurements. A combination of events such as signal quality and specific network characteristics (e.g., preferred PLMN) can be used to determine whether the handover process shall be triggered.

- The MIH function determines that a condition (or a combination of them) has been met and therefore a handover attempt shall be triggered.

- MIH informs the 3GPP component (through MIH_HO_PREPARE) that a handover is imminent on the 802.X side.

MIH HO informs 3GPP side that handover is imminent (MIH management in HO Prepare State)

- The 3GPP side starts the 3GPP functions. The MS initiates cell selection and performs routing area update.

- At this point both LL (radio connection) and HL (connections towards new SGSN) are established

- The new SGSN requests context transfer from the PDG

- A “snap shot” of the current context is taken at the PDG for both uplink and downlink flows.

- The PDG transfers this information to the new SGSN. Right after PDP context is transferred, the PDG stops sending downlink packets towards the MS. Packets received (from GGSN) after this time are buffered. The PDG establishes a new GTP tunnel and sends a duplicate of every packet that is buffering, towards the new SGSN. This is done until a timer expires or the new SGSN is ready to start processing packets.

- When the new network has successfully activated the PDP context, it is now ready to start processing packets.

MIH HO commit (MIH management in HO Commit State)

- The PDP is context is updated at the GGSN and a new GTP tunnel can be established. Packets are now received directly from the GGSN toward the new SGSN. Upon successful PDP context transfer, the 3GPP component informs MIH that Handover has successfully completed (3GPP LL and HL connections are now established

- . MIH can now order release of the 802.x radio connection (E.g., Disassociation).

#### Handover from 3GPP to 802.11

Figure 19 MS Initiated Handover from 3GPP to 802.11

Figure 20 MS Initiated Handover from 3GPP to 802.11 (2/4)

Figure 21 MS Initiated Handover from 3GPP to 802.11 (3/4)

Figure 22 MS Initiated Handover from 3GPP to 802.11 (4/4)

MIHHO Function Receives measurements (MIH management in Steady State)

- The 3GPP component reads system information and it determines whether its content can be used for system reselection to a WLAN system.

- The 3GPP component extracts relevant system information that can be used to determine whether a handover to a WLAN system might be warranted and it forwards this information to the MIH HO function. Alternatively, the 802.x component might execute periodic scanning, either continuously or when prompted by system information received from the 3GPP component

- Relevant 3GPP system information is forwarded to the MIH function.

- The MIH HO component determines that one or several WLAN networks might be suitable for re-selection, based on available information (e.g., Explicit indication, RF signature, geographical location, manual or automatic scanning, specific TMSI assignment).

- The MIH function computes a list of potential candidates for handover selection.

- MIH evaluates candidates based on several aspects such as system operator and known system capabilities.

- The MIH HO finds a target candidate for handover and it triggers 802.x system access through MIH_MACORDER

MIH HO informs 3GPP side that handover is imminent (MIH management in HO Prepare State)

- The MS executes 802.x system association and authentication towards candidate WLAN system.

- When the MS is successfully associated and WLAN authenticated, then Extensible Authentication is used towards relevant 3GPP AAA in accordance to RFC 2284.

- The MS uses the WLAN identity and the associated PLMN to construct a Full Qualified Domain Name (FQDN) and it uses it to obtain the associated PDG address (through DNS query). The MS uses this address to establish an end-to-end tunnel toward the PDG (e.g., using L2TP). Once the tunnel is established, the MS executes routing area update towards the PDG.

- The routing data update received at the PDG triggers a context transfer request towards the old SGSN.

- A “snap shot” of the current context is taken from the source RNC and tranfer to the PDG via old SGSN. Both uplink and downlink context information is captured

- Right after PDP context is transferred, the RNC stops sending downlink packets towards the MS. Packets received (from GGSN) after this time are buffered. When the PDG is ready to start processing packets, the RNC establishes a new GTP tunnel and sends a duplicate of the packets that is buffering towards the PDG, via the old SGSN. This is done until a timer expires.

- The PDP is context is updated at the GGSN and a new GTP tunnel can be established (via Gn’ interface). Packets are now received directly from the GGSN toward the PDG

MIH HO commit (MIH management in HO Commit State)

- 802.x Lower and Higher layers are now established. User data flow from MS to CoN through the 802.x network.

- MIH informs that the handover has completed, though a MIH_HO_COMMIT message

- 3GPP might (optionally) proceed to release RAB resources.

### Intra and Inter 802 Technologies Handover Procedures

#### MS Initiated 802.X to 802.3 Handover

Figure 23 MS Initiated 802.X to 802.3 Handover (1/2)

Figure 24 MS Initiated 802.X to 802.3 Handover (2/2)

MIH detects 802.3 physical connections (MIH management in Steady State)

- While an 802.X connection is up and a data path is established between the 802.X MS and the 802.X AG, an 802.3 physical connection is detected (e.g., RJ45 cable has been plugged)

- Upon detection of an 802.3 physical connection by lower layers, a MIH_PHY_EVENT message is trigger towards the MIH function. This message provides characteristics of the physical link that  might be used to determine whether a handover operation should be executed

- The MIH function continuously process and filters information provided by lower layers.

- This information is categorized to determine whether one or more condition are present in order to trigger a handover procedure.

MIH triggers handover (MIH management in Prepare State)

- The establishment of an 802.3 physical connection prompts the MIH function to trigger handover procedures.  These procedures include transfer of context information (e.g., header compression context, PPP context amongst others) and switching of user data.

- MIH uses MIH_HO_PREPARE in order to triggers both context transfer and MIP procedures at the MIP component. Note that only MIH_HO_PREPARE is par of 802.21. Context transfer procedures and MIP procedures are out of the scope of 802.21 they are shown here for completeness only.

- The MS obtains a new IP address from the 802.3 AG, using the newly established physical connection.

- Either using information coming from the MIH_HO_PREPARE message or using existing MIP message, the MS obtain the IP address of the new AG (i.e., 802.3 gateway). This allows the MS to contact the 802.3 AG to initiate context transfer procedures.

- While context is being transferred to the new AG (802.3 AG), data is forwarded from the old AG (802.X AG). This allows the MS to receive user data before a new CoA is negotiated with the new 802.3 access router (within the 802.3 AG).

- MS negotiates new CoA using existing MIP messages.

- As the new CoA is ready and the lower layers connection is established, the user data path can now be switched from the CoN to the new AG.

- User data is fully is now flowing entirely through the 802.3 network

MIH releases all connections (MIH management in HO Commit State)

- A MIH_HO_COMMIT message is sent in order to inform the MIP side that the old CoA can now be de-registered.

- MIH tears down the 802.X connection though a MIH MACORDER. Optionally MIH might leave the old 802.X connected up, in order to avoid re-associating procedures, should a handover back from 802. 3, be imminent.

#### MS Initiated 802.3 to 802.X Handover

Figure 25 MS Initiated 802.3 to 802.X Handover (1/2)

Figure 26 MS Initiated 802.3 to 802.X Handover (2/2)

MIH detects 802.3 physical connections (MIH management in Steady State)

- An  802.3 connection is up and both lower layers (MAC/PHY) and higher layers (E.g., IP/MIP) are established. User data is flowing between  the 802.3 MS and the 802.3 AG.

- The MIH at the 802.3 AG might optionally provide system information to its peer. Information regarding potential neighbors and their system characteristics can be optionally provided peer to peer through a MIH_SYSINFO message.

- In addition, some events such as availability of A.C. power supply might cause the MIH trigger simultaneous connection of both 802.3 and 802.X stacks. However, the assumptions is that only the 802.3 connection is delivering services to the MS

- The MIH process the MIH_PHY_EVENT message and optionally it might determine that a 802.X system access should be triggered.

- Therefore the MIH function issues an MIH_MACORDER message triggering association and authentication procedures. In order to facilitate transfer of context information, the MS might optionally provide information about the current 802.3 anchor point. An 802.3 anchor point is defined as the 802.3 AG holding context information for the current 802.3 connection. This information will be use by the 802.X AG to request context transfer should a handover be imminent.

- When the 802.3 connection is no longer available (E.g., RJ45 cable is un-plugged), an event is triggered by the lower layers through a MIH_PHY_EVENT message. The MIH function evaluates the conditions that might have generated the trigger (E.g., link is down) and it determines that a 802.X system access (if MS not already associated) and a request for transfer of context information should be issued

- MIH uses an MIH_HO_PREPARE inform to alert the MIP component that a new access network should be used. This should trigger transfer of context information and forwarding of user data from the old AG (803.2) to the new AG (802.X).

Note that details regarding protocols that handle context transfer and user data forwarding (E.g., CTP Context Transfer Protocol) should not be subject to standardization within 802.21. However provision of information that enables optimal transfer of context and data forwarding between AG, should be included with relevant MIH messages (IP address of new and previous access routers).

MIH triggers handover (MIH management in Prepare State)

- The MS obtains a new IP address from the 802.X AG (Recall that MS is associated and authenticated in the 802.X AG). Note that this step might have been executed already. This could have happened when the MS first associated and authenticated with the 802.X AG. Once the 802.X IP address is available, the MS triggers the context transfer procedure and the data forwarding procedure from the old 802.3 AG to the new 802.X AG..

- While context is being transferred to the new AG (802.X AG), data is forwarded from the old AG (802.3 AG). This allows the MS to receive user data before a new CoA is negotiated with the new 802.X access router (within the 802.X AG).

- MS negotiates new CoA using existing MIP messages.

- As the new CoA is ready and the lower layers connection is established, the user data path can now be switched from the CoN to the new AG.

MIH releases all connections (MIH management in HO Commit State)

- A MIH_HO_COMMIT message is sent in order to inform the MIP side that the old CoA can now be de-registered.

#### MS Initiated MS Controlled Inter-802 Wireless Handover

Figure 27 MS Initiated MS Controlled Inter-802 Wireless Handover (Mobile IP v4) – Part 1

Figure 28 MS Initiated MS Controlled Inter-802 Wireless Handover (Mobile IP v4) – Part 2

MIH HO steady state:

- A 802.X session is in progress. The data path is between the MS and the 802.X access network.

- Measurements of MAC/PHY are sent to the MIH function via MIH_PHY_EVENT.

- Remote measurements can also be forwarded to the MS.

- The MIH LLCF is responsible to process the measurements. For example, it can average some measurements and compare them with certain thresholds, and create triggers to indicate MIH HO.

- MIH HO will decide if HO is necessary based on the information it gets from network information services and measurement reports. In order to make intelligent decision, the MIH HO maintains information from system information update, for example, the neighbor list and neighbor networks’ information. The MIH HO makes preventive handover decision in two steps, preparation and commit. Different thresholds and decision making algorithms are to be used for these two steps.

MIH HO preparation state:

- After evaluating the MIH LLCF reports, and if it decides a handover is imminent, it triggers the handover preparation procedures across the network by sending MIH_HO PREPARE to both Mobile IP and MAC/PHY.

- Both MIP and layer 2 can start to prepare HO, for example, get CoA and establish the tunnel. The MIH trigger contains routing info that the MIH function collected when it first accessed the system. This facilitates the acquisition of the new CoA. This example assumes “Multiple CoA registration” so at this point two IP streams are in place.

MIH HO commit state:

- The PHY layer of the MS constantly makes measurements according to the MIH configuration.

- MIH HO continues to process information and checks if a HO should be executed.

When a HO decision is made at MIH HO, it sends a MIH_HO_COMMIT to MIP. The trigger is also sent to the MIH peer in the 802.X access network.

- Upon receiving the MIH HO commit command, the MIP at the MS side and the network side verify that the layer 3 HO is ready.

- The MIH HO orders layer 2 handover by sending MIH_MACORDER.

- When layer 2 handover is finished, the data path should be switched to the new access network and the new MIP tunnel.

- Optionally, the old layer 2 connection can be tore down.

#### Network Initiated Network Controlled Inter-802 Wireless Handover

Figure 29 Network Initiated Network Controlled Inter-802 Handover (Mobile IP v4) – Part 1

Figure 30 Network Initiated Network Controlled Inter-802 Handover (Mobile IP v4) – Part 2

MIH HO steady state:

- A 802.X session is in progress. The data path is between the MS and the 802.X access network.

- Measurements of MAC/PHY are sent to the MIH function in the 802.X access network via MIH_PHY_EVENT.

- Remote measurements from the MS can also be forwarded to the 802.X network.

- The MIH LLCF is responsible to process the measurements. For example, it can average some measurements and compare them with certain thresholds, and create triggers to indicate MIH HO.

- MIH HO will decide if HO is necessary based on the information it gets from network information services and measurement reports. In order to make intelligent decision, the MIH HO maintains information from system information update, for example, the neighbor list and neighbor networks’ information. The MIH HO makes preventive handover decision in two steps, preparation and commit. Different thresholds and decision-making algorithms can be used for these two steps.

MIH HO preparation state:

- After evaluating the MIH LLCF reports, and if the MIH HO decides a handover is imminent, it triggers the handover preparation procedures across the network by sending MIH_HO PREPARE to both Mobile IP and MAC/PHY.

- Both MIP and layer 2 can start to prepare HO, for example, get CoA and establish the tunnel. This example assumes “Multiple CoA registration” so at this point two IP streams are in place.

MIH HO commit state:

- The PHY layer of the 802.X network constantly makes measurements according to the MIH configuration.

- MIH HO continues to process information and checks if a HO should be executed.

When a HO decision is made at MIH HO, it sends a MIH_HO_COMMIT to MIP. The trigger is also sent to the MIH peer in the MS.

- Upon receiving the MIH HO commit command, the MIP at the MS side and the network side verify that the layer 3 HO is ready.

- The MIH HO orders layer 2 handover by sending MIH_MACORDER.

- When layer 2 handover is finished, the data path should be switched to the new access network and the new MIP tunnel.

- Optionally, the old layer 2 connection can be tore down.

## Analysis of Interaction with Mobile IP v6 Enhancements

As stated in IEEE, the 802.21 technology shall focus on enhancing a mobile user’s experience by supporting “seamless handover between heterogeneous networks”. To this end, we outline the following objectives:

- Identify a set of optimal mechanisms for routing to a new location

- Identify methods to efficiently and proactively discover neighbors

- Elaborate on suggested methods to minimize handoff latency

- Evaluate the available options for proprietary signaling

### Optimal mechanisms for routing to a new location

Amongst the various options listed, the Fast Handoff (FMIP) Protocol [9] seems to be one that presents a good approach to minimizing location update latency. Indeed, FMIP incorporates the main ideas of the “Reverse Tunneling” draft, which specifies a tunnel between the Previous CoA and the New CoA. This tunnel is established as soon as an MS sends a “Fast Binding Update” message to its Previous Access Router. This protocol requires the MS to send that update message while being connected to the Previous AR.

On the other hand, the Hierarchical MIP draft [8] tackles the issue of location update latency from a different angle: it presents a solution that separates local from global mobility. This allows intra-domain handoffs to be performed “transparently to the MN’s external component hosts” [8]. In other words, the MS does not need to send its Home Agent or Corresponding Node’s binding updates.

Therefore, FMIP and HMIP are capable of providing efficient mechanism for routing to a new location.

### Neighbor discovery

The Fast Handover Protocol encompasses Neighbor Discovery methods that allow the MS to acquire information about its new access router before moving to it. This provides a good backup in case a connection to one router fails or when the MS moves across multiple domains. Essentially, this is done via a pair of messages exchanged between the MS and the new (future) access router: Neighbor and Router Solicitation, Neighbor and Router Advertisements messages. The resulting information is thus stored at the MS level to allow the MS to keep track of neighboring APs identifiers and their associated router information.

Mobile IPv6 incorporates the above-mentioned fast-handover features and uses them to constantly keep track of neighbors that are reachable and those that are not.

### Minimum Handover Latency

Inherently from its nature, HMIP provides means to reduce handoff latency: by locally handling updates as much as possible, it eliminates unnecessary updates of remote Home Agents.

On the other hand, the “Low Latency Handoffs in Mobile IP” draft [7] proposes 3 methods to minimize handoff-related delays. In the Pre-Registration Handoff Method, the MS is allowed to perform a level 3 handoff before it completes level 2 handoff. This allows the MS to pre-built its registration state on the new (future) FA. The Post-Registration Handoff Methods is based on the essential principle of “Reverse Tunneling“ and uses level 2 triggers along with two-way tunnels between the new and old FA. This permits the MS to continue using its old FA while being connected to a new FA.

### Proprietary signaling

The RFC entitled “Mobile IP Vendor/Organization-specific Extensions” [10] proposes various extensions to allow inclusion of such information in the mobile IP messages. That RFC proposes the Critical (CVSE) and Normal (NVSE) Vendor/Organization Specific Extensions. It also defines error codes to deal with Registration denied by the Foreign Agent or by the Home agent.

### Conclusion

After having analyzed these recommendations, we notice that a limited number of RFCs and Drafts are closely related to the essential objective of 802.21. These are “The Fast Handover Protocol” , “Hierarchical MIP” and “Low-Latency Handoffs in Mobile IP”. We must note at this point that Fast Handover, in fact, covers the idea of pre-registration, which is part of the “Low-Latency Handoffs in Mobile IP” draft.

Furthermore, in the “Low Latency Handoffs in Mobile IP” draft, it is evident that a combination of the proposed Pre- and Post-Registration Handoff Methods will be most effective in our context: the Pre-Registration Method should be the default one performed and the Post-Registration Method should be kept in backup. In fact, the draft mentions that the Post-Registration method is key to a seamless handover, which in effect constitutes a common goal with 802.21.

### Implementation with various MIPv6 Enhancements

#### Fast Handover [9] Implementation

In this section, we are attempting to implement the Fast Handover Protocol in the context of 802.21.  below outlines the various processes involved.

One of the objectives of Fast Handover is to overcome the latency due to MIP registration, and the basic idea is to anticipate movement with the help of link layer (triggers). This implies preparing the network in advance and requires anticipated handover initiated by the MS and the Network. As illustrated in figure 65, this is achieved by commencing the registration process slightly before the actual handover. The rectangles and the arrow in blue represent important milestones in the 802.21 proposal that were mapped into our fast-handover process. The dotted blue arrow represents an optional message.

Application Layer

MIP

MS

Media Access

(Original Network)

Access Gateway

Visiting

Network

New

AR

Domain

Layer

Function

PHY

MAC

MAC

MIH

MIH

Previous

AR

Fast Binding Update

Handover Acknowledge

Fast Binding Acknowledge

MS disconnects with prev AR

Router Solicitation Proxy

Proxy Router Advertisement

Handover Initiate

Fast Binding Acknowledge

Fast Neighbor Advertisement

1

2

3

4

5

6

7

8

9

10

MIH

HO

Connection established with new AR

Prev AR frwds packets to nAR

MIH

LLCF

Packets get delivered to the MS

IP ROUTING

MIH_HANDOVER _PREPARE.Indication

MIH_HANDOVER _PREPARE.Response

MIH_HANDOVER _COMMIT.Indication

Figure 31 Inter-802 Fast Handover Message Flow (MS initiated)

Steps involved in the Inter-802 Fast Handover Message flow:

  - The MS requests information about its neighbors. This is used to build an internal list (at the MS level) of neighboring APs.

  - The MS receives information pertaining to its surrounding APs such as IP and MAC addresses, operating frequency and ESSID information. The MS forwards that information to the MIH via a pre-established link between MIH and MIP (not shown on diagram).

  - The FBU is a message from the MS instructing its (previous) AR to start directing its traffic to the new AR. The FBU is constructed using information extracted from the Router Advertisement Message, and is sent after receiving the 802.21 trigger called “MIH_HANDOVER_COMMIT.Indication”. The objective of the FBU message is to authorize the previous AR to bind the (previous) CoA to the new CoA

  - Having received a FBU, the previous AR will start the procedures for Handover and Tunnel creation. The message sent in this step is a Handover Initiate (HI): this ICMPv6 (Internet Control Message Protocol) message is sent by the previous AR to the new AR to trigger the process of the MS’s handover

  - The Handover Acknowledge (Hack) is a new ICMPv6 message that is sent by the new AR to the previous AR as reply to the HI message. The temporary tunnel has now been established between previous and new ARs

  - The Fback (Fast Binding Acknowledge) message is sent by the previous AR to acknowledge receipt of the FBU message, only after the Previous AR receives the Hack message from the New AR

  - The Previous AR sends the message in (6) to the MS as well for information purposes

  - The previous AR then starts forwarding packets to new AR

  - In order to announce itself to the new AR, and as soon as the MS re-gains connectivity, the MS sends a Fast Neighbor Advertisement (FNA) message.

  - The Handoff is complete: packets are now delivered to the MS from the new AR

#### HMIPv6 [8] Implementation

In this section we are implementing Hierarchical MIPv6. This localized mobility management proposal is aimed at reducing signaling load and handoff latency. To this end, the proposal specifies different sets of procedures for the management of local versus global events.

The concept of global mobility refers to a situation where the MS moves from one MAP to another, thus changing its RCoA. A typical scenario in the case of local mobility would be a MS moving within a same MAP region, but changing from one AR to another. These types of local handoffs are managed locally and transparently to MS’s correspondent hosts.

We will concentrate our analysis on global mobility.  below outlines the various processes involved for that purpose.

Prev MAP frwds packets to new MAP

Domain

Layer

Function

Application Layer

MIP

MS

Media Access

(Original Network)

Access Gateway

HA

PHY

MAC

MAC

MIH

MIH

Prev MAP

Binding Update (including RCoA and LCoA)

Registration (Binding Update containing RCoA)

1

2

3

4

5

6

7

Visiting network

Internet

New MAP

Binding Acknowledge

Tunnel established

CN

Local Binding Update (including new RCoA)

MIH

HO

MIH

LLCF

HANDOVER_COMMIT.Indication

Router Advertisement

Connection with new MAP

IP ROUTING

LBU (includes LCoA)

HANDOVER_PREPARE.Response

HANDOVER_COMMIT.Response

HANDOVER_PREPARE.Indication

Figure 32: Inter-802 HMIPv6 Message Flow (MS initiated)

Steps involved in the Inter-802 HMIPv6 Message flow:

reflects global mobility management within HMIPv6. The solution assumes that MAP neighboring information was initially loaded into the MIH and can be dynamically updated as per .21 proposal. It is also assumed that the station performs regular measurements of its link strength and environing events.

- The MS receives a router advertisement from the new MAP to which he has moved. The MAP’s global address is included herein. Based on the prefix received in the MAP option, the MS forms the new RCoA (Regional Care of Address specific to its MAP).

- The router advertisement, along with the Mobile’s measurement results, is forwarded to the MIH via a pre-established link between MIH and MIP (not shown on diagram). The MIH thus analyses these measurements as per .21 procedures and may decide to send a “MIH_HANDOVER_PREPARE.Indication” trigger to the MIP if the MIH’s measurements show that a handover is advantageous/necessary. The CoAs are herein computed.

- Upon receiving the “MIH_HANDOVER_PREPARE.Indication” trigger, the MIP initiates a pre-binding procedure called LBU with the previous MAP in order to reduce packet loss and to perform a rapid handoff. This is done in view of being consistent with the make-before-break principle. At this point, a “MIH_HANDOVER_COMMIT.Indication” trigger is sent by the MIH to the MIP.

- The MS, having received the .21 trigger indicating handoff, then sends a Binding Update to the new MAP. This message includes a “Home Address” option that contains the RCoA. This BU will bind the MS’s RCoA to its LCoA (On-Link Care of Address).

- The MAP sends a binding acknowledgement to the MS. A bi-directional tunnel is now established between the MS and the new MAP.

- As soon as registration is confirmed with the new MAP (right after step (5)), the MS registers its new RCoA with the HA by sending a BU specifying the biding to the HA.

- This ultimate step is part of the Mobile IPv6 “route optimization” method: a Binding Update similar to that in the previous step is also sent to the MS’s Corresponding Node. This will allow the later to establish a link with the new MAP for the purpose of transmitting packets directly to that new MAP.

Note: The HMIP draft specification does not include step (2) in the same sequence that we are proposing. Instead, it provides for data forwarding between previous and new MAP after step (4). In other words, .21 increases the overall handoff efficiency by having packets forwarded to the new MAP ahead of time.