| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Supplement of D00-02_Ohba_Yoshihiro.USR |  |
| Date Submitted | September 14, 2005 |  |
| Source(s) | Yoshihrio Ohba, Subir Das and Farooq Bari |  |
| Re: | 21-05-0346-00-0000_Supplement_D00_02_Ohba_Yoshihiro.doc |  |
| Abstract | This document is a supplement of D00-02_Ohba_Yoshihiro.USR to contain requests for changes in Tables and Figures. |  |
| Purpose | This document is prepared for discussion in IEEE 802.21 WG session in September 2005 Interim meeting. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

List of Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Yoshiro Ohba | Toshiba America Research Inc. | P.O. Box 429, Piscataway, NJ 08854-4151 | 732-699-5305 | 732-336-3578 | yohba@tari.toshiba.com |
| Subir Das | Telcordia Technologies Inc | One Telcordia Drive, RRC-1B229, Piscataway, NJ 08854-4157 | 732 699 2483 | 732 336 7026 | subir@research.telcordia.com |
| Farooq Bari | Cingular Wireless |  | 425 580 5526 |  | farooq.bari@cingular.com |

- Introduction

This document is a supplement contribution of D00-02_Ohba_Yoshihiro.USR to include requested changes for Tables and Figures in P802-21-D00-02.

- Requested Changes for Tables and Figures

## Requested Changes for Table 4

Table 4 (Section 6.3.2) should be replaced with the following table:

Table 4 – Media-Independent Information Elements

| Name of Information Element | Description | Basic/Extended Set | Comments |
| --- | --- | --- | --- |
| Point of Attachment (PoA) | Identification of the PoA. | Basic Set | A PoA is represented as a pair of PoA type and PoA identifier. A PoA type is a link type of the PoA. For example, RADIUS NAS-Port-Type attribute value which is a unique identifier defined in http://www.iana.org/assignments/radius-types can be used as a link type, e.g., 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - 1X-EV Etc. A PoA identifier is unique within the PoA type, e.g., a BSSID of 802.11 access point. |
| Neighboring Networks | Link types of the networks that are neighbors of a given PoA. | Basic Set | Link type has the same structure as the PoA type of PoA information element. |
| Neighboring PoAs | PoAs that are neighbors of a given PoA. | Basic Set | PoA has the same structure as the PoA information element. |
| Location of PoA | Geographical location of a given PoA. Multiple location types are supported including coordinate-based location information and civic address. | Basic Set | The coordinate-based location information is defined in RFC 3825 and consists of: - Latitude Resolution - Latitude - Longitude Resolution - Longitude - Altitude Type - Altitude Resolution - Altitude - Map Datum The civic address location information is TBD. |
| Network Operator | The operator of a network. | Basic Set | An operator is represented as a pair of operator identifier and operator name. For example, SMI Network Management Private Enterprise Codes defined in http://www.iana.org/assignments/enterprise-numbers can be used as a global unique operator identifier. |
| Roaming Partners | Operators with which the current network operator has direct roaming agreements. | Basic Set | Each operator has the same structure as Network Operator information element. |
| Cost | Indication of cost for service or network usage. | Basic Set | Cost is represented as a binary value, i.e., free or charged. |
| Higher Layer Security Support | A set of binary indications to represent the security characteristics of the higher layers of a network. | Basic Set | The following binary indications are defined: - UAM: indicates whether Universal Access Method is used for authentication method in the network. - PANA: indicates whether Protocol for carrying Authentication for Network Access is used for authentication method in the network. This indication is orthogonal to UAM indication. |
| Higher Layer QoS Support | A set of binary indications to represent the QoS characteristics of the higher layers of a network. | Basic Set | The following binary indications are defined: - Diffserv: indicates whether the IP access network supports Diffserv-based QoS. |

## Requested Changes for Table 5

Table 5 (Section 6.3.2) should be  replaced with the following table:

Table 5 – Media-Dependent Information Elements

| Name of Information Element | Description | Basic/Extended Set | Comments |
| --- | --- | --- | --- |
| Data Rate | The maximum value of data rate supported by the link layer of a given PoA. | Basic Set | A data rate is represented as a 32-bit unsigned integer in unit of Kbps. |
| PHY Type | The media PHY type. | Extended Set | The PHY type can be defined by media-specific MIB. |
| MAC Type | The media MAC type. | Extended Set | The MAC type can be defined by media-specific MIB. |
| Link Layer Security | Security characteristics of the link layer of a given PoA. | Extended Set | Security characteristics are media-dependent and defined in the extended set. The security characteristics can be defined by media specific MIBs. For example, authentication methods and cipher suites can be part of the security characteristics. |
| Link Layer QoS | QoS (Quality of Service) characteristics of the link layer of a given PoA. | Extended Set | QoS characteristics are media-dependent and defined in the extended set. The QoS characteristics can be defined by media specific MIBs. For example, QoS classes, traffic priorities can be part of the QoS characteristics. |

## Requested Changes for Figure 23

Figure 23 (Section 6.3.3.1)  should be replaced with the following figure: