| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Commentary attachments to add events for commands |  |
| Date Submitted | January 16th, 2005 |  |
| Source(s) | Srinivas Sreemanthula and Mathie Peresse |  |
| Ref: | 21-06-0479-00-0000-Events-for-Commands |  |
| Abstract | This contribution provides clarifications to events and commands defined in commentary. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Remedy #1 - Introductory Text

## MIH Events can also be generated due to the receipt of certain MIH commands. The receipt of such commands may indicate a state change in one of the link layers in the local node soon after processing the command. These events are notified to the registered MIH users of such impending status information so that they can be prepared or take appropriate action to provide better user experience.

# Remedy #2 - Add MIH Events

Table-3

| Event Id | Link Event Type | Link Event Name | Local (L), Remote (R) | Remote direction (Client->Network, Network->Client or Network<->Network) | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | State Change | MIH Link Up | L, R |  | Link is initialized and available for use |
| 2 | State Change | MIH Link Down | L, R |  | Link is down and not available |
| 3 | Predictive | MIH Link Going Down | L, R |  | Link conditions are degrading |
| 4 | State Change | MIH Link Detected | L, R |  | A new link is detected |
| 5 | Link Parameters | MIH Link Parameters Change | L, R |  | Link parameters have changed |
| 6 | Administrative | MIH Link Rollback | L, R |  | To roll back a previous event |
| 7 | Link Transmission | MIH Link SDU Transmit Success | L |  | Link transmit success of a higher layer PDU fragments |
| 8 | Link Transmission | MIH Link SDU Transmit Failure | L |  | Link transmit failure of a higher layer PDU fragments |
| 9 | Link Synchronous | MIH Link Handover Imminent | L, R |  |  |
| 10 | Link Synchronous | MIH Link Handover Complete | L, R |  |  |
| 11 | Link Synchronous | MIH Event Cmd-Switch | L |  | Event indicating the command |
| 12 | Link Synchronous | MIH Event Cmd-Handover Initiate | L |  | Event indicating the command |
| 13 | Link Synchronous | MIH Event Cmd-Handover Prepare | L |  | Event indicating the command |
| 14 | Link Synchronous | MIH_Event Cmd-Handover Commit | L |  | Event indicating the command |
| 15 | Link Synchronous | MIH_Event Cmd-Handover Complete | L |  | Event indicating the command |

# Remedy #3 - Add MIH Event SAPs

## The definitions for new MIH events have same definitions corresponding to MIH commands.

## In each of the sections 7.4.7.1, 7.4.10.1, 7.4.11.1, 7.4.12.1 and 7.4.13.1, add the following statement in the "Effect of receipt" section to reflect that a corresponding event exists.

A corresponding event as shown in table 3 can be triggered to send to the registered entities in the local stack. The parameters used for the generated event are same as this command.