| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Service Payload Format |  |
| Date Submitted | February 13, 2006 |  |
| Source(s) | Yoshihiro Ohba, Kenichi Taniuchi and Subir Das |  |
| Re: | IEEE 802.21 Session #NN in March 2006 |  |
| Abstract | This contribution proposes MIH payload format for Information Service. |  |
| Purpose | The purpose of this contribution is for revising 802.21 draft D05. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1 Requested Changes

Change Sections 8.4.4.1 and 8.4.4.2 as follows:

“

8.4.4.1 MIH_Get_Information Request

This message is used by MIH Function to retrieve value of different Information Elements.

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| InfoQueryType | 37 | 1 | Specifies the different types of supported IE formats. 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |
| InfoQueryParameters | 38 | Variable | List of query specific parameters defined in Table 20 |

Table 20: Format of InfoQueryParameters in  MIH_Get_Information Request

| Syntax | Size (octets) | Note |
| --- | --- | --- |
| If (InfoQueryType==1) { | - | - |
| Request_OR_IE_TLVs | Variable | One or more Request or IE TLVs |
| } | - | - |
| Else If (InfoQueryType==2) { | - | - |
| XML_SPARQL_Request _TLV | Variable | One SPARQL Query TLV |
| } | - | - |
| Else If (InfoQueryType==3) { | - | - |
| NULL | 0 | Nothing is carried |
| } | - | - |
| Else If (InfoQueryType==4) { | - | - |
| Extended_Schema_URL_TLV | Variable | Zero or one Extended Schema URL TLV |
| } | - | - |

SPARQL Query TLV

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_SPARQL_QUERY | SPARQL Query | Variable | See Table 21 |

Table 21 SPARQL Query TLV Value Format

| Syntax | Size (octets) | Note |
| --- | --- | --- |
| MIME_Type_TLV (Optional) | Variable | A MIME Type TLV to specify an XML format of SPARQL Query Result. When this TLV is not contained, MIME type “application/xml” is used. MIME type “application/xml” MUST be supported for this type of query. |
| SPARQL_Query_Value | Variable | An octet string of the content of the SPARQL Query. |

MIME Type TLV

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_MIME_TYPE | MIME Type | Variable | An octet string for a MIME type, e.g., “application/xml” |

Extended Schema URL TLV

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_EXTENDED_SCHEMA_URL | URL of Extended Schema | Variable | URL of an Extended Schema encoded in octet string |

8.4.4.2 MIH_Get_Information Response

This message is used by MIH Function to retrieve value of different Information Elements.

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| InfoQueryType | 37 | 1 | Specifies the different types of supported IE formats. 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |
| InfoQueryParameters | 38 | Variable | List of query specific parameters as defined in Table 22 |
| QueryStatus | 40 | 1 | Status of the query operation 0: Success 1: Failure 2: Rejected |

Table 22: Format of InfoQueryParameters in MIH_Get_Information Response TLV

| Syntax | Size (octets) | Note |
| --- | --- | --- |
| If (InfoQueryType==1) { | - | - |
| Response_OR_IE_TLVs | Variable | One or more Response or IE TLVs |
| } | - | - |
| Else If (InfoQueryType==2) { | - | - |
| SPARQL_Query_Results_TLV | Variable | One SPARQL Query Results TLV |
| } | - | - |
| Else If (InfoQueryType==3) { | - | - |
| Extended_Schema_URL_TLV | Variable | One Extended Schema URL TLV |
| } | - | - |
| Else If (InfoQueryType==4) { | - | - |
| Extended Schema_TLV | Variable | One Extended Schema TLV |
| } | - | - |

SPARQL Query Results TLV

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_SPARQL_QUERY_Results | SPARQL Query Results | Variable | See Table 23 |

Table 23  SPQRQL Query Results TLV Value Format

| Syntax | Size (octets) | Note |
| --- | --- | --- |
| MIME_Type_TLV | Variable | A MIME Type TLV to specify an XML format of the SPARQL Query Result. MIME type “application/xml” MUST be supported for this type of query. |
| SPARQL_Query_Results_Value_TLV | Variable | A value that is formatted with the MIME type specified in the MIME_Type_TLV. When the MIME type is “application/xml” this field contains XML text for SPARQL Query Results. |

Extended Schema TLV

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_EXTENDED_SCHEMA | Extended Schema | Variable | See Table 24 |

Table 24  Format of the Value field of Schema TLV

| Syntax | Size (octets) | Note |
| --- | --- | --- |
| MIME_Type_TLV (Optional) | Variable | A MIME Type TLV to specify an XML format of the Schema_Data. When this TLV is not contained, MIME type “application/xml” is used. MIME type “application/xml” MUST be supported for this type of query. |
| Schema_Data | Variable | When MIME_Type_TLV is not contained or the MIME type of MIME_Type_TLV is “application/xml” this field contains XML text for Extended Schema stored in the requested URL. |

“