| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Session Identifier |  |
| Date Submitted | February 17, 2006 |  |
| Source(s) | Yoshihiro Ohba, Qiaobing Xie, Subir Das, Ronny Kim, Srinivas Sreemanthula, Kalyan Koora, Vivek Gupta and Michael Williams |  |
| Re: | IEEE 802.21 Session #NN in March 2006 |  |
| Abstract | This contribution proposes session identifier to be used for maintaining registration for command and event services. |  |
| Purpose | The purpose of this contribution is for revising 802.21 draft D05. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Session Identifier

- Proposed Changes to 802.21 Draft D05

[1] Insert the following section:

“

6.1.4. Session Identifier for Remote Registration

Each remote registration needs to be associated with the MIHF that is registered for particular events and commands. For this purpose, each remote registration is assigned a global and eternally unique session identifier (SessionID). Eternal uniqueness means that  a SessionID assigned to a session created by an MIHF is never reused for future sessions to be created by the MIHF.

A SessionID is assigned by the MIHF that generates a registration request and remains valid during the lifetime of the registration.

The same SessionID may be shared among different services between the same pair of MIHFs.

When an MIHF sends an event indication message or a command request or response message that is associated with a registration, the message contains the SessionID of the registration.

The SessionId format is similar to Diameter Session-Id format [RFC 3588]. The SessionId format is given below:

MIHF-Id;<high 32 bits>;<low 32 bits>[;<optional value>]

MIHF-ID is FQDN or NAI of the sender of the registration request. When NAI is used, the NAI MUST contain not only username part but also realm part (i.e., username@realm) to maintain global uniqueness. MIHF-ID MUST be invariant even if the device obtains different interfaces or other components in its lifetime.

<high 32 bits> and <low 32 bits> are decimal representations of the high and low 32 bits of a monotonically increasing 64-bit value.  The 64-bit value is rendered in two part to simplify formatting by 32-bit processors.  At startup, the high 32 bits of the 64-bit value MAY be initialized to the time, and the low 32 bits MAY be initialized to zero.  This will for practical purposes eliminate the possibility of overlapping Session-Ids after a reboot, assuming the reboot process takes longer than a second.  Alternatively, an implementation MAY keep track of the increasing value in non-volatile memory.

<optional value> is implementation specific but may include a modem's device Id, a layer 2 address, timestamp, etc.

“

[2] Change Section 7.3.3.2, 7.3.4.2, 7.3.5.2, 7.3.6.2:

“

7.3.3.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Register.request (

RequestedEventList

SessionId

}

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| RequestedEvent List | Set of Event IDs | Set of Valid link events | List of link layer events that the endpoint would like to receive indications for, from the Event Source |
| SessionId (Optional) | String | See Section 6.1.4. | SessionId for the registration. This parameter is not carried for local registration. |

7.3.4.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Register.confirm (

ResponseEventList

SessionId,

Lifetime

)

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| ResponseEvent List | Set of Event IDs and their registration status | N /A | List of link layer events along with their registration status |
| SessionId (Optional) | String | See Section 6.1.4. | SessionId for the registration. This parameter is not carried for local registration. |
| Lifetime (Optional) | Integer | 0..2^32-1 | Lifetime of the registration. This parameter is not carried for local registration. |

7.3.5.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Deregister.request (

RequestedEventList

SessionId

)

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| RequestedEvent List | Set of Event IDs | Set of Valid link events | List of link layer events for which indications need to be deregistered from the Event Source |
| SessionId (Optional) | String | See Section 6.1.4. | SessionId for the registration. This parameter is not carried for local registration. |

7.3.6.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Deregister.confirm (

ResponseEventList

SessionId

)

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| ResponseEvent List | Set of Event IDs and their Deregistration status | N /A | List of link layer events along with their deregistration status |
| SessionId (Optional) | String | See Section 6.1.4. | SessionId assigned by the ultimate registrar. This parameter is not carried for local registration. |

“

[3] Add the following row to each Table in Section 7.3.9.1 (Link_Up.indication), 7.3.10.1 (Link_Down.indication), 7.3.11.1 (Link_Going_Down.indication), 7.3.12.1 (Link_Event_Rollback.indication), 7.3.13.1 (Link_Detected.indication),  7.3.14.1 (Link_Parameters_Change indication), 7.3.15.1 (Link_SDU_Transmission_Status.indication), 7.3.16.1 (Link_Handover_Imminent.indication), 7.3.17.1 (Link_Handover_Complete.indication):

“

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| SessionId (Optional) | String | See Section 6.1.4. | SessionId for the registration. This parameter is not carried for local event. |

“

[3] Change section 8.4.2.1, 8.4.2.2, 8.4.2.3 and 8.4.2.4 as follows:

“

8.4.2.1 MIH_Event_Register Request

This message is used by remote MIH Function (the registrant) to register an interest in a particular or group of event types from a particular event origination point.

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| RequestedEventList | 4 | Variable | Array of event codes formatted as messaged-id: Event Code1, Event Code2, Event Code3, .... |
| SessionId (Optional) | To be assigned | Variable | SessionId for the registration. This parameter is not carried for local registration. |

8.4.2.2 MIH_Event_Register Response

The response indicates which of the event types were successfully registered.

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| ResponseEventList | 5 | Variable | Array of event codes formatted as messaged-id: Event Code1, Event Code2, Event Code3, .... |
| SessionId(Optional) | To be assigned | Variable | SessionId for the registration. This parameter is not carried for local registration. |
| Lifetime(Optional) | To be assigned | 4 | Lifetime of the registration. This parameter is not carried for local registration. |

8.4.2.3 MIH_Event_Deregister Request

This message is sent by remote MIHF (the registrant) to deregister for a set of link layer events.

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| RequestedEventList | 4 | Variable | Array of event codes formatted as messaged-id: Event Code1, Event Code2, Event Code3, .... |
| SessionId(Optional) | To be assigned | Variable | SessionId for the registration. This parameter is not carried for local registration. |

8.4.2.4 MIH_Event_Deregister Response

The response indicates which of the event types were successfully deregistered.

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| ResponseEventList | 5 | Variable | Array of event codes formatted as messaged-id: Event Code1, Event Code2, Event Code3, .... |
| SessionId(Optional) | To be assigned | Variable | SessionId for the registration. This parameter is not carried for local registration. |

“

[4] Add the following row to each Table in Sections 8.4.2.5 (MIH_Link_Up Indication), 8.4.2.6 (MIH_Link_Down Indication), 8.4.2.7 (MIH_Link_Going_Down Indication), 8.4.2.8 (MIH_Link_Detected Indication), 8.4.2.9 (MIH_Link_Parameters_Report Indication), 8.4.2.10 (MIH_Link_Event_Rollback Indication), 8.4.2.11 (MIH_Link_Handover_Imminent Indication), 8.4.2.12 (MIH_Link_Handover_Complete Indication)

“

| Name | Type (1 octet) | Length (1 octet) | Value |
| --- | --- | --- | --- |
| SessionId(Optional) | To be assigned | Variable | SessionId for the registration. This parameter is not carried for local registration. |

“