| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Media Independent Handover – Discussion on IDs in primitives |  |
| Date Submitted | May 2006 |  |
| Source(s) | <see list below> |  |
| Re: |  |  |
| Abstract | Discussion on Acquiring Location Based Information over MIIS using TLV format |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

List of Contributors

| Name | Email |
| --- | --- |
| Kalyan Koora Wolfgang Gröting | kalyan.koora@benq.com wolfgang.groeting@benq.com |
| Vivek G. Gupta | vivek.g.gupta@intel.com |
| Yoshihiro Ohba | yohba@tari.toshiba.com |
| Liu Xiaoyu | xiaoyu.liu@samsung.com |

Revision History

| Version | Date | Comment |
| --- | --- | --- |
| 0 | 28.03.06 | Generation of the skeleton doc with introduction |
|  | 31.03.06 | Use cases and discussion points added by Xiaoyu |
|  | 31.03.06 | discussion points added by Kalyan in Red |
| 1 | 06.04.06 | Updated with results form the teleconf on 05.04.06. These results are under section 3 and are envisaged as amendments to the draft standard. |
|  | 24.04.06 | Finalised section 3 |

Contents

1 Introduction	4

2 Proposals:	7

2.1 TLV after TLV in Information Request Primitives	7

2.2 Alternative: Including a condition field within a query	8

2.3 Any other proposals??	9

3 Amendments/Modifications to the Draft Standard	10

4 References	11

# Introduction

Within the IEEE 802.21 MIH standard [1], it is possible for a terminal to get network related information prior to establishing a connection for user data exchange.  This is enabled over Media Independent Information Service (MIIS).

Presently, the MIH information request primitive is defined in draft .21 standard as follows:

MIH_Get_Information.request ( InfoQueryType, InfoQueryParameters )

Query Type: controls the syntax of request (syntax e.g. TLV, XML)

Query Parameters: indicates the type of information the client interested in

It is assumed that the MIIS information providing server in the network is not just beyond the client’s point of attachment but deep in the network. It is also assumed that one MIIS server provides information for different networks at different locations. A simple scenario is depicted in Figure 1-1.

Figure 1-1:   Considered general model.

Problem with the present defined TLV format within draft standard are as follows [2]:

- If a request is received by IS server, it doesn’t know where the requested client/UE/STN/MN is presently located and thus can’t provide exact location based information

- it is difficult to extract the location information just from IP or MAC

The aim of this discussion is to figure out an optimised solution to embed additional parameters such as the location information within a TLV based information request and to enhance the .21 draft standard.

[Xiaoyu]: An example scenario to illustrate the use cases of MIIS Query/Response.

An example scenario is shown in above figure. In this geographical area,

- Two operator own three networks: OP-1 having NW-1 and NW-2; OP-2 having NW3

- Three networks have different billing methods: NW-1 with Cost-1; NW-2 with Cost -2; NW-3 with Cost-3

- Four POAs are shown. POA1 has two heterogeneous neighbors: POA 3 and POA4. POA2 has one heterogeneous  neighbor: POA 4

- One MIIS Server is deployed in depth into the Operator 1’s networks keeping the key information of MIIS IE and the neighboring relationship.

Before handover, a mobile node uses Query/Response procedure to get the heterogeneous network information. In this particular example, the MN communicates with the MIIS Server via the currently connected POA1.

In order to retrieve the information from MIIS Server, three types of information should be provided to the MIIS Server:

- A Unique ID to identify the current POA:

MIIS Server is not deployed in the current POA; In other words, MIIS Server may keep information for many POAs. In this example, MIIS Server keeps information relating to 4 POAs. MIIS Server has to know whose information should be returned to the requesting MN.

The Unique ID for the current POA could be IP/MAC address, or any other IDs.

- Information Type requested by the MN

MIIS Server has to know what information should be returned to the requesting MN. For example, neighboring network type, or neighboring network cost.

- Information Filter

For each information type, there might be many information blocks returned, but only a few of them make sense to the MN. If the MN provides the filtering information to the MIIS server, these unnecessary information would not be returned, thus the network load is reduced.

Among these three types of information submitted to MIIS Server, two of them should be included in the MIIS Query initiated by MN: Information Type and Information Filter. The unique ID of POA is sent from POA to MIIS Server. This problem could be solved by MIIS transport mechanisms.

The following are use cases of the Query/Response and Information Filter. The filtering information may be general information, access network information or POA specific information.

USE CASE 1: One IE requested; Information Filter (one or more) included;

  - General Information

Description: MN wants to know the operators of its neighbors, but it only has the interface cards of NW1 and NW2.

Query: {InfoType == List_of_Operators; InfoFilter == NW2}

Retrieved Info: OP-1

Note: Without the info filter, the returned info would be OP-1 and OP-2. Since OP-2 only provides NW3 services, it is not useful for the MN.

  - Access Network Information

Description: MN wants to know the cost of its neighbors of NW2, but it only subscribes to OP-1 and is not allowed to roam to OP-2.

Query: {InfoType == Cost; InfoFilter ==OP-1 & NW2}

Retrieved Info: Cost-2

Note: Without the info filter, the returned info would be neighboring Cost-2 and Cost -3. Cost-3 is not useful for MN since it can not access to OP-2.

  - POA Specific Information

Description: MN wants to know the cost of its neighboring networks of the particular location of POA1.

Query: {InfoType == Cost; InfoFilter == LOC1}

Retrieved Info: Cost-2, Cost 3

USE CASE 2: Multiple IE requested; Information Filter (one or more) included;

2-1 Similar to above use cases, just one example shown:

Description: MN wants to know the operator IDs and Cost of its neighbors pertaining to the location of POA1.

Query: {InfoType == Operators & Cost ; InfoFilter == LOC1}

Retrieved Info: {OP-1, Cost-2} && {OP-2, Cost-3}

USE CASE 3: Optimized Query

- First query get the rough information: e.g., operator ID, neighboring network types, etc.

- Use the result of previous query as the filter for subsequent Queries. The unnecessary information is filtered out to reduce the signaling load over the networks.

Solution:

The RDF-SCHEMA has good mechanisms to present Filter Information. The problem is where and how to encapsulate the Filter Information for TLV type query. The potential alternatives are:

- Embedded Parameters, i.e., TLV after TLV or nested.

- Set Filter Info in the Query Primitive

The details of these alternatives are explained in the next section.

[Kalyan]

My opinion: it is better not to go for nested TLV’s as first option. Only if we feel that we can’t solve over problem, it would be better to discuss.

# Proposals:

## TLV after TLV in Information Request Primitives

The present draft standard allows sending a list TLVs within a request. Thus, one way is to send location based information TLV within the request. In Figure 2-2 an example is given where a client would like to acquire a list of networks in the vicinity from the PoA. With this TLV  the PoA location is also included.

Figure 2-2:   Example Information Request from Client to Server.

After the information is received by the MIIS server, it parses the TLV list and uses the PoA location information as a sort of filter to retrieve the correct network list within the client’s vicinity and sends the response back.

[Xiaoyu]

My understanding of this proposal is that it is one of the examples in 6.3.6 to show Location Information as the filter to retrieve the MIIS IEs and follow the structure defined in D1.0. This is definitely a meaningful use case.

Discussion points of the TLV after TLV (or nested TLV):

- For USE CASE 1 in Section 1, TLV after TLV (or nested TLV) works fine. This has been shown in the proposed Figure 2-1, and the draft section 6.3.6 and 6.3.7

- For USE CASE 2 in Section 2, the MN queries multiple IEs at one time, the TLV after TLV would be:

(1)

or For each ‘additional parameters’ in TLV query type?  In previous case, how to distinguish the requested TLV and the filtering TLV?

(2)

[Kalyan]

Question: Can the location information embedded in the (1) also solve the purpose of (2)? IMO: it can. At the same time another question is coming to my mind, whether we have to discuss about the granularity of the loc info?

## Alternative: Including a condition field within a query

 Xiaoyu: Could you please elaborate this idea, especially how this “condition“ look like?

The ‘condition’ here may be renamed as ‘InforFilter’ as in the very early draft .

The motivation here is to distinguish the IEs requested and the InfoFilter. It serves the same purpose as ‘additional parameter’ in the embedded TLV query. The only difference from TLV after TLV is how to represent or organize the filtering information.

An example is:

Of course, If we do not care the multiple IEs query use cases, this proposal can be ignored.

[Kalyan]

This is a good start to put the things in order. It is having a similarity with that of (1) as pointed above. Do you mean here to categorise the TYPE_IE_xxx values? That is:

InfoQueryParameters  {TYPE_IE_LIST_OF_NETWORKS, TYPE_IE_LIST_OF_OPERATORS, …}

InfoQueryFilters  {TYPE_IE_POA_LOCATION, TYPE_IE_POA_NAME, …}

This may enable the MIIS server to parse properly the TLVs in the query.

Figure 2-3:   Information Filter in TLV queries.

## Any other proposals??

# Amendments/Modifications to the Draft Standard

New Section:

6.3.4.x.x Information Query Type

Within 802.21 we have different format support for all queries and response to queries of information elements. In order differentiate the content of  the embedded TLVs, the first TLV should have to specify the format of the following TLVs

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_QUERY | Enumerated value indicating the query type in the following TLVs | 1 octet | 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |

Reffer to example 7.4.15.3.1 for use.

In Section:	7.4.15.1.2 Semantics of service primitive

InfoQueryType:

— TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry requests as defined in Section "Information Request and Response".

Embedded TLVs can be clustered into “query parameter cluster” and “query filter cluster”. A request contains minimum of one TLV from query parameter cluster and maximum of one TLV from query filter cluster. All the TLVs from query parameter cluster are independent of each other.

— RDF_DATA: …

— RDF_SCHEMA_URL: …

— RDF_SCHEMA: …

The query filter cluster consists of following TLVs:

- TYPE_IE_POA_LOCATION

- TYPE_IE_POA_ADDRESS

All other TLVs (TYPE_IE_xxx and TYPE_REPORT_xxx) are categorised under query parameter cluster.

In Section:	7.4.15.3.1 Example Query for TLV

A STA enters a hotspot and detects a PoA. It would like to get a list of networks and operators within its vicinity. After discovering the MIIS server, it sends an MIHF frame with the following variable payload in TLV format:

On reception of this TLV query, the MIIS server interprets the request as follows:

- request for list of networks associated with the PoA location and

- request for list of operators associated with the PoA location

An example of the response to this request would be:

# References

- Draft IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services, IEEE P802.21/D01.00

- http://www.ieee802.org/21/march06_meeting_docs/21-06-0525-00-0000-LocInfo_in_IS_request.ppt