| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | 21-06-0626-00-0000-Link_Get_Parameters.doc |  |
| Date Submitted | May, 2006 |  |
| Source(s) | Ajay Rajkumar, Peretz Feder | Lucent Technologies pfeder@lucent.com ajayrajkumar@lucent.com |
| Re: | P802-21-D01.00.pdf |  |
| Abstract | Link_Get_Parameters Command |  |
| Purpose | Replace text in current draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

6.2.5.2 Link Commands

The following set of Link Commands are defined:

Table 5—Link Commands

| No | Link Commands | Comments |
| --- | --- | --- |
| 1 | Link Configure Threshold | Configure the threshold for various link layer events such as Link Going Down |
| 2 | Link Get Parameters | Get parameters measured bt the active link, such as SNR, BER, RSSI |

7.2.2.2 MIH_LINK_SAP

Add entry 14 to table 15:

No 14: Link Get Parameters: Command: Request parameters of air interface

7.3.x.x

7.3.x Link_Get_Parameters.request

7.3.x.x Function

This primitive is used by MIH Function to obtain Link Parameters

.

7.3.x.x Semantics of service primitive

The primitive parameters are as follows:

Link_Get_Parameters.request (

LinkParameterList

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| LinkParamterList | List | NA | A list of the following set of link parameters RSSI SNR C/I BER PER Link Data Rate (goodput) |
| RSSI |  | NA |  |
| SNR |  | NA |  |
| C/I |  | NA |  |
| BER |  | NA |  |
| Data Rate |  | NA |  |

7.3.x.x When generated

This primitive is generated by an upper layer entity that may need to immediately obtain present values of different link parameters.

7.3.x.x Effect on receipt

The recipient responds immediately with Link_Get_parameters confirm primitive.

7.3.x Link_Get_Parameters.response

7.3.x.x Function

This primitive is sent in response to the Link_get_Parameters.request primitive. This primitive provides present value conditions of the link, which may reflect an on going average measurement

.

7.3.x.x Semantics of service primitive

The primitive parameters are as follows:

Link_Get_Parameters.response (

LinkParameterList

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| LinkParamterList | List | NA | A list of the following set of link parameters RSSI SNR C/I BER PER Link Data Rate (goodput) |
| RSSI |  | 0-90dB |  |
| SNR |  | -30-60dB |  |
| C/I |  | -20-50dB |  |
| BER |  | 10e-1 to 10-e-6 |  |
| Data Rate |  | In Mbps |  |

7.3.x.x When generated

This primitive is generated in response to the Link_Get_Parameters.request operation

7.3.x.x Effect on receipt

The recipient immediately receives the present link condition parameters and may pass them to the MIH users