Information Elements

| No | Name of Information Element | Description | Representation | Length (octets) |
| --- | --- | --- | --- | --- |
| General Information Elements |  |  |  |  |
| 1.1 | TYPE_IE_LIST_ OF_NETWORKS | List of Neighboring Access Networks. Link types of the networks that are available in a given geographical area | For example, RADIUS NAS-Port-Type attribute value which is a unique identifier defined in http://www.iana.org/ assignments/radius-types, e.g., 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - 1X-EV etc. | 4 |
| 1.2 | TYPE_IE_NUMB ER_OF_OPERAT ORS | Number of distinct operators for each available link type. There maybe multiple operators for each link type. | If networks of same link type have different operators then they are classified as different networks. Thus there may be three different 802.11 networks in an area, each supported by a different operator | 4 |
| 1.3 | TYPE_IE_LIST_ OF_OPERATORS | Network Operator List for each type of link. This includes the list of network operators for each of the access networks. |  | Variable |
| 1.4 | TYPE_IE_NETWORK_CAPABILITIES | Capabilities of the access networks | For example, if access networks allow VPN services, proxy mobile IP servies, link level security, P2P services, etc | Variable |
| 1.5 | TYPE_IE_OPER ATOR_IDENTIFI ER | Network Operator The operator of a network. | RADIUS Operator-Name attribute defied in draft-ietf-geoprivradius-lo- 05.txt. | Variable |
| 1.6 | TYPE_IE_ROAM ING_PARTNERS | Roaming Partners. Operators with which the current network operator has direct roaming agreements. | Each operator has the same structure as Network Operator information element. | Variable |
| 1.7 | TYPE_IE_COST | Cost Indication of cost for service or network usage | Should include the following: Value: (Actual cost denomination) Unit: (Country/Currency code) - enumerated Time Duration: Duration for which cost is applicable - enumerated Service Info: (whether unrestricted or restricted) | Variable |
| Access Network specific information For each (Link Type and Operator) combination following information may be provided |  |  |  |  |
| 2.1 | TYPE_IE_NUMB ER_POA | Number of Point of Attachments (PoA) for a specific Access Network in the Neighborhood. Number of APs, BSs etc. in the vicinity of client device | This may be retrieved from Neighbor Reports | 4 |
| 2.2 | TYPE_IE_NETW ORK_SECURITY | Security characteristics of the link layer. | Authentication Methods, Cipher Suites The security capabilities type may be defined by media-specific MIB | Variable |
| 2.3 | TYPE_IE_QOS | QoS (Quality of Service) characteristics of the link layer | The QoS classes and traffic type may be defined by media-specific MIB | Variable |
| PoA specific information For each PoA of eachfollowing IEs may be provided |  |  |  |  |
| 3.1 | TYPE_IE_POA_ ADDRESS | MAC Address of PoA | Link-layer address of PoA (Usually the MAC Address of the PoA) | Variable |
| 3.2 | TYPE_IE_POA_L OCATION | Location of PoA Geographical location of a given PoA. Multiple location types are supported including coordinate-based location information and civic address | Two types of location information are supported in draft-ietf-geopriv-radius-lo- 04.txt, i.e., geospatial location information and civic location information. Geospatial location information consists of: - Latitude Resolution - Latitude - Longitude Resolution - Longitude - Altitude Type {Floor, Meters) - Altitude Resolution - Altitude - Map Datum (Co-ordinate reference system) Civic location information consists of: - Country Code - Civic address elements Method of obtaining location information - GPS - Manual | Variable |
| 3.3 | TYPE_IE_POA_ DATA_RATE | Data Rate The minimum and maximum value of data rate supported by the link layer of a given PoA | A data rate is represented as a 32-bit unsigned integer in unit of Kbps. | 4 (32-bit unsigned integer) |
| 3.4 | TYPE_IE_POA_P HY_TYPE | The media PHY type. | Classification of the PHY layer | Variable |
| 3.5 | TYPE_IE_POA_ MAC_TYPE | The media MAC type. | Classification of the MAC layer | Variable |
| 3.6 | TYPE_IE_POA_ CHANNEL_RAN GE | Channel Range/Parameters Spectrum range supported by the Channel for that PoA | (Low, High range) MHz | 4,4 (32-bit unsigned integer) |
| Higher layer services/information per PoA |  |  |  |  |
| 4.1 | TYPE_IE_POA_S UBNET_INFOR MATION | Information about subnets supported by a typical PoA | (IPv4/IPv6) Each PoA may be assigned to multiple subnets. Availability of global or private address spaces. | Variable |
| 4.2 | TYPE_IE_POA_ CAPABILITIES | Bitmap of PoA capabilities | Security Available Y/N QoS Available Y/N Emergency Services available Y/N Others: |  |
| Other Information Elements |  |  |  |  |
| 5.1 | Vendor Specific IEs | Vendor Specific Services | These may be defined by individual vendors. |  |