| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Amendment about OldLinkAction parameter |  |
| DCN | 21-06-0628-00-0000 |  |
| Date Submitted | April, 20, 2006 |  |
| Source(s) | Guo Junxiang |  |
| Re: | IEEE 802.21 Session #14 in Jacksonville, FL |  |
| Abstract | In current draft D01, several handover primitives are inconsistent to their corresponding messages on the definition of OldLinkAction parameter .This contribution proposes to amend this inconsistency. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Problems

In the current draft, each of such primitives has an OldLinkAction or CurrentLinkAction

parameter (these two parameters have the same meaning although with different names):

MIH_Switch.request，

MIH_Handover_Initiate.request，

MIH_Handover_commit.request,

MIH_Handover_commit.response .

But the definition of this parameter in these primitives is inconsistent to that in their corresponding messages. The following will compare the inconsistency and propose amendment for this parameter (OldLinkAction/CurrentLinkAction) in these primitives.

## 1.1 comparison of MIH_Switch.request with MIH_Switch Request

## 7.4.6.1 MIH_Switch.request

7.4.6.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Switch.request 			(

SourceIdentifier,

DestinationIdentifier,

HandoverMode,

OldLinkIdentifier,

OldLinkActions

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group identi fier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| HandoverMode | Enumerate | Make-before- Break Break-before- Make | The handover mode decides the sequence of the execution of link command. If it is make- before-break, the Link Connect of the new link is executed before Link Disconnect of old link; if it is break-before-make, Link Connect is after the Link Disconnect |
| New Link Identifier | Identifier | N/A | Identifier of new link |
| Old Link Identifier | Identifier | N/A | Identifier of old link |
| Old Link Actions | Enumerate | Link Disconnect; Link Sleep; Link Power Down | Upper layers may choose which action the old link should takes. |

      - MIH_Switch Request

This message is used to switch an active session from one link to another.

| Name | Type |
| --- | --- |
| HandoverMode | Handover Mode (12) |
| NewLinkIdentifier | Network Identifier (248) |
| OldLinkIdentifier | Network Identifier (248) |
| OldLinkAction | Link Action (13) |

As the above show, the parameter type of OldLinkAction in MIH_Switch Request message is Link Action (13), which is defined as follows:

      - Link Action

This parameter specifies the action to take on link connectivity during handover.

| Type | Length | Value |
| --- | --- | --- |
| 13 | 4 | Specifies the suggested action on link during handover. Combina tion of below choices are possible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit #5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |

- Problem: Apparently, the type of Old Link Actions parameter in  MIH_Switch.request primitive is inconsistent to that of OldLinkAction parameter in MIH_Switch Request message .

## Comparison of MIH_Handover_Initiate.request with MIH_Handover_Initiate Request

      - MIH_Handover_Initiate.request

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.request 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

SuggestedNewLinkIdentifier,

SuggestedNewPoAIdentifier,

HandoverMode,

CurrentLinkAction,

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| SuggestedNewLinkI dentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks | N/A | This is the identifier of new network to which handover needs to be initi ated. |
| SuggestedNew PoAIdentifier | MAC_ADDRESS (Optional) | N/A | This is the preferred Point of Attach ment (AP/BS) on new network |
| HandoverMode | Enumerated | Make-before_Break Break-before-Make | The handover mode may influence the manner in which links are prepared for handover |
| CurrentLinkAction | Enumerated | LINK_DISCONNECT LINK_LOW_POWER LINK_POWER_DO WN LINK_NO_ACTION | Specifies suggested action on old link once handover procedures have been executed. |
| QueryResourceList | Boolean | N/A | Flag to query the available list of resources on the new link |

      - MIH_Handover_Initiate Request

This message is used for communication between MIH Function on client with MIH Function on network. The function is used to communicate intent of handover initiation. The handover may be initiated either by the client or the network.

| Name | Type |
| --- | --- |
| SuggestedNewLinkIdentifier | Network Identifier (248) |
| SuggestedMACNewPoAIdentifier | PoA MAC Address (250) |
| HandoverMode | Handover Mode (12) |
| CurrentLinkAction | Link Action (13) |

- Problem: Apparently, the type of CurrentLinkAction parameter in  MIH_Handover_Initiate.request primitive is inconsistent to that in MIH_Handover_Initiate Request message.

## Comparison of MIH_Handover_Commit.request with MIH_Handover_Commit Request

      - MIH_Handover_Commit.request

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.request 				(

SourceIdentifier,

DestinationIdentifier,

NewLinkIdentifier,

MACNewPoA,

CurrentLinkAction

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| NewLinkIdentifier | May be one of dif ferent 802 or cel lular networks | N/A | This specifies the new network to which the connection needs to be handed over. |
| MACNewPoA | MAC Address. | N/A | MAC Address of New PoA |
| CurrentLinkAction | Bitmap | 0-4 | Specifies suggested action on old link once handover procedure has been executed: 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4: Reserved |

      - MIH_Handover_Commit Request

This message is used by MIH Function to communicate the intent to commit to a handover request to a specific link and PoA.

| Name | Type (1 octet) |
| --- | --- |
| NewLinkIdentifier | Network Identifier (248) |
| NewPoAMAC | PoA MAC Address (250) |
| CurrentLinkAction | Link Action (13) |

- Problem: Apparently, the type/length of CurrentLinkAction parameter in  MIH_Handover_Commit.request primitive is inconsistent to that in MIH_Handover_Commit Request message.

## Comparison of MIH_Handover_Commit.response with MIH_Handover_Commit Response

      - MIH_Handover_Commit.response

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.response 	(

DestinationIdentifier,

SourceIdentifier,

CurrentLinkAction,

HandoverStatus

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkAction | Bitmap | 0-4 | Specifies suggested action on old link once handover procedure has been exe cuted: 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4: Reserved |
| HandoverStatus | Boolean | N/A | 0: Abort handover 1: Perform handover |

      - MIH_Handover_Commit Response

This message is used by MIH Function to communicate the response of request to commit to a handover request to a specific link and PoA.

| Name | Type |
| --- | --- |
| OldLinkAction | Link Action (13) |

- Problem: Apparently, the type/length of CurrentLinkAction parameter in  MIH_Handover_Commit.response primitive is inconsistent to that of OldLinkAction parameter in MIH_Handover_Commit Response message.

# 2 Amendments

Although with different names, OldLinkAction parameter has the same meaning to

CurrentLinkAction parameter in these primitives, so in order to facilitate understanding,

we unify these two names into one name of OldLinkAction. Also for consistency with

each other, we unify the type of OldLinkAction as Link Action(13) which is defined in

8.4.2.13 of the current draft. The following is the amendments for these chapters (red

words highlight the changes from the original):

## 7.4.6.1 MIH_Switch.request

7.4.6.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Switch.request 			(

SourceIdentifier,

DestinationIdentifier,

HandoverMode,

NewLinkIdentifier,  //editorail issue, missing

OldLinkIdentifier,

OldLinkAction

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group identi fier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| HandoverMode | Enumerate | Make-before- Break Break-before- Make | The handover mode decides the sequence of the execution of link command. If it is make- before-break, the Link Connect of the new link is executed before Link Disconnect of old link; if it is break-before-make, Link Connect is after the Link Disconnect |
| New Link Identifier | Identifier | N/A | Identifier of new link |
| Old Link Identifier | Identifier | N/A | Identifier of old link |
| Old Link Action | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during handover. Combina tion of below choices are possible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit #5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |

7.4.10.1 MIH_Handover_Initiate.request

7.4.10.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.request 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

SuggestedNewLinkIdentifier,

SuggestedNewPoAIdentifier,

HandoverMode,

OldLinkAction

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| SuggestedNewLinkI dentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks | N/A | This is the identifier of new network to which handover needs to be initi ated. |
| SuggestedNew PoAIdentifier | MAC_ADDRESS (Optional) | N/A | This is the preferred Point of Attach ment (AP/BS) on new network |
| HandoverMode | Enumerated | Make-before_Break Break-before-Make | The handover mode may influence the manner in which links are prepared for handover |
| OldLinkAction | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during handover. Combina tion of below choices are possible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit#5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |
| QueryResourceList | Boolean | N/A | Flag to query the available list of resources on the new link |

      - MIH_Handover_Initiate Request

This message is used for communication between MIH Function on client with MIH Function on network. The function is used to communicate intent of handover initiation. The handover may be initiated either by the client or the network.

| Name | Type |
| --- | --- |
| SuggestedNewLinkIdentifier | Network Identifier (248) |
| SuggestedMACNewPoAIdentifier | PoA MAC Address (250) |
| HandoverMode | Handover Mode (12) |
| OldLinkAction | Link Action (13) |

7.4.12.1 MIH_Handover_Commit.request

7.4.12.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.request 				(

SourceIdentifier,

DestinationIdentifier,

NewLinkIdentifier,

MACNewPoA,

OldLinkAction

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| NewLinkIdentifier | May be one of dif ferent 802 or cel lular networks | N/A | This specifies the new network to which the connection needs to be handed over. |
| MACNewPoA | MAC Address. | N/A | MAC Address of New PoA |
| OldLinkAction | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during handover. Combina tion of below choices are possible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit #5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |

      - MIH_Handover_Commit Request

This message is used by MIH Function to communicate the intent to commit to a handover request to a specific link and PoA.

| Name | Type (1 octet) |
| --- | --- |
| NewLinkIdentifier | Network Identifier (248) |
| NewPoAMAC | PoA MAC Address (250) |
| OldLinkAction | Link Action (13) |

7.4.12.2 MIH_Handover_Commit.response

7.4.12.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.response 	(

DestinationIdentifier,

SourceIdentifier,

OldLinkAction,

HandoverStatus

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| OldLinkAction | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during handover. Combina tion of below choices are possible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit#5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |
| HandoverStatus | Boolean | N/A | 0: Abort handover 1: Perform handover |