| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Additional information reports and IEs to clarify functionalities of CS and IS |  |
| DCN | 21-06-0657-00-0000 |  |
| Date Submitted | May 2006 |  |
| Source(s) | Kentaro Ishizu, Yoshia Saito, Masahiro Kuroda | NICT |
| Re: | IEEE 802.21 session #15 in Jacksonville |  |
| Abstract | This document proposes additional information reports and IEs to clarify functionalities of CS and IS. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Additional pre-defined information reports

To realize MIH Get Status, MIH Scan and MIH Network Address Information as IS, following reports shall be added as pre-defined information report. The additional parameters and values in each TLV refer semantics of service primitives for corresponding MIH commands defined in 7.4.5.1.2, 7.4.5.2.4, 7.4.9.1.2, 7.4.9.2.2, 7.4.14.1.2 and 7.4.14.2.2.

      - Terminal Status Report

This report provides information about terminal status of the currently connected and potentially available links. This report may help handover decision on mobile node or network entity.

        - Request for Terminal Status Report

| Type | Length | Additional Parameters |
| --- | --- | --- |
| TYPE_REPORT_TERMINAL_STATUS | Variable | Get Status Request Sets |

NOTE: “Get Status Request Sets” refers to 7.4.5.1.2.

        - Response for Terminal Status Response

| Type | Length | Value |
| --- | --- | --- |
| TYPE_REPORT_TERMINAL_STATUS | Variable | NETWORK_TYPES TLV |
|  |  | DEVICE_INFO TLV |
|  |  | OPERATION_MODE TLV |
|  |  | NETWORK_ID TLV |
|  |  | CHANNEL_ID TLV |
|  |  | CHANNEL_QUALITY TLV |
|  |  | LINK_SPEED TLV |
|  |  | BATTERY_LEVEL TLV |

NOTE: Values described above refer to 7.4.5.2.4.

      - Scan Report

This report provides information about the neighboring PoA of the access networks. This report may be used to survey potential PoA that the MN may later elect to try to join.

        - Request for Scan Report

| Type | Length | Additional Parameters |
| --- | --- | --- |
| TYPE_REPORT_SCAN | Variable | Scan Request Sets |

NOTE: “Scan Request Sets” refers to 7.4.9.1.2.

        - Response for Scan Report

| Type | Length | Additional Parameters |
| --- | --- | --- |
| TYPE_REPORT_SCAN | Variable | Scan Response Sets |

NOTE: “Scan Response Sets ” refers to 7.4.9.2.2.

      - Network Address Information Report

This report provides information about Network Address Information. This report may be used to discover information related to network address of mobile node before handover.

        - Request for Network Address Information Report

| Type | Length | Additional Parameters |
| --- | --- | --- |
| TYPE_REPORT_NETWORK_ ADDRESS_INFORMATION | Variable | Network Address Information Request Sets |

NOTE: “Network Address Information Request Sets” refers to 7.4.14.1.2.

        - Response for Network Address Information Response

| Type | Length | Value |
| --- | --- | --- |
| TYPE_REPORT_NETWORK_ ADDRESS_INFORMATION | Variable | CURRENT_LINK_IDENTIFIER TLV |
|  |  | MAC_MODILE_NODE TLV |
|  |  | HOME_ADDRESS TLV |
|  |  | COA TLV |
|  |  | FA_ADDRESS TLV |
|  |  | ACCESS_ROUTER_ADDRESS TLV |
|  |  | NETWORK_ADDRESS_INFORMATION TLV |

NOTE: Values described above refer to 7.4.14.2.2.

2. Additional IEs

Additional IEs for terminal specific information shall be added to the Table 6, by which Information Service can retrieve the same information as MIH Get Status and MIH Network Address Information. As for MIH Scan, required IEs shall be discussed.

Description, Representation and Length for the new IEs in the table shall be defined.

Table 6 – Information Elements

| No | Name of Information Element | Description | Representation | Length (Octets) |
| --- | --- | --- | --- | --- |
| Terminal specific information |  |  |  |  |
| 5.1 | TYPE_IE_TERMINAL_ NETWORK_TYPES |  |  |  |
| 5.2 | TYPE_IE_TERMINAL_ DEVICE_INFO |  |  |  |
| 5.3 | TYPE_IE_TERMINAL_ OPERATION_MODE |  |  |  |
| 5.4 | TYPE_IE_TERMINAL_ NETWORK_ID |  |  |  |
| 5.5 | TYPE_IE_TERMINAL_ CHANNEL_ID |  |  |  |
| 5.6 | TYPE_IE_TERMINAL_ CHANNEL_QUALITY |  |  |  |
| 5.7 | TYPE_IE_ TERMINAL_ LINK_SPEED |  |  |  |
| 5.8 | TYPE_IE_ TERMINAL_ BATTERY_LEVEL |  |  |  |
| 5.9 | TYPE_IE_TERMINAL_ CURRENT_LINK_IDENTIFIER |  |  |  |
| 5.10 | TYPE_IE_TERMINAL_ MAC_MODILE_NODE |  |  |  |
| 5.11 | TYPE_IE_TERMINAL_ HOME_ADDRESS |  |  |  |
| 5.12 | TYPE_IE_TERMINAL_COA |  |  |  |
| 5.13 | TYPE_IE_TERMINAL_ FA_ADDRESS |  |  |  |
| 5.14 | TYPE_IE_TERMINAL_ ACCESS_ROUTER_ADDRESS |  |  |  |
| 5.15 | TYPE_IE_TERMINAL_ NETWORK_ADDRESS_INFORMATION |  |  |  |
| Other Information Elements |  |  |  |  |
| 6.1 | Vendor Specific IEs |  |  |  |