| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Explanation about Link Action: LB Issue- #36: Comment 615 |  |
| DCN | 21-06-0688-00-0000 |  |
| Date Submitted | July 10, 2006 |  |
| Source(s) | Guo Junxiang |  |
| Re: | IEEE 802.21 Session #15 in San Diego, CA |  |
| Abstract | This contribution is a response to LB Issue #36 in comment assignment and provides explanation about the use case of each link action. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Issue

In current draft section 8.4.2.13 has define different link action choice, but does not provide the detail explanation of what these values mean. This contribution will deal with this problem.

8.4.2.13 Link Action

This parameter specifies the action to take on link connectivity during handover.

| Type | Length | Value |
| --- | --- | --- |
| 13 | 4 | Specifies the suggested action on link during handover. Combina tion of below choices are possible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit #5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |

# 2 Explanation

The following is the description of these values in above table:

LINK_DISCONNECT:

Disconnect the old link connection directly.

LINK_LOW_POWER:

Cause the old link to adjust its RF power level to be low power consumption.

LINK_POWER_DOWN:

Cause the old link to power down so as to stop transmitting and enter idle model.

LINK_NO_ACTION:

The old link need not take any action.

LINK_RESOURCE_RETAIN:

The old link will be disconnected but the resource for the old link connection still remains so as to reestablishing the old link connection later can be more efficient.

DATA_FORWARDING_REQUEST:

When MN handover to the new target network, this indication requires the buffered data at the old PoA entity to be forwarded to the new PoA entity in order to avoid data loss.

BI_CASTING_REQUEST:

During the handover phase or after the handover phase, this indication requires the buffered data at an inner network entity (inter-system mobility anchor) to be bi-casted to both the old PoA and new PoA entities.

HANDOVER_CANCEL:

Handover to a new link is failed and cancelled, the old link should remain unaffected or need to rollback if it is possible. When old link works normally, a handover to another new candidate network may be initiated.