| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | New sections for 7.4 MIH Primitives : LB Issue #15 |
| Date Submitted | July, 2006 |
| Source(s) | Eunah Kim, Yoshihiro Ohba |
| Re: | IEEE 802.21 Session #NN in San Diego, California |
| Abstract | This contribution proposes new subsections for new primitives which were resolved to be added to the draft during the last June ad-hoc meeting. |
| Purpose | Discuss and adopt in the draft. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

1. Introduction

During the last June Ad-hoc meeting, some primitives were proposed to be added to the draft, which was resolved on condition that it would be moved in July plenary meeting. The proposed primitives are not new ones, but different types of existing primitives.

This contribution proposes to change subsection 7.4.10 to 7.4.15 as indicated below :

- Change subsection numbers as follows : response type of primitives

  - 7.4.10.23 MIH_Handover_Initiate.response

  - 7.4.11.23 MIH_Handover_Prepare.response

  - 7.4.12.23 MIH_Handover_Commit.response

  - 7.4.13.23 MIH_Handover_Complete.response

  - 7.4.14.23 MIH_Network_Address_Information.response

  - 7.4.15.23 MIH_Get_Information.response

- Add new subsections as follows : indication and confirm types of primitives

  - 7.4.10.2 MIH_Handover_Initiate.indication

  - 7.4.10.4 MIH_Handover_Initiate.confirm

  - 7.4.11.2 MIH_Handover_Prepare.indication

  - 7.4.11.4 MIH_Handover_Prepare.confirm

  - 7.4.12.2 MIH_Handover_Commit.indication

  - 7.4.12.4 MIH_Handover_Commit.confirm

  - 7.4.13.2 MIH_Handover_Complete.indication

  - 7.4.13.4 MIH_Handover_Complete.confirm

  - 7.4.14.2 MIH_Network_Address_Information.indication

  - 7.4.14.4 MIH_Network_Address_Information.confirm

  - 7.4.15.2 MIH_Get_Information.indication

  - 7.4.15.4 MIH_Get_Information.confirm

2. Proposed Changes

- As for “link identifier” and “link action”, it reflects the last June ad-hoc meeting results.

- Add proposed subsections in 7.4.10 to 7.4.15 as indicated below :

7.4.10.2 MIH_Handover_Initiate.indication

7.4.10.2.1 Function

MIH Function on a mobile node or network indicates to an upper layer that MIH_Handover_Initiate Request message is received from a peer MIH Function.

7.4.10.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.indication (

SourceIdentifier,

SuggestedNewLinkIdentifier,

HandoverMode,

CurrentLinkAction,

QueryResourceList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| SuggestedNewLinkIdentifier | Link Identifier | N/A | This is the identifier of new link to which handover needs to be initiated. |
| HandoverMode | Enumerated | Make-before_Break Break-before-Make | The handover mode may influence the manner in which links are prepared for handover |
| CurrentLinkAction | Link Action | Any valid value of Link Action | Specifies the suggested action on old link during handover |
| QueryResourceList | Boolean | N/A | Flag to query the available list of resources on the new link |

7.4.10.2.3 When generated

It is generated when MIH Function on a mobile node or network receives MIH_Handover_Initiate Request message from a peer MIH Function.

7.4.10.2.4 Effect on receipt

An upper layer receiving this primitive replies with MIH_Handover_Initiate.response primitive. If the recipient of this primitive is a network upper layer, it may generate a MIH_Handover_Prepare.request primitive to exchange a MIH_Handover_Prepare message with the upper layer in the target link under consideration before sending this response primitive.

7.4.10.4.MIH_Handover_Initiate.confirm

7.4.10.4.1 Function

MIH Function on a mobile node or network generates MIH_Handover_Initiate.confirm primitive when it receives MIH_Handover_Initiate Response message from a peer MIH Function or MIH_Handover_Initiate.request primitive from an upper layer for local command operation.

7.4.10.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.confirm (

SourceIdentifier,

HandoverAck,

PreferredLinkIdentifier,

AbortReason,

AvailableResoueceList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the response initiated. This field may be optionally left empty if the command is local. |
| HandoverAck | Boolean | N/A | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a reason code is provided |
| PreferredLinkIdentifier | Link Identifier | N/A | This is the identifier of new link to which handover needs to be initiated. This may be different than the link that was suggested in the handover request. |
| AbortReason | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| Available Resource List | List | N/A | List of available sources on new link |

7.4.10.4.3 When generated

It is generated when MIH Function on a mobile node or network receives MIH_Handover_Initiate Response message from a peer MIH Function. For local command operation, it is issued after that MIH Function on a mobile node or network performs required actions after receiving MIH_Handover_Initiate.request primitive from an upper layer.

7.4.10.4.4 Effect on receipt

On receiving the primitive the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the primitive. If the recipient of the primitive is a client upper layer and in case the handover is to be executed, it may also provide information about selected link identifier on the selected network with MIH_Handover_Commit.request primitive.

7.4.11 MIH Handover Prepare

7.4.11.1 MIH_Handover_Prepare.indication

TBD

7.4.11.2 MIH_Handover_Prepare.confirm

TBD

7.4.12 MIH_Handover_Commit

7.4.12.1 MIH_Handover_Commit.indication

TBD

7.4.12.2 MIH_Handover_Commit.confirm

TBD

7.4.13 MIH_Handover_Complete

7.4.13.1 MIH_Handover_Complete.indication

TBD

7.4.13.2 MIH_Handover_Complete.confirm

TBD

7.4.14 MIH_Network_Address_Information

7.4.14.1 MIH_Network_Address_Information.indication

TBD

7.4.14.2 MIH_Network_Address_Information.confirm

TBD

7.4.15 MIH Get Information

7.4.15.1 MIH_Get_Information.indication

TBD

7.4.15.2 MIH_Get_Information.confirm

TBD