| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | New sections for 7.4 MIH Primitives : LB Issue #15 |
| Date Submitted | July, 2006 |
| Source(s) | Eunah Kim, Junghoon Jee, Yoshihiro Ohba |
| Re: | IEEE 802.21 Session #NN in San Diego, California |
| Abstract | This contribution proposes new subsections for new primitives which were resolved to be added to the draft during the last June ad-hoc meeting. |
| Purpose | Discuss and adopt in the draft. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

1. Introduction

During the last June Ad-hoc meeting, some primitives were proposed to be added to the draft, which was resolved on condition that it would be moved in July plenary meeting. The proposed primitives are not new ones, but different types of existing primitives.

This contribution proposes to change subsection 7.4.10 to 7.4.15 as indicated below :

- Change subsection numbers as follows : response type of primitives and Example Queries

  - 7.4.10.23 MIH_Handover_Initiate.response

  - 7.4.11.23 MIH_Handover_Prepare.response

  - 7.4.12.23 MIH_Handover_Commit.response

  - 7.4.13.23 MIH_Handover_Complete.response

  - 7.4.14.23 MIH_Network_Address_Information.response

  - 7.4.15.23 MIH_Get_Information.response

  - 7.4.15.35 Example Queries

- Add new subsections as follows : indication and confirm types of primitives

  - 7.4.10.2 MIH_Handover_Initiate.indication

  - 7.4.10.4 MIH_Handover_Initiate.confirm

  - 7.4.11.2 MIH_Handover_Prepare.indication

  - 7.4.11.4 MIH_Handover_Prepare.confirm

  - 7.4.12.2 MIH_Handover_Commit.indication

  - 7.4.12.4 MIH_Handover_Commit.confirm

  - 7.4.13.2 MIH_Handover_Complete.indication

  - 7.4.13.4 MIH_Handover_Complete.confirm

  - 7.4.14.2 MIH_Network_Address_Information.indication

  - 7.4.14.4 MIH_Network_Address_Information.confirm

  - 7.4.15.2 MIH_Get_Information.indication

  - 7.4.15.4 MIH_Get_Information.confirm

2. Proposed Changes

- Add proposed subsections in 7.4.10 to 7.4.15 as indicated below :

7.4.10.2 MIH_Handover_Initiate.indication

7.4.10.2.1 Function

This primitive is used by MIH Function to indicate that MIH_Handover_Initiate Request message is received from a peer MIH Function.

7.4.10.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.indication (

SourceIdentifier,

DestinationIdentifier,

SuggestedNewLinkIdentifier,

HandoverMode,

CurrentLinkAction

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of request. |
| SuggestedNewLinkIdentifier | Link Identifier | N/A | This is the identifier of new access link to which handover needs to be initiated. |
| HandoverMode | Handover Mode | Make before Break Break before Make | The handover mode may influence the manner in which links are prepared for handover |
| CurrentLinkAction | Link Action | Any valid value of Link Action | Specifies the suggested action on old link during handover |

7.4.10.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Initiate Request message from a peer MIH Function.

7.4.10.2.4 Effect on receipt

The MIH user receiving this primitive replies with MIH_Handover_Initiate.response primitive. If the recipient of this primitive is a network upper layer, it may generate a MIH_Handover_Prepare.request primitive to exchange a MIH_Handover_Prepare message with the MIH Function in the target link under consideration before sending this response primitive.

7.4.10.4.MIH_Handover_Initiate.confirm

7.4.10.4.1 Function

This primitive is used by MIH Function to confirm that MIH_Handover_Initiate Response message is received from a peer MIH Function.

7.4.10.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.confirm (

SourceIdentifier,

DestinationIdentifier,

HandoverAck,

PreferredLinkIdentifier,

AbortReason

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the response initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of response. |
| HandoverAck | Handover Status | N/A | If the handover has to be aborted then a reason code is provided |
| PreferredLinkIdentifier | Link Identifier | N/A | This is the identifier of new network link to which handover needs to be initiated. This may be different than the link that was suggested in the handover request. |
| AbortReason | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |

7.4.10.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Initiate Response message from a peer MIH Function..

7.4.10.4.4 Effect on receipt

On receiving the primitive the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the primitive. If the recipient of the primitive is a client upper layer and in case the handover is to be executed, it may also provide information about selected link identifier on the selected network with a MIH Handover Commit message.

7.4.11.2 MIH_Handover_Prepare.indication

7.4.11.2.1 Function

The MIH Function on the candidate network indicates that MIH_Handover_Prepare Request message is received from a peer MIH Function on the serving network so that the upper layer entity can identify the link resource usage and prepare resources for the impending handover.

7.4.11.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Prepare.indication (

SourceIdentifier,

DestinationIdentifier,

QueryResourceList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of request. |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

7.4.11.2.3 When generated

This primitive is generated by MIH Function when the MIH Function on the candidate network receives MIH_Handover_Prepare Request message from a peer MIH Function on the serving network.

7.4.11.2.4 Effect on receipt

The MIH user on the candidate network identifies the link resource usage and prepares resources for the impending handover. It also replies with MIH_Handover_Prepare.response primitive.

7.4.11.4 MIH_Handover_Prepare.confirm

7.4.11.4.1 Function

This primitive is used by MIH Function on the serving network to respond the result of resource preparation of the impending handover and to notify the link resource status of the candidate network.

7.4.11.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Prepare.confirm (

SourceIdentifier,

DestinationIdentifier,

ResourceStatus,

AvailableResourceList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the response initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of response. |
| ResourceStatus | Status | N/A | Specifies whether requested resources are available or not at the new PoA. |
| AvailableResourceList | List | N/A | List of resources actually available at the new suggested network and the new networkPoA. |

7.4.11.4.3 When generated

This primitive is generated by MIH Function when the MIH Function on the serving network receives MIH_Handover_Prepare Response message from a peer MIH Function on the candidate network.

7.4.11.4.4 Effect on receipt

After receiving this primitive, the upper layer entity on the serving network may send MIH_Handover_Initiate.response primitive to the MIH Function in case it was indicated with MIH_Handover_Initiate.indication primitive before.

7.4.12.2 MIH_Handover_Commit.indication

7.4.12.2.1 Function

This primitive is used by MIH Function to indicate that MIH_Handover_Commit Request message is received from a peer MIH Function.

7.4.12.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.indication (

SourceIdentifier,

DestinationIdentifier,

NewLinkIdentifier,

CurrentLinkAction

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of request. |
| NewLinkIdentifier | Link Identifier | N/A | This specifies the new network link to which the connection needs to be handed over. |
| CurrentLinkAction | Link Action | Any valid value of Link Action | Specifies the suggested action on old link during handover |

7.4.12.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Commit Request message from a peer MIH Function.

7.4.12.2.4 Effect on receipt

The MIH user receiving this primitive replies with MIH_Handover_Commit.response primitive.

7.4.12.4 MIH_Handover_Commit.confirm

7.4.12.4.1 Function

This primitive is used by MIH Function to confirm that MIH_Handover_Commit Response message is received from a peer MIH Function.

.7.4.12.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkAction,

HandoverStatus

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the response initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of response. |
| CurrentLinkAction | Link Action | Any valid value of Link Action | Specifies the suggested action on old link during handover |
| HandoverStatus | Handover Status | N/A | Whether the handover has to be performed or aborted |

7.4.12.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Commit Response message from a peer MIH Function.

7.4.12.4.4 Effect on receipt

The client recipient may initiate handover process and begin setting up of new layer 2 connection. The network recipient may determine that the handover procedure is in progress to the intended network.

7.4.13.2 MIH_Handover_Complete.indication

7.4.13.2.1 Function

This primitive is used by MIH Function to indicate the status of the handover operation.

7.4.13.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Complete.indication(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverStatus

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of request. |
| CurrentLinkIdentifier | Link Identifier. | Identifier | This identifies the current access network link over which the command needs to be sent. The command is then sent either at L2 or at L3. |
| HandoverStatus | Status | NA | The status of handover operation |

7.4.13.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Complete Request message from a peer MIH Function.

7.4.13.2.4 Effect on receipt

The MIH user receiving this primitive replies with MIH_Handover_Complete.response primitive.

7.4.13.4 MIH_Handover_Complete.confirm

7.4.13.4.1 Function

This primitive is used by MIH Function to confirm that MIH_Handover_Complete Response message is received from a peer MIH Function.

7.4.13.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Complete.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

ResourceStatus

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the response initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of response. |
| CurrentLinkIdentifier | Link Identifier | Identifier | This identifies the current access network link over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| ResourceStatus | Resource Retention | NA | The status of resource retention |

7.4.13.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Complete Response message from a peer MIH Function.

7.4.13.2.4 Effect on receipt

Upon receipt, the client upper layer entity may determine that the handover complete request was processed successfully. Upon receipt, the MIH Function in the new network may forward this message to the client.

7.4.14.2 MIH_Network_Address_Information.indication

7.4.14.2.1 Function

This primitive is used by MIH Function on new PoA to indicate that MIH_Network_Address_Information Request message is received from a peer MIH Function on old PoA.

7.4.14.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.indication (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HomeAddress,

CoA,

OldAccessRouterAddress,

OldFAAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of request. |
| CurrentLinkIdentifier | Link Identifier | Identifier | This identifies the current access network over which the command needs to be sent. The command is then sent either at L2 or at L3. |
| Home Address | IP Address | N/A | Home IP address of Mobile Node |
| CoA | IP Address | N/A | Care of Address of Mobile Node |
| Old Access Router Address | IP Address | N/A | IP address of old Access Router This parameter is only included when mobile node is using IPv6 |
| Old FA Address | IP Address | N/A | IP address of old Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |

7.4.14.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Network_Address_Information Request message from a peer MIH Function.

7.4.14.2.4 Effect on receipt

Upon reception of this primitive, The MIH user on new PoA may generate another MIH_Network_Address_information.request primitive further to deliver the corresponding message to the MIHF in the network, e.g Access Router with MIHF which may have enough information on this message. When this primitive is delivered to the upper layer of the MIHF in the network, e.g, Access Router with MIHF, this may trigger to send network address related information, e.g, Router Advertisement either directly to the mobile node or to MIHF.

7.4.14.4 MIH_Network_Address_Information.confirm

7.4.14.4.1 Function

This primitive is used by MIH Function to confirm that MIH_Network_Address_Information Response message is received from a peer MIH Function.

7.4.14.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HomeAddress,

FAAddress,

AccessRouterAddress,

NetworkAddressInformation

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | Any valid individual or group identifier | The identifier of entity where the response initiated. |
| DestinationIdentifier | MIHF ID | Any valid individual identifier | The destination identifier of response. |
| CurrentLinkIdentifier | Link Identifier | Identifier | This identifies the current access network over which the command needs to be sent. The command is then sent either at L2 or at L3. |
| Home Address | IP Address | N/A | Home IP address of Mobile Node |
| FA Address | IP Address | N/A | IP address of Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Access Router Address | IP Address | N/A | IP address of Access Router. This parameter is only included when mobile node is using IPv6 |
| Network Address Information | IP Address | N/A | In case of Mobile IPv4, Agent Advertisement is encapsulated and in case of Mobile IPv6, Router Advertisement is encapsulated. |

7.4.14.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Network_Address_Information Response message from a peer MIH Function.

7.4.14.4.4 Effect on receipt

The client upper layer entity may use the mobile node’s network address related information when handover starts.

7.4.15.2 MIH_Get_Information.indication

7.4.15.2.1 Function

This primitive is used by MIH Function to indicate that MIH_Get_Information Request message is received from a peer MIH Function.

7.4.15.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.indication (

InfoQueryType,

InfoQueryParameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryType | IS Query Type List | N/A | The type of query that is specified |
| InfoQueryParameters | IS Query Parameters | N/A | Query type specific parameters which indi-cate the type of information the client may be interested in. |

7.4.15.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Get_Information Request message from a peer MIH Function.

7.4.15.2.4 Effect on receipt

The recipient tries to interpret the query request and retrieve the specified information. Once the information is retrieved, the recipient replies with MIH_Get_Information.response primitive.

7.4.15.4 MIH_Get_Information.confirm

7.4.15.4.1 Function

This primitive is used by MIH to respond to a MIH_GET_Information.request primitive.

7.4.15.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.confirm (

InfoQueryType,

MIH_REPORT,

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryType | IS Query Type List | N/A | The type of query that is specified |
| MIH_REPORT | String | N/A | Report consisting of information requested by the MIH User |
| Status | Status | N/A | Specifies whether the information was successfully retrieved or not. |

7.4.15.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Get_Information Response message from a peer MIH Function.

7.4.15.4.4 Effect on receipt

The MIH User that requested the information tries to interpret the MIH_REPORT and takes suitable action.