| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_LINK_SAP primitives mapping with media specific link primitives |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Ronny Kim, Jin Lee LG Electronics, Inc. 533,Hogye-1dong,Dongan-gu, Anyang-shi,Kyongki-do,Korea | Voice: +82-31-450-1879 Fax: +82-31-450-7912 [mailto : {ronnykim, jins978}@lge.com] |
| Re: | IEEE 802.21 Session #14 in San Diego |  |
| Abstract | This contribution is to map media specific primitives with MIH_LINK_SAP primitives. |  |
| Purpose | Update 802.21 draft |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# 7.2.2.2 MIH_LINK_SAP

The key primitives defined as part of MIH_LINK_SAP are described in Table .

Tabke 15 - MIH_LINK_SAP Primitives

| No | Primitives | Service Category | Description | 802.16 | 802.11 |
| --- | --- | --- | --- | --- | --- |
| 1 | Link Event Discover | System Management | Discover link capabilities |  |  |
| 2 | Link Event Register | Event | Register for event notifications |  |  |
| 3 | Link Event Deregister | Event | Deregister for event notifications |  |  |
| 4 | Link Configure Thresholds | Command | Configure link thresholds for Link events | C-HO-REQ (HO-Scan) |  |
| 5 | Link Up | Event | L2 connectivity is established | C-NEM-REQ(Registration) | MLME-LinkUp.indication |
| 6 | Link Down | Event | L2 connectivity is lost | M-MTM-REQ,RSP(Deregistration) C-HO-NOTFY(HO-Scan) -RF signal -BS C-HO-RSP (HO-Scan) -List of signal info-MS | MLME-LinkDown.indication |
| 7 | Link Going Down | Event | L2 connectivity loss is imminent |  | MLME-LinkGoingDown.indication |
| 8 | Link Event Rollback | Event | Predicted event has not occurred and hence event indication must be rolled back. |  | MLME-EventRollback.indication |
| 9 | Link Parameters Change | Event | Link parameters have crossed specified thresholds |  | MLME-LinkParametersChange.indication |
| 10 | Link SDU Transmit Status | Event | Indicate transmission status of all PDU segments |  | MA-UNIDATA-STATUS.indication |
| 11 | Link Handover Imminent | Event | L2 handover is imminent | C-HO-NOTFY(start) | MLME-LinkHandoverImminent.indication |
| 12 | Link Handover Complete | Event | L2 handover has been completed | C-HO-NOTFY(cmplt) | MLME-LinkHandoverComplete.indication |
| 13 | Link Get Information | Information | Request for IEs | C- MIH-NOTIFY | MLME-MIHProtocol.request MLME-MIHProtocol.indication |

# 7.2.1.3 C_SAP

# [Note] At last 802.16 interim meeting, MIH related primitives are consolidated with other 802.16 native primitives.

## The key primitives defined as part of C_SAP are described in the table below.

| No | Primitives | Description |
| --- | --- | --- |
| 1 | C-NEM-REQ (Action Type = Ranging) C-NEM-RSP (Action Type = Ranging) | Upper layers can control ranging procedure with these primitives. Upper layers shall commence 802.16 link setup procedure by sending C-NEM-REQ (Action Type = Ranging) primitive through NCMS. Note) C-NEM-REQ (Action Type = Ranging) primitive with ranging type: “initial” is an 802.16 Link Switch Link Command (link setup) which corresponds to Link Switch MIH command. |
| 2 | M-NEM-REQ (Action Type = Registration) | Upper layers can control registration procedure with these primitives. Upper layers are notified of link setup by M-NEM-RSP (Action Type = Registration). Note) M-NEM-RSP (Action Type = Registration) primitive is an 802.16 Link_Up Link Event. |
| 3 | C-NEM-NOTIFY (Action Type = Neighbor) | When 802.16 MAC receives neighbor advertisement (MOB_NBR-ADV), this primitive is used to deliver the information to upper layers. |
| 4 | C-HO-REQ (HO-Scan) , C-HO-RSP (HO-Scan) | Upper layers can command autonomous scanning with these primitives. Upper layers can also schedule scanning period with BS. During scanning period BS may buffer downlink traffic to the mobile terminal. Note) These primitives are Scan Link Command for MIH Scan command in 802.21. |
| 5 | C-HO-REQ (HO-Scan) , C-HO-RSP (HO-Scan) C-HO-NOTIRY (HO-Scan) | Delivery of the primitives shall be based on the pre-registration procedure between upper layer management entities and NCMS. Scan report can be made remotely to the BS or locally to the upper layer entity depending on the report target value in C-HO-NOTIFY (HO-Scan) |
| 6 | C-HO-REQ, C-HO-RSP, C-HO-NOTIFY | Upper layers can control handover procedure by using these primitives. Note) These primitives can be used as link commands for MIH_Handover_initiate.request/response. Currently only parameters relevant to 802.16 handover are included. Parameters for Media Independent Handover shall be identified and added. |
| 7 | C-HO-NOTIFY | An MS transmits a MOB_HO-IND message for final indication that is about performing a HO. Note) This primitive is a Link Command of MIH_Handover_Commit.request/response MIH Command in the .21 draft. For remote service, 802.16 MAC management message shall be used. (MOB_HO-IND). Currently only parameters relevant to 802.16 handover are included. Parameters for Media Independent Handover shall be identified and added. |
| 8 | M-MTM-REQ M-MTM-RSP | These primitives are used to manage the status of mobile terminal. Upper layer can change the status of mobile terminal into power on/down/hold/de-register, etc. Note) These can be mapped to MIH Configure MIH Command Service. For remote service, 802.16 MAC management messages can be used. (RES-CMD/ DREG-CMD) |