| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements TLV Containers (Comment Assignment #3) |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Qiaobing Xie, Vivek Gupta, Xiaoyu Liu, Soohong Daniel Park, Eric Njedjou, Masahiro Kuroda |  |
| Re: | 21-06-0xxx-00-0000-IE_TLV_container |  |
| Abstract | Containers of Information Elements |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

A) Add the following figure in section 6.3.2 after Table-6.

TYPE_IE_NETWORK_TYPE              = …….

TYPE_IE_OPERATOR_IDENTIFIER = ……..

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = UMTS

TYPE_IE_OPERATOR_IDENTIFIER = Operator_3

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = UMTS

TYPE_IE_OPERATOR_IDENTIFIER = Operator_1

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = 802.16

TYPE_IE_OPERATOR_IDENTIFIER = Operator_3

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = 802.11

TYPE_IE_OPERATOR_IDENTIFIER = Operator_2

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

802.11, Operator_1 PoAK

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_1, PoA3

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_1, PoA2

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

List of Networks

TYPE_IE_NETWORK_TYPE              = 802.11

TYPE_IE_OPERATOR_IDENTIFIER = Operator_1

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

802.11, Operator_1, PoA1

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANGE

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_2, PoAn

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_2, PoA2

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_2, PoA1

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANGE

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

Figure XX – Depicting a List of Neighboring Networks with Information Elements

B) Insert the following new section before the current 6.3.4.1:

6.3.4.0. IE Containers

To effectively represent the heterogeneous neighborhood information in TLV format, three Information Element Containers are defined, namely List of Neighboring Access Networks Container, Access Network Container, and PoA Container:

- List of Neighboring Access Networks Container - contains a list of heterogeneous neighboring access networks for a given geophysical location. 

A List of Neighboring Access Networks Container shall contain at least one Access Network and may contain one or more Vender Specific IEs.

| Type= TYPE_CONTAINER_LIST_OF_NETWORKS | Length= Variable |
| --- | --- |
| Access Network Container #1 |  |
| Access Network Container #2 (optional) |  |
| … |  |
| Access Network Container #k (optional) |  |
| Vender Specific IE (optional) |  |

- Access Network Container – contains all the information depicting an access network. 

An Access Network Container shall contain at least one PoA Container and may contain one or more Vender Specific Network IEs:

| Type=TYPE_CONTAINER_NETWORK | Length= Variable |
| --- | --- |
| Network Type IE |  |
| Operator Identifier IE |  |
| Roaming Partners IE (optional) |  |
| Cost IE (optional) |  |
| Network Security IE (optional) |  |
| QoS IE (optional) |  |
| PoA Container #1 |  |
| PoA Container #2 (optional) |  |
| … |  |
| PoA Container #k (optional) |  |
| Vender Specific Network IE (optional) |  |

- PoA Container – contains all the information depicting a PoA and may include one or more optional Vender Specific PoA IEs:

| Type=TYPE_CONTAINER_POA | Length= Variable |
| --- | --- |
| PoA Location IE |  |
| PoA Data Rate IE |  |
| PoA MAC Type IE |  |
| PoA Channel Range IE |  |
| PoA Subnet Information IE |  |
| PoA Capabilities IE |  |
| PoA PHY Type IE |  |
| PoA Address IE |  |
| Vender Specific PoA IE (optional) |  |

TLVs for the component IEs containing in the Access Network Container and PoA Container are defined in current Sections 6.3.4.1 – 6.3.4.3.

C) Delete sections 6.3.4.1.1, 6.3.4.1.2, 6.3.4.1.3, and 6.3.4.2.1 (no longer needed).

D) Add the following new IE to depicting network type:

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_TYPE | Type of the access network | 1 octets | For example, RADIUS NAS-Port-Type attribute value which is a unique identifier defined in http://www.iana.org/ assignments/radius-types, e.g., 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - 1X-EV etc. |