| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | New sections for 7.4 MIH Primitives : LB Issue #15 |
| Date Submitted | September, 2006 |
| Source(s) | Eunah Kim, Junghoon Jee, Yoshihiro Ohba |
| Re: | IEEE 802.21 Session #NN in Melbourne |
| Abstract | This contribution proposes new subsections for new primitives which were resolved to be added to the draft during the last June ad-hoc meeting. |
| Purpose | Discuss and adopt in the draft. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

1. Introduction

During the last June Ad-hoc meeting, some primitives were proposed to be added to the draft and it was moved at the last July plenary meeting. The proposed primitives are not new ones, but different types of existing primitives.

This contribution proposes to change sections 7.4.10 to 7.4.15 as indicated below :

- Change section numbers for response type of primitives as follows

  - 7.4.10.23 MIH_Handover_Initiate.response

  - 7.4.11.23 MIH_Handover_Prepare.response

  - 7.4.12.23 MIH_Handover_Commit.response

  - 7.4.13.23 MIH_Handover_Complete.response

  - 7.4.14.23 MIH_Network_Address_Information.response

  - 7.4.15.23 MIH_Get_Information.response

- Add new sections for indication and confirm types of primitives as follows

  - 7.4.10.2 MIH_Handover_Initiate.indication

  - 7.4.10.4 MIH_Handover_Initiate.confirm

  - 7.4.11.2 MIH_Handover_Prepare.indication

  - 7.4.11.4 MIH_Handover_Prepare.confirm

  - 7.4.12.2 MIH_Handover_Commit.indication

  - 7.4.12.4 MIH_Handover_Commit.confirm

  - 7.4.13.2 MIH_Handover_Complete.indication

  - 7.4.13.4 MIH_Handover_Complete.confirm

  - 7.4.14.2 MIH_Network_Address_Information.indication

  - 7.4.14.4 MIH_Network_Address_Information.confirm

  - 7.4.15.2 MIH_Get_Information.indication

  - 7.4.15.4 MIH_Get_Information.confirm

2. Proposed Changes

- Add proposed sections, 7.4.10 to 7.4.15 as indicated below :

7.4.10.2 MIH_Handover_Initiate.indication

7.4.10.2.1 Function

This primitive is generated by MIH Function to indicate that MIH_Handover_Initiate Request message is received from a peer MIH Function.

7.4.10.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.indication (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier.

SuggestedNewLinkList,

SuggestedNewPoAList,

HandoverMode,

OldLinkAction,

QueryResourceList

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| SuggestedNewLinkList | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier List of new networks to which handover needs to be initiated. The front is most suitable, backward is less. |
| SuggestedNewPoAList | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| HandoverMode | Enumerated | Make-before_Break Break-before-Make | The handover mode may influence the manner in which links are prepared for handover |
| Old Link Actions | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8-31: Reserved |
| QueryResourceList | Boolean | N/A | Flag to query the available list of resources on the new link |

7.4.10.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Initiate Request message from a peer MIH Function.

7.4.10.2.4 Effect on receipt

The MIH user receiving this primitive replies with MIH_Handover_Initiate.response primitive. If the recipient of this primitive is a network upper layer, it may generate MIH_Handover_Prepare.request primitives to exchange  MIH_Handover_Prepare messages with MIH Functions in one or more candidate networks under consideration before sending this response primitive.

7.4.10.4.MIH_Handover_Initiate.confirm

7.4.10.4.1 Function

This primitive is generated by MIH Function to confirm that MIH_Handover_Initiate Response message is received from a peer MIH Function.

7.4.10.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

PreferredPoAList,

AbortReason,

AvailableResourceList

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request or response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| HandoverAck | Boolean | N/A | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a rea son code is provided |
| PreferredLinkList | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier list of new networks to which handover needs to be initiated. This may be different than the networks that were suggested in the handover request. The front is most suitable, backward is less. |
| PreferredPoAList | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attach ment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| AbortReason | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| Available Resource List | List | N/A | List of available sources on new link |

7.4.10.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Initiate Response message from a peer MIH Function..

7.4.10.4.4 Effect on receipt

On receiving the primitive the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the primitive. If the recipient of the primitive is a client upper layer and in case the handover is to be executed, it may also provide information about selected link identifier on the selected network with a MIH Handover Commit message.

7.4.11.2 MIH_Handover_Prepare.indication

7.4.11.2.1 Function

The MIH Function on the candidate network indicates that MIH_Handover_Prepare Request message is received from a peer MIH Function on the serving network so that the upper layer entity can identify the link resource usage and prepare resources for the impending handover.

7.4.11.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Prepare.indication (

SourceIdentifier,

DestinationIdentifier,

QueryResourceList

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request. |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

7.4.11.2.3 When generated

This primitive is generated by MIH Function when the MIH Function on the candidate network receives MIH_Handover_Prepare Request message from a peer MIH Function on the serving network.

7.4.11.2.4 Effect on receipt

The MIH user on the candidate network identifies the link resource usage and prepares resources for the impending handover. It also replies with MIH_Handover_Prepare.response primitive.

7.4.11.4 MIH_Handover_Prepare.confirm

7.4.11.4.1 Function

This primitive is generated by MIH Function on the serving network to respond the result of resource preparation of the impending handover and to notify the link resource status of the candidate network.

7.4.11.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Prepare.confirm (

SourceIdentifier,

DestinationIdentifier,

ResourceStatus,

AvailableResourceList

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request or response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| ResourceStatus | Enumerate | N/A | Specifies whether requested resources are available or not at the new PoA. |
| AvailableResourceList | List | N/A | List of resources actually available at the new suggested network and the new net workPoA. |

7.4.11.4.3 When generated

This primitive is generated by MIH Function when the MIH Function on the serving network receives MIH_Handover_Prepare Response message from a peer MIH Function on the candidate network.

7.4.11.4.4 Effect on receipt

After receiving this primitive, the upper layer entity on the serving network may send MIH_Handover_Initiate.response primitive to the MIH Function in case it was indicated with MIH_Handover_Initiate.indication primitive before.

7.4.12.2 MIH_Handover_Commit.indication

7.4.12.2.1 Function

This primitive is generated by MIH Function to indicate that MIH_Handover_Commit Request message is received from a peer MIH Function.

7.4.12.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.indication (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

NewLinkIdentifier,

MACNewPoA,

OldLinkAction

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Commit Request message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |
| NewLinkIdentifier | May be one of dif ferent 802 or cel lular networks | N/A | This specifies the new network to which the connection needs to be handed over. |
| MACNewPoA | MAC_ADDRESS | N/A | This is Point of Attach ment List (AP/BS) on new networks. |
| OldLinkActions | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link dur ing handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8-31: Reserved |

7.4.12.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Commit Request message from a peer MIH Function.

7.4.12.2.4 Effect on receipt

The MIH user receiving this primitive replies with MIH_Handover_Commit.response primitive.

7.4.12.4 MIH_Handover_Commit.confirm

7.4.12.4.1 Function

This primitive is generated by MIH Function to confirm that MIH_Handover_Commit Response message is received from a peer MIH Function.

.7.4.12.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

OldLinkAction,

HandoverStatus

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request or response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Commit Request message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |
| NewLinkIdentifier | May be one of dif ferent 802 or cel lular networks | N/A | This specifies the new network to which the connection needs to be handed over. |
| OldLinkActions | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link dur ing handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8-31: Reserved |

7.4.12.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Commit Response message from a peer MIH Function.

7.4.12.4.4 Effect on receipt

Upon receipt, the old serving PoS may start forwarding any pending or remaining packets to the new target PoS. The client recipient may initiate handover process and begin setting up of new layer 2 connection. The network recipient may determine that the handover procedure is in progress to the intended network.

7.4.13.2 MIH_Handover_Complete.indication

7.4.13.2.1 Function

This primitive is generated by MIH Function to indicate the status of the handover operation.

7.4.13.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Complete.indication(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. |
| Destination Identi fier | Identifier | Valid MIHF iden tifier | The destination identifier of request. |
| CurrentLinkIden tifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Complete Request message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |

7.4.13.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Complete Request message from a peer MIH Function.

7.4.13.2.4 Effect on receipt

The MIH user receiving this primitive replies with MIH_Handover_Complete.response primitive.

7.4.13.4 MIH_Handover_Complete.confirm

7.4.13.4.1 Function

This primitive is generated by MIH Function to confirm that MIH_Handover_Complete Response message is received from a peer MIH Function.

7.4.13.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Complete.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (old Network) <> MIHF (new Suggested Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request or response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identi fier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIden tifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Complete Request message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |

7.4.13.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Handover_Complete Response message from a peer MIH Function.

7.4.13.2.4 Effect on receipt

Upon receipt, the client upper layer entity may determine that the handover complete request was processed successfully. Upon receipt, the MIH Function in the new network may forward this message to the client.

7.4.14.2 MIH_Network_Address_Information.indication

7.4.14.2.1 Function

This primitive is generated by MIH Function on new PoA to indicate that MIH_Network_Address_Information Request message is received from a peer MIH Function on old PoA.

7.4.14.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.indication (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HomeAddress,

CoA,

OldAccessRouterAddress,

OldFAAddress

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Home Address | IP Address | N/A | Home IP address of Mobile Node |
| CoA | IP Address | N/A | Care of Address of Mobile Node |
| Old FA Address | IP Address | N/A | IP address of old Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Old Access Router Address | IP Address | N/A | IP address of old Access Router This parameter is only included when mobile node is using IPv6 |

7.4.14.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Network_Address_Information Request message from a peer MIH Function.

7.4.14.2.4 Effect on receipt

Upon reception of this primitive, The MIH user on new PoA may generate another MIH_Network_Address_information.request primitive further to deliver the corresponding message to the MIHF in the network, e.g Access Router with MIHF which may have enough information on this message. When this primitive is delivered to the upper layer of the MIHF in the network, e.g, Access Router with MIHF, this may trigger to send network address related information, e.g, Router Advertisement either directly to the mobile node or to MIHF.

7.4.14.4 MIH_Network_Address_Information.confirm

7.4.14.4.1 Function

This primitive is generated by MIH Function to confirm that MIH_Network_Address_Information Response message is received from a peer MIH Function.

7.4.14.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HomeAddress,

FAAddress,

AccessRouterAddress,

NetworkAddressInformation

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request or response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Home Address | IP Address | N/A | Home IP address of Mobile Node |
| FA Address | IP Address | N/A | IP address of Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Access Router Address | IP Address | N/A | IP address of Access Router This parameter is only included when mobile node is using IPv6 |
| Network Address Infor mation | IP Address | N/A | In case of Mobile IPv4, Agent Advertise ment is encapsulated and in case of Mobile IPv6, Router Advertisement is encapsulated. |

7.4.14.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Network_Address_Information Response message from a peer MIH Function.

7.4.14.4.4 Effect on receipt

The client upper layer entity may use the mobile node’s network address related information when handover starts.

7.4.15.2 MIH_Get_Information.indication

7.4.15.2.1 Function

This primitive is generated by MIH Function to indicate that MIH_Get_Information Request message is received from a peer MIH Function.

7.4.15.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.indication (

InfoQueryType,

InfoQueryParameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryType | An integer value corresponding to one of the following types: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA | N/A | The type of query that is specified |
| InfoQueryParameters | Query type specific parameters | N/A | Query type specific parameters which indicate the type of information the client may be interested in. |

InfoQueryType:

- TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry requests as defined in Section "Information Request and Response".

- RDF_DATA: When this InfoQueryType is specified, InfoQueryParameters is a string which contains a SPARQL (Protocol and RDF Query language) query [2] where the SPARQL query is supposed to contain an appropriate query for obtaining expected RDF/XML data.

- RDF_SCHEMA_URL: When this InfoQueryType is specified, InfoQueryParameters is a null string. This InfoQueryType is used for obtaining the URL of the extended schema.

- RDF_SCHEMA: When this InfoQueryType is specified, InfoQueryParameters carries either the URL of the extended schema the query originator wants to obtain or a null string when the URL of the extended schema is unknown.

7.4.15.2.3 When generated

This primitive is generated by MIH Function on receiving MIH_Get_Information Request message from a peer MIH Function.

7.4.15.2.4 Effect on receipt

The recipient tries to interpret the query request and retrieve the specified information. Once the information is retrieved, the recipient replies with MIH_Get_Information.response primitive.

7.4.15.4 MIH_Get_Information.confirm

7.4.15.4.1 Function

This primitive is generated by MIH to respond to a MIH_GET_Information.request primitive.

7.4.15.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.confirm (

InfoQueryType,

MIH_REPORT,

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryType | An integer value corresponding to one of the following types: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA | N/A | The type of query that is specified |
| MIH_REPORT | String | N/A | Report consisting of information requested by the MIH User |
| Status | Enumerate | 0: success 1: failure 2: rejected | Specifies whether the information was successfully retrieved or not. |

InfoQueryType:

- TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry responses as defined in Section "Information Request and Response".

- RDF_DATA: When this InfoQueryType is specified, InfoQueryResponse is a string which contains a SPARQL (Protocol and RDF Query language) query response [2].

- RDF_SCHEMA_URL: When this InfoQueryType is specified, InfoQueryResponse is an URL of extended schema.

- RDF_SCHEMA: When this InfoQueryType is specified, InfoQueryResponse carries an extended schema.

7.4.15.4.3 When generated

This primitive is generated by MIH Function on receiving MIH_Get_Information Response message from a peer MIH Function.

7.4.15.4.4 Effect on receipt

The MIH User that requested the information tries to interpret the MIH_REPORT and takes suitable action.