| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Link Event and Command IDs : LB Issue #21 (Comment 502) |  |
| Date Submitted | September, 2006 |  |
| Source(s) | Eunah Kim, Junghoon Jee, Hongseok Jeon ETRI | {eakim, jhjee, jeonhs}@etri.re.kr |
| Re: | IEEE 802.21 Session #NN in Melbourne |  |
| Abstract | This contribution provides a solution to the ID assignment for different events and commands to be used in Event List and Command List, respectively. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. LB1 Comment #502

Comment

- To discover what type of events are supported by the Link Layers and to register to receive these events, the primitives use Event IDs. The IEEE 802.21 must define the list of Event IDs in order to support interoperability between different Link Layers.

Suggested Remedy

- Add a new section 7.3.1 called Link Event ID

- Add the following table

Event                                        ID                                                Description

Link Detected	0	A new L2 connectivity has been detected for use

Link Up	1	L2 connectivity is established

Link Down	2	L2 connectivity is lost

Link Going Down	3	L2 connectivity loss is imminent

Link Event Rollback	4	Predicted event has not occurred and hence event indication must be rolled back

Link Parameters Change	5	Link parameters have crossed specified thresholds

Link SDU Transmit Status	6	Indicate transmission status of all PDU segments

Link Handover Imminent	7	L2 handover is imminent

Link Handover Complete	8	L2 handover has been completed

Reserved	9~31	Future use

Resolution of Group

- Accepted-Modified

- Add this in section 8. Add IDs for commands as well

Editor’s Notes

- added as new section - 8.3.2.4 "Link Event and Commend ID".

- did not repeat the primitive description though, only added a ref to table 16

2. Proposed Changes

2.1 Section 8.3.3.4

- List IDs separately for Link Event and Link Command because they are never used together. It will reserve more values for future use for each services.

- Delete Link Get Information because it doesn’t belong to either event or command services.

- Modify section 8.3.3.4 as indicated below and add new section, 8.3.3.5 Link Command ID.

8.3.3.4 Link Event and Commend ID

| Event/Command Name | ID |
| --- | --- |
| Link Event Discover | 0 |
| Link Event Register | 1 |
| Link Event Deregister | 2 |
| Link Configure Thresholds | 3 |
| Link Up | 4 0 |
| Link Down | 5 1 |
| Link Going Down | 6 2 |
| Link Event Rollback | 7 3 |
| Link Detected | 8 4 |
| Link Parameters Report | 9 5 |
| Link SDU Transmit Status | 10 6 |
| Link Handover Imminent | 11 7 |
| Link Handover Complete | 12 8 |
| Link Get Information | 13 |
| (Reserved) | 14 9~31 |

The description of the above events and commands can be found in Table 16  .

8.3.3.5 Link Command ID

| Command Name | ID |
| --- | --- |
| Link Capability Discover | 0 |
| Link Event Register | 1 |
| Link Event Deregister | 2 |
| Link Configure Thresholds | 3 |
| (Reserved) | 4~31 |

The description of the above events and commands can be found in Table 16  .

2.2  Section 8.4.1.1 and 8.4.1.2

- Encoding scheme in primitives for event and command lists is different from the one in messages.

  - Event or command list in primitives (section 7.3.3, 7.3.5, 7.3.21, 7.4.2.1, 7.4.2.2, 7.4.3.1, 7.4.4.1) always occupies 4 octets, while they are variable length in messages (section 8.5.1.1, 8.5.1.2). Each event or command ID in messages is one octet long and doesn’t use a bitmap (section 8.4.1.1, 8.4.1.2).

7.3.3 Link_Event_Register.request (Primitive)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| RequestedEvent List | Set of Event IDs | 4 octets representing the list of events | List of link layer events that the endpoint would like to receive indications for, from the Event Source The encoding uses a bitmap where the bit position represents the event ID. A bit set to 1 indicates that the MIH wishes to receive indication for that event. For example, if bit #0 is set to 1, the MIH requests to receive Link Detected indications. Bits corresponding to reserved event IDs must be set to 0. |

7.3.21 Link_Capability_Discover.confirm (Primitive)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedEvent List | Set of Event IDs | 4 octets representing the list of events | List of link layer events supported by the link layer. The encoding uses a bitmap where the bit position represents the event ID. A bit set to 1 indicates that the Link Layer supports the event. For example, if bit #0 is set to 1, The Link Layer supports Link Detected. Bits corresponding to reserved event IDs must be set to 0. |
| SupportedCommandList | Set of Command IDs | representing the list of commands | List of link layer commands supported by the link layer. The encoding uses a bitmap where the bit position represents the command ID. A bit set to 1 indicates that the Link Layer supports the command. Bits corresponding to reserved command IDs must be set to 0. |

8.5.1.1 MIH_CAPABILITY_DISCOVER REQUEST (Message)

| Name | Type |
| --- | --- |
| SupportedEventList | Event List (255) |
| SupportedCommandList | Command List (254) |
| SupportedTransportList | Transport List (253) |
| SupportedISQueryTypeList | IS Query Type List (252) |

8.4.1.1 Event List

This parameter specifies the list of events that may be carried in a message. Each event is represented by an event code and occupies one octet.

| Type | Length | Value |
| --- | --- | --- |
| 255 | Variable | Array of event codes formatted as messaged-id: Event Code1, Event Code2, Event Code3,.... |

  - Unify the encoding scheme for the values of event and command list for efficiency.

  - Modify sections, 8.4.1.1 and 8.4.1.2 as indicated below

8.4.1.1 Event List

This parameter specifies the list of events that may carried in a message. Each event is represented by a bit position which represents the an event code ID and occupies one octet.

| Type | Length | Value |
| --- | --- | --- |
| 255 | Variable 4 | Array Set of event codes IDs formatted as message-id : Event Code 1, Event Code 2, Event Code 3, … |

8.4.1.2 Command List

This parameter specifies the list of commands that may carried in a message. Each command is represented by a bit position which represents the a command code ID and occupies one octet.

| Type | Length | Value |
| --- | --- | --- |
| 254 | Variable 4 | Array Set of command codes IDs formatted as message-id : Command Code 1, Command Code 2, Command Code 3, … |