| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Suggestion about link parameter threshold configuration: LB Issue- #58: Comment 804 |  |
| DCN | 21-06-0xxx-00-0000 |  |
| Date Submitted | September 7, 2006 |  |
| Source(s) | Guo Junxiang |  |
| Re: | IEEE 802.21 Session #16 in Melbourne, Australia |  |
| Abstract | This contribution is a response to LB Issue #58 in comment assignment and provides amendments about definition of Link Parameter Threshold and the Link Configure Thresholds command, also provides the detail definition of MIH Configure Thresholds command. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

The study of link parameter type shows that it is media specific, i.e. different link type has different definition of link parameter type, and also for a certain link type link parameter threshold varies with specific traffic class. So it is difficult for 802.21 to exactly define the link parameter threshold for each kind of link type, it should be done by SDO of each media. So we suggest that make the following amendments to the description of Link Parameter Threshold and to the definition of Link Configure Thresholds commands, we also regard that it is necessary to add the detail definition of MIH Configure Thresholds into current latest draft.

# 1. Amendment to the description of Link Parameter Threshold

      - Link Parameter Threshold

This specifies the threshold value of a link layer parameter.

| Type | Length | Value |
| --- | --- | --- |
| 16 | Variable | Specifies the threshold value of a link layer parameter. The value depends on the LinkType,TrafficType and ParameterType under consideration.The threshold value is media specific and out of scope of 802.21 and should be defined by SDO.The following example table shows the threshold value range of UMTS. |

The following table lists the value ranges of the UMTS bearer service attributes.

Table 25: Value ranges for UMTS Bearer Service Attributes

| Traffic class | Conversational class | Streaming class | Interactive class | Background class |
| --- | --- | --- | --- | --- |
| Maximum bitrate (kbps) | <= 16 000 | <= 16 000 | <= 16 000 - overhead | <= 16 000 - overhead |
| Delivery order | Yes/No | Yes/No | Yes/No | Yes/No |
| Maximum SDU size (octets) | <=1500 or 1502 | <=1500 or 1502 | <=1500 or 1502 | <=1500 or 1502 |
| SDU format information | TBD by RAN WG3 | TBD by RAN WG3 |  |  |
| Delivery of erroneous SDUs | Yes/No/- | Yes/No/- | Yes/No/- | Yes/No/- |
| Residual BER | 5*10-2, 10-2, 5*10-3, 10-3, 10-4, 10-5, 10-6 | 5*10-2, 10-2, 5*10-3, 10-3, 10-4, 10-5, 10-6 | 4*10-3, 10-5, 6*10-8 | 4*10-3, 10-5, 6*10-8 |
| SDU error ratio | 10-2, 7*10-3, 10-3, 10-4, 10-5 | 10-1, 10-2, 7*10-3, 10-3, 10-4, 10-5 | 10-3, 10-4, 10-6 | 10-3, 10-4, 10-6 |
| Transfer delay (ms) | 100 – maximum value | 300 – maximum value |  |  |
| Guaranteed bit rate (kbps) | <= 16 000 | <= 16 000 |  |  |
| Traffic handling priority |  |  | 1,2,3 |  |
| Allocation/Retention priority | 1,2,3 | 1,2,3 | 1,2,3 | 1,2,3 |
| Source statistic descriptor | Speech/unknown | Speech/unknown |  |  |
| Signalling Indication |  |  | Yes/No |  |

# 2 Amendments to Link_Configure_Thresholds

According to the above description, link parameter threshold value is related not only to link type and parameter type, but also related to traffic class, that is to say, for a certain link type and parameter type, different traffic class has different threshold value. On the other hand, when MIHF issues Link_Configure_Thresholds command to a link layer, the receiver is a certain kind of link, so there is no need to carry the LinkType parameter in this command. So we suggest the Link_Configure_Thresholds command should have following amendments:

    - Link_Configure_Thresholds.request

      - Function

This primitive is used by MIHF to configure thresholds to set threshold values for different link parameters..

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Configure_Thresholds.request			 	(

LinkParameterList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| LinkParameterList | List | N/A | A list of following set of parameters: TrafficType, LinkParameterThresholdList |
| TrafficType | INTEGER | 0x00- All types 0x01- conversational 0x02- streaming 0x03- interactive 0x04- background 0x05-0xff: reserved | Traffic classes for which thresholds may be set. |
| LinkParameterThresholdList | List | N/A | A list of following set of parameters: LinkParameterType, InitiateActionThreshold, RollbackActionThreshold, ExecuteActionThreshold |
| Link ParameterType | Link Quality Parameter Type (8) | N/A | Parameters for which thresholds may be set. LinkParameterType is media specific. |
| InitiateActionThreshold | Link parameter Threshold (16) | N/A | Threshold value which may cause Upper layers to start “setup” type activities in response to actual parameter values crossing this threshold. |
| RollbackActionThreshold | Link parameter Threshold (16) | N/A | Threshold value which may cause Upper layers to cancel or rollback the above setup type operation if the actual parameter values retreat to this threshold. |
| ExecuteActionThreshold | Link parameter Threshold (16) | N/A | Threshold value which may cause Upper layers to execute taking appropriate action if the actual parameter values cross this threshold. |

      - When generated

This primitive is generated by an MIHF that may need to set threshold values for different link parameters.

      - Effect on receipt

The recipient responds immediately with Link_Configure_Threshold.confirm primitive.

    - Link_Configure_Thresholds.confirm

      - Function

This primitive is sent in response to the Link_Configure_Threshold.request primitive. This primitive specifies the status of threshold configuration operation.

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Configure_Thresholds.confirm	(

LinkParameterStatusList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| LinkParameterStatusList | List | N/A | A list of following set of parameters: TrafficType Status |
| TrafficType | INTEGER | 0x00- All types 0x01- conversational 0x02- streaming 0x03- interactive 0x04- background 0x05-0xff: reserved | Traffic class for which thresholds may be set |
| Status | Enumerated | Success Error | Status of operation |

      - When generated

This primitive is generated in response to the Link_Configure_Thresholds.request operation.

      - Effect on receipt

The recipient prepares to receive Link Parameter Change notifications on successful execution of this primitive.

# 3. Add the definition of MIH_Configure_ Thresholds

In draft P802-21-D01-00.pdf, chapter 7.4.8 is about the description of MIH Configure Thresholds, but in draft P802-21-D01-09.pdf  this chapter had been deleted,  as the MIH Configure Thresholds command is different from MIH configure command and they assume different functions, so MIH Configure Thresholds cannot be replaced by MIH configure, this chapter should be remained and be redefined as belows:

    - MIH_Configure_Thresholds

MIH Configure Thresholds may be issued by upper layer entities to set threshold values for different link parameters for different links. MIH Configure Thresholds may contain the configuration commands for multiple lower layer links. When MIH receives MIH Configure Thresholds, it issues the Link Configure Thresholds commands to corresponding links. The configuration parameter except the LinkType are passed on to Link Configure Thresholds commands

7.4.16.1 MIH_Configure_Thresholds.request

7.4.16.1.1  Function

This primitive is issued by upper layer to the MIHF to configure threshold values for different link parameters.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Configure_Thresholds.request			 	(

SourceIdentifier,

DestinationIdentifier,

MultiLinkParameterList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the com mand is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| MultiLinkParameter-List | List | N/A | A list of following set of parameters: LinkType, LinkParameterList |
| >LinkType | Link Identifier (248) | N/A | Identifier of the link to configure. |
| >LinkParameterList | List | N/A | A list of following set of parameters: TrafficType, LinkParameterThresholdList |
| >>TrafficType | INTEGER | 0x00- All types 0x01- conversational 0x02- streaming 0x03- interactive 0x04- background 0x05-0xff: reserved | Traffic class for which thresholds may be set |
| >>LinkParameter-ThresholdList | List | N/A | A list of following set of parameters: LinkParameterType, InitiateActionThreshold, RollbackActionThreshold, ExecuteActionThreshold |
| >>>LinkParameterType | Link Quality Parameter Type (8) | N/A | Parameters for which thresholds may be set. LinkParameterType is media specific. |
| >>>InitiateActionThreshold | Link parameter Threshold (16) | N/A | Threshold value which may cause Upper layers to start “setup” type activities in response to actual parameter values crossing this threshold. |
| >>>RollbackActionThreshold | Link parameter Threshold (16) | N/A | Threshold value which may cause Upper layers to cancel or rollback the above setup type operation if the actual parameter values retreat to this threshold. |
| >>>ExecuteActionThreshold | Link parameter Threshold (16) | N/A | Threshold value which may cause Upper layers to execute taking appropriate action if the actual parameter values cross this threshold. |

        - When generated

This primitive is generated by upper layer that may need to set threshold values for different link parameters.

        - Effect on receipt

A MIH configure Thresholds command causes the MIH to issue a Link Configure Threshold command to set the thresholds for lower layers according to the specified configuration parameters. If multiple links need to be configured then Link Configure Thresholds command should be sent to each of the links.The recipient responds immediately with Link_Configure_Threshold.confirm primitive.

      - MIH_Configure_Thresholds.confirm

        - Function

This primitive is sent in response to the MIH_Configure_Thresholds.request primitive. This primitive specifies the status of threshold configuration operation.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Configure_Thresholds.confirm	(

SourceIdentifier,

DestinationIdentifier,

MultiLinkParameterStatusList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the com mand is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| MultiLinkParameter-StatusList | List | N/A | A list of following set of parameters: LinkType, LinkParameterStatusList |
| >LinkType | Link Identifier (248) | N/A | Identifier of the link to configure. |
| >LinkParameterStatus-List |  | N/A | A list of following set of parameters: TrafficType, Status |
| >>TrafficType | INTEGER | 0x00- All types 0x01- conversational 0x02- streaming 0x03- interactive 0x04- background 0x05-0xff: reserved | Traffic class for which thresholds may be set |
| >>Status | Enumerated | Success Error | Status of operation |

        - When generated

This primitive is generated in response to the MIH_Configure_Thresholds.request operation.

        - Effect on receipt

The recipient prepares to receive Link Parameter Change notifications on successful execution of this primitive.

# 4. Other amendments for consistency

      - MIH_Configure_Thresholds Request

This message is used to configure the thresholds for indications from link layer.

| Name | Type |
| --- | --- |
| LinkType | Link Identifier (248) |
| TrafficType | Traffic Class (256) |
| LinkParameterThresholdList | Link Parameter Threshold Type (26) |

      - MIH_Configure_Thresholds Response

This message returns the response of configuring thresholds for link indications.

| Name | Type |
| --- | --- |
| LinkType | Link Identifier (248) |
| TrafficType | Traffic Class (256) |
| Status | Status (245) |

8.4.2.26 Link Parameter Threshold Type

This specifies the type of parameters and their threshold values.

| Type | Length | Value |
| --- | --- | --- |
| LinkParameterType | Variable | Link Quality Parameter Type (8) |
| InitiateActionThreshold | Variable | Link parameter Threshold (16) |
| RollbackActionThreshold | Variable | Link parameter Threshold (16) |
| ExecuteActionThreshold | Variable | Link parameter Threshold (16) |

8.4.1.13 Traffic Class

This specifies the type of traffic classes.

| Type | Length | Value |
| --- | --- | --- |
| 256 | 1 | 0x00- All types 0x01- conversational 0x02- streaming 0x03- interactive 0x04- background 0x05-0xff: reserved |

      - Link Quality Parameter Type

This specifies the type of link parameter.

| Type | Length | Value |
| --- | --- | --- |
| 8 | Variable | Type of link quality parameters, this is media specific and should be defined by SDO. Examples: Link Speed, Link Bit Error Rate, Link Frame Loss Rate before retransmission, Link Signal Strength, BLER, RxLevNCell SDU Error Rate Transfer Delay |